Image file

The file SPBL7.tif is an extract of a Landsat-7 ETM+ image of St Petersburg, Russia. It was recorded on 12 February 2004 at 15:40Z (Greenwich Mean Time). The image is stored as a 7-band geotiff. The minimum and maximum pixel values for each band are 1 and 255 respectively, and the table shows the corresponding values of the spectral radiance and the wavelengths of the bands.

| band | wavelength | Spectral radiance |
| --- | --- | --- |
|  | µm | W m–2 sr–1 µm–1 |
| 1 | 0.45 → 0.52 | -6.2 → 191.6 |
| 2 | 0.52 → 0.60 | -6.4 → 196.5 |
| 3 | 0.63 → 0.69 | -5.0 → 152.9 |
| 4 | 0.77 → 0.90 | -5.1 → 157.4 |
| 5 | 1.55 → 1.75 | -1.00 → 31.06 |
| 6 | 10.4 → 12.5 | 0.00 → 17.04 |
| 7 | 2.09 → 2.35 | -0.35 → 10.80 |

Georeferencing data are stored with the image.

Exercise 1: preliminaries - Multispec

Find the size of the image in pixels and the number of bytes required to store the image. Comment on the relationship between these values.

Calculate the solar zenith angle and the Earth-Sun distance for the image. Hence calculate the planetary reflectance for bands 1-5 and 7.

Find the image projection, pixel size (in metres) and width and height (in km) represented by the image.

Find the geographical coordinates (latitude and longitude) of some recognisable feature, such as Pulkovo airport (just south of the city), and use Google Earth or Google Maps to confirm the location.

Exercise 2: quantitative radiometric data – Multispec, ImageJ

Calculate the solar zenith angle and the Earth-Sun distance for the image. Hence calculate the planetary reflectances for bands 1-5 and 7 using Multispec. Extract the band 6 image as a separate, single-band tiff image using Multispec. Open this using ImageJ and transform it to a brightness temperature image. Hence estimate the temperature of the aircraft condensation trail relative to that of the water in the Baltic Sea, and the brightness temperature of the ice floes in Lake Ladoga.

Exercise 3: contrast manipulation and spatial filtering – Multispec, ImageJ

Extract a single band of the image using Multispec – band 4 is suggested. Open the image in ImageJ and experiment with adjusting the contrast. If you download the ImageJ plugin 'Equalize_plugin.java' from at http://www.imagingbook.com/index.php?id=98 you can also perform a histogram equalisation.

Convert the image to 32-bit format so that it is capable of representing negative and fractional pixel values, and experiment with different spatial filters using the 'convolve' operation.

Exercise 4: multispectral information – Multispec

Open the image in Multispec and experiment with different band combinations, noting which give insight into particular aspects of the land and water surface. Amongst other combinations, you should try

321 – true-colour composite

432 – false colour infrared

543 – useful for distinguishing areas of snow cover

666 – shows the thermal infrared channel in greyscale

Estimate the dark-pixel values in bands 2, 3, 4 and 5 and hence calculate the NDVI and NDSI from the image.

Perform a Principal Components Analysis and find the percentage of the image variance represente by the first three PCs. Perform a Principal Components Transformation of the image.

Exercise 5: unsupervised classification – Multispec

Perform an unsupervised classification of the image by using the ISODAT clustering procedure. Experiment with different numbers of clusters and try grouping the resultant clusters into different land- and water-cover classes on the basis of the original image and using Google Earth as a source of 'ground truth' data.

Exercise 6: supervised classification – Multispec

Define training areas for different land- and water-cover classes in the image, then perform a supervised classification. Try different classification algorithms. Compare your results with those from the unsupervised classification.

W Burger and MJ Burge, Digital Image Processing - An Algorithmic Introduction using Java (Berlin: Springer-Verlag, 2005).