A COMPUTER PROGRAM FOR THE SOLUTION OF  NAVIER-STOKES SYSTEM OF EQUATIONS USING THE FLOWFIELD-DEPENDENT VARIATION (FDV) METHOD WITH FINITE ELEMENTS

This is a computer program for the solution of Navier-Stokes system of equations in which all features of flows are included to accommodate a wide variety of Mach numbers and Reynolds numbers (compressible, incompressible, inviscid, and viscous flows).  The governing equations are of the form (conservation form of the Navier-Stokes system of equations):

∂U∂t+∂F∂x+∂G∂x=B

The solution is carried out using the Flowfield-Dependent Variation (FDV) method with element-by-element (EBE) assembly via Generalized Minimal Residual (GMRES) solution scheme using finite element discretizations with isoparametric elements and Gaussian quadrature integrations.

The advantages of FDV method are:

- The first order FDV parameters s, s as calculated from the current flowfield variables (Mach numbers and Reynolds numbers) assure the accuracy of solution. They alter the roles of each term in the governing equations in different positions of the domain, reflecting the incompressible behavior very close to the wall and compressible behavior or shock wave discontinuities away from the wall automatically. This can be demonstrated by contour plots of the FDV parameters themselves resembling the actual flowfields. The FDV scheme provides accurate solutions in turbulence with DNS mesh configurations and in supersonic combustion through FDV Jacobians.

- The second order FDV parameters s, s assure the stability of solution process.

- The source term FDV parameters s, s are associated with source terms with s set exponentially proportional to s.

- A single program based on the FDV theory is capable of accommodating all different flow physics, high speed or low speed, compressible or incompressible, viscous or inviscid, in 1-D, 2-D, and 3-D geometries.

- The FDV method can be applied to both FDM and FEM geometries.

Note:

The FDV method is used to solve all problems, demonstrating that incompressible, compressible, viscous, and inviscid flows in 1D, 2D, and 3D can be solved with one code.  FDV parameters as calculated will determine different types of flows throughout the domain and time.

- Incompressible viscous flow

A-1 2-D lid driven cavity

A-2 2-D Backstep

- Compressible inviscid flow (1-D shock tube)

- Compressible viscous flow

C-1  2-D flat plate

C-2  3-D flat plate

C-3  2-D compression corner

C-3-1 Geometry 1

C-3-2 Geometry 2

C-4  3-D compression corner

C-5  Mach 20 Shock wave turbulent boundary layer interaction problem with an impinging shock wave

A.  INCOMPRESSIBLE VISCOUS FLOW

A-1.  TWO-DIMENSIONAL LID-DRIVEN CAVITY

L=1

H=1

U0 = 1, v0 = 0

u = 0, v = 0

∂T/∂n = 0

Solve for the following cases

(1) Re = 1000, (2) Re = 3200, (3) Re = 5000

Solutions:

Using FDV FEM, assume a freestream Mach number of 0.05 and a freestream temperature of 460 R, and use the100 x 100 finite element grid shown below.  Results can be compared with Ghia, U., Ghia, K.N, and Shin, C.T., “High-Re Solutions for Incompressible Flow Using the Navier-Stokes Equations and a Multigrid Method,” Journal of Computational Physics, 48, 387-411 (1982).

(1) Re = 1000

Plots of convection and diffusion FDV parameters, streamlines and velocity vectors are shown below.  The size of the corner vortices agree well with the streamline plots of Ghia, et al.

Convection FDV Parameters (s1)		 Diffusion FDV Parameters (s3)

Streamlines					Velocity Vectors

Below are plots of the horizontal velocity along a vertical line through the geometric center of the cavity and the vertical velocity along a horizontal line through the geometric center.  The results agree well with the results of Ghia, et al.

(2) Re = 3200

Plots of the convection and diffusion FDV parameters, streamlines and velocity vectors are shown below.  The FDV algorithm successfully resolves the vortices that form in the two lower corners.  The size of these vortices agrees well with the results of Ghia, et al.

Convection FDV Parameters (s1)		 Diffusion FDV Parameters (s3)

Streamlines					Velocity Vectors

Plots of horizontal velocity through a vertical line through the geometric center and of the vertical velocity through a horizontal line through the geometric center are shown below.  These plots agree well with the calculations of Ghia, et al.

(3) Re = 5000

Plots of convection and diffusion FDV parameters, streamlines and velocity vectors are shown below.  The FDV algorithm successfully captures the vortices in the lower two corners of the cavity, as well as the vortex in the upper left hand corner.

Convection FDV Parameters (s1)		 Diffusion FDV Parameters (s3)

Streamlines					Velocity Vectors

Reasonable agreement with the calculations of Ghia, et al, is indicated below for plots of the horizontal velocity through a vertical line through the cavity’s geometric center and of the vertical velocity through a horizontal line through the geometric center.

Plot of u-velocity along vertical line through the geometric center of cavity

A-2.  TWO-DIMENSIONAL BACKSTEP

38

6

1

1

x

y

u = v = 0,

Inlet:

v = 0, u parabolic

Initially, interior nodes set to:

Solve and plot the streamline distributions for (1) Re = 150 and (2) Re = 500

Solution: Using FDV FEM and assuming a freestream Mach number of 0.05 and a freestream temperature of 460 R.

(1) For Re = 150, a plot of the streamlines and velocity vectors are shown below.

Streamlines for Re =150				Velocity Vectors near step  for Re = 150

B.    COMPRESSIBLE INVISCID FLOW

(ONE-DIMENSIONAL SHOCK TUBE)

Initial conditions:

Geometry and boundary conditions

State R

Diaphragm

State L

Schematic Diagram of Shock Tube Flow

L

3

2

R

C

V

Expansion Wave

Contact Discontinuity

Shock Wave

Initial position of diaphragm

Governing equations

∂U∂t+∂F∂x=0        U=ρρuρE          F=ρuρuρE+pu

Analytical Solution: The analytical solution can be calculated from the initial conditions as follows.  Referring to the schematic drawing of the shock tube problem shown above, and given the initial pressure ratio, .  The properties at position 2 to the left of the shock wave can be calculated from the pressure ratio as follows:

, 	with .

,

where C is the propagation speed of the shock wave and  is the speed of sound.

The contact surface sustains a discontinuity in density, but the pressure and velocities normal to the surface are continuous.  Therefore, the contact discontinuity propagates at a velocity, , equal to .  Along the contact surface,  and .  The density in region 3 is calculated from , and the acoustic velocity is calculated using .

In region 5 that separates region 3 from region L, there is a continuous variation of the flow variables through the expansion fan.  The flow variables in this region are calculated from

, ,

and .  These relationships are valid for .  This is a time dependent problem.  The location of the shock wave, contact discontinuity, and expansion fan changes with time.  After solving the Euler equations using a given numerical method, calculate the analytical solution based on the elapsed time in your numerical solution.  The shock wave will be locate at , the contact discontinuity will be located at , and the expansion fan will be located at .

Solution (using FDV FEM)

Plots of the density, pressure, and Mach number distribution compared with the exact solution are shown below.  These plots indicate the continuous variation of the density and pressure through the expansion wave that propagates to the left from the diaphragm into the high pressure region.  To the right of the diaphragm, a contact discontinuity and a shock wave propagates into the low pressure region.  There is an instantaneous drop of the density through both the contact discontinuity and the shock wave.  Note that the pressure remains constant through the contact discontinuity, but drops instantaneous through the shock wave.

Shock Tube Density Distribution

Shock Tube Pressure Distribution

Shock Tube Mach number Distribution

C.     COMPRESSIBLE VISCOUS FLOW

Governing Equations (Navier Stokes System of Equations)

U=ρρVρE ,      F=ρVρVV+PδρE+PV,         G=0−τ−τV−q

C-1.   2-D FLAT PLATE

Given:  Flow over a flat plate shown below with the following freestream conditions:

.

An isothermal wall is assumed with the wall temperature set equal to the freestream stagnation temperature, .

Initial 45 x 31 grid (1395 elements)

FDV Parameters

Close-up of velocity vectors near wall, Flat Plate, Mach 3, Re 1000

Wall Pressure Distribution		Wall Skin Friction Distribution

C-2.  Three-Dimensional Flat Plate

Boundary conditions for a three-dimensional version of Carter’s flat plate are shown below.  The plate is assumed to be infinite in width, with symmetry conditions applied on the left and right sides of the plate.  Freestream conditions are the same as for the two-dimensional version of Carter’s flat plate.  One other boundary condition deserves mention.  On the left and right edges of the domain in front of the plate leading edge, there is an intersection between a symmetry condition in the y-z plane (u = 0) and a symmetry conditions in the x-y plane (w = 0).  For these points on the edge, both u and w velocity components are set to zero.

Geometry and Boundary Conditions		Finite Element Grid (52,800 Elements)

Density Contours		Pressure Contours	Mach Number Contours

FDV Parameters

C-3.      2-D Compression Corner

Solve the Navier Stokes System Equations for the following inflow conditions

Case (1) M = 3.0, Re = 1.68 x 104, Isothermal wall , , , h = 1.2

Case (2) M = 4.0, Re = 6.8 x 104, Adiabatic Wall, , , , h = 0.8

Case (3) M = 6.06, Re = 1.5 x 105, Adiabatic Wall, . , , h = 0.6

GEOMETRY 1

1.0

1.0

0.1

h

No-slip wall,

u = v = 0

α

Symmetry, v = 0,

∂T/∂n = τ12 = 0

Inflow

Exit

Case (1) M = 3.0

Contour plots of density and pressure indicate the formation of shock waves at the leading edge of the flat plate section of the domain and at the compression surface.  A plot of pressure at the wall indicates good agreement with the numerical results of Carter.

Finite Element Grid (92 x 50)

FDV Parameters

Density Contours				Pressure Contours

Wall Pressure Distribution

(Carter’s Results from NASA Technical Report NASA TR R-385, July, 1972)

Case (2) Mach = 4

Contour plots of density and pressure show the formation of shock waves at the leading edge of the flat plate section and at the compression surface.  A plot of the pressure at the wall shows good agreement with the experimental results of Lewis, Kubota, and Lees through the pressure rise in the flow separation region.  Some discrepancy is shown near the exit of the domain, possibly due to an inexact boundary condition at the exit.

Finite Element Grid (131 x 48)

FDV Parameters

Density Contours				Pressure Contours

Wall Pressure Distribution

(Experimental Results from Lewis, Kubota, and Lees, AIAA Journal, January, 1968, pp.7-14)

Case (3) M = 6

As above, contour plots of density and pressure indicate the formation of shock waves at the leading edge of the flat plate and at the compression surface.  A plot of pressure at the wall shows good agreement with experimental and prior numerical results, with a slight underprediction of the pressure rise through the flow separation region.

Density Contours				Pressure Contours

Wall Pressure Distribution

(Experimental Results from Lewis, Kubota, and Lees, AIAA Journal, January, 1968, pp.7-14.  Carter’s Results from NASA Technical Report NASA TR R-385, July, 1972)

Geometry 2

1.0

1.0

1.0

1.0

0.8

0.2

Inflow

No-slip, adiabatic wall

u = v = 0, ∂T/∂n = 0

Symmetry: v = 0

∂T/∂n = τ12 = 0

Exit

Inlet Conditions:

Assuming the reference length is 1 inch,

Nondimensional variables:

Solution (using FDV FEM):

Contour plots of density and pressure are shown below.  These plots indicate the presence of a shock wave at the leading edge of the flat plate section due to the compression of the incoming supersonic flow by the formation of the viscous boundary layer and a shock wave at the compression section of the domain.  Both of these shock waves are reflected at the top surface due to the specification of a symmetry (no flow) boundary condition for the top surface.  An expansion fan forms at the beginning of the constant area section following the compression section of the domain.

Finite Element Grid

Density Contours				Pressure Contours

C-4    3-D COMPRESSION CORNER

(a depth of 1.0 in the third dimension and the addition of a no-slip wall on the back side, same inlet conditions as prob 4a (compressible viscous)

z

x

y

No-slip Walls,

u = v = w = 0

∂T/∂n = 0

1.0

1.0

0.6

0.6

Solution (using FDV FEM):

The results are compared with the numerical results in Hung, C.M and MacCormack, R.W., “Numerical Solution of Supersonic Laminar Flow Over a Three-Dimensional Compression Corner,” AIAA Paper 77-694, June 1977.  Density contours at Y/L of 0.3292 indicate the intersection of the two leading edge shocks formed at the leading edge of the flat plate and at the leading edge of side wall.  The intersections of these shock waves form a corner shock in the lower left corner.  Also shown in the plot are the boundary layers on the two surfaces.

Density Contours at Y=0.3292 showing intersection of two leading edge shocks

The density and pressure contour plots below are taking downstream of the compression surface.  The density plot on the left shows the interaction of the wedge shock with the boundary layer on the side wall.  Also, in the upper right hand corner, one observes the formation of a corner shock from the intersection of the reflection of the sidewall leading edge shock from the opposite symmetry side wall with the reflection of the flat plate leading edge shock from the top surface.

Density and Pressure contours at Y = 1.59

C-5   SHOCK WAVE TURBULENT BOUNDARY LAYER INTERACTION

PROBLEM WITH AN IMPINGING SHOCK WAVE

The geometry and boundary conditions for the interaction of an oblique shock wave with a viscous boundary layer is shown below.  The shock wave is generated by the supersonic flow being compressed through an angle, θ, with a resulting shock angle, β.

Shock

Boundary Layer

β

0.25

1.5

0.1

No slip, adiabatic wall

The flow variables are set to the undisturbed freestream conditions on the left side and to post-shock flow conditions on the top surface. The flat plate is a no-slip, adiabatic surface.  The subscript 1 denotes the undisturbed freestream conditions, the subscript 2 denotes the post-shock flow conditions, and the asterisk superscript indicates non-dimensional variables.  The asterisks are omitted in the equations below for simplicity.  The post-shock flow conditions are calculated from the inviscid flow relations as shown below.

Assume freestream conditions of , and a shock deflection angle of .  The shock angle, β, can be obtained from the  chart or the  relation shown below.

For  and , then .  The normal component of Mach number upstream of the shock wave, .  The density and pressure ratios across the shock wave are calculated from

The normal component of the Mach number downstream of the shock wave is calculated from

.

The temperature ratio 	downstream of the shock wave is calculated from

.

Finally, the Mach number downstream of the shock wave is .  From these calculations, one can calculate the input flow variables for the top surface.

The post shock conditions for Mach numbers of 2, 4, 14.1, and 20 are shown in the table below.

| Mach |  | Re | θ | β |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 295 K | 300,000 | 10° | 39.3137° | 1.458 | 1.170 | 0.874 | -0.154 |
| 4 | 295 K | 300,000 | 10° | 22.234° | 1.886 | 1.330 | 0.932 | -0.164 |
| 14.1 | 2385 K | 300,000 | 10° | 13.329° | 4.0729 | 2.986 | 0.960 | -0.169 |
| 20 | 2400 K | 300,000 | 10° | 12.7016° | 4.7685 | 4.7017 | 0.9618 | -0.1696 |

First Order Convection FDV Parameters (s1)

First Order Dissipation FDV Parameters (s3)

Temperature Contours

Pressure Contours

Velocity Vectors

Velocity Vectors near wall in Boundary Layer

Enhanced View of Mach 20 Velocity Vectors near the wall

FLOWCHART FOR FDV PROGRAM

START

WHAT IS   NDIMNN

GENBFAC3

GENBFAC

GAUSINT

A

WHAT IS   NDIMNN

EVALELE

ELELENG

1

OPENING

INPUT

CONSVAR

BLOCK

NDIMN = 3

NDIMN = 2

EVALELE3D

NDIMN = 2

NDIMN = 1

NDIMN = 3

1

CFLTODT

IMPARM

WHAT IS   NDIMNN

CONVFLX

CONVFLX1

CONVFLX3

MEIEQNS

B

SOLVE

C

RMSERR

PRIMVAR

2

GMRES_EBE

NDIMN = 2

NDIMN = 1

NDIMN = 3

TIME

MARCHING

LOOP

2

OUTPUT

OUTIMP

END

A

GAULEG

WHAT

IS

NDIMN?

SHAP1D

SHAPDOM

SHAPSURF

SHAPDOM3

SHAPSURF_3D

RETURN

2

3

1

B

DOMINGRL

BOUINGRL

E2

F

G

WHAT

IS

NDIMN?

ONED_INGRL

1

3

DOMINGRL3D

2

E3

BOUINGRL3D

E1

RETURN

BC

C

RETURN

PRECOND

LOCAL

D2

IS ADAP TRUE?

ELECNCT

ADPCLER

RECHANG

ERR_IMPL

BUFFER

SWITCH

UNREFINE

REFINE

DIVIDE

NODCNCT

RENUMB

NO

YES

DIAG_PRECOND

H

LU_PRECOND

J

EL2NORM

AU_PRODUCT

K

TWODSORT

BLOCK

EVALELE

EXCHANG

RETURN

E1

DERSH1D

DERSHA

DERSHA_3D

EVALDOM

AJACOB1

AJACOB

VISCFLX

BJACOB

CJACOB

AJACOB_3D

VISCFLX_3D

BJACOB_3D

CJACOB_3D

MATXMULT

EVALDOM

RETURN

E2

E3

EVALDOM

MATXMULT

RETURN

MATXMULT

RETURN

Parallel

Element

Loop

F

CALNORM

EVALBOU

AJACOB

VISCFLX

BJACOB

CJACOB

MATXMULT

RETURN

G

CALNORM3

EVALBOU3D

AJACOB_3D

VISCFLX_3D

BJACOB_3D

CJACOB_3D

MATXMULT

RETURN

H

LOCAL

Parallel

Loop

AU_PRODUCT

RETURN

K

J

Parallel

Processing

PRECOND

RETURN

K

LOCAL

Parallel

Processing

LOCAL

RETURN

SUBROUTINE FUNCTIONS

Subroutine OPENING – Opens input/output files and reads in initial parameters and fluid properties

Subroutine INPUT – Reads in element connectivity, nodal point coordinates, initial flow variables, and boundary point information

Subroutine CONSVAR – Calculates conservation variables from primitive variables

Subroutine BLOCK – Groups elements in blocks of non-adjacent elements to allow element operations to be performed in parallel (Ref: Shakib, Hughes, and Johan, 1989, and Hughes, Ferencz, and Hallquist, 1987)

Subroutine GENBFAC – Generates information on boundary face connectivity for 2-D flow

Subroutine GAUINT – Calculates information for Gauss-Legendre integration for the domain and surface integrals.  This subroutine calls GAULEG, SHAPDOM, SHAPSURF, SHAP1D, SHAPDOM3, and SHAPSURF_3D.

Subroutine SHAPDOM – Calculates two-dimensional shape functions and their derivatives for two-dimensional flow calculations.

Subroutine SHAPSURF – Calculates one-dimensional shape functions and their derivatives for evaluating surface integrals in two-dimensional calculations

Subroutine SHAP1D – Calculates one-dimensional shape functions and their derivatives for one-dimensional flow calculations.

Subroutine GAULEG – Calculates weighting functions and abscissas for Gauss-Legendre integration.  Enables user to change number of Gaussian points used on the fly without major reprogramming.

Subroutine ELELENG – Calculates characteristic lengths for one-dimensional elements

Subroutine EVALELE – Calculates characteristic lengths for two-dimensional elements.

Subroutine DERSHA – Calculates spatial derivatives of shape functions in two-dimensions.

Subroutine DERSHA1D – Calculates spatial derivatives of shape functions in one-dimension.

Subroutine PRIMVAR – Calculates primitive variables from conservation variables.

Subroutine CFLTODT –Calculates time step from CFL number and flow conditions.

Subroutine IMPARM – Calculates variation parameters for the elements based on local Mach numbers, Reynolds numbers, and Peclet numbers in element.

Subroutine CONVFLX1 – Calculates convective flux from flow conditions in one-dimensional flow.

Subroutine CONVFLX – Calculates convective flux from flow conditions in two-dimensional flow.

Subroutine MEIEQNS – Main subroutine to begin building process for the FDV equations.

Subroutine ONED_INGRL – Calculates element matrices and right-hand-side (RHS) vector for FDV equations in one-dimensional flow.

Subroutine DOMINGRL – Calculates domain integral terms in element matrices and RHS vector for FDV equations in two-dimensional flow.

Subroutine DOMINGRL3D – Calculates domain integral terms in element matrices and RHS vector for FDV equations in three-dimensional flow.

Subroutine BOUINGRL3D – Calculates surface integral terms in element matrices and RHS vector for FDV equations in three-dimensional flow.

Subroutine BOUINGRL - Calculates surface integral terms in element matrices and RHS vector for FDV equations in two-dimensional flow.

Subroutine EVALBOU – Calculates flow variables at each Gaussian point during evaluation of the surface integrals for two-dimensional flow.

Subroutine APLBNCN2D – Applies boundary conditions to surface integrals for two-dimensional flow.

Subroutine APLBNDCND – Applies boundary conditions to surface integrals for three-dimensional flow.

Subroutine JACBNC2D – Applies boundary conditions to Jacobians in surface integrals for two-dimensional flow (Ref: Chung, 2002).

Subroutine JACBNDCND - Applies boundary conditions to Jacobians in surface integrals for three-dimensional flow (Ref: Chung, 2002).

Subroutine BC – Applies boundary conditions to element matrices before transfer to equation solver.

Subroutine AJACOB1 – Calculates one-dimensional convection Jacobians

Subroutine AJACOB – Calculates two-dimensional convection Jacobians.

Subroutine EVALDOM – Calculates flow variables at each Gaussian point during evaluation of domain integrals.

Subroutine BJACOB – Calculates two-dimensional dissipation Jacobians.

Subroutine CJACOB – Calculates two-dimensional dissipation-gradient Jacobians.

Subroutine VISCFLX – Calculates two-dimensional viscous flux terms.

Subroutine MATXMULT – Performs multiplication of two Jacobians matrices.

Subroutine CALNORM – Calculates direction cosines of unit normal vector to boundary surface for two-dimensional flow calculations.

Subroutine SOLVE – Initiates equation solver routines.

Subroutine GMRES_EBE – Solves system of equations using an element-by-element GMRES algorithm with left preconditioning.

Subroutine PRECOND – Applies LU-preconditioning to system of equations in GMRES algorithm.

Subroutine DIAG_PRECOND – Applies Diagonal preconditioning to system of equations in GMRES algorithm.  Ref: Shakib, 1989.

Subroutine LOCAL – Performs gathering and scattering operations on element matrices for GMRES algorithm.

Subroutines FWDSUBSTN and BACKSUBSTN – perform forward reduction and back substitution operations on element matrices for preconditioning operations.  These subroutines are not used in current version of program.  These operations are incorporated in subroutines PRECOND, LU_PRECOND, and SOR_PRECOND.

Subroutine LU_PRECOND – Performs LU factorization of element matrices and initial preconditioning operations.

Subroutine SOR_PRECOND – Performs preconditioning operations using a successive over-relaxation preconditioner of the form , where  and  are the strict lower and strict upper components of the stiffness matrix, and  is a relaxation factor.  This preconditioner was used for three-dimensional problems to avoid the extra storage required for storing the LU factorization of the element matrices.

Subroutine LU_FACT – Performs LU factorization of a matrix.  This subroutine is not currently used.  This operation is incorporated in subroutine LU_PRECOND.

Subroutine AU_PRODUCT – Calculates product of element matrices with a vector and assembles it into a single column vector.

Subroutine MATX_VEC – Earlier subroutine to calculate a matrix-vector product.  This subroutine is not used in current version.

Subroutine EL2NORM – Calculates L2 norm of a vector in GMRES algorithm.

Subroutine DOTPROD – Calculates dot product of two vectors.

Subroutines CNSTWLTMP and APLBOUN – These subroutines are not used in current version.  These subroutines were used previously to apply the constant wall temperature boundary condition and to apply boundary conditions to the equations.

Subroutine RMSERR – Calculates the RMS error of the system of equations to monitor convergence.

Subroutine OUTPUT – Writes value of element connectivity, nodal coordinates, flow variables, etc., to a file.  The resulting file can be read back into the program as a new input file to restart the calculations at the end of the specified number of time steps.

Subroutine OUTIMP – Writes the values of the variation parameters at the nodal points for plotting purposes.

Subroutine WALLPRES3D – Calculates wall pressure and writes it to a file for plotting purposes for three-dimensional problems.

Subroutine QUICK_SORT_2D – Uses a quick sort routine to sort a list of items ascending in the x any y directions.

Subroutine WALLPROPT – Calculates wall pressure, skin friction, and heat transfer distributions at the wall for two-dimensional problems.

Subroutine QUICK_SORT – Uses a quick sort routine to sort the wall pressure, skin friction, heat transfer ascending in the x direction.

Subroutine GENBFAC3 – Generates information on the boundary face connectivity of a two-dimensional boundary surface in three-dimensional flow.

Subroutine SHAPDOM3 - Calculates three-dimensional shape functions and their derivatives for three-dimensional flow calculations.

Subroutine DERSHA_3D - Calculates spatial derivatives of shape functions in three-dimensions.

Subroutine SHAPSURF_3D - Calculates two-dimensional shape functions and their derivatives for evaluating surface integrals in three-dimensional calculations

Subroutine CONVFLX3 - Calculates convective flux from flow conditions in three-dimensional flow.

Subroutine EVALELE3D – Calculates characteristic lengths for three-dimensional elements.

Subroutine GPG – Calculates Petrov-Galerkin terms.  This subroutine is not used in the current version.

Subroutine AJACOB_3D – Calculates convection Jacobians in three-dimensions.

Subroutine BJACOB_3D – Calculates dissipation Jacobians in three-dimensions.

Subroutine CJACOB_3D – Calculates dissipation gradient Jacobians in three-dimensions.

Subroutine EVALBOU3D - Calculates flow variables at each Gaussian point during evaluation of the surface integrals for three-dimensional flow.

Subroutine CALNORM3 – Calculates the spatial derivatives of the shape functions and the components of the unit normal vector for the evaluation of the surface integrals in three-dimensional flow.

Subroutine D2SHAPSURF – Calculates the second spatial derivatives of the shape functions for the surface integrals in three-dimensional flows.

TABLE OF CONTENTS

Page

Modules										           1

Program ebe_fdv_ver1								           4

Subroutine OPENING								           5

Subroutine INPUT									           6

Subroutine CONSVAR								           8

Subroutine BLOCK									           8

Subroutine GENBFAC								           9

Subroutine GAUSINT								         10

Subroutine SHAPDOM								         10

Subroutine SHAPSURF								         11

Subroutine SHAP1D									         11

Subroutine GAULEG									         11

Subroutine ELELENG								         12

Subroutine EVALELE								         12

Subroutine DERSHA									         12

Subroutine DERSHA1D								         13

Subroutine DERSHA2								         13

Subroutine PRIMVAR								         14

Subroutine CFLTODT								         15

Subroutine IMPARM									         15

Subroutine CONVFLX1								         17

Subroutine CONVFLX								         18

Subroutine MEIEQNS								         18

Subroutine ONED_INGRL								         18

Subroutine DOMINGRL								         21

Subroutine DOMINGRL3D								         23

Subroutine BOUINGRL3D								         25

Subroutine BOUINGRL								         27

Subroutine EVALBOU								         30

Subroutine APLBNCN2D								         31

Subroutine APLBNDCND								         33

Subroutine JACBNC2D								         39

Subroutine JACBNDCND								         41

Subroutine BC									         45

Subroutine AJACOB1								         45

Subroutine AJACOB									         46

Subroutine EVALDOM								         46

Subroutine BJACOB									         47

Subroutine CJACOB									         48

Subroutine VISCFLX									         48

Subroutine MATXMULT								         49

Subroutine CALNORM								         49

Subroutine SOLVE									         50

Subroutine GMRES_EBE								         50

Subroutine PRECOND								         53

Subroutine DIAG_PRECOND							      	         53

Subroutine LOCAL									         54

Subroutine FWDSUBSTN								         55

Subroutine BACKSUBSTN								         55

Subroutine LU_PRECOND								         55

Subroutine SOR_PRECOND								         56

Subroutine LU_FACT								         57

Subroutine AU_PRODUCT								         57

Subroutine MATX_VEC								         57

Subroutine EL2NORM								         57

Subroutine DOTPROD								         58

Subroutine CNSTWLTMP								         58

Subroutine APLBOUN								         58

Subroutine RMSERR									         59

Subroutine OUTPUT									         59

Subroutine OUTIMP									         59

Subroutine WALLPRES3D								         61

Subroutine QUICK_SORT_2D							    	         62

Subroutine WALLPROPT								         63

Subroutine QUICK_SORT								         64

Subroutine GENBFAC3								         65

Subroutine SHAPDOM3								        71

Subroutine DERSHA_3D								        72

Subroutine SHAPSURF_3D								        76

Subroutine CONVFLX3								        76

Subroutine EVALELE3D								        76

Subroutine GPG									        77

Subroutine AJACOB_3D								        78

Subroutine BJACOB_3D								        79

Subroutine CJACOB_3D								        80

Subroutine VISCFLX_3D								        81

Subroutine EVALBOU3D								        81

Subroutine CALNORM3								        82

Subroutine D2SHAPSURF								        84

!    This file contains modules for common global variables used in

!    Program ebe_fdv_ver1.f90

!

!    *************************** Variables **************************

!    mpoin = dimension for number of grid points

!    melem - dimension for number of elements

!    mboun = dimension for number of boundary points

!    mface = dimension for number of boundary faces

!    mdimn = dimension for number of dimensions

!    mnode = dimension for number of nodes per element

!    mequa = dimension for number of unknowns per grid point

!    ngaus = number of Gaussian points used in Gaussian Quadrature

!    mbsize = number of elements per block

!    mconn = dimension for element connectivity array

!    mhng = dimension for hanging node array

!

!    gamma = ratio of specific heats, gamm1 = gamma - 1

!    gm1m2pr = (gamma-1)*Mach^2*prandtl number

!    gm3d2 = (gamma-3)/2, gm1d2 = (gamma-1)/2, gamm3 = gamma - 3

!    cgas = nondimensional gas constant

!    csuth = nondimensional Sutherland constant

!    creyn = Freestream Reynolds number

!    cmach = Freestream Mach number, cmasq = cmach^2

!    cv = specific heat at constant volume

!    cartf = coefficient of artificial viscosity (not currently used)

!    tinf = Freestream static temperature

!    twall = nondimensional wall temperature

!    dense = nondimensional fluid density at grid points

!    energ = nondimensional total energy at grid points

!    press = nondimensional pressure at grid points

!    tempr = nondimensional temperature at grid points

!    u = conservation variables at grid points

!    uold = value of conservation variable from previous time step

!    vel = nondimensional fluid velocity at grid points

!    lnod = element connectivity

!    lpold = element connectivity containing hanging nodes

!    x = Cartesian coordinates of grid points

!    lblock = element block information

!    cleng = element characteristic length

!    alpa = compression corner deflection angle (in radians)

!

!    ibcon = boundary point information:

!            ibcon(1,:) global point number of boundary point

!            ibcon(2,:) boundary condition of boundary point (see below)

!            ibcon(3,:) = 2 for adiabatic wall, otherwise, = 0

!    Boundary conditions (value of ibcon(2,:)):

!       = -12 Top surface on lid-driven cavity, Dirichlet, d/dn.ne.0

!       = -1 Dirichlet inlet

!       = 2,12 Symmetry in x-y plane, w=tau23=tau13=q3=0

!       = 1 No-slip wall in x-y plane, u=v=w=0, T=Twall or q3=0

!       = 4 No-slip Inclined surface in x-y plane, u=v=w=0, T=Twall or q3=0

!       = 6 Outlet, d/dn=0

!       = 7 Top surface, d/dn=0

!       = 8 Compression corner outlet, deriv. wrt flow dir. = 0

!       = 9 Triple shock outlet, deriv. wrt flw dir. = 0

!       = 11,21 No-slip wall in y-z plane, 11 - left side, 21 - right side

!       = 14 No-slip Inclined surface in y-z plane, u=v=w=0, T=Twall or q1=0

!       = 22 Symmetry in y-z plane, u=tau12=tau13=q1=0

!       = 31,41 No-slip wall in x-z plane, 31 - left side, 41 - right side

!

!    indbc,indbd = boundary condition information for each grid point

!    ibouncnd = boundary condition information for each equation

!    rbcon = direction cosines along boundary surface (not used for 3-D)

!    lface,lfacnode = nodal point information for boundary faces (see

!          subroutines genbfac & genbfac3d for complete description)

!

!    Variation Parameter information

!     s1,s2,s3,s4 = Constant-value Variation parameters

!     s1ele,s2ele,s3ele,s4ele = Element variation parameters

!     s3pec,s4pec = dissipation variation parameters calc. from Peclet No.

!     emachold,reyold,pecold = element mach numbers, Reynolds numbers,

!         and Peclet numbers from previous time step

!

!    w,xi = weighting functions and abscissas for Gaussian Quadrature

!    phi,phibou = element shape functions, boundary surface shape func.

!    dpxi = derivatives of shape functions wrt natural coordinates

!    d2phi,d2phidxde,d2phidxdz,d2phidedz = Second derivatives of shape

!          functions wrt to natural coordinates

!    d2shpdx = spatial derivatives of shape functions

!    d2phidx = Second spatial derivatives of shape functions

!

!    FDV equation variables

!       rhs = right hand side vector of FDV equations

!       delu = vector of unknowns

!       Bel = Element stiffness matrix

!       alu = LU factorization of element stiffness matrix

!

!    F = Convective Flux vector

!

!    ****************************************************************

!    Module that contains dimensions for variables

!

Module param_var

!

Implicit None

Save

!

integer, parameter :: mpoin = 8000

integer, parameter :: melem = 8000

integer, parameter :: mboun = 1000,mface = mboun

integer, parameter :: mdimn=2,mnode=4,mequa=4

integer, parameter :: ngaus=2,mgaus=ngaus**mdimn

integer, parameter :: mordr=mpoin*mequa,meleq=mnode*mequa

integer, parameter :: mbsize=128,mblock=melem/64 + 1

!

!    parameters for adaptive mesh

!      integer, parameter :: mconn=24,mhng=36   !  3-D Adaptive Mesh

integer,parameter :: mconn=8,mhng=8       !  2-D Adaptive Mesh

!

end module param_var

!

!  ******************************************************************

!   Module that contains thermodynamic properties

!

Module thermo_propt

!

Implicit None

!

Save

!

real :: gamma,gamm1,gm1m2pr,gm3d2,gm1d2,gamm3

real :: cgas,csuth,creyn,cmach,cmasq,cv,cartf

real :: tinf,twall

!

end Module thermo_propt

!

!    ****************************************************************

!    Module to initialize flow variables

!

Module flow_var

!

Use param_var

!

Implicit None

Save

!

real, dimension(mpoin) :: dense,energ,press,tempr

real, dimension(mordr) :: u,uold

real, dimension(mdimn,mpoin) :: vel

!

End Module flow_var

!

!    ****************************************************************

!    Module to initialize finite element grid data

!

Module grid_info

!

Use param_var

!

Implicit None

Save

!

integer, dimension(mnode,melem) :: lnod  ! Element Nodes

real, dimension(mdimn,mpoin) :: x

integer :: lblock(mblock)   !  Block information

real :: cleng(melem)        !  Element Characteristic Length

real :: alpa         !  Compression Corner Deflection Angle

integer :: imax,jmax,kmax   !  Initial points in x, y, and z dir

!

end module grid_info

!

!     ***************************************************************

!     Module containing boundary condition information

!

Module Bnd_cnd

!

Use param_var

!

Implicit None

Save

!

integer :: ibcon(3,mboun)   !  Boundary Point information

integer, dimension(mpoin) :: indbc,indbd

integer :: ibouncnd(mordr)  !  Boundary Condition Array

real :: rbcon(mdimn,mboun)	!  Direction Cosines along Boundary Surf

!

End Module Bnd_cnd

!     ***************************************************************

!     Module with Boundary Face information

!

Module bface_info

!

Use param_var

!

Implicit None

Save

!

integer,dimension(8,mface) :: lface,lfacnode

!

end module bface_info

!

!    ****************************************************************

!    Module to initialize Variation Parameters

!

Module Var_parm

!

Use param_var

!

Implicit None

Save

!

real :: s1,s2,s3,s4

real, dimension(melem) :: s1ele,s2ele,s3ele,s4ele

real, dimension(melem) :: s3pec,s4pec   !  Variation parameters

!  from Peclet Number

!

end module Var_parm

!

!    ****************************************************************

!    Module for old values of Mach Number, Reynolds number, and Peclet

!    number for calculation of variation parameters

!

Module Mach_old

!

Use Param_var

!

Implicit None

Save

!

real, dimension(mpoin) :: emachold,reyold,pecold   !  Values from

!  previous timestep

end Module Mach_old

!

!    ****************************************************************

!    Module to initialize the domain shape functions for Gaussian Quadrature

!

Module Gaus_quad

!

Use param_var

!

Implicit None

Save

!

real :: w(ngaus),xi(ngaus),ww(mgaus)   !  Gaussian points

!

end module Gaus_quad

!

!    ****************************************************************

!    Module to initialize the domain shape functions for Gaussian Quadrature

!

Module shap_fcns

!

Use param_var

!

Implicit None

Save

!

real :: phi(mnode,mgaus),dpxi(mdimn,mnode,mgaus)  !  Shape Functions

real :: d2phi(mnode,mgaus)    ! Second derivative of shape Functions

!                                                ! of shape functions

end module shap_fcns

!

!    ****************************************************************

!    Module for additional derivative for Three-D shape functions

!

!    d2phidxde - derivative wrt xi and eta

!    d2phidxdz -      "      "  xi and zeta

!    d2phidedz -      "      "  eta and zeta

!

Module Shap3d_fcns

!

Use param_var

!

Implicit None

Save

!

real, dimension(mnode,mgaus) :: d2phidxde,d2phidxdz,d2phidedz

!

end module Shap3d_fcns

!

!    ****************************************************************

!    Shape functions for the boundary surfaces for a 2-D Domain

!

Module Shapbou_2d

!

Use param_var

!

Implicit None

Save

!

real :: phibou(2,ngaus),dpxi(2,ngaus)

!

end module Shapbou_2d

!

!    ****************************************************************

!    Module to initialize shape functions for the boundary surfaces

!    for a Three-dimensional domain

!

Module Shapbou_3d

!

Use param_var

!

Implicit None

Save

!

integer, parameter :: ngsurf=ngaus**(mdimn-1)

real :: phi(4,ngsurf),dpxi(2,4,ngsurf)

real :: d2phi(4,ngsurf), w_w(ngsurf)

real :: dshpdx(mdimn,4,ngsurf),detjac(ngsurf)

real :: d2phidx(mdimn,mdimn,4,ngsurf)

!

end module Shapbou_3d

!

!    *****************************************************************

!    Module for Coefficients for FDV Equations

!

Module FDV_coef

!

Use param_var

!

Implicit None

Save

!

real,dimension(mordr) :: rhs,delu

real,dimension(melem,meleq,meleq) :: Bel

!

end module FDV_coef

!

!    ****************************************************************

!    Module to initialize Convective Flux

!

Module Conv_Flux

!

Use param_var

!

Implicit None

Save

!

real :: F(mpoin,mdimn,mequa)   ! Convective Flux

!

end module Conv_Flux

!

!    *****************************************************************

!    LU Factorization of Stiffness Matrix for GMRES Solve

!

Module GMRES_var

!

Implicit None

!

Save

!

real,allocatable,dimension(:,:,:) :: alu

!

end module GMRES_var

!

!     Flowfield Dependent Variation Finite Element Method with

!     a Element-by-Element GMRES Equation Solver with Left

!     Preconditioning

!

!     Programmed by Gary W. Heard

!                   Graduate Research Assistant

!                   University of Alabama in Huntsville

!

!     This version contains no parallel processing or adaptive mesh

!

!     **************************** Variables **************************

!     npoin = number of grid points, nelem = number of element

!     nboun = number of boundary point, nface = number of boundary faces

!     npold = total number of grid points, including hanging nodes

!     nnode = number of grid points per element

!     nequa = number of unknowns per nodal point

!     ndimn = number of dimensions

!     nordr = total number of equations (npoin*nequa)

!     neleq = number of element equation (nnode*nequa)

!     ntime = number of time steps to be executed

!     ntstp = number of time steps to be executed prior to mesh

!             refinement, if < ntime, no refinement

!     isolve = specifies equation solver, = 4 for EBE GMRES

!     iprecnd = preconditioner for solver, = 0, no preconditioner

!                                          = 1, LU Preconditioner only

!                                          = 2, Diagonal + LU

!                                          = 3, Diagonal only

!                                          = 4, Diagonal + SOR

!     igmres = number of iterations in GMRES cycle

!     nvisc = 1 for N-S Equations, 0 for Euler

!     ierrind = 1 for FDV parameters as error indicators

!             = 2 for primitive variable error indicators

!     imppa = definition of 2nd-order variation parameters,

!           = 7 for current definitiion (0.5(1+s1^eta))

!     nblock = number of element blocks

!     time = total elapsed time

!     cfl = Courant-Frederich-Levy number

!     dt = delta t

!     en = exponent for 2nd-order variation parameter

!     adap = logical variable describing whether grid has been refined

!     artvis, strem = logical variables (not currently used)

!     *****************************************************************

!

PROGRAM ebe_fdv_ver1

!

Use param_var

!      Use FDV_coef

!

Implicit None

!

integer :: npoin,nelem,nboun,nface,npold

integer :: nnode,nequa,ndimn,nordr,neleq

integer :: ntime,ntstp,niter,itime

integer :: isym,nsave,itmax,iunit,isolve,iprecnd,igmres

integer :: nvisc,ierrind,imppa

integer :: koun,iout,iternum,iter

integer :: ifile,jfile,kfile

integer :: nblock

integer :: ierr

integer :: numrefi

integer :: istat    ! Allocation status parameter

real :: time,cfl,dt,en

real :: epstol,tol,omega

LOGICAL :: ADAP, artvis,strem

!

!      CHARACTER*1 NUMB

CHARACTER(len=4) :: NUM

CHARACTER(len=12) :: FILENAME

!

open(unit=23,file='timestep.out')

!

!     READ IN INITIAL DATA

!

CALL OPENING(NPOIN,NELEM,NBOUN,NFACE,NNODE,NEQUA,NDIMN,neleq,&

&             NPOLD,NTIME,NTSTP,NITER,TIME,ISYM,NSAVE,epstol, &

&             TOL,ITMAX,omega,IUNIT,NORDR,CFL,isolve,iprecnd, &

&             adap,igmres,NVISC,ierrind,artvis,strem,         &

&             IMPPA,en)

!

CALL INPUT(NPOIN,NELEM,NBOUN,NFACE,NDIMN,NNODE,NPOLD,        &

&           nequa,adap,TIME,nblock)

!

!     CONVERT PRIMITIVE INPUT DATA TO CONSERVATION VARIABLES

!

call consvar(npoin,nequa,ndimn)

!

if(time.eq.0.000000)  &          ! Block elements for GMRES solver

! Ref: Shakib (1989)

& call block(ndimn,nnode,nelem,nface,npoin,adap,nblock)

!

!     CALCULATE BOUNDARY FACE CONNECTIVITY

!

IF(TIME.EQ.0.000000.AND.NDIMN.eq.2)       &

&   CALL GENBFAC(NELEM,NBOUN,NNODE,NFACE)    ! Two Dimensions

!

if(time.eq.0.000.and.ndimn.eq.3)          &

&   call genbfac3(nelem,nboun,nnode,nface)   ! Three Dimensions

!

!     INITIALIZE VARIABLES FOR PRINTING INTERMEDIATE OUTPUT

!

KOUN = 1

IOUT = NTIME/5  ! Counter for printing intermediate output

!      iout = 100

!

ITERNUM = 0

!

!     CALCULATE SHAPE FUNCTIONS FOR GAUSSIAN QUADRATURE INTEGRATION

!

CALL GAUSINT(ndimn)

!

!  Calculate element characteristic lengths

!

Select Case(ndimn)

Case(1)

call eleleng(nelem)

Case(2)

CALL EVALELE(NNODE,NELEM,adap)

Case(3)

call evalele3d(adap,ndimn,nnode,nelem)

end Select

!

!      print *, ' ENTERING ITERATION LOOP'

!     BEGIN ITERATION LOOP

!

timeloop: DO ITIME = 1, NTIME

ITERNUM = ITERNUM + 1

!      print *, ' CALCULATING TIME STEP'

!

!       if(ndimn>1) then

CALL CFLTODT(NELEM,nnode,ndimn,CFL,DT) ! Calculate timestep from

! CFL number

!       else

!         dt=0.407e-03

!       endif

!

!      print *, ' CALCULATING IMPLICITNESS PARAMETERS'

IF(IMPPA.GE.1)     &

&      CALL IMPARM(NELEM,NPOIN,NNODE,NVISC,IMPPA,ndimn,en,iternum)

!     &                 kount,iout,ITERNUM)

!

TIME = TIME + DT  ! Increment time

!

!      print *, ' CALCULATING CONVECTIVE FLUX'

Select Case(ndimn)

Case(1)

call convflx1(npoin)

Case(2)

CALL CONVFLX(NPOIN)

Case(3)

call convflx3(npoin)

end Select

!

!      write(*,5000)

! 5000 format(' BUILDING MEI EQUATIONS')

CALL MEIEQNS(NELEM,NNODE,NFACE,NEQUA,NDIMN,nordr,neleq,DT,   &

&                nvisc,IMPPA,nblock,adap,artvis,itime)

!

!      print *,' ENTERING EQUATION SOLVER SUBROUTINE'

!

call Solve(nblock,nelem,nnode,nequa,ndimn,nordr,neleq,epstol,tol,&

&            iprecnd,igmres,itime,itmax,iunit,omega,isolve,ierr)

!

CALL RMSERR(npoin,nequa,dt,itime,ierr) ! Calculate RMS error

!

!    Calculate primitive variables from Conservation Variables

!

CALL PRIMVAR(NPOIN,nequa,ndimn)

!

!    Print intermediate output every "iout" timesteps

!

iter=iternum

if((ntime<ntstp.and.iter==koun*iout)&

&.or.iter/ntstp*ntstp==iter) then

!      if(iter/ntstp*ntstp.eq.iter) then   !  Print output prior to remeshing

write(num,'(i4)')iter

if(iter.lt.10) then

filename='t_'//num(4:4)//'.out'

elseif(iter.lt.100) then

filename='t_'//num(3:4)//'.out'

elseif(iter.lt.1000) then

filename='t_'//num(2:4)//'.out'

else

filename='t_'//num(1:4)//'.out'

endif

ifile=17

open(unit=ifile,file=filename)

!

if(iter.lt.10) then

filename='wall'//num(4:4)//'.out'

elseif(iter.lt.100) then

filename='wall'//num(3:4)//'.out'

elseif(iter.lt.1000) then

filename='wall'//num(2:4)//'.out'

else

filename='wall'//num(1:4)//'.out'

endif

jfile = 19

if(ndimn>1) open(unit=jfile,file=filename)

!

CALL output(ifile,jfile,npoin,nelem,nboun,nface,nnode,nequa,ndimn,&

&            adap,ntime,ntstp,time,cfl,igmres,iprecnd,isolve,      &

&            nvisc,ierrind,imppa,en,npold,artvis,strem,nblock)

!

if(iter.lt.10) then

filename='imp'//num(4:4)//'.out'

elseif(iter.lt.100) then

filename='imp'//num(3:4)//'.out'

elseif(iter.lt.1000) then

filename='imp'//num(2:4)//'.out'

else

filename='imp'//num(1:4)//'.out'

endif

!

kfile = 18

open(unit=kfile,file=filename)

call outimp(kfile,nelem,npoin,nnode,npold,adap)

koun=koun+1

endif

!

numrefi = itime/ntstp

!

!

enddo timeloop

!

!   Print final output

!

CALL output(60,70,npoin,nelem,nboun,nface,nnode,nequa,ndimn,    &

&            adap,ntime,ntstp,time,cfl,igmres,iprecnd,isolve,    &

&            nvisc,ierrind,imppa,en,npold,artvis,strem,nblock)

!

IF(IMPPA.GE.1)     &

&   CALL OUTIMP(11,nelem,npoin,nnode,npold,adap)

!

!      if(strem) then

!

!	 call stremfn(npoin,nelem,nequa,nnode,adap,

!     &                isym,nsave,epstol,tol,itmax,ierr)

!

!	 call outstrem(npoin)

!      endif

1500 format(' WARNING:  ALLOCATION OF FDV ARRAYS FAILED, istat = ',i8)

!

STOP

END Program ebe_fdv_ver1

! --------------------------------------------------------------------

!

subroutine opening(npoin,nelem,nboun,nface,nnode,nequa,ndimn,neleq,&

&                   npold,ntime,ntstp,niter,time,isym,nsave,epstol, &

&                   tol,itmax,omega,iunit,nordr,cfl,isolve,iprecnd, &

&                   adap,igmres,nvisc,ierrind,artvis,strem,imppa,en)

!

Use param_var

Use thermo_propt

Use var_parm

Use grid_info

Implicit None

!

logical,intent(out) :: adap, artvis,strem

integer,intent(out) :: npoin,nelem,nboun,nface,npold

integer,intent(out) :: nnode,nequa,ndimn

integer,intent(out) :: ntime,ntstp,niter,nordr,neleq

integer,intent(out) :: isym,nsave,itmax,iunit,isolve,iprecnd,igmres

integer,intent(out) :: nvisc,ierrind,imppa

real,intent(out) :: time,cfl,en

real,intent(out) :: epstol,tol,omega

real :: cpran

!

open(50,file='f.dat')

open(60,file='t.out')

open(70,file='wallprop.out')

open(21,file='rmserror.out')

open(unit=11,file='impparm.out',status='unknown')

!

!      read(50,1)

!      read(50,2)ntstp,adap,remesh,incom,artvis

read(50,1)

read(50,*)nvisc,isolve,ierrind,iprecnd,artvis,imppa

read(50,1)

read(50,*) ntime,ntstp,time,adap,igmres

read(50,1)

read(50,*) cfl,s1,s2,s3,s4

!      read(50,1)

!      read(50,*) igeom, dimcorn, endcorn,alpa

read(50,1)

read(50,*) cmach,creyn,tinf,twall

read(50,1)

read(50,*) en,strem,alpa

READ(50,1)

READ(50,*) NNODE,NEQUA,NDIMN

read(50,1)

if(ndimn.ne.3) then

read(50,*) nelem,npoin,nboun,nface,imax,jmax

else

read(50,*) nelem,npoin,nboun,nface,imax,jmax,kmax

endif

!      if(iacnt.eq.2.or.iacnt.eq.3) read(50,*) npold

if(adap) read(50,*) npold

1 format(20a4)

!    2 format(i7,4L7)

!

! --- NUMBER OF ITERATION OF LUMPING MASS CALCULATION.

niter=3

!

!      nnode=4

!      nequa=4

!      ndimn=2

!

! --- PARAMETERS FOR PGMRES.

isym=0

nsave=10

epstol = 1.0e-09

tol=1.0e-12

itmax=10

omega = 0.0

iunit=10

nordr=npoin*nequa

neleq = nnode*nequa

!

! --- THERMODYNAMICS PROPERTIES.

!

gamma=1.4

gamm1=gamma-1.0

cpran=0.72

cartf=2.0

!

csuth=110./tinf

cmasq=cmach*cmach

cv=1./gamma/gamm1/cmasq

cgas=1./gamma/cmasq

!

gm1m2pr=gamm1*cmasq*cpran

gm1d2=gamm1/2.

gamm3=gamma-3.

gm3d2=gamm3/2.

!

return

end subroutine opening

!

! --------------------------------------------------------------------

!

subroutine input(npoin,nelem,nboun,nface,ndimn,NNODE,npold,   &

&                 nequa,adap,TIME,nblock)

!

Use param_var

Use thermo_propt

Use grid_info

Use flow_var

Use bface_info

Use bnd_cnd

!

Implicit None

!

logical,intent(in) :: adap

integer,intent(in) :: npoin,nelem,nboun,nface,npold

integer,intent(in) :: ndimn,nnode,nequa

integer,intent(out) :: nblock

real,intent(in) :: time

integer :: ielem,ipoin,ip,ie,idimn,iboun,iequa,iordr,iface

integer ::iblock,j    !  Loop counters

real :: energy,velo2

!

read(50,1)

do ielem=1,nelem

read(50,*)ie,(LNOD(j,ielem),j=1,NNODE)  ! Element Connectivity

enddo

!

read(50,1)

do ipoin=1,npoin

read(50,*)ip,(X(j,ipoin),j=1,ndimn)     ! Cartesian Coordinates

enddo

!

! --- NONDIMENSIONALIZED PRIMITIVE VARIABLES.

!

read(50,1)

do ipoin=1,npoin

read(50,*)ip,dense(ipoin),(VEL(idimn,IPOIN),idimn=1,ndimn)   &

&          ,energ(ipoin)

velo2 = 0.0

do idimn = 1, ndimn

velo2 = velo2+vel(idimn,ipoin)*vel(idimn,ipoin)

enddo

energy=energ(ipoin)-0.5*(velo2)

press(ipoin)=dense(ipoin)*gamm1*energy

tempr(ipoin)=energy/cv

enddo

!

!    Boundary points and boundary conditions

!

read(50,1)

do iboun=1,nboun

if(ndimn.lt.3) then

read(50,*)(ibcon(j,iboun),j=1,3),(rbcon(j,iboun),j=1,ndimn)

else

read(50,*)(ibcon(j,iboun),j=1,3)

endif

enddo

!

DO IPOIN = 1, NPOIN

INDBC(IPOIN) = 0

INDBD(IPOIN) = 0

enddo

DO IBOUN = 1, NBOUN

INDBC(IBCON(1,IBOUN)) = IBCON(2,IBOUN)

INDBD(IBCON(1,IBOUN)) = IBCON(3,IBOUN)

enddo

!

! Set boundary conditions on each equations

!

if(ndimn==1) then

iordr = 0

do ipoin = 1,npoin

do iequa = 1,nequa

iordr = iordr + 1

ibouncnd(iordr) = indbc(ipoin)

if(indbc(ipoin)==6) then  ! Constant density at exit

if(iequa==1) ibouncnd(iordr) = -ibouncnd(iordr)

endif

enddo

enddo

endif

!

if(ndimn==2) then

!

IORDR = 0

DO IPOIN = 1, NPOIN

DO IEQUA = 1, NEQUA

IORDR = IORDR + 1

IBOUNCND(IORDR) = INDBC(IPOIN)

IF(INDBC(IPOIN).EQ.1.OR.INDBC(IPOIN).EQ.11.OR.       &

&         INDBC(IPOIN).EQ.21.or.indbc(ipoin).eq.4) THEN

IF(IEQUA.EQ.2.OR.IEQUA.EQ.3)                      &

&            IBOUNCND(IORDR) = -IBOUNCND(IORDR)

IF(INDBD(IPOIN).NE.2.AND.IEQUA.EQ.4)              &

&            IBOUNCND(IORDR) = 100

ENDIF

!

if(indbc(ipoin).eq.3.or.indbc(ipoin).eq.5) then

IF(IEQUA.EQ.2.OR.IEQUA.EQ.3)                      &

&            IBOUNCND(IORDR) = -IBOUNCND(IORDR)

IF(INDBD(IPOIN).NE.2.AND.IEQUA.EQ.4)              &

&            IBOUNCND(IORDR) = 100

ENDIF

!

if(indbc(ipoin).eq.2.or.indbc(ipoin).eq.12) then

if(iequa.eq.3) ibouncnd(iordr) = -ibouncnd(iordr)

endif

!

if(indbc(ipoin).eq.15) then

if(iequa.eq.2) ibouncnd(iordr) = 101

if(iequa.eq.3) ibouncnd(iordr) = -ibouncnd(iordr)

endif

ENDDO

ENDDO

!

endif

!

if(ndimn==3) then

!

IORDR = 0

DO IPOIN = 1, NPOIN

DO IEQUA = 1, NEQUA

IORDR = IORDR + 1

IBOUNCND(IORDR) = INDBC(IPOIN)

! No slip wall on bottom (1), left side (11), and right side (21)

IF(INDBC(IPOIN)==1.OR.INDBC(IPOIN)==11.OR.        &

&         INDBC(IPOIN)==21.or.indbc(ipoin)==31.or.       &

&         indbc(ipoin)==41) THEN

! u = v = w = 0

IF(IEQUA.EQ.2.OR.IEQUA.EQ.3.or.iequa.eq.4)         &

&            IBOUNCND(IORDR) = -IBOUNCND(IORDR)

IF(INDBD(IPOIN).NE.2.AND.IEQUA.EQ.5)               &

&            IBOUNCND(IORDR) = 100    ! T = Twall

ENDIF

!

!  Compression Ramp on bottom (4) and left side (14)

IF(INDBC(IPOIN)==4.OR.INDBC(IPOIN)==14)then

IF(IEQUA.EQ.2.OR.IEQUA.EQ.3.or.iequa.eq.4) & ! u = v = w =0

&            IBOUNCND(IORDR) = -IBOUNCND(IORDR)

IF(INDBD(IPOIN).NE.2.AND.IEQUA.EQ.5)               &

&            IBOUNCND(IORDR) = 100                ! T = Twall

ENDIF

!

if(indbc(ipoin).eq.222) then     ! Bottom corner, u = w = 0

if(iequa.eq.2.or.iequa.eq.4)                       &

&          ibouncnd(iordr) = -ibouncnd(iordr)

endif

!

if(indbc(ipoin).eq.22) then  ! Symmetry on left and right sides

if(iequa.eq.2) ibouncnd(iordr) = -ibouncnd(iordr)  ! u = 0

endif

!

if(indbc(ipoin).eq.2.or.indbc(ipoin).eq.12) then   !  Symmetry on

if(iequa.eq.4) ibouncnd(iordr) = -ibouncnd(iordr)  ! Top and bottom

endif                                               ! Surface w = 0

!

ENDDO

ENDDO

!

endif

!

!   Read in boundary face connectivity

!

if(ndimn/=1) then

IF(adap.or.TIME>0.000000) THEN

read(50,1)

do iface=1,nface

read(50,*) (lface(j,iface),j=1,8)

if(ndimn.eq.3) read(50,*) (lfacnode(j,iface),j=1,8)

enddo

endif

endif

!

!     Element block information   Ref: Shakib, 1989

!

if(time>0.000000) then

read(50,1)

read(50,*)nblock,(lblock(iblock),iblock=1,nblock+1)

endif

!

1 format(20a4)

!

return

end subroutine input

!

!  ***********************************************************

!     THE FOLLOWING SUBROUTINE CONVERTS PRIMITIVE VARIABLES TO

!     CONSERVATION VARIABLES

!

SUBROUTINE CONSVAR(NPOIN,nequa,ndimn)

!

Use flow_var

!

Implicit None

!

integer,intent(in) :: npoin,nequa,ndimn

integer :: L,idimn,iordr   ! Loop counters

!

DO L = 1,NPOIN

iordr = nequa*(L-1)

U(iordr+1) = DENSE(L)

do idimn=1,ndimn

U(iordr+1+idimn)=dense(L)*VEL(idimn,L)

enddo

U(iordr+2+ndimn)=dense(L)*ENERG(L)

enddo

!

RETURN

end subroutine consvar

!     ***************************************************************

!     Subroutine to block elements in nonrecursive blocks

!                adapted from Hughes, Ferencz, and Hallquist (1987)

!

Subroutine block(ndimn,nnode,nelem,nface,npoin,adap,nblock)

!

!   *****************Variables***************************************

!

!     iblock = array storing the block number assigned to each element

!     lblock = pointer identifying the first element number in a block

!     nblock = total number of blocks required for the finite element grid

!     ielblk = array storing the number of elements in each block

!     mbsize = maximum number of element allowed in a block

!

Use param_var

Use grid_info

Use Bface_info

!

Implicit None

!

logical,intent(in) :: adap

integer,intent(in) ::ndimn,nnode,nelem,nface,npoin

integer,intent(inout) :: nblock

!

!

integer :: iblock(nelem),ielblk(mblock),inode(npoin)

integer :: larra(mhng,melem)

integer :: newelno(nelem),ltemp(nelem)

integer :: istart,ifirst,ibsize,j,iel,isum

integer :: ielem,knode,kblock,icount,jelem,iface

integer :: iconn,ihng,jnode,lnode,kelem

integer :: knod1,knod2,knod3,igrp,igrp1,jblock

integer :: kelem1

!

!     initialize IBLOCK array

!

iblock = 0

lblock = 0

!

!     set initial search parameters and number of blocks

!

istart = 1

nblock = 0

100 continue

!

!     Find first element in original ordering not previously

!          assigned to a block

!

do ifirst = istart,nelem

!

if(iblock(ifirst).eq.0) then

!

!           start a new block

!

nblock = nblock + 1        ! increment number of blocks

iblock(ifirst) = nblock	   ! assign element to new block

ibsize = 1                 ! initialize block size

istart = ifirst + 1        ! reset search parameter

inode = 0                  ! zero the INODE array

do j = 1,nnode         ! designate element's nodes as

inode (lnod(j,ifirst)) = 1   ! active within current block

enddo

!

!    Search subsequent elements in original ordering and test

!         unassigned elements for inclusion in current block

!

do iel = istart,nelem

!

if(iblock(iel).eq.0) then

!

!       current element not blocked.  Test if it shares common nodes

!               with any element previously assigned to current block

!               (i.e., have any of its nodes been previously activated)

!

isum = 0

do j = 1,nnode

isum = isum + inode(lnod(j,iel))

enddo

!

if(isum.eq.0) then

!

!         current element does not share nodes with elements

!            in current block, so add element to block

!

iblock(iel) = nblock   ! assign block number to element

ibsize = ibsize + 1    ! increment size of current block

do j = 1,nnode     ! designate element's nodes as

inode(lnod(j,iel)) = 1 !   active within current block

enddo

!

if(ibsize.eq.mbsize) then

!

!           block has reached maximum size

!

ielblk(nblock) = mbsize  ! store size of completed block

goto 100                 !    and begin new search

endif

!

endif

!

endif

enddo

!

!       element list exhausted before block filled to maximum size

!

ielblk(nblock) = ibsize       ! store size of completed block

goto 100                      !     and begin new block

!

endif

!

enddo

!

!     all element assigned to a block, now reorder the lnod array

!

do ielem = 1,nelem

do knode = 1,nnode

larra(knode,ielem) = lnod(knode,ielem)   ! temporary storage

enddo                                       ! of the old connectivity

enddo

!

jelem = 0

!

do kblock = 1,nblock

!

icount = 0

!

do ielem = 1,nelem

!

if(iblock(ielem).eq.kblock) then

!

icount = icount + 1

jelem = jelem + 1

!

do knode = 1,nnode

lnod(knode,jelem) = larra(knode,ielem)

enddo

!

newelno(ielem) = jelem

!

if(icount.eq.1) lblock(kblock) = jelem

!

endif

!

enddo

enddo

!

lblock(nblock+1) = nelem + 1

!

write(45,1000) nblock,(lblock(jblock),jblock=1,nblock+1)

1000 format(8i8)

1050 format(' New Element Numbers')

1100 format(' Nelgrp Information')

!

return

end subroutine block

!

! --------------------------------------------------------------------

!

subroutine genbfac(nelem,nboun,nnode,nface)

!

Use bnd_cnd

Use param_var

Use grid_info

Use bface_info

!

Implicit None

!

integer,intent(in) :: nelem,nboun,nnode

integer,intent(inout) :: nface

integer :: ielem,inode,inod,jnode           ! Loop counter

integer :: iface,iboun,jpoin,ibc1,ibc2

integer :: lflag,lbnode,ipoin,msum,nsum

integer :: lnode(3)

integer :: kboun

!

!     lface(1,iface) = point number for 1st boundary node

!           2        = point number for 2nd boundary node

!           3        = 1st boundary node number

!           4        = 2nd boundary node number

!           5        = 1st local node number

!           6        = 2nd local node number

!           7        = element number

!           8        = boundary condition

!

!

nface=0

!

elemloop: do ielem=1,nelem

!

lbnode=0

do inode=1,nnode

ipoin=lnod(inode,ielem)

lflag=0

if(indbc(ipoin).ne.0) then

lflag=1

lbnode=lbnode+1

goto 350

endif

350 continue

if(lflag.eq.1) lnode(lbnode)=inode

enddo

!

if(lbnode.eq.2) then

nface=nface+1

lface(7,nface)=ielem

lface(1,nface)=lnod(lnode(1),ielem)

lface(2,nface)=lnod(lnode(2),ielem)

if(lnode(1).eq.1.and.lnode(2).eq.4) then

lface(1,nface)=lnod(lnode(2),ielem)

lface(2,nface)=lnod(lnode(1),ielem)

endif

endif

!

if(lbnode.eq.3) then

!

nsum=10

msum=0

do inod=1,3

msum=msum+lnode(inod)

enddo

!

if(msum.eq.9) then

nface=nface+1

lface(7,nface)=ielem

lface(1,nface)=lnod(2,ielem)

lface(2,nface)=lnod(3,ielem)

nface=nface+1

lface(7,nface)=ielem

lface(1,nface)=lnod(3,ielem)

lface(2,nface)=lnod(4,ielem)

endif

!

if(msum.eq.8) then

nface=nface+1

lface(7,nface)=ielem

lface(1,nface)=lnod(3,ielem)

lface(2,nface)=lnod(4,ielem)

nface=nface+1

lface(7,nface)=ielem

lface(1,nface)=lnod(4,ielem)

lface(2,nface)=lnod(1,ielem)

endif

!

if(msum.eq.7) then

nface=nface+1

lface(7,nface)=ielem

lface(1,nface)=lnod(4,ielem)

lface(2,nface)=lnod(1,ielem)

nface=nface+1

lface(7,nface)=ielem

lface(1,nface)=lnod(1,ielem)

lface(2,nface)=lnod(2,ielem)

endif

!

if(msum.eq.6) then

nface=nface+1

lface(7,nface)=ielem

lface(1,nface)=lnod(1,ielem)

lface(2,nface)=lnod(2,ielem)

nface=nface+1

lface(7,nface)=ielem

lface(1,nface)=lnod(2,ielem)

lface(2,nface)=lnod(3,ielem)

endif

!

endif

!

enddo elemloop

!

! --- save the boundary node in the lface

!

lp1: do iface=1,nface

lp2: do inode=1,2

ipoin=lface(inode,iface)

lp3: do iboun=1,nboun

jpoin=ibcon(1,iboun)

if(ipoin.eq.jpoin) then

kboun=iboun

exit lp3

endif

enddo lp3

lface(2+inode,iface)=kboun

enddo lp2

enddo lp1

!

do iface=1,nface

ielem=lface(7,iface)

do inode=1,2

ipoin=lface(inode,iface)

do jnode=1,nnode

jpoin=lnod(jnode,ielem)

if(ipoin.eq.jpoin) lface(4+inode,iface)=jnode

enddo

enddo

enddo

!

do iface=1,nface

ibc1=ibcon(2,lface(3,iface))

ibc2=ibcon(2,lface(4,iface))

lface(8,iface)=ibc1

if(ibc1<ibc2) lface(8,iface)=ibc2

if(ibc1==1.and.ibc2==11) lface(8,iface)=ibc1

if(ibc1==-1.and.ibc2==1) lface(8,iface)=ibc1

if(ibc1==1.and.ibc2==-1) lface(8,iface)=ibc2

if(ibc1==11.and.ibc2==1) lface(8,iface)=ibc1

if(ibc1==21.and.ibc2==-12) lface(8,iface) = ibc2

if(ibc1==9) lface(8,iface) = ibc1

if(ibc1==10) lface(8,iface) = ibc2

if(ibc2==12.or.ibc2==7) lface(8,iface)=ibc1

if(ibc1==12) lface(8,iface) = ibc1

if(ibc1==-2.or.ibc1==-12) lface(8,iface)=ibc1

if(ibc1==5) lface(8,iface) = ibc2

if(ibc2==-2.and.ibc1==21) lface(8,iface) = ibc2

if(ibc2==15.and.ibc1==21) lface(8,iface) = ibc2

if(ibc2==8.or.ibc2==9) lface(8,iface) = ibc2

enddo

!

return

end subroutine genbfac

!

!    *****************************************************************

!

Subroutine GAUSINT(ndimn)

!

Use Gaus_quad

!

Implicit None

!

integer,intent(in) :: ndimn

!

!     CALCULATE GAUSSIAN POINTS AND WEIGHT FUNCTIONS FOR GAUSSIAN

!     QUADRATURE INTEGRATION

!

CALL GAULEG(NGAUS,XI,W,-1.0,1.0)

!

!     CALCULATE SHAPE FUNCTIONS

!     SUBROUTINE SHAPDOM - DOMAIN SHAPE FUNCTIONS

!     SUBROUTINE SHAPSURF - BOUNDARY SHAPE FUNCTIONS

!

Select Case(ndimn)

Case(1)

call shap1d

Case(2)

CALL SHAPDOM

CALL SHAPSURF(XI)

Case(3)

call shapdom3

call shapsurf_3d

end Select

!

RETURN

end subroutine gausint

!

!   ****************************************************************

!     SUBROUTINE TO CALCULATE DOMAIN SHAPE FUNCTIONS AND DERIVATIVES

!

SUBROUTINE SHAPDOM

!

Use param_var

Use Gaus_quad

Use shap_fcns

!

Implicit None

!

integer :: igaus,i,j

!

!     LINEAR INTERPOLATION FUNCTIONS

!

IGAUS=0

DO I = 1, NGAUS

DO J = 1, NGAUS

IGAUS = IGAUS + 1

PHI(1,IGAUS)=0.25*(1.-XI(I))*(1.-XI(J))

PHI(2,IGAUS)=0.25*(1.+XI(I))*(1.-XI(J))

PHI(3,IGAUS)=0.25*(1.+XI(I))*(1.+XI(J))

PHI(4,IGAUS)=0.25*(1.-XI(I))*(1.+XI(J))

!

!     derivatives of linear interpolation functions

!

dpxi(1,1,IGAUS)=-0.25*(1.-xi(j))

dpxi(2,1,IGAUS)=-0.25*(1.-xi(i))

dpxi(1,2,IGAUS)=0.25*(1.-xi(j))

dpxi(2,2,IGAUS)=-0.25*(1.+xi(i))

dpxi(1,3,IGAUS)=0.25*(1.+xi(j))

dpxi(2,3,IGAUS)=0.25*(1.+xi(i))

dpxi(1,4,IGAUS)=-0.25*(1.+xi(j))

dpxi(2,4,IGAUS)=0.25*(1.-xi(i))

WW(IGAUS)=W(I)*W(J)

!

!     Second derivatives of interpolation functions

!

d2phi(1,igaus) = 0.25

d2phi(2,igaus) = -0.25

d2phi(3,igaus) = 0.25

d2phi(4,igaus) = -0.25

!

enddo

enddo

!

RETURN

end subroutine shapdom

!

!     ***************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATE THE SHAPE FUNCTIONS AND THEIR

!     DERIVATIVES FOR A BOUNDARY SURFACE

!

SUBROUTINE SHAPSURF(XI)

!

Use param_var

Use shapbou_2d

!

Implicit None

!

real,intent(in) :: XI(NGAUS)

integer :: i

!

!     TWO-DIMENSIONAL LINEAR SHAPE FUNCTIONS

!

DO I = 1, NGAUS

PHIBOU(1,I)=0.5*(1.-XI(I))

PHIBOU(2,I)=0.5*(1.+XI(I))

!

!     DERIVATIVE OF SHAPE FUNCTIONS

!

DPXI(1,I)=-0.5

DPXI(2,I)=0.5

ENDDO

!

RETURN

end subroutine shapsurf

!

!     ***************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATE THE SHAPE FUNCTIONS AND THEIR

!     DERIVATIVES FOR A BOUNDARY SURFACE

!

SUBROUTINE SHAP1d

!

Use param_var

Use Gaus_quad

Use shap_fcns

!

Implicit None

!

integer :: i

!

!     TWO-DIMENSIONAL LINEAR SHAPE FUNCTIONS

!

DO I = 1, MGAUS

PHI(1,I)=0.5*(1.-XI(I))

PHI(2,I)=0.5*(1.+XI(I))

!

!     DERIVATIVE OF SHAPE FUNCTIONS

!

DPXI(1,1,I)=-0.5

DPXI(1,2,I)=0.5

ww(i)=w(i)

ENDDO

!

RETURN

end subroutine shap1d

!

! --------------------------------------------------------------------

!

subroutine gauleg(n,x,w,x1,x2)

!

! --- given the lower and upper limits of integration x1 and x2, and

!     given N, this routine returns arrays x and w of length N,

!     containing the abscissas and weights of the Gauss-Legendre

!     N point quadrature formula.

!

Implicit None

!

integer,intent(in) :: n

real,intent(out),dimension(n) :: x,w

real,intent(in) :: x1,x2

integer :: m,i,j

real :: xm,xl,z,p1,p2,p3,pp,z1

!

real,parameter :: eps=3.e-14

!

m=(n+1)/2

xm=0.5*(x2+x1)

xl=0.5*(x2-x1)

!

do i=1,m

z=cos(3.141592654*(i-0.25)/(n+0.5))

1  continue

p1=1.0

p2=0.0

do j=1,n

p3=p2

p2=p1

p1=((2.0*j-1.0)*z*p2-(j-1.)*p3)/j

enddo

pp=n*(z*p1-p2)/(z*z-1.0)

z1=z

z=z1-p1/pp

if(abs(z-z1).gt.eps) goto  1

x(i)=xm-xl*z

x(n+1-i)=xm+xl*z

w(i)=2.0*xl/((1.0-z*z)*pp*pp)

w(n+1-i)=w(i)

enddo

!

do i=1,n

if(abs(x(i)).lt.eps) x(i)=0.0

if(abs(w(i)).lt.eps) w(i)=0.0

enddo

!

return

end subroutine gauleg

!

!     ********************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE CHARACTERISTIC

!     LENGTH OF EACH ELEMENT FOR THE ONE-D ELEMENT

!

subroutine eleleng(nelem)

!

Use grid_info

!

Implicit None

!

integer,intent(in) :: nelem

integer :: i,iel,nod1,nod2

real :: dxmin

!

dxmin = 1000.

do iel = 1,nelem

nod1 = lnod(1,iel)

nod2 = lnod(2,iel)

if(nod1==0.or.nod2==0) then

print *,'NODES ARRAY IS ZERO!'

write(*,*)'ELEMENT:  ',iel,' NODES:  ',(lnod(i,iel),i=1,2)

endif

cleng(iel)=abs(x(1,nod2) - x(1,nod1))

if(cleng(iel)<=dxmin) dxmin = cleng(iel)

enddo

!

write(*,100) dxmin

100 format('Delta X min = ',e16.8)

!

return

end subroutine eleleng

!

!     ********************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE CHARACTERISTIC

!     LENGTH OF EACH ELEMENT

!

SUBROUTINE EVALELE(NNODE,NELEM,adap)

!

Use grid_info

Use Gaus_quad

Use shap_fcns

!

Implicit None

!

integer,intent(in) :: nnode,nelem

integer :: iel,inode,jpoin,i,idimn

integer,dimension(nnode) :: iel_nod

real,dimension(mdimn,nnode) :: xloc

real :: ELEAREA(nelem)

real :: dshpdx(mdimn,mnode,mgaus),detjac(mgaus)

real :: d2phidx(mdimn,mdimn,mnode,mgaus)

!

logical,intent(in) :: adap

!

DO IEL=1,NELEM

do inode = 1, nnode

iel_nod(inode) = lnod(inode,iel)

do idimn = 1,mdimn

xloc(idimn,inode) = x(idimn,iel_nod(inode))

enddo

enddo

CALL DERSHA(NNODE,mdimn,xloc,dpxi,d2phi,1,dshpdx,&

detjac,d2phidx)

ELEAREA(IEL)=0.0

DO I=1,MGAUS

ELEAREA(IEL)=ELEAREA(IEL)+DETJAC(I)*ww(i)

enddo

if(elearea(iel).le.0.0) then

write(*,1000)iel

do inode = 1,nnode

jpoin = iel_nod(inode)

write(*,*)jpoin,x(1,jpoin),x(2,jpoin)

enddo

endif

CLENG(IEL)=SQRT(ELEAREA(IEL))

!         write(27,*)iel,cleng(iel)

enddo

1000 format('Element ',i8,' has zero or negative area.',/,   &

&       'Coordinates are:  ')

RETURN

end subroutine evalele

!

!    *************************************************************

SUBROUTINE DERSHA(NNODE,ndimn,xloc,dpxi,d2phi,iflag,  &

&                  dshpdx,detjac,d2phidx)

!

!     THIS SUBROUTINE CALCULATES THE DERIVATIVES OF THE SHAPE FUNCTIONS

!

Use param_var

!      Use Gaus_quad

!      Use shap_fcns

!      Use grid_info

!

Implicit None

!

integer,intent(in) :: nnode,ndimn,iflag

real,dimension(ndimn,nnode),intent(in) :: xloc

real,intent(in) :: dpxi(mdimn,mnode,mgaus),d2phi(mnode,mgaus)

real,intent(out) :: dshpdx(mdimn,mnode,mgaus),detjac(mgaus)

real,intent(out) :: d2phidx(mdimn,mdimn,mnode,mgaus)

integer :: i,igaus,k,L

real,dimension(nnode) :: XELEM,YELEM

real :: dxdxi,dxdeta,dydxi,dydeta

!

DO I=1,nnode

XELEM(I)=Xloc(1,i)

YELEM(I)=Xloc(2,i)

enddo

!

DO IGAUS = 1, MGAUS

DXDXI = 0.0

DXDETA = 0.0

DYDXI = 0.0

DYDETA = 0.0

DO L = 1, NNODE

DXDXI = DXDXI + dpxi(1,L,IGAUS)*xelem(L)

DXDETA = DXDETA + dpxi(2,L,IGAUS)*xelem(L)

DYDXI = DYDXI + dpxi(1,L,IGAUS)*yelem(L)

DYDETA = DYDETA + dpxi(2,L,IGAUS)*yelem(L)

enddo

!

!     determinant of Jacobian

!

detjac(IGAUS)=dxdxi*dydeta - dxdeta*dydxi

!

if(detjac(IGAUS).lt.1.0e-12)then

write(*,*)'detjac = ',detjac(igaus)

write(*,*)'X-Coordinates:  ',(xelem(i),i=1,nnode)

write(*,*)'Y-Coordinates:  ',(yelem(i),i=1,nnode)

stop

endif

!

!

!     CALCULATE DERIVATIVES OF SHAPE FUNCTIONS

!

DO K = 1,NNODE

DSHPDX(1,K,IGAUS)=(DYDETA*DPXI(1,K,IGAUS) -      &

&                DYDXI*DPXI(2,K,IGAUS))/DETJAC(IGAUS)

DSHPDX(2,K,IGAUS)=(-DXDETA*DPXI(1,K,IGAUS) +     &

&                DXDXI*DPXI(2,K,IGAUS))/DETJAC(IGAUS)

enddo

!

if(iflag.eq.2)    &

&       call dersha2(igaus,nnode,ndimn,xloc,dpxi,d2phi,dxdxi,&

&dxdeta,dydxi,dydeta,detjac,d2phidx)

!

!        DETJAC(IGAUS) = DETJAC(IGAUS)*WW(IGAUS)

enddo

!

return

end subroutine dersha

!

!     ***********************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE Derivatives of the

!     ONE-D SHAPE FUNCTIONS

!

SUBROUTINE DERSH1D(nnode,xloc,dpxi,dshpdx,detjac)

!

Use param_var

!      Use Gaus_quad

!      Use shap_fcns

!      Use grid_info

!

Implicit None

!

integer,intent(in) :: nnode

real,dimension(mdimn,nnode),intent(in) :: xloc

real,dimension(mdimn,mnode,mgaus),intent(in) :: dpxi

real,intent(out) :: dshpdx(mdimn,mnode,mgaus),detjac(mgaus)

integer :: inode,igaus

real :: dl

real :: DXDS(MGAUS)

real :: XELEM(2)

DO INODE = 1, 2

XELEM(INODE) = Xloc(1,inode)

enddo

!

DO IGAUS = 1, NGAUS

DXDS(IGAUS) = 0.0

!

DO INODE = 1, 2

DXDS(IGAUS)=DXDS(IGAUS)+DPXI(1,INODE,IGAUS)*XELEM(INODE)

enddo

!

!      write(*,50)igaus,dxds(igaus),dyds(igaus)

!   50 format(i4,2e16.8)

!

DL=DXDS(IGAUS)

DETJAC(IGAUS) = DL

!

!      write(*,55)dl

!   55 format(' DL = ',e16.8)

!

DO INODE = 1, 2

DSHPDX(1,INODE,IGAUS) = dpxi(1,inode,igaus)/dxds(igaus)

enddo

enddo

!

RETURN

end subroutine dersh1d

!

!     ***************************************************************

!     Subroutine to calculate spatial second derivative of shape

!     functions for numerical damping

!

subroutine dersha2(igaus,nnode,ndimn,xloc,dpxi,d2phi,dxdxi,dxdeta,&

&dydxi,dydeta,detjac,d2phidx)

!

!DIR$ INLINEALWAYS dersha2

Use param_var

!      Use Gaus_quad

!      Use shap_fcns

!      Use grid_info

!

Implicit None

!

integer,intent(in) :: igaus,nnode,ndimn

real,dimension(ndimn,nnode),intent(in) :: xloc

real,intent(in) :: dpxi(mdimn,mnode,mgaus),d2phi(mnode,mgaus)

real,intent(in) :: dxdxi,dxdeta,dydxi,dydeta

real,intent(in) :: detjac(mgaus)

real,dimension(mdimn,mdimn,mnode,mgaus),intent(out) ::d2phidx

real :: xelem(nnode), yelem(nnode)

integer :: i,inode

real :: dxxieta,dyxieta,djacdxi,djacdeta,detjac2,detjac3

real :: term1,term2,term3,term4

!

!      print *, 'Entering Subroutine to calculate second derivatives'

do i = 1, nnode

xelem(i) = xloc(1,i)

yelem(i) = xloc(2,i)

enddo

!

dxxieta = 0.0

dyxieta = 0.0

!

do inode = 1, nnode

dxxieta = dxxieta + d2phi(inode,igaus)*xelem(inode)

dyxieta = dyxieta + d2phi(inode,igaus)*yelem(inode)

enddo

!

djacdxi = dxdxi*dyxieta - dxxieta*dydeta

djacdeta = dxxieta*dydeta - dxdeta*dyxieta

!

!       write(*,1100) igaus,detjac(igaus)

1100 format('At Gaussian Point ',i4,'Det of Jacobian = ',e16.8)

detjac2 = 1./detjac(igaus)**2

detjac3 = 1./detjac(igaus)**3

!

do inode = 1, nnode

term1 = -detjac3*djacdxi*(dydeta**2*       &

&              dpxi(1,inode,igaus) - dydeta*  &

&              dydxi*dpxi(2,inode,igaus))

!

term2 = detjac2*(dydeta*dyxieta*dpxi(1,inode,igaus)    &

&             - dydeta*dydxi*d2phi(inode,igaus))

!

term3 = detjac3*djacdeta*(dydxi*dydeta*       &

&             dpxi(1,inode,igaus) - dydxi**2*   &

&             dpxi(2,inode,igaus))

!

term4 = -detjac2*(dydxi*dydeta*                 &

&             d2phi(inode,igaus) - dydxi*dyxieta* &

&             dpxi(2,inode,igaus))

!

!     d2phi/dx2

!

d2phidx(1,1,inode,igaus) = term1 + term2 + term3 + term4

!

term1 = detjac3*djacdxi*(-dxdeta**2*        &

&             dpxi(1,inode,igaus) + dxdeta*   &

&             dxdxi*dpxi(2,inode,igaus))

!

term2 = -detjac2*(-dxdeta*dxxieta*          &

&           dpxi(1,inode,igaus)  + dxdeta*    &

&           dxdxi*d2phi(inode,igaus))

!

term3 = -detjac3*djacdeta*(-dxdxi*dxdeta*    &

&             dpxi(1,inode,igaus) + dxdxi**2*  &

&             dpxi(2,inode,igaus))

!

term4 = detjac2*(-dxdxi*dxdeta*                 &

&             d2phi(inode,igaus) + dxdxi*dxxieta* &

&             dpxi(2,inode,igaus))

!

!     d2phi/dy2

!

d2phidx(2,2,inode,igaus) = term1 + term2 + term3 + term4

!

term1 = -detjac3*djacdxi*(-dydeta*dxdeta*     &

&             dpxi(1,inode,igaus) + dydeta*     &

&           dxdxi*  dpxi(2,inode,igaus))

!

term2 = detjac2*(-dydeta*dxxieta*dpxi(1,inode,igaus)   &

&             + dydeta*dxdxi*d2phi(inode,igaus))

!

term3 = detjac3*djacdeta*(-dydxi*dxdeta*      &

&           dpxi(1,inode,igaus) + dydxi*        &

&           dxdxi*dpxi(2,inode,igaus))

!

term4 = -detjac2*(-dydxi*dxdeta*	                &

&             d2phi(inode,igaus) + dydxi*dxxieta*  &

&             dpxi(2,inode,igaus))

!

!     d2phi/dxdy

!

d2phidx(1,2,inode,igaus) = term1 + term2 + term3 + term4

!

term1 = detjac3*djacdxi*(dxdeta*dydeta*       &

&           dpxi(1,inode,igaus) - dxdeta*     &

&         dydxi*dpxi(2,inode,igaus))

!

term2 = -detjac2*(dxdeta*dyxieta*dpxi(1,inode,igaus)   &

&           - dxdeta*dydxi*d2phi(inode,igaus))

!

term3 = -detjac3*djacdeta*(dxdxi*dydeta*      &

&           dpxi(1,inode,igaus) - dxdxi*      &

&          dydxi*dpxi(2,inode,igaus))

!

term4 = detjac2*(dxdxi*dydeta*                 &

&         d2phi(inode,igaus) - dxdxi*dyxieta*  &

&         dpxi(2,inode,igaus))

!

!     d2phidydx

!

d2phidx(2,1,inode,igaus) = term1 + term2 + term3 + term4

!

enddo

!

!         write(55,1000)igaus

!         write(55,'(4e16.8)')(d2phidx2(knode,igaus), knode = 1, nnode)

!         write(55,'(4e16.8)')(d2phidy2(knode,igaus), knode = 1, nnode)

!         write(55,'(4e16.8)')(d2phidxy(knode,igaus), knode = 1, nnode)

!         write(55,'(4e16.8)')(d2phidyx(knode,igaus), knode = 1, nnode)

!

1000 format(' Gaussian Point ',i8)

!      print *,'Finished calculating second derivatives'

!

return

end subroutine dersha2

!

!   ****************************************************************

!     SUBROUTINE TO CONVERT CONSERVATION VARIABLES TO PRIMITIVE

!     VARIABLES

!

SUBROUTINE PRIMVAR(NPOIN,nequa,ndimn)

!

Use thermo_propt

Use flow_var

!

Implicit None

!

integer,intent(in) :: npoin,nequa,ndimn

integer :: j,iordr,idimn

real ::energy

!

DO J=1,NPOIN

iordr = nequa*(J-1)

DENSE(J) = U(iordr+1)

ENERG(J)=U(iordr+2+ndimn)/U(iordr+1)

ENERGY = ENERG(J)

do idimn = 1,ndimn

VEL(idimn,j)=U(iordr+1+idimn)/U(iordr+1)

ENERGY=ENERGY-0.5*VEL(idimn,J)*VEL(idimn,J)

enddo

TEMPR(J)=ENERGY/CV

PRESS(J)=DENSE(J)*GAMM1*ENERGY

enddo

!

RETURN

end subroutine primvar

!

!   *********************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE TIME STEP

!     FROM THE CFL CONDITION

!

SUBROUTINE CFLTODT(NELEM,nnode,ndimn,CFL,DT)

!

Use thermo_propt

Use grid_info

Use flow_var

!

Implicit None

!

integer,intent(in) :: nelem,nnode,ndimn

real,intent(in) :: cfl

real,intent(out) :: dt

integer :: iel,jnode,idimn,L

real :: dxoumx,dxou,velo,acou,denom,dxouele

real :: amaxele,amax,velmax,vmax

integer :: ipoin,jpoin

!

DXOUMX=1000.

DO IEL=1,NELEM

DXOU=1000.

DO JNODE=1,nnode

L=LNOD(JNODE,iel)

velo = 0.0

do idimn = 1, ndimn

velo=velo + vel(idimn,L)*vel(idimn,L)

enddo

VELO=SQRT(VELO)

IF(TEMPR(L).LT.1.0E-12) TEMPR(L)=0.0

ACOU=SQRT(GAMMA*CGAS*TEMPR(L))

DENOM=ACOU+VELO

if(denom.eq.0.0) cycle

DXOUELE=CLENG(IEL)/DENOM

IF(DXOUELE.LE.DXOU) then

DXOU = DXOUELE

ipoin = L;amaxele=acou;velmax=velo

endif

enddo

IF(DXOU.LE.DXOUMX) then

DXOUMX = DXOU

jpoin = ipoin; amax=amaxele; vmax=velmax

endif

enddo

DT=CFL*DXOUMX

!

write(23,150) jpoin,amax,vmax

!

if(dt.le.1.0e-10) then

write(*,100) dt

stop

endif

!

100 format(' ALGORITHM MAY BE UNSTABLE!  DT = ',e16.8)

150 format('Maximum velocity at point ',i8,' Max Acou vel:  ',e12.6, &

&       /,'Max vel:  ',e12.6)

RETURN

end subroutine cfltodt

!

!  *******************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE IMPLICITNESS PARAMETERS

!     BASED ON THE ELEMENT MACH NUMBER AND REYNOLDS NUMBER

!

Subroutine imparm(NELEM,NPOIN,NNODE,NVISC,IMPPA,ndimn,en,iternum)

!

Use thermo_propt

Use grid_info

Use flow_var

Use var_parm

Use Mach_old

!

Implicit None

!

integer,intent(in) :: nelem,npoin,nnode,nvisc,ndimn

integer,intent(in) :: imppa,iternum

integer :: ipoin,idimn,iel,i,jpoin

real,intent(in) :: en

real :: pwr,acou,velo,emmax,emmin,emach

real :: s1spat,ri,emmaxt,emmint,s1time

real :: cvisc,cp,hk,remax,remin,pemax,pemin,re,pe

real :: s3spat,pratio,remaxt,remint,pemaxt,pemint

real :: s3time,s3pspat,s3ptime

real :: EM(nPOIN),REYNO(nPOIN),pecno(npoin)

!

!      CHARACTER*4 NUM

!      CHARACTER*12 FILENAME

!

!      en = 6.0

pwr = en

!

!     CALCULATE CONVECTIVE IMPLICITNESS PARAMETERS BASED ON MACH

!     NUMBER

!

!

DO IPOIN = 1, NPOIN

IF(TEMPR(IPOIN).LT.1.0E-12) TEMPR(IPOIN)=0.0

ACOU = SQRT(GAMMA*CGAS*TEMPR(IPOIN))

velo = 0.0

do idimn = 1, ndimn

velo = velo +vel(idimn,ipoin)*vel(idimn,ipoin)

enddo

VELO=SQRT(velo)

IF(ACOU.LT.1.0E-12) THEN

EM(IPOIN)=CMACH

ELSE

EM(IPOIN)=VELO/ACOU

ENDIF

enddo

!

DO IEL = 1, NELEM

IPOIN = LNOD(1,IEL)

EMMAX = EM(IPOIN)

EMMIN = EMMAX

DO I= 2, NNODE

JPOIN = LNOD(i,IEL)

EMACH = EM(JPOIN)

IF(EMACH.GE.EMMAX) EMMAX = EMACH

IF(EMACH.LE.EMMIN) EMMIN = EMACH

enddo

!

IF(EMMIN.EQ.0.0)THEN

S1SPAT=1.

!            if(emmin==0.0.and.emmax==0.0) s1spat = 0.0

ELSE

ri=sqrt(emmax**2-emmin**2)/emmin

IF(RI.GE.0.01)THEN

S1SPAT=AMIN1(RI,1.0)

!               if(emmax-emmin<=1.0e-06) s1spat = 0.0

ELSE

S1SPAT=0.

ENDIF

ENDIF

!

s1ele(iel) = s1spat

!

if(iternum.gt.1) then

IPOIN = LNOD(1,IEL)

EMMAXT = EMACHOLD(IPOIN)

EMMINT = EMMAXT

DO I= 2, NNODE

JPOIN = LNOD(i,IEL)

EMACH = EMACHOLD(JPOIN)

IF(EMACH.GE.EMMAXT) EMMAXT = EMACH

IF(EMACH.LE.EMMINT) EMMINT = EMACH

ENDDO

!

IF(EMMAX.GT.EMMAXT) EMMAXT = EMMAX

IF(EMMIN.LT.EMMINT) EMMINT = EMMIN

IF(EMMINt.EQ.0.0)THEN

S1TIME=1.

!            if(emmint==0.0.and.emmaxt==0.0) s1time = 0.0

ELSE

ri=sqrt(emmaxt**2-emmint**2)/emmint

IF(RI.GE.0.01)THEN

S1TIME=AMIN1(RI,1.0)

!               if(emmaxt-emmint<=1.0e-06) s1time = 0.0

ELSE

S1TIME=0.

ENDIF

ENDIF

!

if(s1time>s1spat) s1ele(iel) = s1time

!

endif

!

Select Case(imppa)

Case(1)

S2ELE(IEL) = S1ELE(IEL)

Case(2)

S2ELE(IEL) = 1. - S1ELE(IEL)

Case(3)

S2ELE(IEL) = s2

Case(4)

S2ELE(IEL) = MAX(1.-S1ELE(IEL),0.5)

Case(5)

s2ele(iel) = sqrt(1. - s1ele(iel)**2)

Case(6)

s2ele(iel) = s1ele(iel)**pwr

Case(7)

s2ele(iel) = 0.5*(1. + s1ele(iel)**pwr)

end Select

enddo

!

!     CALCULATE DISSIPATIVE IMPLICITNESS PARAMETER BASED ON

!     REYNOLDS NUMBER

!

IF(NVISC.EQ.1) THEN

!

DO IPOIN = 1, NPOIN

!

!     CALCULATE COEFFICIENT OF VISCOSITY USING SUTHERLAND'S

!     RELATIONSHIP

!

IF(TEMPR(IPOIN).LT.1.0E-12) THEN

CVISC = 1.0

ELSE

CVISC=(1.0+CSUTH)/(TEMPR(IPOIN)+CSUTH)*TEMPR(IPOIN)**1.5

ENDIF

!

velo = 0.0

do idimn = 1, ndimn

velo=velo+vel(idimn,ipoin)*vel(idimn,ipoin)

enddo

VELO=SQRT(velo)

REYNO(IPOIN) = DENSE(IPOIN)*VELO/CVISC

cp = gamma*cv

hk = cvisc/gm1m2pr

pecno(ipoin) = cp*velo*dense(ipoin)/hk

enddo

!

DO IEL = 1, NELEM

IPOIN = LNOD(1,IEL)

REMAX = REYNO(IPOIN)

REMIN = REMAX

pemax = pecno(ipoin)

pemin = pemax

DO I= 2, NNODE

JPOIN = LNOD(i,IEL)

RE = REYNO(JPOIN)

IF(RE.GE.REMAX) REMAX = RE

IF(RE.LE.REMIN) REMIN = RE

pe = pecno(jpoin)

if(pe.ge.pemax) pemax = pe

if(pe.le.pemin) pemin = pe

enddo

!

!     s3 based on Reynolds number

!

if(remin.lt.1.0e-12) remin = 0.0

!

IF(REMIN.EQ.0.0)THEN

S3SPAT=1.

ELSE

ri = sqrt(remax**2-remin**2)/remin

IF(RI.GE.0.01)THEN

S3SPAT=AMIN1(RI,1.0)

ELSE

S3SPAT=0.

ENDIF

ENDIF

!

s3ele(iel) = s3spat

!

!     s3 based on Peclet number

!

if(pemin.lt.1.0e-12) pemin = 0.0

!

IF(PEMIN.EQ.0.0)THEN

S3PSPAT=1.

ELSE

pratio = sqrt(pemax**2 - pemin**2)/pemin

IF(pratio.GE.0.01)THEN

S3PSPAT=AMIN1(pratio,1.0)

ELSE

S3PSPAT=0.

ENDIF

ENDIF

!

s3pec(iel) = s3pspat

!

if(iternum.gt.1) then

IPOIN = LNOD(1,IEL)

REMAXT = REYOLD(IPOIN)

REMINT = REMAXT

pemaxt = pecold(ipoin)

pemint = pemaxt

DO I= 2, NNODE

JPOIN = LNOD(i,IEL)

RE = REYOLD(JPOIN)

IF(RE.GE.REMAXT) REMAXT = RE

IF(RE.LE.REMINT) REMINT = RE

pe = pecold(jpoin)

if(pe.ge.pemaxt) pemaxt = pe

if(pe.le.pemint) pemint = pe

enddo

!

if(remint.lt.1.0e-12) remint = 0.0

if(pemint.lt.1.0e-12) pemint = 0.0

!

if(remax.gt.remaxt) remaxt = remax

if(remin.lt.remint) remint = remin

if(pemax.gt.pemaxt) pemaxt = pemax

if(pemin.lt.pemint) pemint = pemin

!

IF(REMINT.EQ.0.0)THEN

S3TIME=1.

ELSE

ri = sqrt(remaxt**2-remint**2)/remint

IF(RI.GE.0.01)THEN

S3TIME=AMIN1(RI,1.0)

ELSE

S3TIME=0.

ENDIF

ENDIF

!

IF(PEMINT.EQ.0.0)THEN

S3PTIME=1.

ELSE

pratio = sqrt(pemaxt**2 - pemint**2)/pemint

IF(pratio.GE.0.01)THEN

S3PTIME=AMIN1(pratio,1.0)

ELSE

S3PTIME=0.

ENDIF

ENDIF

!

if(s3time>s3spat) s3ele(iel) = s3time

if(s3ptime>s3pspat) s3pec(iel) = s3ptime

endif

!

Select Case(imppa)

Case(1)

S4ELE(IEL) = S3ELE(IEL)

s4pec(iel) = s3pec(iel)

Case(2)

S4ELE(IEL) = 1. - S3ELE(IEL)

s4pec(iel) = 1. - s3pec(iel)

Case(3)

S4ELE(IEL) = s4

s4pec(iel) = s4

Case(4)

S4ELE(IEL) = MAX(1.-S3ELE(IEL),0.5)

s4pec(iel) = max(1. - s3pec(iel),0.5)

Case(5)

s4ele(iel) = sqrt(1. - s3ele(iel)**2)

s4pec(iel) = sqrt(1. - s3pec(iel)**2)

Case(6)

s4ele(iel) = s3ele(iel)**pwr

s4pec(iel) = s3pec(iel)**pwr

Case(7)

s4ele(iel) = 0.5*(1. + s3ele(iel)**pwr)

s4pec(iel) = 0.5*(1. + s3pec(iel)**pwr)

end Select

enddo

!

else

do iel = 1,nelem

s3ele(iel) = 0.0

s4ele(iel) = 0.0

s3pec(iel) = 0.0

s4ele(iel) = 0.0

enddo

!

ENDIF

!

do ipoin = 1, npoin

emachold(ipoin) = em(ipoin)

if(nvisc.eq.1) then

reyold(ipoin) = reyno(ipoin)

pecold(ipoin) = pecno(ipoin)

endif

enddo

!

RETURN

end subroutine imparm

!

!  **************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE CONVECTIVE FLUX

!     AT A NODAL POINT (ONE-D)

!

SUBROUTINE CONVFLX1(NPOIN)

!

Use flow_var

Use Conv_flux

!

Implicit None

!

integer,intent(in) :: npoin

integer :: k

real :: r,uvel,p,e

!

DO K=1,NPOIN

!

R = DENSE(K)

UVEL = VEL(1,K)

P = PRESS(K)

E = ENERG(K)

!

F(K,1,1)=R*UVEL

F(K,1,2)=R*UVEL*UVEL + P

F(K,1,3)=R*E*UVEL + P*UVEL

!

enddo

!

RETURN

end subroutine CONVFLX1

!

!  **************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE CONVECTIVE FLUX

!     AT A NODAL POINT

!

SUBROUTINE CONVFLX(NPOIN)

!

Use flow_var

Use Conv_flux

!

Implicit None

!

integer,intent(in) :: npoin

integer :: k

real :: r,uvel,vvel,p,e

!

DO K=1,NPOIN

!

R = DENSE(K)

UVEL = VEL(1,K)

VVEL = VEL(2,K)

P = PRESS(K)

E = ENERG(K)

!

F(K,1,1)=R*UVEL

F(K,1,2)=R*UVEL*UVEL + P

F(K,1,3)=R*UVEL*VVEL

F(K,1,4)=R*E*UVEL + P*UVEL

!

F(K,2,1)=R*VVEL

F(K,2,2)=R*UVEL*VVEL

F(K,2,3)=R*VVEL*VVEL + P

F(K,2,4)=R*E*VVEL + P*VVEL

enddo

!

RETURN

end subroutine CONVFLX

!

!  ***************************************************************

!     SUBROUTINE TO BUILD EQUATIONS FOR MEIFEM METHOD

!

SUBROUTINE MEIEQNS(NELEM,NNODE,NFACE,NEQUA,NDIMN,nordr,neleq,DT,  &

&                   nvisc,IMPPA,nblock,adap,artvis,itime)

!

Use grid_info

Use flow_var

Use FDV_coef

!

Implicit None

!

integer,intent(in) :: nelem,nnode,nface,nequa,ndimn

integer,intent(in) :: nordr,nvisc,imppa,nblock,itime

integer,intent(in) :: neleq

real,intent(in) :: dt

integer :: ndbou

LOGICAL :: ADAP,ARTVIS

!

if(ndimn.eq.2) ndbou = 2

if(ndimn.eq.3) ndbou = 4

!

!     INITIALIZE FDV VARIABLES

!

rhs = 0.0                  ! zero array

delu = 0.0                 ! zero array

!

Select Case(ndimn)

Case(1)

call ONED_INGRL(NELEM,nblock,NEQUA,NNODE,NDIMN,neleq,nordr,&

&              NVISC,DT,IMPPA,adap,artvis)

!

Case(2)

!   Calculate Domain intergral FDV equations

CALL DOMINGRL(NELEM,nblock,NEQUA,NNODE,NDIMN,neleq,&

&                 nordr,NVISC,DT,IMPPA,adap,artvis)

!

!   Calculate Surface integral FDV equations

CALL BOUINGRL(nface,dt,nequa,ndimn,ndbou,neleq,nvisc,imppa)

!

Case(3)

!   Calculate Domain intergral FDV equations

CALL DOMINGRL3D(NELEM,nblock,NEQUA,NNODE,NDIMN,neleq,nordr, &

&                   NVISC,DT,IMPPA,adap,artvis)

!   Calculate Surface integral FDV equations

call bouingrl3d(nface,dt,nnode,nequa,ndimn,neleq,nvisc,     &

&                   ndbou,imppa,itime)

end Select

!

!     Apply boundary conditions

!

call bc (nnode,nordr,ndimn,ndbou,nequa,nface,neleq,nblock)

!

!

1000 format(' WARNING:  DIAGONAL AT EQUATION ',i8,' IS NEGATIVE')

1100 FORMAT(' NODE:  ',i8,' DIAGONAL = ',e16.8,' BOUNDARY CONDITION '  &

&         ,I8)

2005 format(' IORDR     RHS VECTOR     DIAGONAL ENTRY')

2010 format(i8,2(1x,e16.8))

!

RETURN

end subroutine meieqns

!

!  *****************************************************************

!   SUBROUTINE TO BUILD DOMAIN INTEGRALS (One Dimension)

!

SUBROUTINE ONED_INGRL(NELEM,nblock,NEQUA,NNODE,NDIMN,neleq,nordr,&

&                    NVISC,DT,IMPPA,adap,artvis)

!

Use grid_info

Use flow_var

Use thermo_propt

Use Gaus_quad

Use shap_fcns

Use FDV_coef

Use Conv_flux

Use var_parm

Use bnd_cnd

!

Implicit None

!

integer,intent(in) :: nelem,nequa,nnode,ndimn,neleq,nvisc,imppa

integer,intent(in) :: nordr,nblock

real,intent(in) :: dt

LOGICAL,intent(in) :: ADAP, artvis

!

real :: dt2ov2,reinv,re2inv

real :: e,T,vis,hk,vkk

real :: term1,term2,dfluxsum,fluxsum

real :: termb1,termb2

real :: aaba,abbb,detww,apb,pipj,dpixpj,dpixdpjx

integer :: inode,igaus,iel,koun,kequa,ieleq,jeleq

integer :: ig,idimn,jdimn,ir,is,nel,iordr,iequa

integer :: nel,mel,irow,icol

integer :: nal

integer :: ipoin,jpoin

integer :: ia,ja,jnode,jordr,kcol

integer,dimension(nnode) :: iel_nod

integer :: iblock,ieone,ie0,ielast,iv,nvec

!

real,dimension(ndimn,nnode) :: xloc,vloc

real,dimension(nnode) :: Eloc,Tloc

real :: Uloc(nnode,nequa),Floc(nnode,ndimn,nequa)

real :: DUNDX(ndimn,nEQUA),unkn(nequa),velo(ndimn)

real :: CFLUX(nDIMN,nEQUA),DFJDXJ(nEQUA)

real :: G(nDIMN,nEQUA),tau(ndimn,ndimn)

real :: dgjdxj(nequa)

real :: dudx(ndimn,ndimn),dtdx(ndimn)

real :: d2tdx2(ndimn),d2udx2(ndimn,ndimn,ndimn)

real :: e1(ndimn,nequa,nequa), e2(ndimn,ndimn,nequa,nequa)

real :: dshpdx(mdimn,mnode,mgaus),detjac(mgaus)

real :: d2phidx(mdimn,mdimn,mnode,mgaus)

!      real delx(mdimn),S(mdimn,mdimn,mdimn,mdimn)

!

real,dimension(ndimn,nequa,nequa) :: aj,bj

real,dimension(ndimn,ndimn,nequa,nequa) :: cj

real :: xjac_flux(nequa)

real :: aa(ndimn,ndimn,nequa,nequa)

real :: ab(ndimn,ndimn,nequa,nequa)

real :: ba(ndimn,ndimn,nequa,nequa)

real :: bb(ndimn,ndimn,nequa,nequa)

!      real dphidx(mdimn,mnode),d2shdx2(mdimn,mdimn,mnode)

!      real ht(mnode,mnode)

!      real hxi(mdimn)

real :: s1imp(nequa),s2imp(nequa),s3imp(nequa),s4imp(nequa)

!      real Fbar(mordr)

!      real psi(mnode)

!      real artdis(mordr)

!

DT2OV2 = DT*DT/2.

reinv = 1./creyn

re2inv = reinv/creyn

!

!      do kequa = 1, nordr

!         artdis(kequa) = 0.0

!      enddo

!

koun = 1

!

blkloop:  do iblock = 1,nblock

ieone = lblock(iblock)

ie0 = ieone - 1

ielast = lblock(iblock+1) - 1

nvec = lblock(iblock+1) - ieone

!

elemntlp:  DO iv = 1, nvec

!

iel = ie0 + iv

!

IF(IMPPA.GE.1) THEN

do kequa = 1,nequa

S1IMP(kequa) = S1ELE(IEL)

S2IMP(kequa) = S2ELE(IEL)

S3IMP(kequa) = S3ELE(IEL)

if(kequa.eq.nequa) s3imp(kequa) = s3pec(iel)

S4IMP(kequa) = S4ELE(IEL)

if(kequa.eq.nequa) s4imp(kequa) = s4pec(iel)

enddo

ELSE

do kequa = 1,nequa

S1IMP(kequa) = S1

S2IMP(kequa) = S2

S3IMP(kequa) = S3

S4IMP(kequa) = S4

enddo

ENDIF

!

do ieleq = 1,neleq

do jeleq = 1,neleq

bel(iel,ieleq,jeleq) = 0.0

enddo

enddo

!

do inode = 1, nnode

iel_nod(inode) = lnod(inode,iel)

ipoin = lnod(inode,iel)

Eloc(inode) = energ(ipoin)

Tloc(inode) = tempr(ipoin)

do idimn = 1,ndimn

xloc(idimn,inode) = x(idimn,ipoin)

vloc(idimn,inode) = vel(idimn,ipoin)

enddo

do iequa = 1,nequa

iordr = nequa*(ipoin-1) + iequa

uloc(inode,iequa) = u(iordr)

do idimn = 1,ndimn

Floc(inode,idimn,iequa) = F(ipoin,idimn,iequa)

enddo

enddo

enddo

!

call dersh1d(nnode,xloc,dpxi,dshpdx,detjac)

d2phidx = 0.0

!

!        if(koun.le.100) then

!           do ig = 1,mgaus

!           do idimn = 1,ndimn

!              write(26,105)(dshpdx(idimn,inode,ig),inode = 1,nnode)

!           enddo

!           enddo

!        endif

105 format(4(1x,e16.8))

!

koun = koun + 1

!

!  101 format(' Element No. ',i8)

!	 do igaus = 1, mgaus

!	    detjac(igaus) = detjac(igaus)*ww(igaus)

!	 enddo

!

!

gausloop:  DO IG=1,MGAUS

!

!         if(iel.lt.200) then

!         write(38,2001)iel,ig

2001 format(' Shape function derivatives for element ',i8,     &

&       ' at Gaussian point ',i4)

!         do kdimn = 1,ndimn

!            write(38,*)(dshpdx(kdimn,knode,ig),knode=1,nnode)

!         enddo

!         endif

!

CALL EVALDOM(NEQUA,NDIMN,NNODE,ig,vloc,Eloc,Tloc, &

&                    Uloc,Floc,phi,dshpdx,&

&                  d2phidx,VELO,E,T,unkn,DUDX,DTDX,DUNDX,CFLUX, &

&                  DFJDXJ,d2tdx2,d2udx2)

!

!

CALL AJACOB1(VELO,E,gamma,gamm1,gm3d2,gm1d2,gamm3,aj)

!

!     CALCULATE VISCOUS TERMS ASSOCIATED WITH THE NAVIER-STOKES

!     EQUATIONS

!

!     CALCULATE THE VISCOSITY USING SUTHERLAND'S RELATIONSHIP

!

IF(T.LT.1.E-12) THEN

VIS=1.0

ELSE

VIS = (1.0 +CSUTH)/(T + CSUTH)*T**1.5

ENDIF

!

HK = VIS/GM1M2PR

!

if(nvisc.eq.1) then

vkk = 0.0

do idimn = 1,ndimn

vkk = vkk + dudx(idimn,idimn)

enddo

!

do idimn = 1,ndimn

do jdimn = 1,ndimn

tau(idimn,jdimn) = vis*(dudx(idimn,jdimn) +     &

&                         dudx(jdimn,idimn))

if(idimn.eq.jdimn)    &

&         tau(idimn,jdimn) = tau(idimn,jdimn) - 2.*vis/3.*vkk

enddo

enddo

!

2000 format(' Element No. ',i6,' Gaussian Point ',i4,/,        &

&       ' Viscous Flux')

2025 format(5(e12.6,1x))

!

2010 format(' Element No. ',i6,' Gaussian Point ',i4,          &

&       ' Diffusion Jacobian')

2015 format(' Element No. ',i6,' Gaussian Point ',i4,          &

&       'Diffusion Gradient Jacobian')

!

endif

!

IF(NVISC.NE.1) THEN

!

do ir = 1,nequa

xjac_flux(ir) = 0.0

enddo

!

do is = 1,nequa

do ir = 1,nequa

xjac_flux(ir) = xjac_flux(ir) + aj(1,ir,is)*    &

&                                dfjdxj(is)

enddo

enddo

!

DO NAL = 1, NNODE

DO IR = 1, NEQUA

iordr=nequa*(iel_nod(nal)-1)+ir

!

TERM1= DSHPDX(1,NAL,ig)*CFLUX(1,IR)

TERM2 = xjac_flux(ir)*DSHPDX(1,NAL,ig)

!

RHS(iordr)=RHS(iordr) + (DT*TERM1          &

&                      -DT2OV2*TERM2)*DetJAC(ig)*ww(ig)

enddo

enddo

!

ELSE

!

do ir = 1,nequa

xjac_flux(ir) = 0.0

enddo

!

do is = 1,nequa

dfluxsum = dfjdxj(is) + dgjdxj(is)

do ir = 1,nequa

apb = aj(1,ir,is) + bj(1,ir,is)

xjac_flux(ir) = xjac_flux(ir) + apb*dfluxsum

enddo

enddo

!

DO NAL = 1, NNODE

DO IR = 1, NEQUA

!

iordr=nequa*(iel_nod(nal)-1)+ir

fluxsum = cflux(1,ir) + g(1,ir)

TERM1=DSHPDX(1,NAL,ig)*fluxsum

TERM2= xjac_flux(ir)*DSHPDX(1,NAL,ig)

!

RHS(iordr)=RHS(iordr) + (DT*TERM1           &

&                      -DT2OV2*TERM2)*DETJAC(ig)*ww(ig)

enddo

enddo

ENDIF

!

!     CALCULATE LHS MATRIX

!

call matxmult(ndimn,nequa,aj,aj,aa)

!

IF(NVISC.NE.1) THEN

!

do ir = 1,nequa

do is = 1,nequa

e2(1,1,ir,is) = -dt2ov2*s2imp(ir)*aa(1,1,ir,is)

e1(1,ir,is) = dt*s1imp(ir)*aj(1,ir,is)

enddo

enddo

!

else

!

call matxmult(ndimn,nequa,aj,bj,ab)

call matxmult(ndimn,nequa,bj,aj,ba)

call matxmult(ndimn,nequa,bj,bj,bb)

!

do ir = 1,nequa

do is = 1,nequa

aaba = aa(1,1,ir,is) + ba(1,1,ir,is)

abbb = ab(1,1,ir,is) + bb(1,1,ir,is)

e2(1,1,ir,is) = dt*s3imp(ir)*cj(1,1,ir,is)   &

&                               -dt2ov2*(s2imp(ir)*aaba           &

&                               +s4imp(ir)*abbb)

e1(1,ir,is) = dt*(s1imp(ir)*aj(1,ir,is)+     &

&                                  s3imp(ir)*bj(1,ir,is))

enddo

enddo

endif

!

detww = detjac(ig)*ww(ig)

!

DO NEL=1,NNODE

DO MEL=1,NNODE

!

pipj = phi(nel,ig)*phi(mel,ig)*detww

dpixpj = dshpdx(1,nel,ig)*phi(mel,ig)*detww

dpixdpjx = dshpdx(1,nel,ig)*dshpdx(1,mel,ig)*detww

!

DO IR=1,NEQUA

DO IS=1,NEQUA

!

TERMB1= -e1(1,ir,is)*dpixpj

TERMB2= -e2(1,1,ir,is)*dpixdpjx

!

irow = nequa*(nel-1) + ir

icol = nequa*(mel-1) + is

!

BEL(iel,irow,icol)=BEL(iel,irow,icol) +	      &

&                           (TERMB1+TERMB2)

IF(IR.EQ.IS)     &

&                 BEL(iel,irow,icol) = BEL(iel,irow,icol) + pipj

!

!

enddo

enddo

enddo

enddo

!

ENDDO gausloop

!

!     Apply boundary conditions

!

do inode = 1,nnode

ipoin = iel_nod(inode)

do ir = 1,nequa

iordr = nequa*(ipoin-1) + ir

do jnode = 1,nnode

jpoin = iel_nod(jnode)

do is = 1,nequa

jordr = nequa*(jpoin-1) + is

if(ibouncnd(jordr)<0) then

irow = nequa*(inode-1) + ir

icol = nequa*(jnode-1) + is

bel(iel,irow,icol) = 0.0

if(irow==icol) then

do kcol = 1,neleq

Bel(iel,irow,kcol) = 0.0   ! Zero column

enddo

Bel(iel,irow,icol) = 1.0

rhs(iordr) = 0.0

endif

endif

enddo

enddo

enddo

enddo

!

enddo elemntlp

!

enddo blkloop

!

RETURN

end subroutine oned_ingrl

!

!  ********************************************************************

!   Subroutine to calculate domain integrals for the FDV Equations

!

Subroutine DOMINGRL(NELEM,nblock,NEQUA,NNODE,NDIMN,neleq,nordr,&

&                    NVISC,DT,IMPPA,adap,artvis)

!

Use param_var

Use grid_info

Use flow_var

Use thermo_propt

Use Gaus_quad

Use shap_fcns

Use FDV_coef

Use Conv_flux

Use Var_parm

!

IMplicit None

!

integer,intent(in) :: nelem,nequa,nnode,ndimn,neleq,nvisc,imppa

integer,intent(in) :: nordr,nblock

real,intent(in) :: dt

LOGICAL,intent(in) :: ADAP, artvis

!

integer :: i,iblock,icol,idimn,iel,iel0,ielno,iequa

integer :: ig,igaus,inode,iordr,ipoin,ir,irow,is,iv

integer :: j,jdimn

integer :: nal,na,nb,nvec

integer,dimension(nnode) :: iel_nod

!

real :: dt2ov2,E,reinv,T,vis,vkk,hk

real,dimension(nequa) :: s1imp,s2imp,s3imp,s4imp

real,dimension(nnode,mgaus) :: shap

real,dimension(mgaus) :: detjac

real,dimension(nnode) :: Eloc,Tloc

real,dimension(nnode,ndimn,nequa) :: Floc

real,dimension(nnode,nequa) :: Uloc

real,dimension(ndimn,nnode) :: vloc,xloc

real,dimension(nequa) :: dfjdxj,dgjdxj,unkn

real,dimension(ndimn,nequa) :: cflux,dundx,G

real,dimension(ndimn) :: dtdx,d2tdx2,velo

real,dimension(ndimn,ndimn) :: dudx,tau

real,dimension(ndimn,ndimn,ndimn) :: d2udx2

real,dimension(ndimn,nnode,mgaus) :: dshpdx

real,dimension(ndimn,ndimn,nnode,mgaus) :: d2phidx

real,dimension(ndimn,nequa,nequa) :: aj,bj,e1

real,dimension(ndimn,ndimn,nequa,nequa) :: cj,aa,ba,ab,bb,e2

real :: detww,term1,term2

real :: aapba,abpbb,pipj,dpixpj,dpiypj,dpixdpjx,dpixdpjy

real :: dpiydpjx,dpiydpjy

real :: axpbx,aypby,dfluxsum,xfluxsum,yfluxsum

real,dimension(nequa) :: xjac_flux,yjac_flux

!

dt2ov2 = dt*dt/2.

reinv = 1./creyn

!

blocklp:  do iblock = 1,nblock

iel = lblock(iblock)

iel0 = iel - 1

nvec = lblock(iblock+1) - iel

!

eleloop: do iv = 1,nvec

ielno = iel0 + iv

!

!     Variation Parameters

!

do iequa = 1,nequa

if(imppa>0) then

s1imp(iequa) = s1ele(ielno)

s2imp(iequa) = s2ele(ielno)

s3imp(iequa) = s3ele(ielno)

s4imp(iequa) = s4ele(ielno)

else

s1imp(iequa) = s1

s2imp(iequa) = s2

s3imp(iequa) = s3

s4imp(iequa) = s4

endif

enddo

!    Zero element stiffness matrix

!

Bel(ielno,1:neleq,1:neleq) = 0.0

!

do inode = 1,nnode

iel_nod(inode) = lnod(inode,ielno)

ipoin = lnod(inode,ielno)

Eloc(inode) = energ(ipoin)

Tloc(inode) = tempr(ipoin)

!

do idimn = 1,ndimn

vloc(idimn,inode) = vel(idimn,ipoin)

xloc(idimn,inode) = x(idimn,iel_nod(inode))

do iequa = 1,nequa

Floc(inode,idimn,iequa) = F(ipoin,idimn,iequa)

enddo

enddo

!

do iequa = 1,nequa

iordr = nequa*(ipoin-1) + iequa

Uloc(inode,iequa) = U(iordr)

enddo

enddo

!

shap = phi

!

!

!    Calculate spatial derivatives of shape functions

!

CALL DERSHA(NNODE,ndimn,xloc,dpxi,d2phi,2,dshpdx,&

detjac,d2phidx)

!

!   Gaussian Quadrature loop

!

gausloop:  do ig = 1,mgaus

!

!   Calculate value of flow variables at Gaussian Point

!

call evaldom(nequa,ndimn,nnode,ig,vloc,Eloc,Tloc,Uloc,  &

&                  Floc,shap,dshpdx,d2phidx,velo,E,T,unkn,   &

&                  dudx,dtdx,dundx,cflux,dfjdxj,d2tdx2,d2udx2)

!

!   Calculate viscosity, thermal Conductivity, and stress tensor

!

if(T<1.0e-12) then

vis = 1.0

else

vis = (1.0 + csuth)/(T + csuth)*T**1.5

endif

!

hk = vis/gm1m2pr

!

vkk = 0.0

do idimn = 1,ndimn

vkk = vkk + dudx(idimn,idimn)

enddo

!

do idimn = 1,ndimn

do jdimn = 1,ndimn

tau(idimn,jdimn) = vis*(dudx(idimn,jdimn) +  &

dudx(jdimn,idimn))

if(idimn==jdimn)     &

tau(idimn,jdimn) = tau(idimn,jdimn) -2./3.*vis  &

&*vkk

enddo

enddo

!

!   Calculate Jacobians and Viscous Flux

!

call ajacob(velo,E,gamma,gamm1,gm3d2,gm1d2,gamm3,aj)

!

call viscflx(reinv,tau,hk,vis,dudx,dtdx,velo,d2tdx2, &

d2udx2,G,dgjdxj)

!

call bjacob(reinv,cv,vis,hk,tau,dundx,unkn,velo,E,bj)

!

call cjacob(reinv,cv,vis,hk,unkn,cj)

!

!     Calculate Right-Hand-Side Vector

!

detww = detjac(ig)*ww(ig)

!

do ir = 1,nequa

xjac_flux(ir) = 0.0

yjac_flux(ir) = 0.0

enddo

!

do is = 1,nequa

dfluxsum = dfjdxj(is) + dgjdxj(is)

do ir = 1,nequa

axpbx = aj(1,ir,is) + bj(1,ir,is)

aypby = aj(2,ir,is) + bj(2,ir,is)

xjac_flux(ir) = xjac_flux(ir) + axpbx*dfluxsum

yjac_flux(ir) = yjac_flux(ir) + aypby*dfluxsum

enddo

enddo

!

do nal = 1,nnode

ipoin = iel_nod(nal)

do ir = 1,nequa

iordr = nequa*(ipoin-1) + ir

!

xfluxsum = cflux(1,ir) + G(1,ir)

yfluxsum = cflux(2,ir) + G(2,ir)

term1 = xfluxsum*dshpdx(1,nal,ig) + yfluxsum*   &

&dshpdx(2,nal,ig)

term2 = xjac_flux(ir)*dshpdx(1,nal,ig) +        &

yjac_flux(ir)*dshpdx(2,nal,ig)

!

rhs(iordr) = rhs(iordr) + (dt*term1 - dt2ov2*term2) &

*detww

enddo

enddo

!

!    Calculate LHS Matrix

!

call matxmult(ndimn,nequa,aj,aj,aa)

call matxmult(ndimn,nequa,bj,aj,ba)

call matxmult(ndimn,nequa,aj,bj,ab)

call matxmult(ndimn,nequa,bj,bj,bb)

!

do i = 1,ndimn

do ir = 1,nequa

do is = 1,nequa

e1(i,ir,is) = dt*(s1imp(ir)*aj(i,ir,is)   &

+ s3imp(ir)*bj(i,ir,is))

aapba = aa(i,1,ir,is) + ba(i,1,ir,is)

abpbb = ab(i,1,ir,is) + bb(i,1,ir,is)

e2(i,1,ir,is) = dt*s3imp(ir)*cj(i,1,ir,is) &

- dt2ov2*(s2imp(ir)*aapba  &

+ s4imp(ir)*abpbb)

!

aapba = aa(i,2,ir,is) + ba(i,2,ir,is)

abpbb = ab(i,2,ir,is) + bb(i,2,ir,is)

e2(i,2,ir,is) = dt*s3imp(ir)*cj(i,2,ir,is) &

- dt2ov2*(s2imp(ir)*aapba  &

+ s4imp(ir)*abpbb)

enddo

enddo

enddo

!

do na = 1,nnode

do nb = 1,nnode

pipj = shap(na,ig)*shap(nb,ig)*detww

dpixpj = dshpdx(1,na,ig)*shap(nb,ig)*detww

dpiypj = dshpdx(2,na,ig)*shap(nb,ig)*detww

dpixdpjx = dshpdx(1,na,ig)*dshpdx(1,nb,ig)*detww

dpixdpjy = dshpdx(1,na,ig)*dshpdx(2,nb,ig)*detww

dpiydpjx = dshpdx(2,na,ig)*dshpdx(1,nb,ig)*detww

dpiydpjy = dshpdx(2,na,ig)*dshpdx(2,nb,ig)*detww

!

do ir = 1,nequa

do is = 1,nequa

irow = nequa*(na-1) + ir

icol = nequa*(nb-1) + is

!

term1 = e1(1,ir,is)*dpixpj + e1(2,ir,is)* &

dpiypj

term2 = e2(1,1,ir,is)*dpixdpjx +   &

e2(1,2,ir,is)*dpixdpjy +   &

e2(2,1,ir,is)*dpiydpjx +   &

e2(2,2,ir,is)*dpiydpjy

!

Bel(ielno,irow,icol) = Bel(ielno,irow,icol) &

&- term1 - term2

if(ir==is)   &

Bel(ielno,irow,icol) = Bel(ielno,irow,icol) &

+ pipj

!

enddo

enddo

enddo

enddo

!

enddo gausloop

enddo eleloop

!

enddo blocklp

!

return

end subroutine domingrl

!

!  ********************************************************************

!   Subroutine to calculate domain integrals for the FDV Equations

!

Subroutine DOMINGRL3D(NELEM,nblock,NEQUA,NNODE,NDIMN,neleq,nordr,&

&                    NVISC,DT,IMPPA,adap,artvis)

!

Use param_var

Use grid_info

Use flow_var

Use thermo_propt

Use Gaus_quad

Use shap_fcns

Use Shap3d_fcns

Use FDV_coef

Use Conv_flux

Use Var_parm

!

IMplicit None

!

integer,intent(in) :: nelem,nequa,nnode,ndimn,neleq,nvisc,imppa

integer,intent(in) :: nordr,nblock

real,intent(in) :: dt

LOGICAL,intent(in) :: ADAP, artvis

!

integer :: i,iblock,icol,idimn,iel,iel0,ielno,iequa

integer :: ig,igaus,inode,iordr,ipoin,ir,irow,is,iv

integer :: j,jdimn

integer :: nal,na,nb,nvec

integer,dimension(nnode) :: iel_nod

!

real :: dt2ov2,E,reinv,T,vis,vkk,hk

real,dimension(nequa) :: s1imp,s2imp,s3imp,s4imp

real,dimension(nnode,mgaus) :: shap

real,dimension(mgaus) :: detjac

real,dimension(nnode) :: Eloc,Tloc

real,dimension(nnode,ndimn,nequa) :: Floc

real,dimension(nnode,nequa) :: Uloc

real,dimension(ndimn,nnode) :: vloc,xloc

real,dimension(nequa) :: dfjdxj,dgjdxj,unkn

real,dimension(ndimn,nequa) :: cflux,dundx,G

real,dimension(ndimn) :: dtdx,d2tdx2,velo

real,dimension(ndimn,ndimn) :: dudx,tau

real,dimension(ndimn,ndimn,ndimn) :: d2udx2

real,dimension(ndimn,nnode,mgaus) :: dshpdx

real,dimension(ndimn,ndimn,nnode,mgaus) :: d2phidx

real,dimension(ndimn,nequa,nequa) :: aj,bj,e1

real,dimension(ndimn,ndimn,nequa,nequa) :: cj,aa,ba,ab,bb,e2

real :: detww,term1,term2

real :: aapba,abpbb,pipj,dpixpj,dpiypj,dpizpj,dpixdpjx,dpixdpjy

real :: dpixdpjz,dpiydpjx,dpiydpjy,dpiydpjz,dpizdpjx,dpizdpjy

real :: dpizdpjz

real :: axpbx,aypby,azpbz,dfluxsum,xfluxsum,yfluxsum,zfluxsum

real,dimension(nequa) :: xjac_flux,yjac_flux,zjac_flux

!

dt2ov2 = dt*dt/2.

reinv = 1./creyn

!

blocklp:  do iblock = 1,nblock

iel = lblock(iblock)

iel0 = iel - 1

nvec = lblock(iblock+1) - iel

!

eleloop: do iv = 1,nvec

ielno = iel0 + iv

!

!     Variation Parameters

!

do iequa = 1,nequa

if(imppa>0) then

s1imp(iequa) = s1ele(ielno)

s2imp(iequa) = s2ele(ielno)

s3imp(iequa) = s3ele(ielno)

s4imp(iequa) = s4ele(ielno)

else

s1imp(iequa) = s1

s2imp(iequa) = s2

s3imp(iequa) = s3

s4imp(iequa) = s4

endif

enddo

!    Zero element stiffness matrix

!

Bel(ielno,1:neleq,1:neleq) = 0.0

!

do inode = 1,nnode

iel_nod(inode) = lnod(inode,ielno)

ipoin = lnod(inode,ielno)

Eloc(inode) = energ(ipoin)

Tloc(inode) = tempr(ipoin)

!

do idimn = 1,ndimn

vloc(idimn,inode) = vel(idimn,ipoin)

xloc(idimn,inode) = x(idimn,iel_nod(inode))

do iequa = 1,nequa

Floc(inode,idimn,iequa) = F(ipoin,idimn,iequa)

enddo

enddo

!

do iequa = 1,nequa

iordr = nequa*(ipoin-1) + iequa

Uloc(inode,iequa) = U(iordr)

enddo

enddo

!

shap = phi

!

!    Calculate spatial derivatives of shape functions

!

call dersha_3d(nnode,ndimn,xloc,dpxi,d2phidxde,d2phidxdz, &

&                   d2phidedz,1,0,dshpdx,detjac,d2phidx)

!

!

!   Gaussian Quadrature loop

!

gausloop:  do ig = 1,mgaus

!

!   Calculate value of flow variables at Gaussian Point

!

call evaldom(nequa,ndimn,nnode,ig,vloc,Eloc,Tloc,Uloc,  &

&                  Floc,shap,dshpdx,d2phidx,velo,E,T,unkn,   &

&                  dudx,dtdx,dundx,cflux,dfjdxj,d2tdx2,d2udx2)

!

!   Calculate Convection Jacobian

!

call ajacob_3d(velo,E,gamma,gamm1,gm3d2,gm1d2,gamm3,aj)

!

!   For Viscous Flow,

!       Calculate viscosity, thermal Conductivity, and stress tensor

!

if(nvisc==1) then

if(T<1.0e-12) then

vis = 1.0

else

vis = (1.0 + csuth)/(T + csuth)*T**1.5

endif

!

hk = vis/gm1m2pr

!

vkk = 0.0

do idimn = 1,ndimn

vkk = vkk + dudx(idimn,idimn)

enddo

!

do idimn = 1,ndimn

do jdimn = 1,ndimn

tau(idimn,jdimn) = vis*(dudx(idimn,jdimn) +  &

dudx(jdimn,idimn))

if(idimn==jdimn)     &

tau(idimn,jdimn) = tau(idimn,jdimn) -2./3.*vis  &

&*vkk

enddo

enddo

!   Calculate Viscous Flux and Diffusion Jacobians

!

call viscflx_3d(reinv,tau,hk,vis,dudx,dtdx,velo,  &

d2tdx2,d2udx2,G,dgjdxj)

!

call bjacob_3d(reinv,cv,vis,hk,tau,dundx,unkn,velo,  &

E,bj)

!

call cjacob_3d(reinv,cv,vis,hk,unkn,cj)

!

endif

!

!     Calculate Right-Hand-Side Vector

!

detww = detjac(ig)*ww(ig)

!

do ir = 1,nequa

xjac_flux(ir) = 0.0

yjac_flux(ir) = 0.0

zjac_flux(ir) = 0.0

enddo

!

if(nvisc/=1) then

do is = 1,nequa

do ir = 1,nequa

xjac_flux(ir) = xjac_flux(ir) + aj(1,ir,is)*dfjdxj(is)

yjac_flux(ir) = yjac_flux(ir) + aj(2,ir,is)*dfjdxj(is)

zjac_flux(ir) = zjac_flux(ir) + aj(3,ir,is)*dfjdxj(is)

enddo

enddo

!

do nal = 1,nnode

ipoin = iel_nod(nal)

do ir = 1,nequa

iordr = nequa*(ipoin-1) + ir

term1 = cflux(1,ir)*dshpdx(1,nal,ig) + &

cflux(2,ir)*dshpdx(2,nal,ig) + &

cflux(3,ir)*dshpdx(3,nal,ig)

term2 = xjac_flux(ir)*dshpdx(1,nal,ig) +        &

yjac_flux(ir)*dshpdx(2,nal,ig) +        &

zjac_flux(ir)*dshpdx(3,nal,ig)

!

rhs(iordr) = rhs(iordr) + (dt*term1 -  &

dt2ov2*term2)*detww

enddo

enddo

!

else

!

do is = 1,nequa

dfluxsum = dfjdxj(is) + dgjdxj(is)

do ir = 1,nequa

axpbx = aj(1,ir,is) + bj(1,ir,is)

aypby = aj(2,ir,is) + bj(2,ir,is)

azpbz = aj(3,ir,is) + bj(3,ir,is)

xjac_flux(ir) = xjac_flux(ir) + axpbx*dfluxsum

yjac_flux(ir) = yjac_flux(ir) + aypby*dfluxsum

zjac_flux(ir) = zjac_flux(ir) + azpbz*dfluxsum

enddo

enddo

!

do nal = 1,nnode

ipoin = iel_nod(nal)

do ir = 1,nequa

iordr = nequa*(ipoin-1) + ir

!

xfluxsum = cflux(1,ir) + G(1,ir)

yfluxsum = cflux(2,ir) + G(2,ir)

zfluxsum = cflux(3,ir) + G(3,ir)

term1 = xfluxsum*dshpdx(1,nal,ig) + yfluxsum*   &

&dshpdx(2,nal,ig) + zfluxsum*dshpdx(3,nal,ig)

term2 = xjac_flux(ir)*dshpdx(1,nal,ig) +        &

yjac_flux(ir)*dshpdx(2,nal,ig) +        &

zjac_flux(ir)*dshpdx(3,nal,ig)

!

rhs(iordr) = rhs(iordr) + (dt*term1 - dt2ov2*term2) &

*detww

enddo

enddo

endif

!

!    Calculate LHS Matrix

!

call matxmult(ndimn,nequa,aj,aj,aa)

!

if(nvisc/=1) then

do i = 1,ndimn

do ir = 1,nequa

do is = 1,nequa

e1(i,ir,is) = dt*s1imp(ir)*aj(i,ir,is)

e2(i,1,ir,is) = -dt2ov2*s2imp(ir)*aa(i,1,ir,is)

e2(i,2,ir,is) = -dt2ov2*s2imp(ir)*aa(i,2,ir,is)

e2(i,3,ir,is) = -dt2ov2*s2imp(ir)*aa(i,3,ir,is)

enddo

enddo

enddo

!

else

call matxmult(ndimn,nequa,bj,aj,ba)

call matxmult(ndimn,nequa,aj,bj,ab)

call matxmult(ndimn,nequa,bj,bj,bb)

!

do i = 1,ndimn

do ir = 1,nequa

do is = 1,nequa

e1(i,ir,is) = dt*(s1imp(ir)*aj(i,ir,is)   &

+ s3imp(ir)*bj(i,ir,is))

aapba = aa(i,1,ir,is) + ba(i,1,ir,is)

abpbb = ab(i,1,ir,is) + bb(i,1,ir,is)

e2(i,1,ir,is) = dt*s3imp(ir)*cj(i,1,ir,is) &

- dt2ov2*(s2imp(ir)*aapba  &

+ s4imp(ir)*abpbb)

!

aapba = aa(i,2,ir,is) + ba(i,2,ir,is)

abpbb = ab(i,2,ir,is) + bb(i,2,ir,is)

e2(i,2,ir,is) = dt*s3imp(ir)*cj(i,2,ir,is) &

- dt2ov2*(s2imp(ir)*aapba  &

+ s4imp(ir)*abpbb)

!

aapba = aa(i,3,ir,is) + ba(i,3,ir,is)

abpbb = ab(i,3,ir,is) + bb(i,3,ir,is)

e2(i,3,ir,is) = dt*s3imp(ir)*cj(i,3,ir,is) &

- dt2ov2*(s2imp(ir)*aapba  &

+ s4imp(ir)*abpbb)

enddo

enddo

enddo

endif

!

do na = 1,nnode

do nb = 1,nnode

pipj = shap(na,ig)*shap(nb,ig)*detww

dpixpj = dshpdx(1,na,ig)*shap(nb,ig)*detww

dpiypj = dshpdx(2,na,ig)*shap(nb,ig)*detww

dpizpj = dshpdx(3,na,ig)*shap(nb,ig)*detww

dpixdpjx = dshpdx(1,na,ig)*dshpdx(1,nb,ig)*detww

dpixdpjy = dshpdx(1,na,ig)*dshpdx(2,nb,ig)*detww

dpixdpjz = dshpdx(1,na,ig)*dshpdx(3,nb,ig)*detww

dpiydpjx = dshpdx(2,na,ig)*dshpdx(1,nb,ig)*detww

dpiydpjy = dshpdx(2,na,ig)*dshpdx(2,nb,ig)*detww

dpiydpjz = dshpdx(2,na,ig)*dshpdx(3,nb,ig)*detww

dpizdpjx = dshpdx(3,na,ig)*dshpdx(1,nb,ig)*detww

dpizdpjy = dshpdx(3,na,ig)*dshpdx(2,nb,ig)*detww

dpizdpjz = dshpdx(3,na,ig)*dshpdx(3,nb,ig)*detww

!

do ir = 1,nequa

do is = 1,nequa

irow = nequa*(na-1) + ir

icol = nequa*(nb-1) + is

!

term1 = e1(1,ir,is)*dpixpj + e1(2,ir,is)* &

dpiypj + e1(3,ir,is)*dpizpj

term2 = e2(1,1,ir,is)*dpixdpjx +   &

e2(1,2,ir,is)*dpixdpjy +   &

e2(1,3,ir,is)*dpixdpjz +   &

e2(2,1,ir,is)*dpiydpjx +   &

e2(2,2,ir,is)*dpiydpjy +   &

e2(2,3,ir,is)*dpiydpjz +   &

e2(3,1,ir,is)*dpizdpjx +   &

e2(3,2,ir,is)*dpizdpjy +   &

e2(3,3,ir,is)*dpizdpjz

!

Bel(ielno,irow,icol) = Bel(ielno,irow,icol) &

&- term1 - term2

if(ir==is)   &

Bel(ielno,irow,icol) = Bel(ielno,irow,icol) &

+ pipj

!

enddo

enddo

enddo

enddo

!

enddo gausloop

enddo eleloop

!

enddo blocklp

!

return

end subroutine domingrl3d

!

!    ******************************************************************

!    Subroutine to evaluate the boundary integrals for a three-D problem

SUBROUTINE BOUINGRL3d(NFACE,DT,nnode,NEQUA,NDIMN,neleq,NVISC,  &

&                      ndbou,IMPPA,itime)

!

Use param_var

Use thermo_propt

Use grid_info

Use flow_var

Use Conv_flux

Use Bface_info

Use Bnd_cnd

Use Gaus_quad

Use shapbou_3d

Use FDV_coef

Use Var_parm

!

Implicit None

!

integer,intent(in) :: ndbou

!

integer,intent(in) :: nface,nnode,nequa,ndimn,neleq,ndbou

integer,intent(in) :: nvisc,imppa,itime

real,intent(in) :: dt

!

logical :: adiabatic

integer :: iface,ielno,ibndcnd,ifacdir,iequa,inode,ig

integer :: idimn,jdimn,ipoin,iordr,i,ir,is,j,na,nb

integer :: ia,ib,irow,icol

integer,dimension(ndbou) :: iel_nod

!

real :: dt2ov2,E,reinv,T

real,dimension(nequa) :: s1imp,s2imp,s3imp,s4imp

real,dimension(ndimn) :: dtdx,d2tdx2,enorm,velo

real,dimension(ndimn,ngsurf) :: en

real,dimension(nequa) :: dgjdxj,unkn

real,dimension(ndimn,ndimn) :: dudx,tau

real,dimension(ndimn,ndimn,ndimn) :: d2udx2

real,dimension(ndimn,nequa) :: cflux,dfxdx,dundx,G

real :: hk,vis,vkk

real,dimension(ndimn,nequa,nequa) :: aj,bj,e1

real,dimension(ndimn,ndimn,nequa,nequa) :: cj,e2,aa,ab,ba,bb

real :: aapba,abpbb,dfluxsum,term1,term2

real :: pipj,pidpjx,pidpjy,pidpjz

real,dimension(nequa) :: xjac_flux,yjac_flux,zjac_flux

!

dt2ov2 = dt*dt/2.

reinv = 1./creyn

!

faceloop: do iface = 1,nface

ielno = lface(7,iface)   !  Element containing boundary face

ibndcnd = lface(8,iface) !  Boundary type of face

ifacdir = lface(6,iface) !  Direction of unit normal to face

!

!    Element Variation Parameters

!

do iequa = 1,nequa

if(imppa>0) then

s1imp(iequa) = s1ele(ielno)

s2imp(iequa) = s2ele(ielno)

s3imp(iequa) = s3ele(ielno)

s4imp(iequa) = s4ele(ielno)

else

s1imp(iequa) = s1

s2imp(iequa) = s2

s3imp(iequa) = s3

s4imp(iequa) = s4

endif

enddo

!

do inode = 1,ndbou

iel_nod(inode) = lface(inode,iface)

enddo

!

!   Calculate spatial derivatives of shape functions and normal vector

!

call calnorm3(ndimn,ndbou,ifacdir,iel_nod,enorm,en)

!

!    Gaussian Quadrature loop

!

gausloop:  do ig = 1,ngsurf

!

!     Calculate value of flow variables at Gaussian point

!

call EVALBOU3D(NDIMN,NEQUA,iel_nod,dshpdx,IG,VELO,   &

&                   E,T,unkn,DUDX,dtdx,dundx,cflux,dfxdx,&

&                   d2tdx2,d2udx2)

!

!    Apply boundary conditions

!

call aplbndcnd(ndimn,nequa,ndbou,iface,ielno,ibndcnd,  &

&iel_nod,ig,adiabatic,E,T,unkn,velo,dtdx,&

&d2tdx2,dudx,d2udx2,dundx,dfxdx,enorm)

!

!    Calculate Convective Jacobian

!

call ajacob_3d(velo,E,gamma,gamm1,gm3d2,gm1d2,gamm3,aj)

!

!    For Viscous Flow, Calculate viscosity, thermal conductivity

!        and stress tensor

if(nvisc==1) then

if(T<1.0e-12) then

vis = 1.0

else

vis = (1.0 + csuth)/(T + csuth)*T**1.5

endif

!

hk = vis/gm1m2pr

!

vkk = 0.0

do idimn = 1,ndimn

vkk = vkk + dudx(idimn,idimn)

enddo

!

do idimn = 1,ndimn

do jdimn = 1,ndimn

tau(idimn,jdimn) = vis*(dudx(idimn,jdimn) +  &

dudx(jdimn,idimn))

if(idimn==jdimn)     &

tau(idimn,jdimn) = tau(idimn,jdimn) -2./3.*vis  &

&*vkk

enddo

enddo

!

!   Calculate viscous flux and diffusion Jacobians

call viscflx_3d(reinv,tau,hk,vis,dudx,dtdx,velo,  &

d2tdx2,d2udx2,G,dgjdxj)

call bjacob_3d(reinv,cv,vis,hk,tau,dundx,unkn,velo,E,bj)

call cjacob_3d(reinv,cv,vis,hk,unkn,cj)

endif

!

!    Apply boundary conditions to Jacobians

!

call jacbndcnd(ndimn,nequa,ibndcnd,adiabatic,hk,vis,unkn,  &

&dundx,E,reinv,tau,velo,aj,bj,cj)

!

!    Calculate Right-Hand-Side Vector

!

do ir = 1,nequa

xjac_flux(ir) = 0.0

yjac_flux(ir) = 0.0

zjac_flux(ir) = 0.0

enddo

!

if(nvisc/=1) then

!

do is = 1,nequa

dfluxsum = dfxdx(1,is) + dfxdx(2,is) + dfxdx(3,is)

do ir = 1,nequa

xjac_flux(ir) = xjac_flux(ir) + aj(1,ir,is)*dfluxsum

yjac_flux(ir) = yjac_flux(ir) + aj(2,ir,is)*dfluxsum

zjac_flux(ir) = zjac_flux(ir) + aj(3,ir,is)*dfluxsum

enddo

enddo

!

do na = 1,ndbou

ipoin = iel_nod(na)

do ir = 1,nequa

iordr = nequa*(ipoin-1) + ir

!

term1 = cflux(1,ir)*en(1,ig) + cflux(2,ir)*en(2,ig) + &

cflux(3,ir)*en(3,ig)

term2 = xjac_flux(ir)*en(1,ig) + yjac_flux(ir)*&

&en(2,ig) + zjac_flux(ir)*en(3,ig)

!

rhs(iordr) = rhs(iordr) + (-dt*term1 + dt2ov2*term2) &

&*phi(na,ig)

enddo

enddo

!

else

!

do is = 1,nequa

dfluxsum = dfxdx(1,is) + dfxdx(2,is) + dfxdx(3,is) + &

dgjdxj(is)

do ir = 1,nequa

xjac_flux(ir) = xjac_flux(ir) + (aj(1,ir,is) +    &

bj(1,ir,is))*dfluxsum

yjac_flux(ir) = yjac_flux(ir) + (aj(2,ir,is) +    &

bj(2,ir,is))*dfluxsum

zjac_flux(ir) = zjac_flux(ir) + (aj(3,ir,is) +    &

bj(3,ir,is))*dfluxsum

enddo

enddo

!

do na = 1,ndbou

ipoin = iel_nod(na)

do ir = 1,nequa

iordr = nequa*(ipoin-1) + ir

!

term1 = (cflux(1,ir) + G(1,ir))*en(1,ig) +     &

(cflux(2,ir) + G(2,ir))*en(2,ig) +     &

(cflux(3,ir) + G(3,ir))*en(3,ig)

term2 = xjac_flux(ir)*en(1,ig) + yjac_flux(ir)*&

&en(2,ig) + zjac_flux(ir)*en(3,ig)

!

rhs(iordr) = rhs(iordr) + (-dt*term1 + dt2ov2*term2) &

&*phi(na,ig)

enddo

enddo

endif

!

if(ibndcnd<0) cycle  !  No boundary conditions applied to

!  LHS for inflow

!

!  Calculate LHS Matrix

!

call matxmult(ndimn,nequa,aj,aj,aa)

if(nvisc/=1) then

!

do i = 1,ndimn

do ir = 1,nequa

do is = 1,nequa

e1(i,ir,is) = dt*s1imp(ir)*aj(i,ir,is)

e2(i,1,ir,is) = -dt2ov2*s2imp(ir)*aa(i,1,ir,is)

e2(i,2,ir,is) = -dt2ov2*s2imp(ir)*aa(i,2,ir,is)

e2(i,3,ir,is) = -dt2ov2*s2imp(ir)*aa(i,3,ir,is)

enddo

enddo

enddo

!

else

!

call matxmult(ndimn,nequa,bj,aj,ba)

call matxmult(ndimn,nequa,aj,bj,ab)

call matxmult(ndimn,nequa,bj,bj,bb)

!

do i = 1,ndimn

do ir = 1,nequa

do is = 1,nequa

e1(i,ir,is) = dt*(s1imp(ir)*aj(i,ir,is)   &

+ s3imp(ir)*bj(i,ir,is))

aapba = aa(i,1,ir,is) + ba(i,1,ir,is)

abpbb = ab(i,1,ir,is) + bb(i,1,ir,is)

e2(i,1,ir,is) = dt*s3imp(ir)*cj(i,1,ir,is) &

- dt2ov2*(s2imp(ir)*aapba  &

+ s4imp(ir)*abpbb)

aapba = aa(i,2,ir,is) + ba(i,2,ir,is)

abpbb = ab(i,2,ir,is) + bb(i,2,ir,is)

e2(i,2,ir,is) = dt*s3imp(ir)*cj(i,2,ir,is) &

- dt2ov2*(s2imp(ir)*aapba  &

+ s4imp(ir)*abpbb)

aapba = aa(i,3,ir,is) + ba(i,3,ir,is)

abpbb = ab(i,3,ir,is) + bb(i,3,ir,is)

e2(i,3,ir,is) = dt*s3imp(ir)*cj(i,3,ir,is) &

- dt2ov2*(s2imp(ir)*aapba  &

+ s4imp(ir)*abpbb)

enddo

enddo

enddo

endif

!

do na = 1,ndbou

ia = lfacnode(na,iface)

do nb = 1,ndbou

ib = lfacnode(nb,iface)

pipj = phi(na,ig)*phi(nb,ig)

pidpjx = phi(na,ig)*dshpdx(1,nb,ig)

pidpjy = phi(na,ig)*dshpdx(2,nb,ig)

pidpjz = phi(na,ig)*dshpdx(3,nb,ig)

do ir = 1,nequa

irow = nequa*(ia-1) + ir

do is = 1,nequa

icol = nequa*(ib-1) + is

!

term1 = (e1(1,ir,is)*en(1,ig) + e1(2,ir,is)* &

en(2,ig) + e1(3,ir,is)*en(3,ig))*pipj

term2 = (e2(1,1,ir,is)*en(1,ig) + e2(2,1,ir,is)*&

en(2,ig) + e2(3,1,ir,is)*en(3,ig))* &

pidpjx +(e2(1,2,ir,is)*en(1,ig) +   &

e2(2,2,ir,is)*en(2,ig) + e2(3,2,ir,is)&

*en(3,ig))*pidpjy + (e2(1,3,ir,is)* &

en(1,ig) + e2(2,3,ir,is)*en(2,ig) + &

e2(3,3,ir,is)*en(3,ig))*pidpjz

!

Bel(ielno,irow,icol) = Bel(ielno,irow,icol) &

& + term1 + term2

enddo

enddo

enddo

enddo

!

enddo gausloop

enddo faceloop

!

return

end subroutine bouingrl3d

!

!     ************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES SURFACE INTEGRAL "N SUB

!     ALPHA R AT TIME STEP N"

!

SUBROUTINE BOUINGRL(NFACE,DT,NEQUA,NDIMN,ndbou,neleq,NVISC,IMPPA)

!

Use param_var

Use thermo_propt

Use grid_info

Use flow_var

Use Conv_flux

Use var_parm

Use Gaus_quad

Use Shapbou_2d,phi=>phibou

Use FDV_coef

Use bnd_cnd

Use Bface_info

!

Implicit None

!

integer,intent(in) :: ndbou,nface,nequa,ndimn,neleq,nvisc,imppa

real,intent(in) :: dt

real :: dt2ov2,reinv,re2inv

integer :: ibndcnd,iface,ielem,iequa,jequa,ipp1,ig

integer :: ipoin,inode,iordr,jordr,idimn,jdimn,iboun

integer :: kequa,lnode,lnod1,jpoin

real :: E,T

real :: dy,x1,y1,tan_a

real :: dprdy,u1,v1,t1,p1,P

real :: cos_a,sin_a,en1,en2,tan_n

real :: enum,denom,dprdx

real :: rh1,re1,dx

integer :: ir,is,nal,i

integer,dimension(mnode) :: iel_nod

real :: dfluxsum,term1,term2

real :: vis,hk,vkk,axpbx,aypby

real :: xfluxsum,yfluxsum,ckor2cv,cvisr

integer :: mb,ia,ib,irow,icol

real :: pipj,dpixpj,dpiypj,e1n,termb1,termb2

real :: aaba,abbb

real,dimension(ndimn,nequa,nequa) :: aj,bj

real,dimension(ndimn,ndimn,nequa,nequa) :: cj

!

real :: DGJDXJ(nequa)

real :: DPIDX(nDIMN,2,NGAUS),EN(2,NGAUS)

REAL :: DUNDX(nDIMN,nEQUA),unkn(nequa),velo(ndimn)

REAL :: CFLUX(NDIMN,NEQUA),DFXDX(NDIMN,NEQUA)

REAL :: G(NDIMN,NEQUA),tau(ndimn,ndimn)

REAL :: DUDX(NDIMN,NDIMN),DTDX(NDIMN)

real :: d2tdx2(ndimn),d2udx2(ndimn,ndimn,ndimn)

REAL :: dfxdy(nequa)

REAL :: e1(ndimn,nequa,nequa), e2(ndimn,ndimn,nequa,nequa)

real :: s1imp(nequa),s2imp(nequa),s3imp(nequa),s4imp(nequa)

!

real :: xjac_flux(nequa),yjac_flux(nequa)

real :: aa(ndimn,ndimn,nequa,nequa)

real :: ab(ndimn,ndimn,nequa,nequa)

real :: ba(ndimn,ndimn,nequa,nequa)

real :: bb(ndimn,ndimn,nequa,nequa)

!

logical :: adiabatic

!

DT2OV2 = DT*DT/2.

reinv = 1./creyn

re2inv = reinv/creyn

!

ifaceloop: DO IFACE = 1, NFACE

!

IELEM=LFACE(7,IFACE)

ibndcnd=LFACE(8,IFACE)

!

IF(IMPPA.GE.1) THEN

do kequa = 1,nequa

S1IMP(kequa) = S1ELE(IELEM)

S2IMP(kequa) = S2ELE(IELEM)

S3IMP(kequa) = S3ELE(IELEM)

if(kequa.eq.nequa) s3imp(kequa) = s3pec(ielem)

S4IMP(kequa) = S4ELE(IELEM)

if(kequa.eq.nequa) s4imp(kequa) = s4pec(ielem)

enddo

ELSE

do kequa=1,nequa

S1IMP(kequa) = S1

S2IMP(kequa) = S2

S3IMP(kequa) = S3

S4IMP(kequa) = S4

enddo

ENDIF

!

DO INODE = 1, 2

iel_nod(INODE) = LFACE(inode,IFACE)

enddo

!

CALL CALNORM(iel_nod,DPIDX,W,EN)

!

gausloop: DO IG=1,NGAUS

!

CALL EVALBOU(NDIMN,NEQUA,DPIDX,IG,iel_nod,VELO,      &

&                   E,T,Unkn,DUDX,DTDX,DUNDX,CFLUX,DFXDX,   &

&                   d2tdx2,d2udx2)

!

CALL AJACOB(velo,E,gamma,gamm1,gm3d2,gm1d2,gamm3,aj)

!

!   Appy boundary conditions

!

call aplbncn2d(ndimn,nequa,ndbou,iface,ielem,ibndcnd,  &

&                     iel_nod,ig,adiabatic,E,T,unkn,velo,dtdx,&

&                     dudx,dundx,dfxdx)

!

IF(NVISC.NE.1) THEN

!

do ir = 1,nequa

xjac_flux(ir) = 0.0

yjac_flux(ir) = 0.0

enddo

!

do is = 1,nequa

dfluxsum = DFXDX(1,IS)+DFXDX(2,IS)

do ir = 1,nequa

xjac_flux(ir) = xjac_flux(ir) + aj(1,ir,is)*dfluxsum

yjac_flux(ir) = yjac_flux(ir) + aj(2,ir,is)*dfluxsum

enddo

enddo

!

DO NAL=1,2

DO IR=1,nequa

iordr=nequa*(iel_nod(nal)-1)+ir

!

TERM1=PHI(NAL,IG)*(CFLUX(1,IR)*EN(1,IG) + cflux(2,ir)*   &

&             en(2,ig))

!

TERM2 = PHI(NAL,IG)*(xjac_flux(ir)*en(1,ig) +          &

&               yjac_flux(ir)*en(2,ig))

!

RHS(iordr) = RHS(iordr) - (DT*TERM1 -	         &

&                            DT2OV2*TERM2)

enddo

enddo

!

ELSE

!

!     CALCULATE VISCOUS TERMS ASSOCIATED WITH THE NAVIER-STOKES

!     EQUATIONS

!

!     CALCULATE THE VISCOSITY USING SUTHERLAND'S RELATIONSHIP

!

IF(T<1.E-12) THEN

VIS=1.0

ELSE

VIS = (1.0 +CSUTH)/(T + CSUTH)*T**1.5

ENDIF

!

HK = VIS/GM1M2PR

!

vkk = 0.0

do idimn = 1,ndimn

vkk = vkk + dudx(idimn,idimn)

enddo

!

do idimn = 1,ndimn

do jdimn = 1,ndimn

tau(idimn,jdimn) = vis*(dudx(idimn,jdimn) +       &

&                         dudx(jdimn,idimn))

if(idimn.eq.jdimn)        &

&         tau(idimn,jdimn) = tau(idimn,jdimn) - 2.*vis/3.*vkk

enddo

enddo

!

!     CALCULATE VISCOUS FLUX

!

CALL VISCFLX(reinv,TAU,HK,vis,dudx,dtdx,velo,d2tdx2,d2udx2,  &

&                G,dgjdxj)

!

!   CALCULATE "B" JACOBIAN

CALL BJACOB(reinv,cv,VIS,HK,TAU,DUNDX,Unkn,velo,E,bj)

!

!   CALCULATE "C" JACOBIAN

CALL CJACOB(reinv,cv,VIS,HK,Unkn,cj)

!

do ir = 1,nequa

xjac_flux(ir) = 0.0

yjac_flux(ir) = 0.0

enddo

!

do is = 1,nequa

dfluxsum = DFXDX(1,IS) + DFXDX(2,IS) + dgjdxj(is)

do ir = 1,nequa

axpbx = aj(1,ir,is) + bj(1,ir,is)

aypby = aj(2,ir,is) + bj(2,ir,is)

xjac_flux(ir) = xjac_flux(ir) + axpbx*dfluxsum

yjac_flux(ir) = yjac_flux(ir) + aypby*dfluxsum

enddo

enddo

!

DO NAL=1,2

DO IR=1,nequa

iordr=nequa*(iel_nod(nal)-1)+ir

!

xfluxsum = cflux(1,ir) + g(1,ir)

yfluxsum = cflux(2,ir) + g(2,ir)

!

TERM1 = (xfluxsum*en(1,ig) + yfluxsum*en(2,ig))*    &

&                 PHI(NAL,ig)

!

TERM2 = phi(nal,ig)*(xjac_flux(ir)*en(1,ig) +       &

&                    yjac_flux(ir)*en(2,ig))

!

RHS(iordr) = RHS(iordr) - (DT*TERM1 -               &

&                            DT2OV2*TERM2)

enddo

enddo

!

ENDIF

!

!     CALCULATED LHS MATRIX

!

IF(NVISC.NE.1) THEN

!

call matxmult(ndimn,nequa,aj,aj,aa)

!

do i = 1,ndimn

do ir = 1,nequa

do is = 1,nequa

e1(i,ir,is) = dt*s1imp(ir)*aj(i,ir,is)

e2(i,1,ir,is) = -dt2ov2*s2imp(ir)*aa(i,1,ir,is)

e2(i,2,ir,is) = -dt2ov2*s2imp(ir)*aa(i,2,ir,is)

enddo

enddo

enddo

!

ELSE

if(ibndcnd<0) goto 8000

!

!   Apply boundary conditions to Jacobians

!

call jacbnc2d(ndimn,nequa,ibndcnd,adiabatic,hk,vis,  &

&unkn,dundx,E,reinv,tau,velo,aj,bj,cj)

!

call matxmult(ndimn,nequa,aj,aj,aa)

call matxmult(ndimn,nequa,aj,bj,ab)

call matxmult(ndimn,nequa,bj,aj,ba)

call matxmult(ndimn,nequa,bj,bj,bb)

!

do i = 1,ndimn

do ir = 1,nequa

do is = 1,nequa

e1(i,ir,is) = dt*(s1imp(ir)*aj(i,ir,is)+   &

&                                  s3imp(ir)*bj(i,ir,is))

!

aaba = aa(i,1,ir,is) + ba(i,1,ir,is)

abbb = ab(i,1,ir,is) + bb(i,1,ir,is)

e2(i,1,ir,is) = dt*s3imp(ir)*cj(i,1,ir,is)  &

&                               -dt2ov2*(s2imp(ir)*aaba          &

&                               +s4imp(ir)*abbb)

!

aaba = aa(i,2,ir,is) + ba(i,2,ir,is)

abbb = ab(i,2,ir,is) + bb(i,2,ir,is)

e2(i,2,ir,is) = dt*s3imp(ir)*cj(i,2,ir,is)   &

&                               -dt2ov2*(s2imp(ir)*aaba           &

&                               +s4imp(ir)*abbb)

enddo

enddo

enddo

!

DO NAL = 1, 2

DO MB = 1, 2

!

pipj = phi(nal,ig)*phi(mb,ig)

dpixpj = dpidx(1,mb,ig)*phi(nal,ig)

dpiypj = dpidx(2,mb,ig)*phi(nal,ig)

!

DO IR = 1, nequa

DO IS = 1,nequa

!

e1n = e1(1,ir,is)*en(1,ig) + e1(2,ir,is)*en(2,ig)

!

TERMB1 = pipj*e1n

!

TERMB2 = (e2(1,1,ir,is)*en(1,ig) + e2(2,1,ir,is)*    &

&                      en(2,ig))*dpixpj + (e2(1,2,ir,is)*en(1,ig) &

&                      + e2(2,2,ir,is)*en(2,ig))*dpiypj

!

ia = lface(nal+4,iface)

ib = lface(mb+4,iface)

irow = nequa*(ia-1) + ir

icol = nequa*(ib-1) + is

!

BEL(ielem,irow,icol)=BEL(ielem,irow,icol) +            &

&                           (TERMB1+TERMB2)

enddo

enddo

enddo

enddo

ENDIF

!

8000  CONTINUE

!

enddo gausloop

!

!     ASSEMBLE ELEMENT MATRIX INTO GLOBAL MATRIX

!

!      CALL ebe_vec(2,NEQUA,Delu,Fbar,Diag)

!

enddo ifaceloop

!

RETURN

end subroutine bouingrl

!

!    **************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE "U" VECTOR AND

!     JACOBIANS AT THE GAUSSIAN POINT OF INTEREST FOR THE

!     BOUNDARY TERMS

!

SUBROUTINE EVALBOU(NDIMN,NEQUA,DPIDX,IG,ilbou,VELO,         &

&                   E,T,unkn,DUDX,DTDX,DUNDX,CFLUX,DFXDX,     &

d2tdx2,d2udx2)

!

Use param_var

Use flow_var

Use thermo_propt

Use conv_flux

Use shapbou_2d

!

implicit none

!

integer,intent(in) :: ndimn,nequa,ig

integer,intent(in) :: ILBOU(MNODE)

real,intent(out) :: e,T

real,intent(in) :: DPIDX(MDIMN,2,NGAUS)

REAL,intent(out) :: CFLUX(MDIMN,MEQUA),DFXDX(MDIMN,MEQUA)

real,intent(out) :: DUNDX(MDIMN,MEQUA),unkn(mequa),velo(mdimn)

REAL,intent(out) :: DUDX(MDIMN,MDIMN),DTDX(MDIMN)

real,intent(out) :: d2tdx2(mdimn),d2udx2(mdimn,mdimn,mdimn)

integer :: idimn,jdimn,kdimn,iequa,i,L,iordr,ipoin

!

T = 0.0

E=0.0

!

do idimn = 1, ndimn

velo(idimn) = 0.0

DTDX(IDIMN) = 0.0

d2tdx2(idimn) = 0.0

do jdimn = 1,ndimn

dudx(idimn,jdimn) = 0.0

do kdimn = 1,ndimn

d2udx2(idimn,jdimn,kdimn) = 0.0

enddo

enddo

enddo

DO IEQUA = 1, NEQUA

unkn(iequa) = 0.00

DO JDIMN = 1,NDIMN

DUNDX(JDIMN,IEQUA) = 0.0

DFXDX(JDIMN,IEQUA) = 0.0

CFLUX(JDIMN,IEQUA) = 0.0

ENDDO

ENDDO

!

DO I=1,2

L=ILBOU(I)

iordr = nequa*(L-1)

E=E+ENERG(L)*PHIBOU(I,IG)

T = T + TEMPR(L)*PHIBOU(I,IG)

!

do idimn = 1, ndimn

VELO(idimn)=VELO(idimn)+VEL(idimn,L)*PHIBOU(I,IG)

DTDX(IDIMN) = DTDX(IDIMN) + DPIDX(IDIMN,I,IG)*TEMPR(L)

DO JDIMN = 1,NDIMN

DUDX(IDIMN,JDIMN)=DUDX(IDIMN,JDIMN)+DPIDX(JDIMN,I,IG)*   &

&                     VEL(IDIMN,L)

enddo

enddo

!

DO IEQUA = 1, NEQUA

Unkn(iequa)=Unkn(iequa)+U(iordr+iequa)*PHIBOU(I,IG)

DO JDIMN = 1,NDIMN

DUNDX(JDIMN,IEQUA) = DUNDX(JDIMN,IEQUA) + DPIDX(JDIMN,I,IG)* &

&                  U(iordr+IEQUA)

ENDDO

ENDDO

enddo

!

DO IEQUA = 1, NEQUA

DO I = 1, 2

IPOIN = ILBOU(I)

DO JDIMN = 1,NDIMN

DFXDX(JDIMN,IEQUA) = DFXDX(JDIMN,IEQUA) + DPIDX(JDIMN,I,IG)*  &

&                     F(IPOIN,JDIMN,IEQUA)

CFLUX(JDIMN,IEQUA) = CFLUX(JDIMN,IEQUA)+PHIBOU(I,IG)*       &

&                         F(IPOIN,JDIMN,IEQUA)

ENDDO

enddo

enddo

!

RETURN

end subroutine evalbou

!

!  ********************************************************************

!    Subroutine to apply boundary conditions to FDV surface integrals

!

Subroutine aplbncn2d(ndimn,nequa,ndbou,iface,ielno,ibndcnd,  &

&                     iel_nod,ig,adiabatic,E,T,unkn,velo,dtdx,&

&                     dudx,dundx,dfxdx)

!

!DIR$ INLINEALWAYS aplbncn2d

!

Use Conv_Flux

Use bface_info

Use Bnd_cnd

Use flow_var

Use grid_info

Use shapbou_3d

Use thermo_propt

!

Implicit None

!

integer,intent(in) :: ndbou,ndimn,nequa,iface,ielno,ibndcnd,ig

integer,intent(in),dimension(ndbou) :: iel_nod

logical,intent(out) :: adiabatic

real,intent(in) :: E,T,unkn(nequa)

real,intent(inout),dimension(ndimn) :: dtdx,velo

real,intent(inout),dimension(ndimn,ndimn) :: dudx

real,intent(inout),dimension(ndimn,nequa) :: dundx,dfxdx

!

integer :: iboun,idimn,iequa,inode,ipoin,lnode,lnod1

integer :: iordr,ipp1,jordr

!

real :: dx,dy

real,dimension(nequa) :: dfxdy

real :: P,dprdx,dprdy,tan_a

real :: p1,t1,u1,v1

real :: x1,y1

!

Select Case(ibndcnd)

Case(2,12)   !  Symmetry Condition, v=tau12=q2=0

dudx(1,2)=0.0

dudx(2,2)=0.0

dtdx(2)=0.0

do iequa=1,nequa

dundx(2,iequa)=0.0

dfxdx(2,iequa)=0.0

enddo

if(indbc(iel_nod(2)).eq.1.or.indbc(iel_nod(2)).eq.4) then

ipoin=iel_nod(2)

ipp1=lnod(3,ielno)

dy=x(2,ipp1)-x(2,ipoin)

dudx(1,2)=phi(2,IG)*(vel(1,ipp1)-vel(1,ipoin))/dy

dudx(2,2)=phi(2,IG)*(vel(2,ipp1)-vel(2,ipoin))/dy

dtdx(2)=phi(2,IG)*(tempr(ipp1)-tempr(ipoin))/dy

do iequa=1,nequa

dundx(2,iequa)=phi(2,IG)*(u(NEQUA*(ipp1-1)+iequa)    &

&                       -u(NEQUA*(ipoin-1)+iequa))/dy

dfxdx(2,iequa)=phi(2,IG)*(f(ipp1,2,iequa)            &

&                      -f(ipoin,2,iequa))/dy

enddo

endif

!

Case(1)  ! No-slip wall, u=v=0, T=Twall or dTdn=0

dudx(1,2)=0.0

dudx(2,2)=0.0

dtdx(2)=0.0

do iequa=1,nequa

dundx(2,iequa)=0.0

dfxdx(2,iequa)=0.0

enddo

do inode=1,2

ipoin=iel_nod(inode)

if(inode.eq.1) then

ipp1=lnod(4,ielno)

if(lface(5,iface).eq.3) ipp1 = lnod(2,ielno)

elseif(inode.eq.2) then

ipp1=lnod(3,ielno)

if(lface(6,iface).eq.4) ipp1 = lnod(1,ielno)

endif

dy=x(2,ipp1)-x(2,ipoin)

dudx(1,2)=dudx(1,2)+phi(inode,IG)*vel(1,ipp1)/dy

dudx(2,2)=dudx(2,2)+phi(inode,IG)*vel(2,ipp1)/dy

! --- for constant wall temperature.

if(indbd(ipoin).ne.2) then

adiabatic=.false.

dtdx(2)=dtdx(2)+phi(inode,IG)*(tempr(ipp1)-tempr(ipoin))/dy

do iequa=1,nequa

dundx(2,iequa)=dundx(2,iequa)      &

&   +phi(inode,IG)*(u(NEQUA*(ipp1-1)+iequa)-      &

&   u(NEQUA*(ipoin-1)+iequa))/dy

dfxdx(2,iequa)=dfxdx(2,iequa)+phi(inode,IG)   &

&             *(f(ipp1,2,iequa)-f(ipoin,2,iequa))/dy

enddo

! --- for adiabatic wall boundary condition.

else

adiabatic=.true.

P = cgas*unkn(1)*T

dtdx(2)=0.0

dundx(2,1) = 0.0

dundx(2,2)=dundx(2,2) + phi(inode,ig)*           &

&              unkn(1)*vel(1,ipp1)/dy

dundx(2,3) = dundx(2,3) + phi(inode,ig)*unkn(1)*  &

&                vel(2,ipp1)/dy

dundx(2,4)= 0.0

!

dfxdx(2,1)=dfxdx(2,1)+phi(inode,IG)*unkn(1)*      &

&              vel(2,ipp1)/dy

dfxdx(2,2) = 0.0

dfxdx(2,3) = 0.0

dfxdx(2,4)=dfxdx(2,4)+phi(inode,IG)        &

&              *(unkn(4) + P)*vel(2,ipp1)/dy

endif

enddo

!

Case(4)   ! No-slip compression corner

dudx(1,2)=0.0

dudx(2,2)=0.0

dtdx(2)=0.0

dprdy = 0.0

x1 = 0.0

y1 = 0.0

tan_a = 0.0

tan_a = -1./tan(alpa)

do iequa=1,nequa

dundx(2,iequa)=0.0

dfxdx(2,iequa)=0.0

enddo

do inode=1,2

ipoin=iel_nod(inode)

iboun = lface(inode+2,iface)

if(inode.eq.1) then

ipp1=lnod(4,ielno)

if(lface(5,iface).eq.3) ipp1 = lnod(2,ielno)

elseif(inode.eq.2) then

ipp1=lnod(3,ielno)

if(lface(6,iface).eq.4) ipp1 = lnod(1,ielno)

endif

x1 = x1 + phi(inode,ig)*x(1,ipoin)

y1 = y1 + phi(inode,ig)*x(2,ipoin)

!      tan_a = tan_a + phi(inode,ig)*rbcon(2,iboun)/(-rbcon(1,iboun))

dy=x(2,ipp1)-x(2,ipoin)

dudx(1,2)=dudx(1,2)+phi(inode,IG)*vel(1,ipp1)/dy

dudx(2,2)=dudx(2,2)+phi(inode,IG)*vel(2,ipp1)/dy

dprdy = dprdy + phi(inode,ig)*(press(ipp1)-press(ipoin))/dy

dtdx(2)=dtdx(2)+phi(inode,IG)*(tempr(ipp1)-tempr(ipoin))/dy

!

do iequa=1,nequa

dundx(2,iequa)=dundx(2,iequa)              &

&   +phi(inode,IG)*(u(NEQUA*(ipp1-1)+iequa)-   &

&   u(NEQUA*(ipoin-1)+iequa))/dy

dfxdx(2,iequa)=dfxdx(2,iequa)+phi(inode,IG)      &

&             *(f(ipp1,2,iequa)-f(ipoin,2,iequa))/dy

enddo

!

enddo

!

ipoin = iel_nod(1)

ipp1 = lnod(4,ielno)

dx = x1 - x(1,ipoin)

u1 = vel(1,ipoin) + (y1-x(2,ipoin))*(vel(1,ipp1)-vel(1,ipoin))/   &

&     (x(2,ipp1)-x(2,ipoin))

v1 = vel(2,ipoin) + (y1-x(2,ipoin))*(vel(2,ipp1)-vel(2,ipoin))/   &

&     (x(2,ipp1)-x(2,ipoin))

t1 = tempr(ipoin) + (y1-x(2,ipoin))*(tempr(ipp1)-tempr(ipoin))/   &

&     (x(2,ipp1)-x(2,ipoin))

p1 = press(ipoin) + (y1-x(2,ipoin))*(press(ipp1)-press(ipoin))/   &

&     (x(2,ipp1)-x(2,ipoin))

P = unkn(1)*cgas*T

dudx(1,1) = (velo(1)-u1)/dx

dudx(2,1) = (velo(2)-v1)/dx

dtdx(1) = (T - t1)/dx

dprdx = (P - p1)/dx

!

! --- for constant wall temperature.

if(indbd(ipoin).ne.2) then

adiabatic=.false.

dundx(1,2) = unkn(1)*dudx(1,1)

dundx(1,3) = unkn(1)*dudx(2,1)

dundx(1,4) = E*dundx(1,1) + unkn(1)*cv*dtdx(1)

else

!

! --- for adiabatic wall boundary condition.

!

adiabatic=.true.

dundx(1,1) = -dundx(2,1)*tan_a

dundx(1,2) = unkn(1)*dudx(1,1)

dundx(1,3) = unkn(1)*dudx(2,1)

dundx(1,4) = E*dundx(1,1) + unkn(1)*cv*dtdx(1)

dtdx(1) = -dtdx(2)*tan_a

dprdx = -dprdy*tan_a

endif

!

dfxdx(1,1) = dundx(1,2)

dfxdx(1,2) = dprdx

dfxdx(1,3) = 0.0

dfxdx(1,4) = (unkn(4)+P)*dudx(1,1)

!

Case(-12)   ! Top surface of driven cavity

! Dirichlet on Flow Variables

! Flow Gradients Non-Zero

dudx(1,2) = 0.0

dudx(2,2) = 0.0

dtdx(2) = 0.0

velo(2) = 0.0

!

do iequa = 1,nequa

dundx(2,iequa) = 0.0

dfxdx(2,iequa) = 0.0

enddo

!

do inode = 1,2

ipoin = iel_nod(inode)

if(inode.eq.1) ipp1 = lnod(2,ielno)

if(inode.eq.2) ipp1 = lnod(1,ielno)

dy = x(2,ipp1) - x(2,ipoin)

do idimn = 1,ndimn

dudx(idimn,2) = dudx(idimn,2) + phi(inode,ig)*    &

(vel(idimn,ipp1) - vel(idimn,ipoin))/dy

enddo

dtdx(2) = dtdx(2) + phi(inode,ig)*(tempr(ipp1) -  &

tempr(ipoin))/dy

!

do iequa = 1,nequa

iordr = nequa*(ipoin-1) + iequa

jordr = nequa*(ipp1-1) + iequa

dundx(2,iequa) = dundx(2,iequa) + phi(inode,ig)*   &

(U(jordr) - U(iordr))/dy

dfxdx(2,iequa) = dfxdx(2,iequa) + phi(inode,ig)*   &

(f(ipp1,2,iequa) - f(ipoin,2,iequa))/dy

enddo

!

enddo

!

!

Case(11,21)   !  Side wall of driven cavity

dudx(1,1)=0.0

dudx(2,1)=0.0

dtdx(1)=0.0

do iequa=1,nequa

dundx(1,iequa)=0.0

dfxdx(1,iequa)=0.0

enddo

do inode=1,2

ipoin=iel_nod(inode)

lnode=lface(inode+4,iface)

lnod1=lnode+1

if(ibndcnd.eq.11) then

if(lnode.eq.4) lnod1 = 3

elseif(ibndcnd.eq.21) then

if(lnode.eq.2) lnod1 = 1

endif

ipp1 = lnod(lnod1,ielno)

dx=x(1,ipp1)-x(1,ipoin)

dudx(1,1)=dudx(1,1)+phi(inode,ig)*vel(1,ipp1)/dx

dudx(2,1)=dudx(2,1)+phi(inode,ig)*vel(2,ipp1)/dx

! --- for constant wall temperature.

if(indbd(ipoin).ne.2) then

adiabatic=.false.

dtdx(1)=dtdx(1)+phi(inode,ig)*(tempr(ipp1)-tempr(ipoin))/dx

do iequa=1,nequa

dundx(1,iequa)=dundx(1,iequa)                &

&   +phi(inode,ig)*(u(nequa*(ipp1-1)+iequa)-   &

&    u(nequa*(ipoin-1)+iequa))/dx

dfxdx(1,iequa)=dfxdx(1,iequa)+phi(inode,ig)  &

&             *(f(ipp1,1,iequa)-f(ipoin,1,iequa))/dx

enddo

! --- for adiabatic wall boundary condition.

else

adiabatic=.true.

P = cgas*unkn(1)*T

dtdx(1)=0.0

dundx(1,1) = 0.0

dundx(1,2)=dundx(1,2) + phi(inode,ig)*unkn(1)*   &

&           (vel(1,ipp1) - vel(1,ipoin))/dx

dundx(1,3) = dundx(1,3) + phi(inode,ig)*unkn(1)*  &

&             (vel(2,ipp1) - vel(2,ipoin))/dx

dundx(1,4) = 0.0

!

dfxdx(1,1)=dfxdx(1,1)+phi(inode,ig)*unkn(1)       &

&             *(vel(1,ipp1) - vel(1,ipoin))/dx

dfxdx(1,2)= 0.0

dfxdx(1,3)= 0.0

dfxdx(1,4)=dfxdx(1,4)+phi(inode,ig)                    &

&      *(unkn(4) + P)*vel(1,ipp1)/dx

endif

enddo

!

Case(-1,6)  !  Inlet or Exit, normal derivatives = 0

dudx(1,1)=0.0

dudx(2,1)=0.0

dtdx(1)=0.0

do iequa=1,nequa

dundx(1,iequa)=0.0

dfxdx(1,iequa)=0.0

enddo

!

Case(7)    !   Top surface, normal derivatives = 0

dudx(1,2)=0.0

dudx(2,2)=0.0

dtdx(2)=0.0

do iequa=1,nequa

dundx(2,iequa)=0.0

dfxdx(2,iequa)=0.0

enddo

!

Case(8)  !  Exit for compression corner, Derivatives normal

!  to flow direction = 0

dudx(1,1)=0.0

dudx(2,1)=0.0

dtdx(1)=0.0

tan_a=tan(alpa)

!        tan_a = velo(2)/velo(1)

do iequa=1,nequa

dundx(1,iequa)=0.0

dfxdx(1,iequa)=0.0

dfxdy(iequa)=0.0

enddo

!

do inode = 1, 2

ipoin = iel_nod(inode)

if(inode.eq.1) lnode=1

if(inode.eq.2) lnode=4

ipp1=lnod(lnode,ielno)

!           tan_a=tan_a + phi(inode,ig)*

!     &           (x(2,ipp1)-x(2,ipoin))/(x(1,ipp1)-x(1,ipoin))

do iequa=1,nequa

dfxdy(iequa)=dfxdy(iequa)+dshpdx(2,inode,ig)*      &

&                  f(ipoin,1,iequa)

enddo

enddo

!

dudx(1,1)=-dudx(1,2)*tan_a

dudx(2,1)=-dudx(2,2)*tan_a

dtdx(1)=-dtdx(2)*tan_a

!

do iequa=1,nequa

dundx(1,iequa)=-dundx(2,iequa)*tan_a

dfxdx(1,iequa)=-dfxdy(iequa)*tan_a

enddo

!

End Select

!

return

end subroutine aplbncn2d

!

!  ********************************************************************

!    Subroutine to apply boundary conditions to FDV surface integrals

!

Subroutine aplbndcnd(ndimn,nequa,ndbou,iface,ielno,ibndcnd,  &

&                     iel_nod,ig,adiabatic,E,T,unkn,velo,dtdx,&

&                     d2tdx2,dudx,d2udx2,dundx,dfxdx,enorm)

!

!DIR$ INLINEALWAYS aplbndcnd

!

Use Conv_Flux

Use bface_info

Use Bnd_cnd

Use flow_var

Use grid_info

Use shapbou_3d

Use thermo_propt

!

Implicit None

!

integer,intent(in) :: ndimn,nequa,ndbou,iface,ielno,ibndcnd,ig

integer,intent(in),dimension(ndbou) :: iel_nod

logical,intent(out) :: adiabatic

real,intent(in),dimension(ndimn) :: enorm,velo

real,intent(in) :: E,T,unkn(nequa)

real,intent(inout),dimension(ndimn) :: dtdx,d2tdx2

real,intent(inout),dimension(ndimn,ndimn) :: dudx

real,intent(inout),dimension(ndimn,ndimn,ndimn) :: d2udx2

real,intent(inout),dimension(ndimn,nequa) :: dundx,dfxdx

!

integer :: idimn,iequa,inode,inod1,ipoin,lnode,lnod1

integer :: iordr,jordr,ipoin1,ipoin2,ipoin3,ipoin4

integer :: ipp1,ipp2,ipp3,ipp4,iordr1,iordr2,iordr3,iordr4

integer :: jnode,jordr1,jordr2,jordr3,jordr4,jpoin,jpp1

!

real :: beta,dx,dx1,dx2,dx3,dx4,dy,dz,dz1,dz2,delx,dely

real :: dx1a,dx1b,dx2a,dx2b,drdy,r1,rho1a,rho1b

real :: dprdx,dprdy,dprdz,angl_b,tan_a,tan_b,pi

real :: P,p1a,p1b,p1,t1a,t1b,t1,u1a,u1b,u1,v1a,v1b,v1

real :: w1a,w1b,w1,x1,y1,z1

real,dimension(nequa) :: dfydx,dfydz

!

Select Case(ibndcnd)

Case(2,12)    !  Symmetry in x-y plane, w=tau13=tau23=q3=0

dudx(1,3) = 0.0

dudx(2,3) = 0.0

dudx(3,3) = 0.0

dtdx(3) = 0.0

d2tdx2(3) = 0.0

do idimn = 1,ndimn

d2udx2(1,idimn,3) = 0.0

d2udx2(2,idimn,3) = 0.0

d2udx2(3,idimn,3) = 0.0

d2udx2(1,3,idimn) = 0.0

d2udx2(2,3,idimn) = 0.0

d2udx2(3,3,idimn) = 0.0

enddo

!

do iequa = 1,nequa

dundx(3,iequa) = 0.0

dfxdx(3,iequa) = 0.0

enddo

!

do inode = 1,ndbou

ipoin = iel_nod(inode)

lnode = lfacnode(inode,iface)

inod1 = lnode + ndbou

if(ibndcnd==12) inod1 = lnode - ndbou

ipp1 = lnod(inod1,ielno)

dz = x(3,ipp1) - x(3,ipoin)

dudx(3,3) = dudx(3,3) + phi(inode,ig)*vel(3,ipp1)/dz

enddo

!

p = unkn(1)*cgas*T

dfxdx(3,5) = (unkn(5) + P)*dudx(3,3)

!

if(indbc(iel_nod(3))==1) then    ! Interface with no-slip wall

do inode = 3,4

ipoin = iel_nod(inode)

lnode = lfacnode(inode,iface)

inod1 = lnode + ndbou

if(ibndcnd==12) inod1 = lnode - ndbou

ipp1 = lnod(inod1,ielno)

dz = x(3,ipp1) - x(3,ipoin)

if(indbd(iel_nod(3))/=2)      &

dtdx(3) = dtdx(3) + phi(inode,ig)*(tempr(ipp1) -  &

tempr(ipoin))/dz

do idimn = 1,2

dudx(idimn,3) = dudx(idimn,3) + phi(inode,ig)*   &

(vel(idimn,ipp1) - vel(idimn,ipoin))&

&/dz

enddo

!

dfxdx(3,1:nequa) = 0.0

!

do iequa = 1,nequa

iordr = nequa*(ipoin-1) + iequa

jordr = nequa*(ipp1-1) + iequa

dundx(3,iequa) = dundx(3,iequa) + phi(inode,ig)*  &

(u(jordr) - u(iordr))/dz

dfxdx(3,iequa) = dfxdx(3,iequa) + phi(inode,ig)*  &

(F(ipp1,3,iequa) - F(ipoin,3,iequa))&

&/dz

enddo

enddo

endif

!

Case(22)    ! Symmetry in y-z plane, u=tau12=tau13=q1=0

dudx(1,1) = 0.0

dudx(2,1) = 0.0

dudx(3,1) = 0.0

dtdx(1) = 0.0

d2tdx2(1) = 0.0

do idimn = 1,ndimn

d2udx2(1,idimn,1) = 0.0

d2udx2(2,idimn,1) = 0.0

d2udx2(3,idimn,1) = 0.0

d2udx2(1,1,idimn) = 0.0

d2udx2(2,1,idimn) = 0.0

d2udx2(3,1,idimn) = 0.0

enddo

!

do iequa = 1,nequa

dundx(1,iequa) = 0.0

dfxdx(1,iequa) = 0.0

enddo

!

lnode = lfacnode(1,iface)

!

ipoin1 = iel_nod(1)

ipoin2 = iel_nod(2)

ipoin3 = iel_nod(3)

ipoin4 = iel_nod(4)

!

if(lnode==1) then

ipp1 = lnod(2,ielno)

ipp2 = lnod(3,ielno)

ipp3 = lnod(7,ielno)

ipp4 = lnod(6,ielno)

elseif(lnode==2) then

ipp1 = lnod(1,ielno)

ipp2 = lnod(4,ielno)

ipp3 = lnod(8,ielno)

ipp4 = lnod(5,ielno)

endif

!

dx1 = x(1,ipp1) - x(1,ipoin1)

dx2 = x(1,ipp2) - x(1,ipoin2)

dx3 = x(1,ipp3) - x(1,ipoin3)

dx4 = x(1,ipp4) - x(1,ipoin4)

dudx(1,1) = phi(1,ig)*vel(1,ipp1)/dx1 + phi(2,ig)*vel(1,ipp2)/dx2 &

&+ phi(3,ig)*vel(1,ipp3)/dx3 + phi(4,ig)*vel(1,ipp4)/dx4

!

p = unkn(1)*cgas*T

dfxdx(1,5) = (unkn(5) + P)*dudx(1,1)

!

if(indbc(iel_nod(3))==14) then    ! Interface with no-slip wall

if(indbd(iel_nod(3))/=2)       &

dtdx(1) = phi(2,ig)*(tempr(ipp2) - tempr(ipoin2))/dx2 &

+ phi(3,ig)*(tempr(ipp3) - tempr(ipoin3))/dx3

do idimn = 2,3

dudx(idimn,1) = phi(2,ig)*(vel(idimn,ipp2)-             &

&vel(idimn,ipoin2))/dx2+ phi(3,ig)*&

(vel(idimn,ipp3) - vel(idimn,ipoin3))/dx3

enddo

!

do iequa = 1,nequa

iordr2 = nequa*(ipoin2-1) + iequa

jordr2 = nequa*(ipp2-1) + iequa

iordr3 = nequa*(ipoin3-1) + iequa

jordr3 = nequa*(ipp3-1) + iequa

dundx(1,iequa) = phi(2,ig)*(u(jordr2) - u(iordr2))/dx2 &

+ phi(3,ig)*(u(jordr3) - u(iordr3))/dx3

dfxdx(1,iequa) = phi(2,ig)*(F(ipp2,1,iequa)            &

&- F(ipoin2,1,iequa))/dx2 + phi(3,ig)* &

&(F(ipp3,1,iequa) - F(ipoin3,1,iequa))/dx3

enddo

endif

!

Case(1)   !  No slip wall, u=v=w=0

dudx(1,3) = 0.0

dudx(2,3) = 0.0

dudx(3,3) = 0.0

dtdx(3) = 0.0

d2tdx2(3) = 0.0

do idimn = 1,ndimn

d2udx2(1,idimn,3) = 0.0

d2udx2(2,idimn,3) = 0.0

d2udx2(3,idimn,3) = 0.0

d2udx2(1,3,idimn) = 0.0

d2udx2(2,3,idimn) = 0.0

d2udx2(3,3,idimn) = 0.0

enddo

!

do iequa = 1,nequa

dundx(3,iequa) = 0.0

dfxdx(3,iequa) = 0.0

enddo

!

p = unkn(1)*cgas*T

!

do inode = 1,ndbou

ipoin = iel_nod(inode)

lnode = lfacnode(inode,iface)

inod1 = lnode + ndbou

ipp1 = lnod(inod1,ielno)

dz = x(3,ipp1) - x(3,ipoin)

do idimn = 1,ndimn

dudx(idimn,3) = dudx(idimn,3) + phi(inode,ig)*  &

&vel(idimn,ipp1)/dz

enddo

!   For constant wall temperature

if(indbd(ipoin)/=2) then

adiabatic = .false.

dtdx(3) = dtdx(3) + phi(inode,ig)*(tempr(ipp1) -  &

&tempr(ipoin))/dz

do iequa = 1,nequa

iordr = nequa*(ipoin-1) + iequa

jordr = nequa*(ipp1-1) + iequa

dundx(3,iequa) = dundx(3,iequa) + phi(inode,ig)* &

&(u(jordr) - u(iordr))/dz

dfxdx(3,iequa) = dfxdx(3,iequa) + phi(inode,ig)* &

&(f(ipp1,3,iequa) - f(ipoin,3,iequa))/dz

enddo

!  for adiabatic wall condition

else

adiabatic = .true.

dtdx(3) = 0.0

dundx(3,1) = 0.0

dundx(3,2) = dundx(3,2) + phi(inode,ig)*unkn(1)*   &

&vel(1,ipp1)/dz

dundx(3,3) = dundx(3,3) + phi(inode,ig)*unkn(1)*   &

&vel(2,ipp1)/dz

dundx(3,4) = dundx(3,4) + phi(inode,ig)*unkn(1)*   &

&vel(3,ipp1)/dz

dundx(3,5) = 0.0

dfxdx(3,1) = dfxdx(3,1) + phi(inode,ig)*unkn(1)*   &

&vel(3,ipp1)/dz

dfxdx(3,2) = 0.0

dfxdx(3,3) = 0.0

dfxdx(3,4) = 0.0

dfxdx(3,5) = dfxdx(3,5) + phi(inode,ig)*(unkn(5) + &

&P)*vel(3,ipp1)/dz

endif

enddo

!

Case(4)    ! Compression Corner in x-y plane

dudx(1,3)=0.0

dudx(2,3)=0.0

dudx(3,3) = 0.0

dtdx(3)=0.0

dprdz = 0.0

dprdx = 0.0

dprdy = 0.0

x1 = 0.0

y1 = 0.0

z1 = 0.0

tan_a = -1./tan(alpa)

do iequa=1,nequa

dundx(3,iequa)=0.0

dfxdx(3,iequa)=0.0

enddo

do inode=1,ndbou

ipoin=iel_nod(inode)

lnode = lfacnode(inode,iface)

jnode = lnode + ndbou

ipp1 = lnod(jnode,ielno)

x1 = x1 + phi(inode,ig)*x(1,ipoin)

y1 = y1 + phi(inode,ig)*x(2,ipoin)

z1 = z1 + phi(inode,ig)*x(3,ipoin)

dz=x(3,ipp1)-x(3,ipoin)

dudx(1,3)=dudx(1,3)+phi(inode,IG)*vel(1,ipp1)/dz

dudx(2,3)=dudx(2,3)+phi(inode,IG)*vel(2,ipp1)/dz

dudx(3,3) = dudx(3,3) + phi(inode,ig)*vel(3,ipp1)/dz

dprdz = dprdz + phi(inode,ig)*(press(ipp1)-press(ipoin))/dz

dtdx(3)=dtdx(3)+phi(inode,IG)*(tempr(ipp1)-tempr(ipoin))/dz

dprdx = dprdx + dshpdx(1,inode,ig)*press(ipoin)

dprdy = dprdy + dshpdx(2,inode,ig)*press(ipoin)

!

do iequa=1,nequa

dundx(3,iequa)=dundx(3,iequa)              &

&   +phi(inode,IG)*(u(NEQUA*(ipp1-1)+iequa)-	&

&   u(NEQUA*(ipoin-1)+iequa))/dz

dfxdx(3,iequa)=dfxdx(3,iequa)+phi(inode,IG)      &

&             *(f(ipp1,3,iequa)-f(ipoin,3,iequa))/dz

enddo

!

enddo

!

ipoin = iel_nod(1)

ipp1 = lnod(5,ielno)

if(ipoin==ipp1) ipp1 = lnod(1,ielno)

jpoin = iel_nod(2)

jpp1 = lnod(6,ielno)

if(jpoin==jpp1) jpp1 = lnod(2,ielno)

dy = y1 - x(2,ipoin)

dx = abs(x(1,jpoin) - x(1,ipoin))

dx1 = abs(x1 - x(1,ipoin))

dx2 = abs(x1 - x(1,jpoin))

u1a = vel(1,ipoin) + (z1-x(3,ipoin))*(vel(1,ipp1)-vel(1,ipoin))/ &

&     (x(3,ipp1)-x(3,ipoin))

u1b = vel(1,jpoin) + (z1-x(3,jpoin))*(vel(1,jpp1)-vel(1,jpoin))/ &

&     (x(3,jpp1)-x(3,jpoin))

v1a = vel(2,ipoin) + (z1-x(3,ipoin))*(vel(2,ipp1)-vel(2,ipoin))/ &

&     (x(3,ipp1)-x(3,ipoin))

v1b = vel(2,jpoin) + (z1-x(3,jpoin))*(vel(2,jpp1)-vel(2,jpoin))/ &

&     (x(3,jpp1)-x(3,jpoin))

w1a = vel(3,ipoin) + (z1-x(3,ipoin))*(vel(3,ipp1)-vel(3,ipoin))/ &

&     (x(3,ipp1)-x(3,ipoin))

w1b = vel(3,jpoin) + (z1-x(3,jpoin))*(vel(3,jpp1)-vel(3,jpoin))/ &

&     (x(3,jpp1)-x(3,jpoin))

t1a = tempr(ipoin) + (z1-x(3,ipoin))*(tempr(ipp1)-tempr(ipoin))/ &

&     (x(3,ipp1)-x(3,ipoin))

t1b = tempr(jpoin) + (z1-x(3,jpoin))*(tempr(jpp1)-tempr(jpoin))/ &

&     (x(3,jpp1)-x(3,jpoin))

p1a = press(ipoin) + (z1-x(3,ipoin))*(press(ipp1)-press(ipoin))/ &

&     (x(3,ipp1)-x(3,ipoin))

p1b = press(jpoin) + (z1-x(3,jpoin))*(press(jpp1)-press(jpoin))/ &

&     (x(3,jpp1)-x(3,jpoin))

u1 = (u1a*dx1 + u1b*dx2)/dx

v1 = (v1a*dx1 + v1b*dx2)/dx

w1 = (w1a*dx1 + w1b*dx2)/dx

p1 = (p1a*dx1 + p1b*dx2)/dx

t1 = (t1a*dx1 + t1b*dx2)/dx

P = unkn(1)*cgas*T

dudx(1,2) = (velo(1)-u1)/dy

dudx(2,2) = (velo(2)-v1)/dy

dudx(3,2) = (velo(3)-w1)/dy

dtdx(2) = (T - t1)/dy

dprdy = (P - p1)/dy

!

! --- for constant wall temperature.

if(indbd(ipoin).ne.2) then

adiabatic=.false.

dundx(2,2) = unkn(1)*dudx(1,2)

dundx(2,3) = unkn(1)*dudx(2,2)

dundx(2,4) = unkn(1)*dudx(3,2)

dundx(2,5) = E*dundx(2,1) + unkn(1)*cv*dtdx(2)

else

!

! --- for adiabatic wall boundary condition.

!

adiabatic=.true.

dundx(2,1) = -dundx(3,1)*tan_a

dundx(2,2) = unkn(1)*dudx(1,2)

dundx(2,3) = unkn(1)*dudx(2,2)

dundx(2,4) = unkn(1)*dudx(3,2)

dtdx(2) = - dtdx(3)*tan_a

dundx(2,5) = E*dundx(2,1) + unkn(1)*cv*dtdx(2)

dprdy = -dprdz*tan_a

endif

!

dfxdx(2,1) = dundx(2,3)

dfxdx(2,2) = 0.0

dfxdx(2,3) = dprdy

dfxdx(2,4) = 0.0

dfxdx(2,5) = (unkn(5)+P)*dudx(2,2)

!

Case(14)    ! Compression Corner in y-z plane

dudx(1,1)=0.0

dudx(2,1)=0.0

dudx(3,1) = 0.0

dtdx(1)=0.0

dprdx = 0.0

dprdz = 0.0

x1 = 0.0

y1 = 0.0

z1 = 0.0

ipoin = iel_nod(1)

jpoin = iel_nod(2)

delx = x(1,jpoin) - x(1,ipoin)

dely = x(2,jpoin) - x(2,ipoin)

pi = acos(-1.)

beta = 15.*pi/180.

tan_a = -1./tan(beta)

do iequa=1,nequa

dundx(1,iequa)=0.0

dfxdx(1,iequa)=0.0

enddo

do inode=1,ndbou

ipoin=iel_nod(inode)

lnode = lfacnode(inode,iface)

if(inode==2.or.inode==3) then

jnode = lnode - 1

else

jnode = lnode + 1

endif

ipp1 = lnod(jnode,ielno)

x1 = x1 + phi(inode,ig)*x(1,ipoin)

y1 = y1 + phi(inode,ig)*x(2,ipoin)

z1 = z1 + phi(inode,ig)*x(3,ipoin)

dx=x(1,ipp1)-x(1,ipoin)

dudx(1,1)=dudx(1,1)+phi(inode,IG)*vel(1,ipp1)/dx

dudx(2,1)=dudx(2,1)+phi(inode,IG)*vel(2,ipp1)/dx

dudx(3,1) = dudx(3,1) + phi(inode,ig)*vel(3,ipp1)/dx

dprdx = dprdx + phi(inode,ig)*(press(ipp1)-press(ipoin))/dx

dtdx(1)=dtdx(1)+phi(inode,IG)*(tempr(ipp1)-tempr(ipoin))/dx

dprdz = dprdz + dshpdx(3,inode,ig)*press(ipoin)

!

do iequa=1,nequa

dundx(1,iequa)=dundx(1,iequa)              &

&   +phi(inode,IG)*(u(NEQUA*(ipp1-1)+iequa)-	&

&   u(NEQUA*(ipoin-1)+iequa))/dx

dfxdx(1,iequa)=dfxdx(1,iequa)+phi(inode,IG)      &

&             *(f(ipp1,1,iequa)-f(ipoin,1,iequa))/dx

enddo

!

enddo

!

ipoin = iel_nod(1)

ipp1 = lnod(2,ielno)

if(ipoin==ipp1) ipp1 = lnod(1,ielno)

jpoin = iel_nod(4)

jpp1 = lnod(6,ielno)

if(jpoin==jpp1) jpp1 = lnod(5,ielno)

dy = y1 - x(2,ipoin)

dz = abs(x(3,jpoin) - x(3,ipoin))

dz1 = abs(z1 - x(3,ipoin))

dz2 = abs(z1 - x(3,jpoin))

u1a = vel(1,ipoin) + (x1-x(1,ipoin))*(vel(1,ipp1)-vel(1,ipoin))/ &

&     (x(1,ipp1)-x(1,ipoin))

u1b = vel(1,jpoin) + (x1-x(1,jpoin))*(vel(1,jpp1)-vel(1,jpoin))/ &

&     (x(1,jpp1)-x(1,jpoin))

v1a = vel(2,ipoin) + (x1-x(1,ipoin))*(vel(2,ipp1)-vel(2,ipoin))/ &

&     (x(1,ipp1)-x(1,ipoin))

v1b = vel(2,jpoin) + (x1-x(1,jpoin))*(vel(2,jpp1)-vel(2,jpoin))/ &

&     (x(1,jpp1)-x(1,jpoin))

w1a = vel(3,ipoin) + (x1-x(1,ipoin))*(vel(3,ipp1)-vel(3,ipoin))/ &

&     (x(1,ipp1)-x(1,ipoin))

w1b = vel(3,jpoin) + (x1-x(1,jpoin))*(vel(3,jpp1)-vel(3,jpoin))/ &

&     (x(1,jpp1)-x(1,jpoin))

t1a = tempr(ipoin) + (x1-x(1,ipoin))*(tempr(ipp1)-tempr(ipoin))/ &

&     (x(1,ipp1)-x(1,ipoin))

t1b = tempr(jpoin) + (x1-x(1,jpoin))*(tempr(jpp1)-tempr(jpoin))/ &

&     (x(1,jpp1)-x(1,jpoin))

p1a = press(ipoin) + (x1-x(1,ipoin))*(press(ipp1)-press(ipoin))/ &

&     (x(1,ipp1)-x(1,ipoin))

p1b = press(jpoin) + (x1-x(1,jpoin))*(press(jpp1)-press(jpoin))/ &

&     (x(1,jpp1)-x(1,jpoin))

u1 = (u1a*dz1 + u1b*dz2)/dz

v1 = (v1a*dz1 + v1b*dz2)/dz

w1 = (w1a*dz1 + w1b*dz2)/dz

p1 = (p1a*dz1 + p1b*dz2)/dz

t1 = (t1a*dz1 + t1b*dz2)/dz

P = unkn(1)*cgas*T

dudx(1,2) = (velo(1)-u1)/dy

dudx(2,2) = (velo(2)-v1)/dy

dudx(3,2) = (velo(3)-w1)/dy

dtdx(2) = (T - t1)/dy

dprdy = (P - p1)/dy

!

! --- for constant wall temperature.

if(indbd(ipoin).ne.2) then

adiabatic=.false.

dundx(2,2) = unkn(1)*dudx(1,2)

dundx(2,3) = unkn(1)*dudx(2,2)

dundx(2,4) = unkn(1)*dudx(3,2)

dundx(2,5) = E*dundx(2,1) + unkn(1)*cv*dtdx(2)

else

!

! --- for adiabatic wall boundary condition.

!

adiabatic=.true.

dundx(2,1) = -dundx(1,1)*tan_a

dundx(2,2) = unkn(1)*dudx(1,2)

dundx(2,3) = unkn(1)*dudx(2,2)

dundx(2,4) = unkn(1)*dudx(3,2)

dtdx(2) = -dtdx(1)*tan_a

dundx(2,5) = E*dundx(2,1) + unkn(1)*cv*dtdx(2)

dprdy = -dprdx*tan_a

endif

!

dfxdx(2,1) = dundx(2,3)

dfxdx(2,2) = 0.0

dfxdx(2,3) = dprdy

dfxdx(2,4) = 0.0

dfxdx(2,5) = (unkn(5)+P)*dudx(2,2)

!

Case(11,21)   ! No slip wall in y-z plane

dudx(1,1)=0.0

dudx(2,1)=0.0

dudx(3,1) = 0.0

dtdx(1)=0.0

do iequa=1,nequa

dundx(1,iequa)=0.0

dfxdx(1,iequa)=0.0

enddo

do inode=1,ndbou

ipoin=iel_nod(inode)

lnode=lfacnode(inode,iface)

lnod1=lnode+1

if(ibndcnd==11) then ! left side wall

if(lnode==8) lnod1 = 7

if(lnode==4) lnod1 = 3

elseif(ibndcnd==21) then ! right side wall

if(lnode==2) lnod1 = 1

if(lnode==6) lnod1 = 5

endif

ipp1 = lnod(lnod1,ielno)

dx=x(1,ipp1)-x(1,ipoin)

dudx(1,1)=dudx(1,1)+phi(inode,ig)*vel(1,ipp1)/dx

dudx(2,1)=dudx(2,1)+phi(inode,ig)*vel(2,ipp1)/dx

dudx(3,1)=dudx(3,1)+phi(inode,ig)*vel(3,ipp1)/dx

! --- for constant wall temperature.

if(indbd(ipoin).ne.2) then

adiabatic=.false.

dtdx(1)=dtdx(1)+phi(inode,ig)*(tempr(ipp1)-tempr(ipoin))/dx

do iequa=1,nequa

dundx(1,iequa)=dundx(1,iequa)      &

&   +phi(inode,ig)*(u(nequa*(ipp1-1)+iequa)-    &

&    u(nequa*(ipoin-1)+iequa))/dx

dfxdx(1,iequa)=dfxdx(1,iequa)+phi(inode,ig)       &

&             *(f(ipp1,1,iequa)-f(ipoin,1,iequa))/dx

enddo

! --- for adiabatic wall boundary condition.

else

adiabatic=.true.

P = cgas*unkn(1)*T

dtdx(1)=0.0

dundx(1,1) = 0.0

dundx(1,2)=dundx(1,2) + phi(inode,ig)*unkn(1)*    &

&           (vel(1,ipp1) - vel(1,ipoin))/dx

dundx(1,3) = dundx(1,3) + phi(inode,ig)*unkn(1)*	 &

&             (vel(2,ipp1) - vel(2,ipoin))/dx

dundx(1,4) = dundx(1,4) + phi(inode,ig)*unkn(1)*	 &

&             (vel(3,ipp1) - vel(3,ipoin))/dx

dundx(1,5)= 0.0

!

dfxdx(1,1)=dfxdx(1,1)+phi(inode,ig)*unkn(1)      &

&             *(vel(1,ipp1) - vel(1,ipoin))/dx

dfxdx(1,2) = 0.0

dfxdx(1,3) = 0.0

dfxdx(1,4) = 0.0

dfxdx(1,5)=dfxdx(1,5)+phi(inode,ig)              &

&      *(unkn(5) + P)*vel(1,ipp1)/dx

endif

enddo

!

Case(31,41)   ! No slip wall in x-z plane

dudx(1,2)=0.0

dudx(2,2)=0.0

dudx(3,2) = 0.0

dtdx(2)=0.0

do iequa=1,nequa

dundx(2,iequa)=0.0

dfxdx(2,iequa)=0.0

enddo

do inode=1,ndbou

ipoin=iel_nod(inode)

lnode=lfacnode(inode,iface)

if(ibndcnd==31) then ! left side wall

lnod1=lnode+1

if(lnode==1) lnod1 = 4

if(lnode==5) lnod1 = 8

elseif(ibndcnd==41) then ! right side wall

lnod1=lnode-1

if(lnode==4) lnod1 = 1

if(lnode==8) lnod1 = 5

endif

ipp1 = lnod(lnod1,ielno)

dy=x(2,ipp1)-x(2,ipoin)

dudx(1,2)=dudx(1,2)+phi(inode,ig)*vel(1,ipp1)/dy

dudx(2,2)=dudx(2,2)+phi(inode,ig)*vel(2,ipp1)/dy

dudx(3,2)=dudx(3,2)+phi(inode,ig)*vel(3,ipp1)/dy

! --- for constant wall temperature.

if(indbd(ipoin).ne.2) then

adiabatic=.false.

dtdx(2)=dtdx(2)+phi(inode,ig)*(tempr(ipp1)-tempr(ipoin))/dy

do iequa=1,nequa

dundx(2,iequa)=dundx(2,iequa)                &

&   +phi(inode,ig)*(u(nequa*(ipp1-1)+iequa)-	&

&    u(nequa*(ipoin-1)+iequa))/dy

dfxdx(2,iequa)=dfxdx(2,iequa)+phi(inode,ig)  &

&             *(f(ipp1,2,iequa)-f(ipoin,2,iequa))/dy

enddo

! --- for adiabatic wall boundary condition.

else

adiabatic=.true.

P = cgas*unkn(1)*T

dtdx(2)=0.0

dundx(2,1) = 0.0

dundx(2,2)=dundx(2,2) + phi(inode,ig)*unkn(1)*      &

&           (vel(1,ipp1) - vel(1,ipoin))/dy

dundx(2,3) = dundx(2,3) + phi(inode,ig)*unkn(1)*	   &

&             (vel(2,ipp1) - vel(2,ipoin))/dy

dundx(2,4) = dundx(2,4) + phi(inode,ig)*unkn(1)*	   &

&             (vel(3,ipp1) - vel(3,ipoin))/dy

dundx(2,5)= 0.0

!

dfxdx(2,1)=dfxdx(2,1)+phi(inode,ig)*unkn(1)         &

&             *(vel(2,ipp1) - vel(2,ipoin))/dy

dfxdx(2,2)= 0.0

dfxdx(1,3)= 0.0

dfxdx(1,4)= 0.0

dfxdx(1,5)=dfxdx(1,5)+phi(inode,ig)                 &

&      *(unkn(5) + P)*vel(2,ipp1)/dy

endif

enddo

!

Case(-12)    ! Top Surface of Lid-Driven Cavity

! Dirichlet on Flow variables

! Normal Gradients Non-zero

dudx(1:3,3) = 0.0

d2udx2(1:ndimn,1:ndimn,3) = 0.0

d2udx2(1:ndimn,3,1:ndimn) = 0.0

dtdx(3) = 0.0

d2tdx2(3) = 0.0

dundx(3,1:nequa) = 0.0

dfxdx(3,1:nequa) = 0.0

!

do inode = 1,ndbou

ipoin = iel_nod(inode)

lnode = lfacnode(inode,iface)

inod1 = lnode - ndbou

ipp1 = lnod(inod1,ielno)

dz = x(3,ipp1) - x(3,ipoin)

do idimn = 1,ndimn

dudx(idimn,3) = dudx(idimn,3) + phi(inode,ig)*  &

(vel(idimn,ipp1) - vel(idimn,ipoin))&

/dz

enddo

!

dtdx(3) = dtdx(3) + phi(inode,ig)*(tempr(ipp1) - &

tempr(ipoin))/dz

do iequa = 1,nequa

iordr = nequa*(ipoin-1) + iequa

jordr = nequa*(ipp1-1) + iequa

dundx(3,iequa) = dundx(3,iequa) + phi(inode,ig)*  &

(u(jordr) - u(iordr))/dz

dfxdx(3,iequa) = dfxdx(3,iequa) + phi(inode,ig)*  &

(f(ipp1,3,iequa) - f(ipoin,3,iequa))&

/dz

enddo

enddo

!

Case(-1,6)   ! Inlet and Exit, normal derivatives = 0

dudx(1,2)=0.0

dudx(2,2)=0.0

dudx(3,2) = 0.0

dtdx(2)=0.0

d2tdx2(2) = 0.0

do idimn = 1,ndimn

d2udx2(1,idimn,2) = 0.0

d2udx2(2,idimn,2) = 0.0

d2udx2(3,idimn,2) = 0.0

d2udx2(1,2,idimn) = 0.0

d2udx2(2,2,idimn) = 0.0

d2udx2(3,2,idimn) = 0.0

enddo

do iequa=1,nequa

dundx(2,iequa)=0.0

dfxdx(2,iequa)=0.0

enddo

!

Case(7)   !  Normal derivatives on top surface = 0

dudx(1,3)=0.0

dudx(2,3)=0.0

dudx(3,3) = 0.0

dtdx(3)=0.0

d2tdx2(3) = 0.0

do idimn = 1,ndimn

d2udx2(1,idimn,3) = 0.0

d2udx2(2,idimn,3) = 0.0

d2udx2(3,idimn,3) = 0.0

d2udx2(1,3,idimn) = 0.0

d2udx2(2,3,idimn) = 0.0

d2udx2(3,3,idimn) = 0.0

enddo

do iequa=1,nequa

dundx(3,iequa)=0.0

dfxdx(3,iequa)=0.0

enddo

!

Case(8)   ! Compression Corner exit, derivatives in Flow Dir 0

dudx(1,2)=0.0

dudx(2,2)=0.0

dudx(3,2) = 0.0

dtdx(2)=0.0

tan_a=tan(alpa)

!        tan_a = velo(2)/velo(1)

do iequa=1,nequa

dundx(2,iequa)=0.0

dfxdx(2,iequa)=0.0

dfydz(iequa)=0.0

enddo

!

do inode = 1, ndbou

ipoin = iel_nod(inode)

if(inode==1) lnode=1

if(inode==2) lnode=2

if(inode==3) lnode=6

if(inode==4) lnode=5

ipp1=lnod(lnode,ielno)

!           tan_a=tan_a + phi(inode,ig)*

!     &           (x(2,ipp1)-x(2,ipoin))/(x(1,ipp1)-x(1,ipoin))

do iequa=1,nequa

dfydz(iequa)=dfydz(iequa)+dshpdx(3,inode,ig)*     &

&                        f(ipoin,2,iequa)

enddo

enddo

!

dudx(1,2)=-dudx(1,3)*tan_a

dudx(2,2)=-dudx(2,3)*tan_a

dudx(3,2) = -dudx(3,3)*tan_a

dtdx(2)=-dtdx(3)*tan_a

!

do iequa=1,nequa

dundx(2,iequa)=-dundx(3,iequa)*tan_a

dfxdx(2,iequa)=-dfydz(iequa)*tan_a

enddo

!

!

Case(9)   ! Triple Shock exit, derivatives in Flow Dir 0

dudx(1,2)=0.0

dudx(2,2)=0.0

dudx(3,2) = 0.0

dtdx(2)=0.0

pi= acos(-1.)

angl_b = 15.*pi/180.

tan_a=tan(alpa)

tan_b = tan(angl_b)

do iequa=1,nequa

dundx(2,iequa)=0.0

dfxdx(2,iequa)=0.0

dfydx(iequa) = 0.0

dfydz(iequa)=0.0

enddo

!

do inode = 1, ndbou

ipoin = iel_nod(inode)

if(inode==1) lnode=1

if(inode==2) lnode=2

if(inode==3) lnode=6

if(inode==4) lnode=5

ipp1=lnod(lnode,ielno)

do iequa=1,nequa

dfydx(iequa) = dfydx(iequa) + dshpdx(1,inode,ig)* &

f(ipoin,2,iequa)

dfydz(iequa)=dfydz(iequa)+dshpdx(3,inode,ig)*     &

&                        f(ipoin,2,iequa)

enddo

enddo

!

dudx(1,2) = -dudx(1,1)*tan_b - dudx(1,3)*tan_a

dudx(2,2) = -dudx(2,1)*tan_b - dudx(2,3)*tan_a

dudx(3,2) = -dudx(3,1)*tan_b - dudx(3,3)*tan_a

dtdx(2) = -dtdx(1)*tan_b - dtdx(3)*tan_a

!

do iequa=1,nequa

dundx(2,iequa) = -dundx(1,iequa)*tan_b - dundx(3,iequa)*tan_a

dfxdx(2,iequa) = -dfydx(iequa)*tan_b - dfydz(iequa)*tan_a

enddo

!

Case(5)   !  Compression corner exit, non-zero gradients

!

x1 = 0.0

y1 = 0.0

z1 = 0.0

do inode = 1,ndbou

ipoin = iel_nod(inode)

x1 = x1 + phi(inode,ig)*x(1,ipoin)

y1 = y1 + phi(inode,ig)*x(2,ipoin)

z1 = z1 + phi(inode,ig)*x(3,ipoin)

enddo

ipp1 = lnod(1,ielno)

jpp1 = lnod(2,ielno)

dx1a = x1 - x(1,ipp1)

dx1b = x(1,jpp1) - x1

dx1 = x(1,jpp1) - x(1,ipp1)

u1a = (vel(1,ipp1)*dx1a + vel(1,jpp1)*dx1b)/dx1

v1a = (vel(2,ipp1)*dx1a + vel(2,jpp1)*dx1b)/dx1

w1a = (vel(3,ipp1)*dx1a + vel(3,jpp1)*dx1b)/dx1

rho1a = (dense(ipp1)*dx1a + dense(jpp1)*dx1b)/dx1

t1a = (tempr(ipp1)*dx1a + tempr(jpp1)*dx1b)/dx1

p1a = (press(ipp1)*dx1a + press(jpp1)*dx1b)/dx1

!

ipp1 = lnod(5,ielno)

jpp1 = lnod(6,ielno)

dx2a = x1 - x(1,ipp1)

dx2b = x(1,jpp1) - x1

dx2 = x(1,jpp1) - x(1,ipp1)

u1b = (vel(1,ipp1)*dx2a + vel(1,jpp1)*dx2b)/dx2

v1b = (vel(2,ipp1)*dx2a + vel(2,jpp1)*dx2b)/dx2

w1b = (vel(3,ipp1)*dx2a + vel(3,jpp1)*dx2b)/dx2

rho1b = (dense(ipp1)*dx2a + dense(jpp1)*dx2b)/dx2

t1b = (tempr(ipp1)*dx2a + tempr(jpp1)*dx2b)/dx2

p1b = (press(ipp1)*dx2a + press(jpp1)*dx2b)/dx2

!

ipp1 = lnod(1,ielno)

jpp1 = lnod(5,ielno)

dz1 = z1 - x(3,ipp1)

dz2 = x(3,jpp1) - z1

dz = x(3,jpp1) - x(3,ipp1)

u1 = (u1a*dz1 + u1b*dz2)/dz

v1 = (v1a*dz1 + v1b*dz2)/dz

w1 = (w1a*dz1 + w1b*dz2)/dz

r1 = (rho1a*dz1 + rho1b*dz2)/dz

t1 = (t1a*dz1 + t1b*dz2)/dz

p1 = (p1a*dz1 + p1b*dz2)/dz

!

P = unkn(1)*cgas*T

ipoin = lnod(1,ielno)

dy = y1 - x(1,ipoin)

dudx(1,2) = (velo(1) - u1)/dy

dudx(2,2) = (velo(2) - v1)/dy

dudx(3,2) = (velo(3) - w1)/dy

dtdx(2) = (T - t1)/dy

drdy = (unkn(1) - r1)/dy

dprdy = (P -p1)/dy

!

dundx(2,1) = drdy

dundx(2,2) = velo(1)*drdy + unkn(1)*dudx(1,2)

dundx(2,3) = velo(2)*drdy + unkn(1)*dudx(2,2)

dundx(2,4) = velo(3)*drdy + unkn(1)*dudx(3,2)

dundx(2,5) = E*drdy +unkn(1)*(cv*dtdx(2) +   &

velo(1)*dudx(1,2) + velo(2)*dudx(2,2) +   &

velo(3)*dudx(3,2))

!

dfxdx(2,1) = dundx(2,3)

dfxdx(2,2) = velo(1)*velo(2)*drdy + unkn(1)*(velo(1)*  &

dudx(2,2) + velo(2)*dudx(1,2))

dfxdx(2,3) = velo(2)*velo(2)*drdy + unkn(1)*2.*velo(2)* &

dudx(2,2) + dprdy

dfxdx(2,4) = velo(2)*velo(3)*drdy + unkn(1)*(velo(2)*  &

dudx(3,2) + velo(3)*dudx(2,2))

dfxdx(2,5) = (unkn(5) + P)*dudx(2,2) + velo(2)*(dundx(2,5)&

+ dprdy)

!

End Select

!

return

end subroutine aplbndcnd

!

!   *******************************************************************

!     Subroutine to apply boundary conditions to  2-D Jacobians

!

Subroutine jacbnc2d(ndimn,nequa,ibndcnd,adiabatic,hk,vis,  &

&unkn,dundx,E,reinv,tau,velo,aj,bj,cj)

!

!DIR$ INLINEALWAYS jacbnc2d

!

Use thermo_propt

!

Implicit None

!

integer,intent(in) :: ndimn,nequa,ibndcnd

logical,intent(in) :: adiabatic

real,intent(in) :: E,hk,reinv,vis

real,intent(in),dimension(ndimn,ndimn) :: tau

real,intent(in),dimension(nequa) :: unkn

real,intent(in),dimension(ndimn,nequa) :: dundx

real,intent(in),dimension(ndimn) :: velo

real,intent(inout),dimension(ndimn,nequa,nequa) :: aj,bj

real,intent(inout),dimension(ndimn,ndimn,nequa,nequa) :: cj

!

integer :: iequa,jequa

real :: r,ru,rv,re,ru2,rv2,uvel,vvel,u2,v2

real :: drdx,dldx,dmdx,dedx,drdy,dldy,dmdy,dedy

real :: rinv,r2inv,ckor2cv,cvisr

!

r = unkn(1)

ru = unkn(2)

rv = unkn(3)

re = unkn(4)

uvel = velo(1)

vvel = velo(2)

u2 = uvel*uvel

v2 = vvel*vvel

ru2 = ru*ru

rv2 = rv*rv

!

drdx = dundx(1,1)

dldx = dundx(1,2)

dmdx = dundx(1,3)

dedx = dundx(1,4)

drdy = dundx(2,1)

dldy = dundx(2,2)

dmdy = dundx(2,3)

dedy = dundx(2,4)

!

rinv = 1./r

r2inv = rinv/r

CKOR2CV = HK*r2inv/CV

CVISR = 4./3.*VIS

!

Select Case(ibndcnd)

Case(2,12)   ! Symmetry, v=tau12=dT/dn=0

AJ(1,3,1)=0.0

AJ(1,3,2)=0.0

AJ(1,3,3)=0.0

AJ(2,1,3)=0.0

AJ(2,2,1)=0.0

AJ(2,2,2)=0.0

AJ(2,2,3)=0.0

AJ(2,3,1)=gamm1/2.*velo(1)*velo(1)

AJ(2,3,2)=-gamm1*velo(1)

AJ(2,3,3)=0.0

AJ(2,3,4)=gamm1

AJ(2,4,1)=0.0

AJ(2,4,2)=0.0

AJ(2,4,3)=0.0

AJ(2,4,4)=0.0

BJ(1,3,1)=0.0

BJ(1,3,2)=0.0

BJ(1,3,3)=0.0

BJ(1,4,1)=velo(1)*BJ(1,2,1)+(unkn(2)/unkn(1)**2*TAU(1,1)    &

&          -ckor2cv*(-dundx(1,4)+2*velo(1)*dundx(1,2)        &

&                    +(2*E-3*velo(1)**2)*dundx(1,1)))*reinv

BJ(1,4,2)=velo(1)*BJ(1,2,2) + (-tau(1,1)/unkn(1)              &

&          -ckor2cv*(-dundx(1,2)+2.*velo(1)*dundx(1,1)))*reinv

BJ(1,4,3)=velo(1)*BJ(1,2,3)+ckor2cv*dundx(1,3)*reinv

BJ(1,4,4)=ckor2cv*dundx(1,1)*reinv

BJ(2,2,1)=0.0

BJ(2,2,2)=0.0

BJ(2,2,3)=0.0

BJ(2,4,1)=0.0

BJ(2,4,2)=0.0

BJ(2,4,3)=0.0

BJ(2,4,4)=0.0

CJ(1,1,3,1)=0.0

CJ(1,1,3,3)=0.0

CJ(1,1,4,1)=(cvisr*velo(1)*velo(1)/unkn(1)                 &

&        -HK/cv*(-E/unkn(1)+velo(1)*velo(1)/unkn(1)))*reinv

CJ(1,1,4,3)=0.0

CJ(1,2,3,1)=0.0

CJ(1,2,3,2)=0.0

CJ(1,2,4,1)=0.0

CJ(1,2,4,2)=0.0

CJ(2,1,2,1)=0.0

CJ(2,1,2,3)=0.0

CJ(2,1,4,1)=0.0

CJ(2,1,4,2)=0.0

CJ(2,1,4,3)=0.0

CJ(2,2,2,1)=0.0

CJ(2,2,2,2)=0.0

CJ(2,2,4,1)=0.0

CJ(2,2,4,2)=0.0

CJ(2,2,4,3)=0.0

CJ(2,2,4,4)=0.0

!

Case(1)    ! No-slip wall, u=v=0, T=Twall or dT/dn=0

do iequa=1,nequa

do jequa=1,nequa

AJ(1,iequa,jequa)=0.0

AJ(2,iequa,jequa)=0.0

enddo

enddo

AJ(1,2,4)=gamm1

AJ(2,3,4)=gamm1

BJ(1,4,1)=-ckor2cv*(-dundx(1,4)+2*E*dundx(1,1))*reinv

BJ(1,4,2)=ckor2cv*dundx(1,2)*reinv

BJ(1,4,3)=ckor2cv*dundx(1,3)*reinv

BJ(1,4,4)=ckor2cv*dundx(1,1)*reinv

BJ(2,4,1)=-ckor2cv*(-dundx(2,4)+2*E*dundx(2,1))*reinv

BJ(2,4,2)=ckor2cv*dundx(2,2)*reinv

BJ(2,4,3)=ckor2cv*dundx(2,3)*reinv

BJ(2,4,4)=ckor2cv*dundx(2,1)*reinv

! -for no heat flux

if(adiabatic) then

do iequa=1,nequa

BJ(2,4,iequa)=0.0

enddo

endif

!

CJ(1,1,4,1)=HK*E/unkn(1)/cv*reinv

CJ(1,1,4,2)=0.0

CJ(1,1,4,3)=0.0

CJ(1,1,4,4)=-HK/Unkn(1)/cv*reinv

CJ(1,2,4,1)=0.0

CJ(1,2,4,2)=0.0

CJ(1,2,4,3)=0.0

CJ(2,1,4,1)=0.0

CJ(2,1,4,2)=0.0

CJ(2,1,4,3)=0.0

CJ(2,2,4,1)=CJ(1,1,4,1)

CJ(2,2,4,2)=0.0

CJ(2,2,4,3)=0.0

CJ(2,2,4,4)=CJ(1,1,4,4)

! -for no heat flux

if(adiabatic) then

CJ(2,2,4,1)=0.0

CJ(2,2,4,4)=0.0

endif

!

Case(4)  ! No-slip compression corner, u=v=0, T=Twall or dT/dn=0

do iequa=1,nequa

do jequa=1,nequa

AJ(1,iequa,jequa)=0.0

AJ(2,iequa,jequa)=0.0

enddo

enddo

AJ(1,2,4)=gamm1

AJ(2,3,4)=gamm1

BJ(1,4,1)=-ckor2cv*(-dundx(1,4)+2*E*dundx(1,1))*reinv

BJ(1,4,2)=ckor2cv*dundx(1,2)*reinv

BJ(1,4,3)=ckor2cv*dundx(1,3)*reinv

BJ(1,4,4)=ckor2cv*dundx(1,1)*reinv

BJ(2,4,1)=-ckor2cv*(-dundx(2,4)+2*E*dundx(2,1))*reinv

BJ(2,4,2)=ckor2cv*dundx(2,2)*reinv

BJ(2,4,3)=ckor2cv*dundx(2,3)*reinv

BJ(2,4,4)=ckor2cv*dundx(2,1)*reinv

!

CJ(1,1,4,1)=HK*E/unkn(1)/cv*reinv

CJ(1,1,4,2)=0.0

CJ(1,1,4,3)=0.0

CJ(1,1,4,4)=-HK/Unkn(1)/cv*reinv

CJ(1,2,4,1)=0.0

CJ(1,2,4,2)=0.0

CJ(1,2,4,3)=0.0

CJ(2,1,4,1)=0.0

CJ(2,1,4,2)=0.0

CJ(2,1,4,3)=0.0

CJ(2,2,4,1)=CJ(1,1,4,1)

CJ(2,2,4,2)=0.0

CJ(2,2,4,3)=0.0

CJ(2,2,4,4)=CJ(1,1,4,4)

!

Case(15) ! Top surface of Driven Cavity

AJ(1,3,1)=0.0

AJ(1,3,2)=0.0

AJ(1,3,3)=0.0

AJ(2,1,3)=0.0

AJ(2,2,1)=0.0

AJ(2,2,2)=0.0

AJ(2,2,3)=0.0

AJ(2,3,1)=gamm1/2.*velo(1)*velo(1)

AJ(2,3,2)=-gamm1*velo(1)

AJ(2,3,3)=0.0

AJ(2,3,4)=gamm1

AJ(2,4,1)=0.0

AJ(2,4,2)=0.0

AJ(2,4,3)=0.0

AJ(2,4,4)=0.0

!

Case(11,21)   ! No-slip side walls on Driven Cavity

do iequa=1,nequa

do jequa=1,nequa

AJ(1,iequa,jequa)=0.0

AJ(2,iequa,jequa)=0.0

enddo

enddo

AJ(1,2,4)=gamm1

AJ(2,3,4)=gamm1

BJ(1,4,1)=-ckor2cv*(-dundx(1,4)+2*E*dundx(1,1))*reinv

BJ(1,4,2)=ckor2cv*dundx(1,2)*reinv

BJ(1,4,3)=ckor2cv*dundx(1,3)*reinv

BJ(1,4,4)=ckor2cv*dundx(1,1)*reinv

BJ(2,4,1)=-ckor2cv*(-dundx(2,4)+2*E*dundx(2,1))*reinv

BJ(2,4,2)=ckor2cv*dundx(2,2)*reinv

BJ(2,4,3)=ckor2cv*dundx(2,3)*reinv

BJ(2,4,4)=ckor2cv*dundx(2,1)*reinv

! -for no heat flux

if(adiabatic) then

do iequa=1,nequa

BJ(1,4,iequa)=0.0

enddo

endif

!

CJ(1,1,4,1)=HK*E/unkn(1)/cv*reinv

CJ(1,1,4,2)=0.0

CJ(1,1,4,3)=0.0

CJ(1,1,4,4)=-HK/Unkn(1)/cv*reinv

CJ(1,2,4,1)=0.0

CJ(1,2,4,2)=0.0

CJ(1,2,4,3)=0.0

CJ(2,1,4,1)=0.0

CJ(2,1,4,2)=0.0

CJ(2,1,4,3)=0.0

CJ(2,2,4,1)=CJ(1,1,4,1)

CJ(2,2,4,2)=0.0

CJ(2,2,4,3)=0.0

CJ(2,2,4,4)=CJ(1,1,4,4)

! -for no heat flux

if(adiabatic) then

CJ(1,1,4,1)=0.0

CJ(1,1,4,4)=0.0

endif

!

End Select

!

return

end subroutine jacbnc2d

!

!   *******************************************************************

!     Subroutine to apply boundary conditions to 3-D Jacobians

!

Subroutine jacbndcnd(ndimn,nequa,ibndcnd,adiabatic,hk,vis,  &

&unkn,dundx,E,reinv,tau,velo,aj,bj,cj)

!

!DIR$ INLINEALWAYS jacbndcnd

!

Use thermo_propt

!

Implicit None

!

integer,intent(in) :: ndimn,nequa,ibndcnd

logical,intent(in) :: adiabatic

real,intent(in) :: E,hk,reinv,vis

real,intent(in),dimension(ndimn,ndimn) :: tau

real,intent(in),dimension(nequa) :: unkn

real,intent(in),dimension(ndimn,nequa) :: dundx

real,intent(in),dimension(ndimn) :: velo

real,intent(inout),dimension(ndimn,nequa,nequa) :: aj,bj

real,intent(inout),dimension(ndimn,ndimn,nequa,nequa) :: cj

!

integer :: iequa,jequa

real :: r,ru,rv,rw,re,ru2,rv2,rw2,uvel,vvel,wvel,u2,v2,w2

real :: drdx,dldx,dmdx,dndx,dedx,drdy,dldy,dmdy,dndy,dedy

real :: drdz,dldz,dmdz,dndz,dedz,rinv,r2inv,ckor2cv,cvisr

!

r = unkn(1)

ru = unkn(2)

rv = unkn(3)

rw = unkn(4)

re = unkn(5)

uvel = velo(1)

vvel = velo(2)

wvel = velo(3)

u2 = uvel*uvel

v2 = vvel*vvel

w2 = wvel*wvel

ru2 = ru*ru

rv2 = rv*rv

rw2 = rw*rw

!

drdx = dundx(1,1)

dldx = dundx(1,2)

dmdx = dundx(1,3)

dndx = dundx(1,4)

dedx = dundx(1,5)

drdy = dundx(2,1)

dldy = dundx(2,2)

dmdy = dundx(2,3)

dndy = dundx(2,4)

dedy = dundx(2,5)

drdz = dundx(3,1)

dldz = dundx(3,2)

dmdz = dundx(3,3)

dndz = dundx(3,4)

dedz = dundx(3,5)

!

rinv = 1./r

r2inv = rinv/r

CKOR2CV = HK*r2inv/CV

CVISR = 4./3.*VIS

!

Select Case(ibndcnd)

Case(2,12)   !  Symmetry in x-y plane, w=0,tau13=tau23=q3=0

!

AJ(1,4,1)=0.0

AJ(1,4,2)=0.0

AJ(1,4,3)=0.0

AJ(1,4,4) = 0.0

AJ(2,4,1)=0.0

AJ(2,4,2)=0.0

AJ(2,4,3)=0.0

AJ(2,4,4) = 0.0

do iequa = 1,nequa

do jequa = 1,nequa

aj(3,iequa,jequa) = 0.0

enddo

enddo

!

aj(3,4,1) = gamm1/2*(u2 + v2)

aj(3,4,2) = -gamm1*uvel

aj(3,4,3) = -gamm1*vvel

aj(3,4,5) = gamm1

BJ(1,4,1)=0.0

BJ(1,4,2)=0.0

BJ(1,4,3)=0.0

BJ(1,4,4)=0.0

BJ(1,5,1)=uvel*BJ(1,2,1)+vvel*BJ(1,3,1)+(rinv*(uvel*tau(1,1) +   &

&          vvel*TAU(1,2))-ckor2cv*(-dedx + 2.*uvel*dldx + 2.*vvel*&

&          dmdx + (2.*E - 3.*u2 - 3.*v2)*drdx))*reinv

BJ(1,5,2)= uvel*BJ(1,2,2) + vvel*BJ(1,3,2) +    &

&          (-tau(1,1)*rinv-ckor2cv*(-dldx + 2.*uvel*drdx))*reinv

BJ(1,5,3)=uvel*BJ(1,2,3) + vvel*BJ(1,3,3) + (-TAU(1,2)*rinv     &

&          -ckor2cv*(2.*vvel*drdx - dmdx))*reinv

BJ(1,5,4)=uvel*BJ(1,2,4) + vvel*BJ(1,3,4) + ckor2cv*dndx*reinv

BJ(1,5,5)=ckor2cv*drdx*reinv

BJ(2,4,1)=0.0

BJ(2,4,2)=0.0

BJ(2,4,3)=0.0

BJ(2,4,4)=0.0

BJ(2,5,1)=uvel*BJ(2,2,1) + vvel*BJ(2,3,1)+(rinv*(uvel*tau(2,1)    &

&          + vvel*TAU(2,2)) - ckor2cv*(-dedy + (2.*E-3.*u2 - 3.*v2)&

&          *drdy + 2.*uvel*dldy + 2.*vvel*dmdy))*reinv

BJ(2,5,2)=uvel*BJ(2,2,2) + vvel*BJ(2,3,2) + (-TAU(2,1)*rinv      &

&          - ckor2cv*(2.*uvel*drdy-dldy))*reinv

BJ(2,5,3)=uvel*BJ(2,2,3) + vvel*BJ(2,3,3) + (-TAU(2,2)*rinv      &

&          - ckor2cv*(2.*vvel*drdy - dmdy))*reinv

BJ(2,5,4)=uvel*BJ(2,2,4) + vvel*BJ(2,3,4) + ckor2cv*dndy*reinv

BJ(3,2,1)=0.0

BJ(3,2,2)=0.0

BJ(3,2,3)=0.0

BJ(3,2,4)=0.0

BJ(3,3,1)=0.0

BJ(3,3,2)=0.0

BJ(3,3,3)=0.0

BJ(3,3,4)=0.0

BJ(3,5,1)=0.0

BJ(3,5,2)=0.0

BJ(3,5,3)=0.0

BJ(3,5,4)=0.0

BJ(3,5,5)=0.0

CJ(1,1,4,1)=0.0

CJ(1,1,4,3)=0.0

CJ(1,1,4,4)=0.0

CJ(1,1,5,1)=uvel*CJ(1,1,2,1) + vvel*CJ(1,1,3,1) - ckor2cv*   &

&            (-re + (ru2 + rv2)*rinv)*reinv

CJ(1,1,5,4)=0.0

CJ(1,2,4,1)=0.0

CJ(1,2,4,2)=0.0

CJ(1,2,4,3)=0.0

CJ(1,2,4,4)=0.0

CJ(1,3,4,1)=0.0

CJ(1,3,4,2)=0.0

CJ(1,3,4,3)=0.0

Cj(1,3,4,4)=0.0

CJ(1,3,5,1)=uvel*CJ(1,3,2,1)

CJ(1,3,5,2)=0.0

CJ(2,1,4,1)=0.0

CJ(2,1,4,2)=0.0

CJ(2,1,4,3)=0.0

CJ(2,1,4,4)=0.0

CJ(2,2,4,1)=0.0

CJ(2,2,4,2)=0.0

CJ(2,2,4,3)=0.0

CJ(2,2,4,4)=0.0

CJ(2,2,5,1)=uvel*CJ(2,2,2,1) + vvel*CJ(2,2,3,1) - ckor2cv*  &

&            (-re + (ru2 + rv2)*rinv)*reinv

CJ(2,2,5,4)=0.0

CJ(2,3,4,1)=0.0

CJ(2,3,4,2)=0.0

CJ(2,3,4,3)=0.0

CJ(2,3,4,4)=0.0

CJ(2,3,5,1)=vvel*CJ(2,3,3,1)

CJ(2,3,5,3)=0.0

CJ(3,1,2,1)=0.0

CJ(3,1,2,2)=0.0

CJ(3,1,2,3)=0.0

CJ(3,1,2,4)=0.0

CJ(3,1,3,1)=0.0

CJ(3,1,3,2)=0.0

CJ(3,1,3,3)=0.0

CJ(3,1,3,4)=0.0

CJ(3,1,5,1)=0.0

CJ(3,1,5,2)=0.0

CJ(3,1,5,3)=0.0

CJ(3,1,5,4)=0.0

CJ(3,1,5,5)=0.0

CJ(3,2,2,1)=0.0

CJ(3,2,2,2)=0.0

CJ(3,2,2,3)=0.0

CJ(3,2,2,4)=0.0

CJ(3,2,3,1)=0.0

CJ(3,2,3,2)=0.0

CJ(3,2,3,3)=0.0

CJ(3,2,3,4)=0.0

CJ(3,2,5,1)=0.0

CJ(3,2,5,2)=0.0

CJ(3,2,5,3)=0.0

CJ(3,2,5,4)=0.0

CJ(3,2,5,5)=0.0

CJ(3,3,2,1)=0.0

CJ(3,3,2,2)=0.0

CJ(3,3,2,3)=0.0

CJ(3,3,2,4)=0.0

CJ(3,3,3,1)=0.0

CJ(3,3,3,2)=0.0

CJ(3,3,3,3)=0.0

CJ(3,3,3,4)=0.0

CJ(3,3,5,1)=0.0

CJ(3,3,5,2)=0.0

CJ(3,3,5,3)=0.0

CJ(3,3,5,4)=0.0

CJ(3,3,5,5)=0.0

!

Case(22)  ! Symmetry in y-z plane, u=0, tau21=tau31=q1=0

do iequa = 1,nequa

do jequa = 1,nequa

aj(1,iequa,jequa) = 0.0

enddo

enddo

!

AJ(1,2,1)=gamm1/2.*(v2 + w2)

AJ(1,2,2)=0.0

AJ(1,2,3)=-gamm1*vvel

AJ(1,2,4) = -gamm1*wvel

AJ(1,2,5) = gamm1

AJ(2,2,1)=0.0

AJ(2,2,2)=0.0

AJ(2,2,3)=0.0

AJ(2,2,4) = 0.0

aj(3,2,1) = 0.0

aj(3,2,2) = 0.0

aj(3,2,3) = 0.0

aj(3,2,4) = 0.0

BJ(1,3,1)=0.0

BJ(1,3,2)=0.0

BJ(1,3,3)=0.0

BJ(1,3,4) = 0.0

BJ(1,4,1)=0.0

BJ(1,4,2)=0.0

BJ(1,4,3)=0.0

BJ(1,4,4)=0.0

BJ(1,5,1)=0.0

BJ(1,5,2)=0.0

BJ(1,5,3)=0.0

BJ(1,5,4)=0.0

BJ(1,5,5)=0.0

BJ(2,2,1)=0.0

BJ(2,2,2)=0.0

BJ(2,2,3)=0.0

BJ(2,2,4)=0.0

BJ(2,5,1)=vvel*BJ(2,3,1)+wvel*BJ(2,4,1)+(rinv*(vvel*    &

&           TAU(2,2)+wvel*TAU(2,3))-ckor2cv*(-dedy +	  &

&           (2.*E - 3.*v2 - 3.*w2)*drdy				  &

&           +2.*vvel*dmdy + 2.*wvel*dndy))*reinv

BJ(2,5,2)=vvel*BJ(2,3,2) + wvel*BJ(2,4,2) + ckor2cv*dldy*reinv

BJ(2,5,3)=vvel*BJ(2,3,3) + wvel*BJ(2,4,3) + (-TAU(2,2)*rinv   &

&          - ckor2cv*(2.*vvel*drdy - dmdy))*reinv

BJ(2,5,4)=vvel*BJ(2,3,4) + wvel*BJ(2,4,4) + (-TAU(2,3)*rinv   &

&          - ckor2cv*(2.*wvel*drdy - dndy))*reinv

BJ(3,2,1)=0.0

BJ(3,2,2)=0.0

BJ(3,2,3)=0.0

BJ(3,2,4)=0.0

BJ(3,5,1)=vvel*BJ(3,3,1) + wvel*BJ(3,4,1) + (rinv*(vvel         &

&          *TAU(3,2) + wvel*TAU(3,3)) - ckor2cv*(-dedz	 &

&          + (2.*E - 3.*v2 - 3.*w2)*drdz + 2.*vvel*dmdz + 2.*wvel*&

&          dndz))*reinv

BJ(3,5,2)=vvel*BJ(3,3,2)+wvel*BJ(3,4,2) + ckor2cv*dldz*reinv

BJ(3,5,3)= vvel*bj(3,3,3) + wvel*bj(3,4,3) +   &

&          (-tau(2,3)*rinv - ckor2cv*(-dmdz + 2.*vvel*drdz))*reinv

BJ(3,5,4)=vvel*BJ(3,3,4) + wvel*BJ(3,4,4) + (-TAU(3,3)*rinv     &

&          - ckor2cv*(2.*wvel*drdz - dndz))*reinv

CJ(1,1,3,1)=0.0

CJ(1,1,3,2)=0.0

CJ(1,1,3,3)=0.0

CJ(1,1,3,4)=0.0

CJ(1,1,4,1)=0.0

CJ(1,1,4,2)=0.0

CJ(1,1,4,3)=0.0

CJ(1,1,4,4)=0.0

CJ(1,1,5,1)=0.0

CJ(1,1,5,2)=0.0

CJ(1,1,5,3)=0.0

CJ(1,1,5,4)=0.0

CJ(1,1,5,5)=0.0

CJ(1,2,3,1)=0.0

CJ(1,2,3,2)=0.0

CJ(1,2,3,3)=0.0

CJ(1,2,3,4)=0.0

CJ(1,2,4,1)=0.0

CJ(1,2,4,2)=0.0

CJ(1,2,4,3)=0.0

CJ(1,2,4,4)=0.0

CJ(1,2,5,1)=0.0

CJ(1,2,5,2)=0.0

CJ(1,2,5,3)=0.0

CJ(1,2,5,4)=0.0

CJ(1,2,5,5)=0.0

CJ(1,3,3,1)=0.0

CJ(1,3,3,2)=0.0

CJ(1,3,3,3)=0.0

CJ(1,3,3,4)=0.0

CJ(1,3,4,1)=0.0

CJ(1,3,4,2)=0.0

CJ(1,3,4,3)=0.0

CJ(1,3,4,4)=0.0

CJ(1,3,5,1)=0.0

CJ(1,3,5,2)=0.0

CJ(1,3,5,3)=0.0

CJ(1,3,5,4)=0.0

CJ(1,3,5,5)=0.0

CJ(2,1,2,1)=0.0

CJ(2,1,2,2)=0.0

CJ(2,1,2,3)=0.0

CJ(2,1,2,4)=0.0

CJ(2,1,5,1)=vvel*CJ(2,1,3,1)

CJ(2,1,5,3)=0.0

CJ(2,2,2,1)=0.0

CJ(2,2,2,2)=0.0

CJ(2,2,2,3)=0.0

CJ(2,2,2,4)=0.0

CJ(2,2,5,1)=vvel*CJ(2,2,3,1) + wvel*CJ(2,2,4,1) - ckor2cv*    &

&            (-re + (rv2 + rw2)*rinv)*reinv

CJ(2,2,5,2) = 0.0

CJ(2,3,2,1)=0.0

CJ(2,3,2,2)=0.0

CJ(2,3,2,3)=0.0

CJ(2,3,2,4)=0.0

CJ(3,1,2,1)=0.0

CJ(3,1,2,2)=0.0

CJ(3,1,2,3)=0.0

CJ(3,1,2,4)=0.0

CJ(3,1,5,1) = wvel*CJ(3,1,4,1)

CJ(3,1,5,4)=0.0

CJ(3,2,2,1)=0.0

CJ(3,2,2,2)=0.0

CJ(3,2,2,3)=0.0

CJ(3,2,2,4)=0.0

CJ(3,3,2,1)=0.0

CJ(3,3,2,2)=0.0

CJ(3,3,2,3)=0.0

CJ(3,3,2,4)=0.0

CJ(3,3,5,1)=vvel*CJ(3,3,3,1) + wvel*CJ(3,3,4,1) - ckor2cv*   &

&            (-re + (rv2 + rw2)*rinv)*reinv

CJ(3,3,5,2)=0.0

!

Case(1,11,21,31,41)  ! No-slip wall,u=v=w=0,T=Twall or dT/dn = 0

!

aj(1:ndimn,1:nequa,1:nequa) = 0.0

!

AJ(1,2,5)=gamm1

AJ(2,3,5)=gamm1

aj(3,4,5) = gamm1

BJ(1,5,1)=-ckor2cv*(-dedx+2*E*drdx)*reinv

BJ(1,5,2)=ckor2cv*dldx*reinv

BJ(1,5,3)=ckor2cv*dmdx*reinv

bj(1,5,4)=ckor2cv*dndx*reinv

BJ(1,5,5)=ckor2cv*drdx*reinv

BJ(2,5,1)=-ckor2cv*(-dedy+2*E*drdy)*reinv

BJ(2,5,2)=ckor2cv*dldy*reinv

BJ(2,5,3)=ckor2cv*dmdy*reinv

BJ(2,5,4)=ckor2cv*dndy*reinv

BJ(2,5,5)=ckor2cv*drdy*reinv

BJ(3,5,1)=-ckor2cv*(-dedz+2*E*drdz)*reinv

BJ(3,5,2)=ckor2cv*dldz*reinv

BJ(3,5,3)=ckor2cv*dmdz*reinv

BJ(3,5,4)=ckor2cv*dndz*reinv

BJ(3,5,5)=ckor2cv*drdz*reinv

! -for no heat flux

if(adiabatic) then

if(ibndcnd==1) bj(3,5,:) = 0.0  !  perpendicular to z-axis

if(ibndcnd==11.or.ibndcnd==21)  &

&      bj(1,5,:) = 0.0              !  perpendicular to x-axis

if(ibndcnd==31.or.ibndcnd==41)  &

&      bj(2,5,:) = 0.0              !  perpendicular ot y-axis

endif

!

CJ(1,1,5,1)=ckor2cv*re*reinv

CJ(1,1,5,2)=0.0

CJ(1,1,5,3)=0.0

CJ(1,1,5,4)=0.0

CJ(1,1,5,5)=-ckor2cv*r*reinv

CJ(1,2,5,1)=0.0

CJ(1,2,5,2)=0.0

CJ(1,2,5,3)=0.0

CJ(1,2,5,4)=0.0

CJ(1,3,5,1)=0.0

CJ(1,3,5,2)=0.0

CJ(1,3,5,4)=0.0

CJ(2,1,5,1)=0.0

CJ(2,1,5,2)=0.0

CJ(2,1,5,3)=0.0

CJ(2,1,5,4)=0.0

CJ(2,2,5,1)=CJ(1,1,5,1)

CJ(2,2,5,2)=0.0

CJ(2,2,5,3)=0.0

CJ(2,2,5,4)=0.0

CJ(2,2,5,5)=CJ(1,1,5,5)

CJ(2,3,5,1)=0.0

CJ(2,3,5,2)=0.0

CJ(2,3,5,3)=0.0

CJ(2,3,5,4)=0.0

CJ(3,1,5,1)=0.0

CJ(3,1,5,2)=0.0

CJ(3,1,5,4)=0.0

CJ(3,2,5,1)=0.0

CJ(3,2,5,3)=0.0

CJ(3,2,5,4)=0.0

CJ(3,3,5,1)=cj(1,1,5,1)

CJ(3,3,5,2)=0.0

CJ(3,3,5,3)=0.0

CJ(3,3,5,4)=0.0

CJ(3,3,5,5)=CJ(1,1,5,5)

! -for no heat flux

if(adiabatic) then

if(ibndcnd==1) then

CJ(3,3,5,1)=0.0

CJ(3,3,5,5)=0.0

elseif(ibndcnd==11.or.ibndcnd==21) then

CJ(1,1,5,1)=0.0

CJ(1,1,5,5)=0.0

elseif(ibndcnd==31.or.ibndcnd==41) then

CJ(2,2,5,1)=0.0

CJ(2,2,5,5)=0.0

endif

endif

!

Case(4,14)  !  No-slip compression corner

!  u=v=w=0,  T=Twall or dT/dn =0

!

aj(1:ndimn,1:nequa,1:nequa) = 0.0

!

AJ(1,2,5)=gamm1

AJ(2,3,5)=gamm1

aj(3,4,5) = gamm1

BJ(1,5,1)=-ckor2cv*(-dedx+2*E*drdx)*reinv

BJ(1,5,2)=ckor2cv*dldx*reinv

BJ(1,5,3)=ckor2cv*dmdx*reinv

bj(1,5,4)=ckor2cv*dndx*reinv

BJ(1,5,5)=ckor2cv*drdx*reinv

BJ(2,5,1)=-ckor2cv*(-dedy+2*E*drdy)*reinv

BJ(2,5,2)=ckor2cv*dldy*reinv

BJ(2,5,3)=ckor2cv*dmdy*reinv

BJ(2,5,4)=ckor2cv*dndy*reinv

BJ(2,5,5)=ckor2cv*drdy*reinv

BJ(3,5,1)=-ckor2cv*(-dedz+2*E*drdz)*reinv

BJ(3,5,2)=ckor2cv*dldz*reinv

BJ(3,5,3)=ckor2cv*dmdz*reinv

BJ(3,5,4)=ckor2cv*dndz*reinv

BJ(3,5,5)=ckor2cv*drdz*reinv

!

CJ(1,1,5,1)=ckor2cv*re*reinv

CJ(1,1,5,2)=0.0

CJ(1,1,5,3)=0.0

CJ(1,1,5,4)=0.0

CJ(1,1,5,5)=-ckor2cv*r*reinv

CJ(1,2,5,1)=0.0

CJ(1,2,5,2)=0.0

CJ(1,2,5,3)=0.0

CJ(1,2,5,4)=0.0

CJ(1,3,5,1)=0.0

CJ(1,3,5,2)=0.0

CJ(1,3,5,4)=0.0

CJ(2,1,5,1)=0.0

CJ(2,1,5,2)=0.0

CJ(2,1,5,3)=0.0

CJ(2,1,5,4)=0.0

CJ(2,2,5,1)=CJ(1,1,5,1)

CJ(2,2,5,2)=0.0

CJ(2,2,5,3)=0.0

CJ(2,2,5,4)=0.0

CJ(2,2,5,5)=CJ(1,1,5,5)

CJ(2,3,5,1)=0.0

CJ(2,3,5,2)=0.0

CJ(2,3,5,3)=0.0

CJ(2,3,5,4)=0.0

CJ(3,1,5,1)=0.0

CJ(3,1,5,2)=0.0

CJ(3,1,5,4)=0.0

CJ(3,2,5,1)=0.0

CJ(3,2,5,3)=0.0

CJ(3,2,5,4)=0.0

CJ(3,3,5,1)=cj(1,1,5,1)

CJ(3,3,5,2)=0.0

CJ(3,3,5,3)=0.0

CJ(3,3,5,4)=0.0

CJ(3,3,5,5)=cj(1,1,5,5)

!

End Select

!

return

end subroutine jacbndcnd

!

!     **************************************************************

!     Subroutine to apply boundary conditions

!

Subroutine bc (nnode,nordr,ndimn,ndbou,nequa,nface,neleq,nblock)

!

Use bnd_cnd

Use thermo_propt

Use grid_info

Use FDV_coef

Use bface_info

Implicit None

!

integer,intent(in) :: nnode,nordr,ndimn,ndbou,nequa,nface

integer,intent(in) :: neleq,nblock

integer :: iface,inode,ielem,ipoin,iordr,ir,jnode

integer :: jpoin,jordr,is,ia,ja,irow,icol,kcol

integer :: iblock,iel,iel0,ielast,nvec,i,iv

integer :: ibpnt,jcol

real :: en1oven2

!

integer :: locnode(ndbou)

real :: dloc(mbsize,meleq)

!

if(ndimn/=1) then

!

do iface = 1,nface

!

do inode = 1,ndbou

if(ndimn.eq.2) then

locnode(inode) = lface(inode+4,iface)

elseif(ndimn.eq.3) then

locnode(inode) = lfacnode(inode,iface)

endif

enddo

!

!     Apply boundary conditions

!

ielem = lface(7,iface)

!

do inode = 1,ndbou

ipoin = lface(inode,iface)

do ir = 1,nequa

iordr = nequa*(ipoin-1) + ir

do jnode = 1,ndbou

jpoin = lface(jnode,iface)

do is = 1,nequa

jordr = nequa*(jpoin-1) + is

if(ibouncnd(jordr).lt.0) then    ! Dirichlet bc

ia = locnode(inode)

ja = locnode(jnode)

irow = nequa*(ia-1) + ir

icol = nequa*(ja-1) + is

!               rhs(iordr) = rhs(iordr) - bel(ielem,irow,icol)*

!     &                      delu(jordr)

bel(ielem,irow,icol) = 0.0

if(irow.eq.icol) then

do kcol = 1,neleq

Bel(ielem,irow,kcol) = 0.0   ! Zero column

enddo

Bel(ielem,irow,icol) = 1.0  ! Diagonal

if(ndimn==2) Bel(ielem,irow,icol) = 0.5

if(ndimn==3) Bel(ielem,irow,icol) = 0.25

rhs(iordr) = 0.0

endif

elseif(ibouncnd(jordr).eq.100) then   ! u=v=0, T = Twall

ia = locnode(inode)

ja = locnode(jnode)

irow = nequa*(ia-1) + ir

icol = nequa*(ja-1) + is

jcol = icol - (ndimn + 1)

if(irow.ne.icol) then

Bel(ielem,irow,jcol) = Bel(ielem,irow,jcol) +      &

&                                 Bel(ielem,irow,icol)*cv*Twall

Bel(ielem,irow,icol) = 0.0

else

do kcol = 1,neleq

Bel(ielem,irow,kcol) = 0.0

enddo

Bel(ielem,irow,jcol) = -cv*Twall

Bel(ielem,irow,icol) = 1.0

if(ndimn==2) Bel(ielem,irow,icol) = 0.5

if(ndimn==3) Bel(ielem,irow,icol) = 0.25

rhs(iordr) = 0.0

endif

elseif(ibouncnd(jordr).eq.5) then  ! Inviscid slip wall

ia = locnode(inode)

ja = locnode(jnode)

irow = nequa*(ia-1) + ir

icol = nequa*(ja-1) + is

jcol = icol - 1

ibpnt = lface(jnode+2,iface)

en1oven2 = -rbcon(2,ibpnt)/rbcon(1,ibpnt)

if(irow.ne.icol) then

Bel(ielem,irow,jcol) = Bel(ielem,irow,jcol) +      &

&                                 Bel(ielem,irow,icol)*en1oven2

Bel(ielem,irow,icol) = 0.0

else

do kcol = 1,neleq

Bel(ielem,irow,kcol) = 0.0

enddo

Bel(ielem,irow,jcol) = en1oven2

Bel(ielem,irow,icol) = 1.0

rhs(iordr) = 0.0

endif

endif

enddo

enddo

!

enddo

enddo

!

enddo

!

endif

!

return

end subroutine bc

!

!  ****************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE ONE-D JACOBIAN MATRICES

!     "A SUB I" FOR A SAMPLE POINT

!

!     THESE MATRICES ARE FOR THE NAVIER-STOKES SYSTEM OF EQUATION

!     WITHOUT PRESSURE CORRECTIONS

!

SUBROUTINE AJACOB1(velo,E,gamma,gamm1,gm3d2,gm1d2,gamm3,aj)

!

!DIR$ INLINEALWAYS AJACOB1

!

Use param_var

!      Use thermo_propt

!

Implicit None

real,intent(in) :: velo(mdimn),E

real,intent(in) :: gamma,gamm1,gm3d2,gm1d2,gamm3

real,intent(out),dimension(mdimn,mequa,mequa) :: aj

real :: v1,v11

!

!     Zero jacobian

aj = 0.0

!

v1 = velo(1)

v11 = v1*v1

!

aj(1,1,2)=1.0

aj(1,2,1)=GM3D2*v11

aj(1,2,2)=-GAMM3*v1

aj(1,2,3)=GAMM1

aj(1,3,1)=-GAMMA*E*v1+GAMM1*v1*v11

aj(1,3,2)=GAMMA*E-GM1D2*(3.*v11)

aj(1,3,3)=GAMMA*v1

!

RETURN

end subroutine AJACOB1

!

!  ****************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE TWO-D JACOBIAN MATRICES

!     "A SUB I" FOR A SAMPLE POINT

!

!     THESE MATRICES ARE FOR THE NAVIER-STOKES SYSTEM OF EQUATION

!     WITHOUT PRESSURE CORRECTIONS

!

SUBROUTINE AJACOB(velo,E,gamma,gamm1,gm3d2,gm1d2,gamm3,aj)

!

!DIR$ INLINEALWAYS AJACOB

!

Use param_var

!      Use thermo_propt

!

Implicit None

real,intent(in) :: velo(mdimn),E

real,intent(in) :: gamma,gamm1,gm3d2,gm1d2,gamm3

real,dimension(mdimn,mequa,mequa),intent(out) :: aj

real :: v1,v2,v11,v12,v22,v11p22

!

!    Zero Jacobian

aj = 0.0

!

v1  = velo(1)

v2  = velo(2)

v11 = v1*v1

v12 = v1*v2

v22 = v2*v2

v11p22 = v11 + v22

!

aj(1,1,2) = 1.0

aj(1,2,1) = gm3d2*v11+gm1d2*v22

aj(1,2,2) =-gamm3*v1

aj(1,2,3) =-gamm1*v2

aj(1,2,4) = gamm1

aj(1,3,1) =-v12

aj(1,3,2) = v2

aj(1,3,3) = v1

aj(1,4,1) =-gamma*e*v1+gamm1*v1*v11p22

aj(1,4,2) = gamma*e-gm1d2*(3.*v11 + v22)

aj(1,4,3) =-gamm1*v12

aj(1,4,4) = gamma*v1

!

aj(2,1,3) = 1.0

aj(2,2,1) =-v12

aj(2,2,2) = v2

aj(2,2,3) = v1

aj(2,3,1) = gm1d2*v11+gm3d2*v22

aj(2,3,2) =-gamm1*v1

aj(2,3,3) =-gamm3*v2

aj(2,3,4) = gamm1

aj(2,4,1) =-gamma*e*v2+gamm1*v2*v11p22

aj(2,4,2) =-gamm1*v12

aj(2,4,3) = gamma*e-gm1d2*(v11+3.*v22)

aj(2,4,4) = gamma*v2

!

RETURN

end subroutine AJACOB

!    **************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE "U" VECTOR AND

!     JACOBIANS AT THE GAUSSIAN POINT OF INTEREST

!

SUBROUTINE EVALDOM(NEQUA,NDIMN,NNODE,IG,vloc,Eloc,Tloc,&

&                   Uloc,Floc,phi,dshpdx,  &

&                   d2phidx,VELO,E,T,UNKN,DUDX,DTDX,DUNDX,CFLUX,&

&                   DFJDXJ,d2tdx2,d2udx2)

!

Use param_var

!      Use flow_var

!      Use thermo_propt

!      Use Conv_flux

!      Use shap_fcns

!

Implicit None

!

integer,intent(in) :: nequa,ndimn,nnode,ig

real,intent(in),dimension(ndimn,nnode) :: vloc

real,intent(in),dimension(nnode) :: Eloc,Tloc

real,intent(in) :: Uloc(nnode,nequa),Floc(nnode,ndimn,nequa)

real,intent(in) :: dshpdx(mdimn,mnode,mgaus),phi(mnode,mgaus)

real,intent(in) :: d2phidx(mdimn,mdimn,mnode,mgaus)

real,intent(out) :: T,E

real,intent(out) :: CFLUX(nDIMN,nEQUA)

REAL,intent(out) :: DUNDX(nDIMN,nEQUA),unkn(nequa),velo(ndimn)

REAL,intent(out) :: DUDX(nDIMN,nDIMN),DTDX(nDIMN)

real,intent(out) :: DFJDXJ(nEQUA)

real,intent(out) :: d2tdx2(ndimn),d2udx2(ndimn,ndimn,ndimn)

real :: DFXDX(ndimn,nEQUA)

integer :: iequa,idimn,jdimn,kdimn,i,L,iordr

integer :: ipoin

!

T = 0.0

E=0.0

!

DO IEQUA = 1, NEQUA

UNKN(IEQUA) = 0.0

do idimn = 1,ndimn

DUNDX(IDIMN,IEQUA) = 0.0

enddo

ENDDO

!

DO IDIMN = 1, NDIMN

VELO(IDIMN) = 0.0

DTDX(IDIMN) = 0.0

D2TDX2(IDIMN) = 0.0

DO JDIMN = 1,NDIMN

DUDX(IDIMN,JDIMN) = 0.0

DO KDIMN = 1,NDIMN

D2UDX2(IDIMN,JDIMN,KDIMN) = 0.0

ENDDO

ENDDO

ENDDO

!

DO I=1,NNODE

E= E + Eloc(I)*PHI(I,IG)

T = T + Tloc(I)*PHI(I,IG)

!

DO IDIMN = 1,NDIMN

DO JDIMN = 1,NDIMN

DUDX(IDIMN,JDIMN)=DUDX(IDIMN,JDIMN) + DSHPDX(JDIMN,I,IG)    &

&                  *Vloc(IDIMN,I)

DO KDIMN = 1,NDIMN

d2udx2(IDIMN,JDIMN,KDIMN) = d2udx2(IDIMN,JDIMN,KDIMN) +  &

&                            d2phidx(JDIMN,KDIMN,i,ig)*vloc(IDIMN,I)

ENDDO

ENDDO

VELO(IDIMN)=VELO(IDIMN)+Vloc(IDIMN,I)*PHI(I,IG)

d2tdx2(IDIMN) = d2tdx2(idimn) + d2phidx(idimn,idimn,i,ig)* &

&                   Tloc(I)

DTDX(IDIMN)=DTDX(IDIMN)+DSHPDX(IDIMN,I,IG)*Tloc(I)

ENDDO

!

DO IEQUA = 1, NEQUA

UNKN(IEQUA)=UNKN(IEQUA)+Uloc(I,IEQUA)*PHI(I,IG)

do idimn = 1,ndimn

DUNDX(idimn,IEQUA) = DUNDX(idimn,IEQUA) + DSHPDX(idimn,I,IG)  &

&                      *Uloc(I,IEQUA)

enddo

ENDDO

enddo

!

DO IEQUA = 1,NEQUA

DO I = 1, NDIMN

CFLUX(I,IEQUA) = 0.0

DFXDX(I,IEQUA) = 0.0

enddo

dfjdxj(iequa) = 0.0

enddo

!

DO IEQUA = 1, NEQUA

DO I = 1, NNODE

do idimn = 1,ndimn

DFXDX(idimn,IEQUA) =DFXDX(idimn,IEQUA) + DSHPDX(idimn,I,ig)*   &

&                        Floc(I,idimn,IEQUA)

CFLUX(idimn,IEQUA) = CFLUX(idimn,IEQUA)+PHI(I,ig)*     &

&                       Floc(I,idimn,IEQUA)

enddo

enddo

enddo

!

DO IEQUA =1, NEQUA

do idimn = 1,ndimn

DFJDXJ(IEQUA) = dfjdxj(iequa) + DFXDX(idimn,IEQUA)

enddo

enddo

!

RETURN

end subroutine evaldom

!

!  ******************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE JACOBIAN "B SUB I"

!     FOR A SAMPLE POINT

!

SUBROUTINE BJACOB(reinv,cv,VIS,HK,TAU,DUNDX,UNKN,VELO,e,bj)

!

!DIR$ INLINEALWAYS BJACOB

!

Use param_var

!      Use thermo_propt

!

Implicit None

!

real,intent(in) :: reinv,vis,hk,e,cv

real,intent(in) :: DUNDX(mdimn,MEQUA),VELO(MDIMN),UNKN(MEQUA)

real,intent(in) :: tau(mdimn,mdimn)

real,dimension(mdimn,mequa,mequa),intent(out) :: bj

real :: vislam,visr,denom,term1

real :: r,v1,v2,v11,v22,v11p22

real :: drdx,dldx,dmdx,dedx,drdy,dldy,dmdy,dedy

!

!     Zero Jacobian

bj = 0.0

!

VISLAM=-2.*VIS/3.

visr=2.*vis+vislam

DENOM=1./UNKN(1)**2

TERM1=HK*DENOM/CV

!

r = unkn(1)

v1 = velo(1)

v2 = velo(2)

v11 = v1*v1

v22 = v2*v2

v11p22 = v11 + v22

!

drdx = dundx(1,1)

dldx = dundx(1,2)

dmdx = dundx(1,3)

dedx = dundx(1,4)

drdy = dundx(2,1)

dldy = dundx(2,2)

dmdy = dundx(2,3)

dedy = dundx(2,4)

!

bj(1,2,1)=(DENOM*(VISR*Dldx+VISLAM*Dmdy-2.*(visr*      &

&         v1*drdx+vislam*v2*drdy)))*reinv

bj(1,2,2)=VISR*drdx*DENOM*reinv

bj(1,2,3)=VISLAM*drdy*DENOM*reinv

bj(1,3,1)=(VIS*DENOM*(dldy+dmdx-2.*(v1*drdy+           &

&         v2*drdx)))*reinv

bj(1,3,2)=VIS*Drdy*DENOM*reinv

bj(1,3,3)=VIS*Drdx*DENOM*reinv

bj(1,4,1)=v1*bj(1,2,1)+v2*bj(1,3,1)+((v1*TAU(1,1)+v2*TAU(2,1))/r &

&         -TERM1*(-dedx+(2.*E-3.*  &

&         v11-3.*v22)*drdx+2.*v1*dldx+2.*v2*dmdx))*reinv

bj(1,4,2)=v1*bj(1,2,2)+v2*bj(1,3,2)+(-TAU(1,1)/r-         &

&        TERM1*(2.*v1*drdx-dldx))*reinv

bj(1,4,3)=v1*bj(1,2,3)+v2*bj(1,3,3)+(-TAU(2,1)/r-         &

&         TERM1*(-dmdx+2.*v2*drdx))*reinv

bj(1,4,4)=TERM1*drdx*reinv

!

bj(2,2,1)=bj(1,3,1)

bj(2,2,2)=bj(1,3,2)

bj(2,2,3)=bj(1,3,3)

bj(2,3,1)=(DENOM*(VISLAM*(dldx+4.*v2*drdy)+VISR*       &

&         (v1*drdx+dmdy)))*reinv

bj(2,3,2)=VISLAM*drdx*DENOM*reinv

bj(2,3,3)=VISR*drdy*DENOM*reinv

bj(2,4,1)=v1*bj(2,2,1)+v2*bj(2,3,1)+((v1*TAU(1,2)+v2*TAU(2,2))/r &

&         -TERM1*(-dedy+(2.*E-3.*  &

&         v11-3.*v22)*drdy+2.*v1                       &

&         *dldy+2.*v2*dmdy))*reinv

bj(2,4,2)=v1*bj(2,2,2)+v2*bj(2,3,2)+(-TAU(2,1)/r -        &

&         TERM1*(-dldy+2.*v1*drdy))*reinv

bj(2,4,3)=v1*bj(2,2,3)+v2*bj(2,3,3)+(-TAU(2,2)/r-         &

&         TERM1*(2.*v2*drdy-dmdy))*reinv

bj(2,4,4)=TERM1*drdy*reinv

!

RETURN

end subroutine bjacob

!

!    **************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE JACOBIAN "C SUB IJ"

!     ON A SAMPLE POINT

!

SUBROUTINE CJACOB(reinv,cv,VIS,HK,UNKN,cj)

!

!DIR$ INLINEALWAYS CJACOB

!

Use param_var

!      Use thermo_propt

!

Implicit None

real,intent(in) :: reinv,vis,hk,cv

real,intent(in) :: unkn(mequa)

real,dimension(mdimn,mdimn,mequa,mequa),intent(out) :: cj

real :: vislam,visr

real :: r,denom,denom2

real :: rv1,rv2,re,rv11,rv12,rv22,rv11p22,hkocv

!

!     Zero Jacobian

cj = 0.0

!

VISLAM=-2.*VIS/3.

visr=2.*vis+vislam

!

r      = unkn(1)

denom  = 1./r**2

denom2 = denom/r

!

rv1     = unkn(2)

rv2     = unkn(3)

re      = unkn(4)

rv11    = rv1 * rv1

rv12    = rv1 * rv2

rv22    = rv2 * rv2

rv11p22 = rv11 + rv22

hkocv   = hk/cv

!

cj(1,1,2,1) =  visr*rv1*denom*reinv

cj(1,1,2,2) = -visr/r*reinv

cj(1,1,3,1) =  vis*rv2*denom*reinv

cj(1,1,3,3) = -vis/r*reinv

cj(1,1,4,1) =((visr*rv11*denom2+vis*rv22*denom2)      &

&             -hkocv*(-re*denom+(rv11p22)*denom2))*reinv

cj(1,1,4,2) =(-visr+hkocv)*rv1*denom*reinv

cj(1,1,4,3) =(-vis +hkocv)*rv2*denom*reinv

cj(1,1,4,4) = -hkocv/r*reinv

!

cj(1,2,2,1) = vislam*rv2*denom*reinv

cj(1,2,2,3) =-vislam/r*reinv

cj(1,2,3,1) = vis*rv1*denom*reinv

cj(1,2,3,2) =-vis/r*reinv

cj(1,2,4,1) =(vislam+vis)*rv12*denom2*reinv

cj(1,2,4,2) =-vis*rv2*denom*reinv

cj(1,2,4,3) =-vislam*rv1*denom*reinv

!

cj(2,1,2,1) = vis*rv2*denom*reinv

cj(2,1,2,3) =-vis/r*reinv

cj(2,1,3,1) = vislam*rv1*denom*reinv

cj(2,1,3,2) =-vislam/r*reinv

cj(2,1,4,1) =(vis+vislam)*rv12*denom2*reinv

cj(2,1,4,2) =-vislam*rv2*denom*reinv

cj(2,1,4,3) =-vis*rv1*denom*reinv

!

cj(2,2,2,1) = vis*rv1*denom*reinv

cj(2,2,2,2) =-vis/r*reinv

cj(2,2,3,1) = visr*rv2*denom*reinv

cj(2,2,3,3) =-visr/r*reinv

cj(2,2,4,1) =((vis*rv11*denom2+visr*rv22*denom2)        &

&             -hkocv*(-re*denom+rv11p22*denom2))*reinv

cj(2,2,4,2) =(-vis +hkocv)*rv1*denom*reinv

cj(2,2,4,3) =(-visr+hkocv)*rv2*denom*reinv

cj(2,2,4,4) =-hkocv/r*reinv

!

RETURN

end subroutine cjacob

!

!     ************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE VISCOUS FLUX AT A

!     NODAL POINT

!

SUBROUTINE VISCFLX(reinv,TAU,HK,VIS,DUDX,DTDX,VELO,d2tdx2,d2udx2,&

&                   G,dgjdxj)

!

!DIR$ INLINEALWAYS VISCFLX

!

Use param_var

!

Implicit None

!

real,intent(in) :: reinv,hk,vis

real,intent(out) :: g(mdimn,mequa),dgjdxj(mequa)

REAL,intent(in) :: velo(mdimn)

real,intent(in) :: dudx(mdimn,mdimn),dtdx(mdimn)

real,intent(in) :: tau(mdimn,mdimn)

real,intent(in) :: d2tdx2(mdimn),d2udx2(mdimn,mdimn,mdimn)

real :: DG1DX1(MEQUA),DG2DX2(MEQUA)

real :: vis43,vis23

!

vis43 = 4./3.*vis

vis23 = 2./3.*vis

G(1,1)=0.0

G(1,2)=-TAU(1,1)*reinv

G(1,3)=-TAU(1,2)*reinv

G(1,4)=(-TAU(1,1)*VELo(1) - TAU(1,2)*VELo(2) - HK*DTDX(1))*reinv

G(2,1)=0.0

G(2,2)=-TAU(2,1)*reinv

G(2,3)=-TAU(2,2)*reinv

G(2,4)=(-TAU(2,1)*VELO(1) - TAU(2,2)*VELO(2) - HK*DTDX(2))*reinv

!

dG1DX1(1) = 0.0

DG1DX1(2) = (-vis43*D2UDX2(1,1,1)+vis23*D2UDX2(2,1,2))*reinv

DG1DX1(3) = -vis*(D2UDX2(1,1,2)+D2UDX2(2,1,1))*reinv

DG1DX1(4) = DG1DX1(2)*VELO(1)+DG1DX1(3)*VELO(2)+G(1,2)*DUDX(1,1)  &

&            +G(1,3)*DUDX(2,1)-HK*D2TDX2(1)*reinv

!

DG2DX2(1) = 0.0

DG2DX2(2) = -vis*(D2UDX2(1,2,2)+D2UDX2(2,1,2))*reinv

DG2DX2(3) = (-vis43*D2UDX2(2,2,2)+vis23*D2UDX2(1,1,2))*reinv

DG2DX2(4) = DG2DX2(2)*VELO(1)+DG2DX2(3)*VELO(2)+G(2,2)*DUDX(1,2)   &

&            +G(2,3)*DUDX(2,2)-HK*D2TDX2(2)*reinv

!

DGJDXJ(1) = DG1DX1(1) + DG2DX2(1)

DGJDXJ(2) = DG1DX1(2) + DG2DX2(2)

DGJDXJ(3) = DG1DX1(3) + DG2DX2(3)

DGJDXJ(4) = DG1DX1(4) + DG2DX2(4)

!

RETURN

end subroutine viscflx

!

!     ***********************************************************

!     Subroutine to multiply Jacobian Matrices

!

subroutine matxmult(ndimn,nequa,a,b,c)

!

!DIR$ INLINEALWAYS matxmult

!

Implicit None

!

integer,intent(in) :: ndimn,nequa

real,intent(in) :: a(ndimn,nequa,nequa), b(ndimn,nequa,nequa)

real,intent(out) :: c(ndimn,ndimn,nequa,nequa)

integer :: i,j,ir,is,iq     ! Loop counters

!

c = 0.0

!

do i = 1, ndimn

do j = 1, ndimn

do ir = 1, nequa

do is = 1, nequa

do iq = 1, nequa

c(i,j,ir,is) = c(i,j,ir,is) + a(i,ir,iq)*    &

&                            b(j,iq,is)

enddo

enddo

enddo

enddo

enddo

!

return

end subroutine matxmult

!

!     ***********************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE DIRECTION COSINES

!     OF THE NORMAL VECTOR TO A CONTROL SURFACE

!

SUBROUTINE CALNORM(ilbou,DPIDX,W,EN)

!

Use param_var

Use bface_info

Use shapbou_2d

Use grid_info

!

Implicit None

!

integer,intent(in) :: ilbou(mnode)

real,intent(in) :: w(ngaus)

real,intent(out) :: dpidx(mdimn,2,ngaus),en(2,ngaus)

real :: DPDX(2),DPDY(2)

real :: DXDS(NGAUS),DYDS(NGAUS)

real :: XELEM(2),YELEM(2)

integer :: inode,igaus      !  Loop Counters

real :: delx,dely,dl

!

DO INODE = 1, 2

XELEM(INODE) = X(1,ilbou(inode))

YELEM(INODE) = X(2,ilbou(inode))

enddo

!

DELX = XELEM(2) - XELEM(1)

DELY = YELEM(2) - YELEM(1)

!

IF(ABS(DELX).LT.1.0E-12) THEN

DPDX(1) = 0.0

DPDX(2) = 0.0

ELSE

DPDX(1) = -1.0/DELX

DPDX(2) = 1.0/DELX

ENDIF

!

IF(ABS(DELY).LT.1.0E-12) THEN

DPDY(1) = 0.0

DPDY(2) = 0.0

ELSE

DPDY(1) = -1.0/DELY

DPDY(2) = 1.0/DELY

ENDIF

!

DO IGAUS = 1, NGAUS

DXDS(IGAUS) = 0.0

DYDS(IGAUS) = 0.0

!

DO INODE = 1, 2

DXDS(IGAUS)=DXDS(IGAUS)+dpxi(INODE,IGAUS)*XELEM(INODE)

DYDS(IGAUS)=DYDS(IGAUS)+dpxi(INODE,IGAUS)*YELEM(INODE)

enddo

!

!      write(*,50)igaus,dxds(igaus),dyds(igaus)

!   50 format(i4,2e16.8)

!

DL=SQRT(DXDS(IGAUS)**2+DYDS(IGAUS)**2)

!

!      write(*,55)dl

!   55 format(' DL = ',e16.8)

!

EN(1,IGAUS)=DYDS(IGAUS)/DL

EN(2,IGAUS)=-DXDS(IGAUS)/DL

!

EN(1,IGAUS) = EN(1,IGAUS)*DL*w(igaus)

EN(2,IGAUS) = EN(2,IGAUS)*DL*w(igaus)

!

DO INODE = 1, 2

DPIDX(1,INODE,IGAUS) = DPDX(INODE)

DPIDX(2,INODE,IGAUS) = DPDY(INODE)

enddo

enddo

!

RETURN

end subroutine calnorm

!

!    ****************************************************************

!    Subroutine to initiate Equation Solver routines

!

Subroutine Solve(nblock,nelem,nnode,nequa,ndimn,nordr,neleq,epstol,  &

&                tol,iprecnd,igmres,itime,itmax,iunit,omega,isolve,   &

&                ierr)

!

Use flow_var

Use FDV_coef

Use grid_info

Use bnd_cnd

!

Implicit None

!

!     Routine ebe_jacb - Jacobian Element-by-Element Solver

!     Routine ebe_cgm  - Element-by-Element Conjugate Gradient Solver

!

integer,intent(in) :: nblock,nelem,nnode,nequa,ndimn,nordr

integer,intent(in) :: neleq

real,intent(in) :: epstol,tol,omega

integer,intent(in) :: iprecnd,igmres,itime,itmax,iunit,isolve

integer,intent(out) :: ierr

integer :: iordr,locatn

real :: delumax

!

if(isolve.eq.4)         &

&   call GMRES_ebe(nelem,nnode,nequa,nordr,neleq,ndimn,igmres,iprecnd,  &

&                  nblock,itmax,epstol,tol,itime,iunit,ierr)

!

delumax = 0.0

!

do iordr = 1,nordr

if(ibouncnd(iordr).ge.0) then

if(abs(delu(iordr)).ge.delumax) then

delumax = delu(iordr)

locatn = iordr

endif

uold(iordr) = u(iordr)

u(iordr) = u(iordr) + delu(iordr)

endif

enddo

!

write(53,1000) itime,delumax,locatn

1000 format(' At time step ',i6,' Max Delu is ',e16.8,     &

&       ' at Equation ',i8)

!

return

end subroutine Solve

!

!     ***************************************************************

!     Element-by-Element GMRES Equation Solver

!

Subroutine GMRES_ebe(nelem,nnode,nequa,nordr,neleq,ndimn,igmres, &

&                     iprecnd,nblock,itmax,epstol,tol,itime,iunit,&

ierr)

!

Use param_var

Use thermo_propt

Use FDV_coef, only: rhs,delu

Use grid_info

Use GMRES_var

Use bnd_cnd

!

Implicit None

!

integer,intent(in) :: nelem,nnode,nequa,nordr,ndimn,igmres

integer,intent(in) :: iprecnd,nblock,itmax,itime,iunit

integer,intent(in) :: neleq

real,intent(in) :: epstol

real,intent(inout) :: tol

integer,intent(out) :: ierr

!

integer :: iblock

!

integer :: iordr,iter,lflag

integer :: i,jiter,j,ig,jordr,n

integer :: status      ! Allocatable array status, = 0 for success

!

real,dimension(nordr) :: Diag,Fbar

real :: Diag_m12(nordr)      !  Diagonal**(-0.5)

!

real :: uloc(neleq),Fbl(neleq)

real :: H(igmres+1,igmres),beta(igmres+1,igmres+1)

real :: Ebar(igmres+1)

real :: utemp(nordr),u1temp(nordr)

real :: c(igmres),s(igmres)

real :: Error(nordr,igmres+1)

real :: y(igmres)

real :: rhsnorm,epsilon,resid,hnorm,r

real :: CvTw

real :: h1temp,h2temp

!

!      print *, 'nordr = ',nordr

!

!     initialize UNEW

!

do iordr = 1,nordr

if(ibouncnd(iordr).ge.0)     &

&      delu(iordr) = 0.0

enddo

!

!     Apply preconditioning to matrix

!

Select Case(iprecnd)

Case(0)

call Au_product(nblock,nnode,nordr,nequa,neleq,delu,Fbar)

!

Case(1)

allocate(alu(nelem,neleq,neleq),stat=status)

if(status/=0) write(*,101) status

101 format('ALLOCATION FAILED, STAT = ',i8)

call LU_precond(nelem,neleq,nordr,nequa,nnode,nblock,Fbar)

!

Case(2)

call Diag_precond(nblock,nordr,neleq,nelem,nequa,nnode,     &

&                     ndimn,Diag,Diag_m12,Fbar)

allocate(alu(nelem,neleq,neleq),stat=status)

if(status/=0) write(*,101) status

call LU_precond(nelem,neleq,nordr,nequa,nnode,nblock,Fbar)

!

Case(3)

call Diag_precond(nblock,nordr,neleq,nelem,nequa,nnode,     &

&                     ndimn,Diag,Diag_m12,Fbar)

!

Case(4)

call Diag_precond(nblock,nordr,neleq,nelem,nequa,nnode,     &

&                     ndimn,Diag,Diag_m12,Fbar)

call SOR_Precond(nblock,nelem,neleq,nordr,nequa,nnode,Fbar)

call SOR_Precond(nblock,nelem,neleq,nordr,nequa,nnode,rhs)

End Select

!

if(itime.eq.1) write(iunit,125)igmres

write(iunit,150) itime

!

!      write(47,5000)(Fbar(iordr),iordr=1,nordr)

5000 format(4e16.8)

!

call el2norm(nordr,rhs,rhsnorm)

!      print *,' RHSNORM = ',rhsnorm

!

epsilon = epstol*rhsnorm

tol = epsilon

lflag = 0

resid = 1.0

ierr = 0

!

iterloop: do iter = 1,itmax

!

if(iter.gt.1) then

call Au_product(nblock,nnode,nordr,nequa,neleq,delu,Fbar)

!

if(iprecnd>=2) then

!

do iordr = 1,nordr

if(ibouncnd(iordr).eq.100) then

jordr = iordr - (ndimn+1)

Fbar(iordr) = Diag_m12(iordr)*Diag_m12(iordr)*     &

&                           delu(iordr) - Diag_m12(iordr)*Cv*    &

&                           Twall*Diag_m12(jordr)*delu(jordr)

elseif(ibouncnd(iordr).eq.101) then

jordr = iordr - 1

Fbar(iordr) = Diag_m12(iordr)*Diag_m12(iordr)*     &

&                           delu(iordr) - Diag_m12(iordr)*       &

&                           Diag_m12(jordr)*delu(jordr)

endif

enddo

!

else

!

do iordr = 1,nordr

if(ibouncnd(iordr).eq.100) then

Fbar(iordr) = delu(iordr) - Cv*Twall*delu(iordr -   &

&                           (nequa-1))

endif

enddo

!

endif

!

if(iprecnd.eq.1.or.iprecnd.eq.2)                &

&          call precond(nblock,nelem,neleq,nordr,nequa, &

&                       nnode,Fbar,Fbar)

!

if(iprecnd==4)  call SOR_Precond(nblock,nelem,neleq,nordr,  &

nequa,nnode,Fbar)

endif

!

!     Calculate initial residual

!

do iordr = 1,nordr

if(ibouncnd(iordr).lt.0) then

Error(iordr,1) = 0.0

else

Error(iordr,1) = rhs(iordr) - Fbar(iordr)

endif

utemp(iordr) = Error(iordr,1)

enddo

!

call el2norm(nordr,utemp,Ebar(1))

!

do iordr = 1,nordr

Error(iordr,1) = utemp(iordr)/Ebar(1)

enddo

!

!         write(45,*)(Error(iordr,1),iordr=1,nordr)

!         print *,'Ebar(1) = ',Ebar(1)

!

!     Start of GMRES iteration

!

gmresloop: do i = 1,igmres

!

jiter = (iter-1)*igmres + i

!

do iordr = 1,nordr

utemp(iordr) = Error(iordr,i)

u1temp(iordr) = 0.0

enddo

!

call Au_product(nblock,nnode,nordr,nequa,neleq,utemp,     &

&                      u1temp)

!

!     Preconditioning - Error(i+1) = (L-Inverse)*A*(U-Inverse)*Errbar

!

if(iprecnd.eq.1.or.iprecnd.eq.2)                           &

&         call precond(nblock,nelem,neleq,nordr,nequa,nnode,&

&                     u1temp,u1temp)

!

if(iprecnd==4)   call SOR_Precond(nblock,nelem,neleq,nordr,  &

nequa,nnode,u1temp)

!

do iordr = 1,nordr

Error(iordr,i+1) = u1temp(iordr)

enddo

!

!            write(45,*)i,(Error(iordr,i+1), iordr=1,nordr)

!

!     Modified Gram-Schmidt Orthogonalization

!

beta(i+1,1) = 0.0

do n = 1,nordr

beta(i+1,1) = beta(i+1,1) + Error(n,i+1)*Error(n,1)

enddo

!

do j = 1,i

beta(i+1,j+1) = 0.0

!

do iordr = 1,nordr

Error(iordr,i+1) = Error(iordr,i+1) - beta(i+1,j)*  &

&                               Error(iordr,j)

beta(i+1,j+1) = beta(i+1,j+1) + Error(iordr,i+1)*   &

&                            Error(iordr,j+1)

enddo

enddo

!

beta(i+1,i+1) = sqrt(beta(i+1,i+1))

!

hnorm = beta(i+1,i+1)

!

!     Build ith column of h

!

do j = 1,i+1

h(j,i) = beta(i+1,j)

enddo

!

!            print *,j,' H(j+1,j) = ',h(j+1,j)

!

!     Check for convergence

!

if(h(i+1,i).le.tol) then

lflag = 1

ig = i

goto 305

endif

!

!

do iordr = 1,nordr

Error(iordr,i+1) = Error(iordr,i+1)/H(i+1,i)

enddo

305  continue

!

!     Q-R Algorithm (Givens Rotations)

!

if(i.gt.1) then

!

do j = 1,i-1

!

h1temp = c(j)*H(j,i) + s(j)*H(j+1,i)

h2temp = -s(j)*H(j,i) + c(j)*H(j+1,i)

H(j,i) = h1temp

H(j+1,i) = h2temp

!

enddo

!

endif

!

r = sqrt(H(i,i)*H(i,i) + H(i+1,i)*H(i+1,i))

c(i) = H(i,i)/r

s(i) = H(i+1,i)/r

!

H(i,i) = r

H(i+1,i) = 0.0

!

Ebar(i+1) = -s(i)*Ebar(i)

Ebar(i) = c(i)*Ebar(i)

!

write(iunit,200) jiter,abs(Ebar(i+1)),hnorm

!

!     Convergence Check

!

if(abs(Ebar(i+1)).le.tol) then

lflag = 1

ig = i

goto 405

endif

!

if(lflag.eq.1) goto 405

!

enddo gmresloop

!

ig = igmres

!

405    continue

!

!

!     Solve for y by back Substitution

!

y(ig) = Ebar(ig)/H(ig,ig)

!

do i = ig-1,1,-1

do j = i+1,ig

Ebar(i) = Ebar(i) - H(i,j)*y(j)

enddo

y(i) = Ebar(i)/H(i,i)

enddo

!

!        Update Solution

!

do j = 1,ig

do iordr = 1,nordr

if(ibouncnd(iordr).ge.0) then

if(ibouncnd(iordr).eq.100) then

jordr = iordr - (ndimn+1)

!

if(iprecnd>=2) then

CvTw = Diag(iordr)*Diag_m12(jordr)*cv*Twall

else

CvTw = cv*Twall

endif

!

delu(iordr) = CvTw*delu(jordr)

elseif(ibouncnd(iordr).eq.101) then

jordr = iordr - 1

!

if(iprecnd>=2) then

delu(iordr) = Diag(iordr)*Diag_m12(jordr)*     &

&                             delu(jordr)

else

delu(iordr) = delu(jordr)

endif

!

else

delu(iordr) = delu(iordr) + y(j)*Error(iordr,j)

endif

endif

enddo

enddo

!

!

!         write(iunit,200) iter,Ebar(ig+1),H(ig+1,ig)

!          write(iunit,200) iter,h(ig+1,ig),resid

!

!     Convergence Check

!

if(lflag.eq.1) then

ierr = 0

goto 1005

endif

!

if(iter.eq.itmax) then

ierr = 2

write(6,100) iter,Ebar(ig+1)

!         write(6,100) iter,resid

endif

!

enddo iterloop

1005 continue

!

!

if(iprecnd>=2) then

do iordr = 1,nordr

delu(iordr) = delu(iordr)*Diag_m12(iordr)

enddo

endif

!

deallocate (alu,stat=status)    ! Deallocating ALU Matrix

!

!      write(35,*)(delu(iordr), iordr = 1,nordr)

!

100 format('  FAILED TO CONVERGE IN ',i8,' CURRENT RESIDUAL IS ',  &

&          e16.8)

125 format(' ELEMENT-BY-ELEMENT GMRES SOLVER',' GMRES PARAMETER ', &

&       i8)

150 format(' ITERATION        RESIDUAL FOR TIME STEP ',i8)

200 format(i8,2e16.8)

!

return

end subroutine GMRES_ebe

!

!    ****************************************************************

!    Subroutine to apply preconditioning for Conjugate Gradient Algorithm

!

Subroutine precond(nblock,nelem,neleq,nordr,nequa,nnode,R,Z)

!

Use param_var

Use grid_info

Use GMRES_var

Use bnd_cnd

!

Implicit None

!

integer,intent(in) :: nblock,nelem,neleq,nordr,nequa,nnode

real,intent(in) :: R(nordr)

real,intent(out) :: Z(nordr)

!

integer :: iordr,iblock,iel,iel0,ielast,nvec,iv,i,j

!

real :: zloc(mbsize,neleq)

!

do iordr = 1,nordr

Z(iordr) = R(iordr)

enddo

!

!     Forward reduction

!

do iblock = 1,nblock

iel = lblock(iblock)

iel0 = iel - 1

ielast = lblock(iblock+1) - 1

nvec = lblock(iblock+1) - iel

!

!      print *,' Forward reduction (local)'

call local(nnode,nordr,nequa,neleq,iel,ielast,z,zloc,     &

&              'localize')

!

do iv = 1,nvec

do i = 1,neleq

do j = 1,i-1

zloc(iv,i) = zloc(iv,i) - alu(iel0+iv,i,j)*zloc(iv,j)

enddo

enddo

enddo

!

!      print *,' Forward reduction (global)'

call local(nnode,nordr,nequa,neleq,iel,ielast,z,zloc,   &

&              'globaliz')

enddo

!

!     Back Substitution

!

do iblock = nblock+1,2,-1

iel = lblock(iblock-1)

iel0 = iel - 1

ielast = lblock(iblock) - 1

nvec = lblock(iblock) - iel

!

!      print *,' Back Substitution (local)'

call local(nnode,nordr,nequa,neleq,iel,ielast,z,zloc,     &

&              'localize')

!

do iv = nvec,1,-1

zloc(iv,neleq) = zloc(iv,neleq)/alu(iel0+iv,neleq,neleq)

enddo

!

do i = neleq-1,1,-1

do j = i+1,neleq

do iv = nvec,1,-1

zloc(iv,i) = zloc(iv,i) - alu(iel0+iv,i,j)*zloc(iv,j)

enddo

enddo

!

do iv = nvec,1,-1

zloc(iv,i) = zloc(iv,i)/alu(iel0+iv,i,i)

enddo

!

enddo

!

!      print *,' Back Substitution (global)'

call local(nnode,nordr,nequa,neleq,iel,ielast,z,zloc,   &

&              'globaliz')

!

enddo

!

return

end subroutine precond

!

!     **************************************************************

!     Subroutine to apply Diagonal preconditioning to stiffness matrix

!

Subroutine Diag_precond(nblock,nordr,neleq,nelem,nequa,nnode,   &

&                        ndimn,Diag,Diag_m12,Fbar)

!

Use param_var

Use thermo_propt

Use grid_info

Use flow_var

Use FDV_coef,only: Bel,Delu,rhs

!      Use GMRES_var

Use bnd_cnd

!

Implicit None

!

integer,intent(in) :: nblock,nordr,neleq,nelem,nequa,nnode,ndimn

real,intent(out),dimension(nordr) :: Diag,Diag_m12

real,intent(inout) :: Fbar(nordr)

!

real :: Dloc(mbsize,meleq)

integer :: iordr,ipoin,iblock,i,j,iv,jordr

integer :: idimn,iel,iel0,ielast,nvec

!

!     Extract Diagonal of stiffness matrix for use in preconditioning

!

Diag = 0.0

!

do iblock = 1,nblock

iel = lblock(iblock)

iel0 = iel - 1

ielast = lblock(iblock+1) - 1

nvec = lblock(iblock+1) - iel

!

do i = 1,neleq

do iv = 1,nvec

!

dloc(iv,i) = Bel(iel0+iv,i,i)

enddo

enddo

!

call local(nnode,nordr,nequa,neleq,iel,ielast,Diag,dloc,    &

&              'add glob')

!

enddo

!

do iordr = 1,nordr

if(ibouncnd(iordr).lt.0.or.ibouncnd(iordr).eq.100.or.       &

&      ibouncnd(iordr).eq.101) then

Diag(iordr) = 1.0

endif

enddo

!

do iordr = 1,nordr

!

Fbar(iordr) = 0.0

!

if(Diag(iordr).lt.0.0) then

write(*,1000)iordr

ipoin = 1 + (iordr-1)/nequa

write(*,1100)ipoin

write(*,1200)Diag(iordr)

write(*,1550) dense(ipoin),(vel(idimn,ipoin),     &

&                     idimn=1,ndimn)

!            Diag(iordr)=abs(Diag(iordr))

endif

!

Diag(iordr) = sqrt(Diag(iordr))

Diag_m12(iordr) = 1./Diag(iordr)

enddo

!

do iordr = 1,nordr

delu(iordr) = delu(iordr)*Diag(iordr)

rhs(iordr) = rhs(iordr)*Diag_m12(iordr)

enddo

!

do iblock = 1,nblock

iel = lblock(iblock)

iel0 = iel - 1

ielast = lblock(iblock+1) - 1

nvec = lblock(iblock+1) - iel

!

!        write(47,1500)iblock,iel,ielast,nvec

1500 format(' Block No.:  ',i4,' IEL:  ',i8,' IELast:  ',i8,   &

&         ' NVEC:  ',i8)

!

!      print *,' Diagonal Preconditioning'

call local(nnode,nordr,nequa,neleq,iel,ielast,       &

&              Diag_m12,Dloc,'localize')

!

!

do iv = 1,nvec

do i = 1,neleq,2

do j = 1,neleq,2

!

Bel(iel0+iv,i  ,j  ) = Bel(iel0+iv,i  ,j  )*       &

&                                   Dloc(iv,i  )*Dloc(iv,j  )

Bel(iel0+iv,i+1,j  ) = Bel(iel0+iv,i+1,j  )*       &

&                                   Dloc(iv,i+1)*Dloc(iv,j  )

Bel(iel0+iv,i  ,j+1) = Bel(iel0+iv,i  ,j+1)*       &

&                                   Dloc(iv,i  )*Dloc(iv,j+1)

Bel(iel0+iv,i+1,j+1) = Bel(iel0+iv,i+1,j+1)*       &

&                                   Dloc(iv,i+1)*Dloc(iv,j+1)

!

enddo

enddo

enddo

enddo

!

call Au_product(nblock,nnode,nordr,nequa,neleq,delu,Fbar)

!

do iordr = 1,nordr

if(ibouncnd(iordr).eq.100) then

jordr = iordr - (ndimn+1)

Fbar(iordr) = Diag_m12(iordr)*Diag_m12(iordr)*    &

&                           delu(iordr) - Diag_m12(iordr)*Cv*   &

&                           Twall*Diag_m12(jordr)*delu(jordr)

elseif(ibouncnd(iordr).eq.101) then

jordr = iordr - 1

Fbar(iordr) = Diag_m12(iordr)*Diag_m12(iordr)*    &

&                           delu(iordr) - Diag_m12(iordr)       &

&                           *Diag_m12(jordr)*delu(jordr)

endif

enddo

!

!

1000 format(' WARNING:  DIAGONAL FOR EQUATION ',I8,' MAY BE NEGATIVE')

1100 format(' OCCURING AT NODE ',I8)

1200 format(' VALUE OF DIAGONAL IS ',e16.8)

1550 format(' VALUES OF DENSITY AND VELOCITY ARE: ',4(e12.6,1x))

!

return

end subroutine Diag_precond

!

!     **************************************************************

!     Subroutine to recover the local element components of a assembled

!     global vector

!

Subroutine local(nnode,nordr,nequa,neleq,iel,ielast,a,aloc,   &

&                 task)

!

!DIR$ INLINEALWAYS local

!

Use param_var

Use grid_info

!

Implicit None

!

integer,intent(in) :: nnode,nordr,nequa,neleq,iel,ielast

real,intent(inout) :: a(nordr),aloc(mbsize,neleq)

character(len=8),intent(in) :: task

integer :: k,inode,iequa,ielem,kel,ipoin,iordr

integer :: ieleq

!

!      print *,'LOCAL input variable:  nnode,nordr,nequa,neleq,iel,ielast'

!      print *,nnode,nordr,nequa,neleq,iel,ielast

!

if(task.eq.'localize') then

k = 0

!

do inode=1,nnode

do iequa = 1,nequa

kel = 0

k = k+1

!

!         if(inode==1.and.iequa==1)      &

!         write(41,*)nnode,nordr,nequa,neleq,iel,ielast

!

do ielem = iel,ielast

kel = kel + 1

ipoin = lnod(inode,ielem)

iordr = nequa*(ipoin-1) + iequa

aloc(kel,k) = a(iordr)

enddo

enddo

!

enddo

!

return

!

endif

!

if(task.eq.'globaliz') then

!

do ieleq = 1,neleq

inode = 1 + (ieleq-1)/nequa

!

kel = 0

do ielem = iel,ielast

kel = kel + 1

ipoin = lnod(inode,ielem)

iordr = nequa*(ipoin-1) + ieleq -(ieleq-1)/nequa*nequa

!

a(iordr) = aloc(kel,ieleq)

!

enddo

!

enddo

return

endif

!

if(task.eq.'add glob')then

!

!

do ieleq = 1,neleq

inode = 1 + (ieleq-1)/nequa

kel = 0

!

do ielem = iel,ielast

kel = kel + 1

ipoin = lnod(inode,ielem)

iordr = nequa*(ipoin-1) + ieleq -(ieleq-1)/nequa*nequa

!

a(iordr) = a(iordr) + aloc(kel,ieleq)

!

enddo

!

enddo

!

return

endif

!

end subroutine local

!

!     ***********************************************************

!     Subroutine to solve a system of equations of the form Lx = B

!     by Forward Substitution where L is a unit lower triangular

!     matrix and x and B are column vectors

!

Subroutine Fwdsubstn(n,L,B)

!

!DIR$ INLINEALWAYS Fwdsubstn

!

Implicit None

!

integer,intent(in) :: n

real,intent(in) :: L(n,n)

real,intent(inout) :: B(n)

integer :: i,j

!

do i = 1,n

do j = 1,i-1

B(i) = B(i) - L(i,j)*B(j)

enddo

enddo

!

return

end subroutine Fwdsubstn

!

!     ************************************************************

!     Subroutine to solve the system of equations Uy = B by backward

!     substitution where U is a upper triangular matrix and y and B

!     are column vectors

!

Subroutine Backsubstn(n,U,B)

!

!DIR$ INLINEALWAYS Backsubstn

!

Implicit None

!

integer,intent(in) :: n

real,intent(in) :: U(n,n)

real,intent(inout) :: B(n)

integer i,j

!

B(n) = B(n)/U(n,n)

!

do i = n-1,1,-1

do j = i+1,n

B(i) = B(i) - U(i,j)*B(j)

enddo

!

B(i) = B(i)/U(i,i)

!

enddo

!

return

end subroutine Backsubstn

!

!     ***********************************************************

!     Subroutine to perform LU Preconditioning on the system of

!     equations

!

Subroutine LU_precond(nelem,neleq,nordr,nequa,nnode,nblock,Fbar)

!

Use FDV_coef, only: Bel,rhs

Use GMRES_var

Use grid_info

!

Implicit None

!

integer,intent(in) :: nelem,neleq,nordr,nequa,nnode,nblock

real,intent(inout) :: Fbar(nordr)

integer :: i,j,ielem,iblock,iel,ielast,k

!      real rhsloc(neleq),tmp(neleq),uloc(neleq)

!

!     Initialize alu to Bel

!

!

do ielem = 1,nelem

do j = 1,neleq

do i = 1,neleq

if(i/=j) then

alu(ielem,i,j) = Bel(ielem,i,j)

else

!

alu(ielem,i,i) = 1.0       ! Winget Regularization

endif

!

enddo

enddo

enddo

!

!     Perform LU Factorization

!

do iblock = 1,nblock

iel = lblock(iblock)

ielast = lblock(iblock+1) - 1

!

do ielem = iel,ielast

do i = 2,neleq

do k = 1,i-1

!

alu(ielem,i,k) = alu(ielem,i,k)/alu(ielem,k,k)

!

do j = k+1, neleq

alu(ielem,i,j) = alu(ielem,i,j)                &

&                              - alu(ielem,i,k)*alu(ielem,k,j)

!

enddo

!

enddo

enddo

enddo

!

enddo

!

!     Apply preconditioning to RHS and Fbar

!

call precond(nblock,nelem,neleq,nordr,nequa,nnode,rhs,rhs)

!

call precond(nblock,nelem,neleq,nordr,nequa,nnode,Fbar,Fbar)

!

500 format(' Element No. ',i8,' Nodes:  ',4i8)

return

end subroutine LU_precond

!

!    ******************************************************************

!    Subroutine to apply Successive Overrelaxation Preconditioning

!

!    Preconditioning matrix, M = (I-wE)*D*(I-wF), where -E is the

!    strict-lower part of the stiffness matrix, -F is the strict-upper

!    part of the stiffness matrix, and w is a relaxation factor

!

Subroutine SOR_Precond(nblock,nelem,neleq,nordr,nequa,nnode,Z)

!

Use param_var

Use grid_info

Use FDV_coef, only: Bel

!

Implicit None

!

integer,intent(in) :: nblock,nelem,neleq,nordr,nequa,nnode

real,intent(inout) :: z(nordr)

!

integer :: iblock,iel,iel0,ielast,nvec,iv,i,j

real :: zloc(mbsize,neleq),omega

!

!   Relaxation Factor, Omega

!

omega = 0.75

!

!   Forward Reduction, I-wE

!

do iblock = 1,nblock

iel = lblock(iblock)

iel0 = iel - 1

ielast = lblock(iblock+1) - 1

nvec = lblock(iblock+1) - iel

!

call local(nnode,nordr,nequa,neleq,iel,ielast,Z,zloc,   &

'localize')

!

do iv = 1,nvec

do i = 2,neleq

do j = 1,i-1

zloc(iv,i) = zloc(iv,i) - omega*Bel(iel0+iv,i,j)*&

&zloc(iv,j)

enddo

enddo

enddo

!

call local(nnode,nordr,nequa,neleq,iel,ielast,Z,zloc,   &

'globaliz')

!

enddo

!

!     Backward Substitution, I-wF

!

do iblock = nblock+1,2,-1

iel = lblock(iblock-1)

iel0 = iel - 1

ielast = lblock(iblock) - 1

nvec = lblock(iblock) - iel

!

call local(nnode,nordr,nequa,neleq,iel,ielast,Z,zloc,   &

'localize')

!

do iv = nvec,1,-1

do i = neleq-1,1,-1

do j = i+1,neleq

zloc(iv,i) = zloc(iv,i) - omega*Bel(iel0+iv,i,j)*&

&zloc(iv,j)

enddo

enddo

enddo

!

call local(nnode,nordr,nequa,neleq,iel,ielast,Z,zloc,   &

'globaliz')

!

enddo

!

return

end subroutine SOR_Precond

!

!     **************************************************************

!     Subroutine to perform LU Factorization of a dense matrix

!

Subroutine LU_Fact(n,a)

!

Implicit None

!

integer,intent(in) :: n

integer i,j,k

real,intent(inout) :: a(n,n)

!

do i = 2,n

do k = 1,i-1

!

a(i,k) = a(i,k)/a(k,k)

!

do j = k+1, n

a(i,j) = a(i,j) - a(i,k)*a(k,j)

!

enddo

enddo

enddo

!

return

end subroutine LU_Fact

!

!     ***************************************************************

!     Subroutine to construct the product of the element stiffness

!     matrices with a vector

!

Subroutine Au_product(nblock,nnode,nordr,nequa,neleq,u,u2)

!

Use param_var

Use grid_info

Use FDV_coef

!

Implicit None

!

integer,intent(in) :: nblock,nnode,nordr,nequa,neleq

real,intent(in) :: u(nordr)

real,intent(out) :: u2(nordr)

real :: uloc(mbsize,neleq), u2loc(mbsize,neleq)

integer :: iblock,iel,iel0,nvec,ielast,i,j,iv

integer :: istart

!

u2 = 0.0        ! Zero Array

!

!      print *,'nordr = ',nordr

!

do iblock = 1,nblock

iel = lblock(iblock)

iel0 = iel - 1

nvec = lblock(iblock + 1) - iel

ielast = lblock(iblock+1) - 1

!

u2loc = 0.0       ! Zero matrix

!

!      print *,' Au_Product (local)'

call local(nnode,nordr,nequa,neleq,iel,ielast,u,uloc,     &

&              'localize')

!

!   Preconditioning loops for loop unrolling

!

istart = MOD(neleq,4)

!

do iv = 1,nvec

do i = 1,istart

do j = 1,neleq,2

u2loc(iv,i) = u2loc(iv,i) +      &

&                          Bel(iel0+iv,i,j  )*uloc(iv,j  ) + &

Bel(iel0+iv,i,j+1)*uloc(iv,j+1)

enddo

enddo

enddo

!

!

do iv = 1,nvec

do i = istart+1,neleq,4

do j = 1,neleq,2

!

u2loc(iv,i) = u2loc(iv,i)                         &

&                          + Bel(iel0+iv,i  ,j  )*uloc(iv,j  ) &

&                          + Bel(iel0+iv,i  ,j+1)*uloc(iv,j+1)

!

u2loc(iv,i+1) = u2loc(iv,i+1)                     &

&                          + Bel(iel0+iv,i+1,j  )*uloc(iv,j  ) &

&                          + Bel(iel0+iv,i+1,j+1)*uloc(iv,j+1)

!

u2loc(iv,i+2) = u2loc(iv,i+2)                     &

&                          + Bel(iel0+iv,i+2,j  )*uloc(iv,j  ) &

&                          + Bel(iel0+iv,i+2,j+1)*uloc(iv,j+1)

!

u2loc(iv,i+3) = u2loc(iv,i+3)                     &

&                          + Bel(iel0+iv,i+3,j  )*uloc(iv,j  ) &

&                          + Bel(iel0+iv,i+3,j+1)*uloc(iv,j+1)

!

enddo

enddo

enddo

!

!

!      print *,'Au_Product (global)'

call local(nnode,nordr,nequa,neleq,iel,ielast,u2,u2loc,   &

&              'add glob')

!

enddo

!

return

end subroutine Au_product

!

!     **************************************************************

!     Subroutine to assemble element stiffness matrices into a

!     column vector for element-by-element solver

!

Subroutine matx_vec(ielem,Num_Nodes,Num_Eqs,neleq,nelem,nordr,   &

&                    ND_CNCDNC,Bel,Delu,Fbar)

!

Implicit None

!

integer,intent(in) :: ielem,Num_Nodes,Num_Eqs,nelem,nordr

integer,intent(in) :: neleq

integer,intent(in) :: ND_CNCDNC(Num_Nodes)

real,intent(in) :: Bel(nelem,neleq,neleq),Delu(nordr)

real,intent(inout) :: Fbar(nordr)

!

real :: uloc(neleq)

integer :: k,inode,ipoin,iequa,iordr,irow,inode

integer :: icol

real :: sum

!

!     convert global vector to local vector

!

k = 0

!

do inode = 1,Num_Nodes

ipoin = ND_CNCDNC(inode)

do iequa = 1,Num_Eqs

iordr = Num_Eqs*(ipoin-1) + iequa

k = k + 1

uloc(k) = delu(iordr)

enddo

enddo

!

do irow = 1,neleq

inode = 1 + (irow-1)/Num_Eqs

ipoin = ND_CNCDNC(inode)

iordr = Num_Eqs*(ipoin-1) + irow - (irow-1)/Num_Eqs*Num_Eqs

sum = 0.0

do icol = 1,neleq

sum = sum + Bel(ielem,irow,icol)*uloc(icol)

!

enddo

!

Fbar(iordr) = Fbar(iordr) + sum

!

enddo

!

return

end subroutine matx_vec

!

!     ***************************************************************

!     Subroutine to calculate the L2NORM of a vector

!

Subroutine el2norm(n,a,b)

!

Implicit None

!

integer,intent(in) :: n

real,intent(in) :: a(n)

real,intent(out) :: b

integer :: i

real :: sum

!

sum = 0.0

!

do i = 1,n

sum = sum + a(i)*a(i)

enddo

!

b = sqrt(sum)

!

return

end subroutine el2norm

!

!    ****************************************************************

!     Function to calculate the inner (or dot) product of two vectors

!

Real Function DotProd(n,a,b)

!

Implicit None

!

integer,intent(in) :: n

real,intent(in) :: a(n),b(n)

integer :: i

real :: sum

!

sum = 0.0

!

do i = 1,n

sum = sum + a(i)*b(i)

enddo

!

DotProd = sum

!

return

end function DotProd

!

!    ****************************************************************

!

subroutine cnstwltmp(nboun,nface,nequa)

!

Use thermo_propt

Use grid_info

Use Bface_info

Use bnd_cnd

Use Flow_var

!

Implicit None

!

integer,intent(in) :: nboun,nface,nequa

integer :: iface,ielem,ibidx,inode,ibnod,ipoin,jordr

integer :: iordr

integer :: iboun

real :: cvtemp

!

! --- NO SLIP WALL BOUNDARY CONDITIONS.

!

do iface=1,nface

ielem=lface(7,iface)

ibidx=lface(8,iface)

!

!

! ... CONSTANT WALL TEMPERATURE B.C.

!

if(ibidx.eq.1.or.ibidx.eq.4) then

do inode=1,2

ibnod=lface(inode+2,iface)

ipoin=ibcon(1,ibnod)

jordr = nequa*(ipoin-1)

!

if(indbd(ipoin).ne.2) then

cvtemp = cv*twall

!       cvtemp=cv*2.6245

u(jordr+4)=u(jordr+1)*cvtemp

endif

!

enddo

endif

enddo

!

do iboun = 1,nboun

if(indbc(iboun).eq.15) then

ipoin = ibcon(1,iboun)

iordr = nequa*(ipoin-1) + 1

jordr = nequa*(ipoin-1) + 2

u(jordr) = u(iordr)

endif

enddo

!

return

end subroutine cnstwltmp

!

!     *************************************************************

!     Subroutine to apply adiabatic wall boundary condition

!

Subroutine aplboun(nface,nequa)

!

Use flow_var

Use grid_info

Use bface_info

Use bnd_cnd

!

Implicit None

!

integer,intent(in) :: nface,nequa

!

integer :: nodadj(2)

integer :: iface,ielem,ibidx,lnode,ibnod,ipoin

integer :: lnode1,ipp1,jordr,iordr,inode

real :: energy,cvtemp

!

do iface = 1,nface

ielem = lface(7,iface)

ibidx = lface(8,iface)

!

!     Determine adjacent nodes

!

lnode = lface(5,iface)

if(lnode.eq.1) then

nodadj(1) = 4

nodadj(2) = 3

elseif(lnode.eq.2) then

nodadj(1) = 1

nodadj(2) = 4

elseif(lnode.eq.3) then

nodadj(1) = 2

nodadj(2) = 1

elseif(lnode.eq.4) then

nodadj(1) = 3

nodadj(2) = 2

endif

!

if(ibidx.eq.1.or.ibidx.eq.11.or.ibidx.eq.21.or.ibidx.eq.2 &

&      .or.ibidx.eq.4) then

!

do inode = 1,2

ibnod = lface(inode+2,iface)

ipoin = ibcon(1,ibnod)

if(ibcon(2,ibnod).lt.0) cycle

if(ibcon(2,ibnod).ne.2.and.ibcon(3,ibnod).ne.2) cycle

lnode = lface(inode+4,iface)

lnode1 = nodadj(inode)

ipp1 = lnod(lnode1,ielem)

!

jordr = nequa*(ipp1-1)

iordr = nequa*(ipoin-1)

!

u(iordr+1) = u(jordr+1)

energy = u(jordr+4) - 0.5*(u(jordr+2)*u(jordr+2)  &

&                     + u(jordr+3)*u(jordr+3))/u(jordr+1)

cvtemp = energy/u(jordr+1)

u(iordr+4) = u(iordr+1)*cvtemp

!

enddo

endif

!

enddo

!

return

end subroutine aplboun

!

!     *************************************************************

!     THE FOLLOWING SUBROUTINE CHECKS FOR CONVERGENCE AND CALCULATES

!     THE RMS ERROR

!

SUBROUTINE RMSERR(npoin,nequa,dt,itime,ierr)

!

Use flow_var

Use FDV_coef

Use bnd_cnd

!

Implicit None

!

integer,intent(in) :: npoin,nequa,itime,ierr

real,intent(in) :: dt

real :: RMS(NEQUA)

real :: enum,denom

integer :: j,i,iordr,iequa

!

DO J=1,nequa

ENUM=0.0

DENOM=0.0

DO I=1,NPOIN

!	    if(indbc(i).lt.0) cycle

iordr = nequa*(i-1)+j

if(ibouncnd(iordr).lt.0) cycle

ENUM=ENUM+delu(iordr)*delu(iordr)

DENOM=DENOM+Uold(iordr)*Uold(iordr)

enddo

!

if(denom.lt.1.0e-12) then

rms(j) = 0.0

else

RMS(J) = SQRT(ENUM/DENOM)

endif

!

enddo

PRINT 100, ITIME,IERR,dt

PRINT 200, (RMS(IEQUA), IEQUA = 1, NEQUA)

WRITE(21,300)ITIME,(RMS(IEQUA),IEQUA = 1, NEQUA)

!

if(ierr.ne.0) then

print 400, ierr

stop

endif

!

100 FORMAT(1X,'ITERATION NO. ',I8,2X, 'IERR = ',I8,' TIME STEP = ', &

&       E14.6)

200 FORMAT(2X,6E12.4)

300 FORMAT(2X,I8,4(1X,E16.8))

400 format(1x,'POSSIBLE PROBLEM WITH SOLVER, IERR = ',i4)

RETURN

end subroutine rmserr

!

! --------------------------------------------------------------------

!

subroutine output(ifile,jfile,npoin,nelem,nboun,nface,nnode,nequa,  &

&                ndimn,adap,ntime,ntstp,time,cfl,igmres,iprecnd,     &

&                isolve,nvisc,ierrind,imppa,en,npold,artvis,         &

&                strem,nblock)

!

Use param_var

Use grid_info

Use flow_var

Use bface_info

Use bnd_cnd

Use thermo_propt

Use Var_parm

!

Implicit None

!

integer,intent(in) :: ifile,jfile,npoin,nelem,nboun,nface,nnode

integer,intent(in) :: nequa,ndimn,ntime,igmres,iprecnd,isolve

integer,intent(in) :: nvisc,ierrind,imppa,npold,nblock

integer,intent(in) :: ntstp

real,intent(in) :: time,cfl,en

logical,intent(in) :: adap,artvis,strem

integer :: ielem,ipoin,iordr,idimn,iboun,j,iface,iblock

integer :: inode,jpoin

integer :: lflag

!

!      write(ifile,2001)

!      write(ifile,1001)ntstp,adap,remesh,incom,artvis

write(ifile,2003)

write(ifile,1060)nvisc,isolve,ierrind,iprecnd,artvis,imppa

write(ifile,2000)

write(ifile,1000) ntime,ntstp,time,adap,igmres

write(ifile,2090)

write(ifile,1090) cfl,s1,s2,s3,s4

!      write(ifile,2002)

!      write(ifile,1002) igeom, dimcorn,endcorn,alpa

write(ifile,2095)

write(ifile,1095) cmach,creyn,tinf,twall

write(ifile,2096)

write(ifile,1096) en,strem,alpa

write(ifile,2005)

write(ifile,1005) nnode, nequa, ndimn

write(ifile,2010)

if(ndimn.ne.3) then

write(ifile,1010) nelem,npoin,nboun,nface,imax,jmax

else

write(ifile,1011) nelem,npoin,nboun,nface,imax,jmax,kmax

endif

!      if(iacnt.eq.1.or.iacnt.eq.2) write(ifile,'(i10)') npold

if(adap) write(ifile,'(i10)') npold

!

write(ifile,2020)

do ielem=1,nelem

write(ifile,1020) ielem,(lnod(inode,ielem),inode=1,nnode)

enddo

!

write(ifile,2030)

do ipoin=1,npoin

write(ifile,1030) ipoin,(X(idimn,ipoin),idimn=1,ndimn)

enddo

!

write(ifile,2040)

do ipoin=1,npoin

iordr = nequa*(ipoin-1) + 1

dense(ipoin)=u(iordr)

do idimn = 1,ndimn

vel(idimn,ipoin)=u(iordr+idimn)/u(iordr)

enddo

energ(ipoin)=u(iordr+ndimn+1)/u(iordr)

write(ifile,1040) ipoin,dense(ipoin),(vel(idimn,ipoin),    &

&                  idimn=1,ndimn),energ(ipoin)

enddo

!

write(ifile,2050)

do iboun=1,nboun

if(ndimn.lt.3) then

write(ifile,1050) (ibcon(j,iboun),j=1,3),          &

&                        (rbcon(j,iboun),j=1,ndimn)

else

write(ifile,1005) (ibcon(j,iboun),j=1,3)

endif

enddo

!

if(ndimn.gt.1) then

write(ifile,2080)

do iface=1,nface

write(ifile,1080) (lface(j,iface),j=1,8)

if(ndimn.eq.3) write(ifile,1080) (lfacnode(j,iface),j=1,8)

enddo

endif

!

write(ifile,3000)

write(ifile,1020) nblock,(lblock(iblock),iblock=1,nblock+1)

!

close(ifile,status='keep')

!

! --- WRITE WALL PRESSURE, SKIN FRICTION COEFFICIENT, AND HEAT TRANSFER

!

if(ndimn.eq.2) then

!

call wallpropt(jfile,nelem,nface,nboun,npoin,lnod)

endif

!

if(ndimn.eq.3) then

call wallpres3d(jfile,nboun)

!

!         call skinfric3d(nface,nboun,npoin,lnod)

endif

!

close(jfile,status='keep')

!

1000 format(2i8,e20.10,L7,i8)

1001 format(i7,4L7)

1002 format(i10,3e18.8)

1005 format(3i8)

1010 format(6i8)

1011 format(7i8)

1020 format(9i8)

1030 format(i8,3e18.8)

1040 format(i8,5e18.8)

1050 format(3i8,3e18.8)

1060 format(4i8,L7,i8)

1070 format(8i8)

1080 format(8i9)

1090 format(5f12.6)

1095 format(e16.8,3e18.8)

1096 format(f10.6,L7,e16.8)

6005 format('    x         P/Pinf        Pressure Coefficient')

6010 format(f10.5,f10.5,f10.6)

6500 format(6f10.5)

!

2000 format('ntime  ntstp  time   adap    igmres     ')

2001 format(' ntstp    adap     remesh    incom     artvis')

2002 format(' igeom      dimcorn   endcorn     angle')

2003 format('nvisc   isolve  ierrind  iprecnd   artvis     imppa')

2005 format(' nnode      nequa     ndimn')

2010 format('nelem  npoin  nboun  nface   imax   jmax')

2020 format('element and nodes distribution')

2030 format('x & y coordinates of each point')

2040 format('calculated unknown variables')

2050 format(' boundary conditions')

2060 format(' Adaptive Mesh Information')

2070 format(' hanging nodes')

2080 format(' boundary surfaces information')

2090 format(' cfl#    s1     s2     s3     s4')

2095 format(' mach#    Reynolds#   Temp(inf)     Twall')

2096 format(' Exponent for Implicitness Parameters      strm  ')

3000 format(' Element Block Information')

!

return

end subroutine output

!

! --------------------------------------------------------------------

!

subroutine outimp(ifile,nelem,npoin,nnode,npold,adap)

!

Use grid_info

Use Var_parm

!

Implicit None

!

integer,intent(in) :: ifile,nelem,npoin,nnode,npold

!

logical,intent(in) :: adap

integer,allocatable,dimension(:) :: npnum

real,allocatable,dimension(:) :: s1poi,s3poi

real,allocatable,dimension(:) :: s2poi,s4poi

real,allocatable,dimension(:) :: s3pcoi,s4pcoi

integer :: npoi1,ipoin,ielem,jnode

integer :: status

!

npoi1 = npoin

if(adap.and.mdimn>1) npoi1 = npold

!

allocate(npnum(npoi1),STAT=status)

allocate(s1poi(npoi1),s2poi(npoi1),s3poi(npoi1),stat=status)

allocate(s4poi(npoi1),s3pcoi(npoi1),s4pcoi(npoi1),stat=status)

!

do ipoin=1,npoi1

npnum(ipoin)=0

s1poi(ipoin)=0.0

s3poi(ipoin)=0.0

s2poi(ipoin)=0.0

s4poi(ipoin)=0.0

s3pcoi(ipoin) = 0.0

s4pcoi(ipoin) = 0.0

enddo

!

if(ifile.eq.11)       &

& open(unit=31,file='imp_ele.out')

if(ifile.eq.11)       &

&   open(unit=51,file='diff_var.out')

do ielem=1,nelem

if(ifile.eq.11) then

write(31,1000)ielem, s1ele(ielem), s2ele(ielem),  &

&                    s3ele(ielem), s4ele(ielem)

endif

!

do jnode=1,nnode

ipoin=lnod(jnode,ielem)

npnum(ipoin)=npnum(ipoin)+1

s1poi(ipoin)=s1poi(ipoin)+s1ele(ielem)

s2poi(ipoin)=s2poi(ipoin)+s2ele(ielem)

s3poi(ipoin)=s3poi(ipoin)+s3ele(ielem)

s4poi(ipoin)=s4poi(ipoin)+s4ele(ielem)

s3pcoi(ipoin) = s3pcoi(ipoin)+s3pec(ielem)

s4pcoi(ipoin) = s4pcoi(ipoin)+s4pec(ielem)

enddo

enddo

!

npoi1 = npoin

if(adap.and.mdimn>1) npoi1 = npold

do ipoin=1,npoi1

if(npnum(ipoin).eq.0) write(*,1100) ipoin

s1poi(ipoin)=s1poi(ipoin)/float(npnum(ipoin))

s2poi(ipoin)=s2poi(ipoin)/float(npnum(ipoin))

s3poi(ipoin)=s3poi(ipoin)/float(npnum(ipoin))

s4poi(ipoin)=s4poi(ipoin)/float(npnum(ipoin))

s3pcoi(ipoin)=s3pcoi(ipoin)/float(npnum(ipoin))

s4pcoi(ipoin)=s4pcoi(ipoin)/float(npnum(ipoin))

write(ifile,1000)ipoin,s1poi(ipoin),s2poi(ipoin),   &

&              s3poi(ipoin),s4poi(ipoin)

if(ifile.eq.11)       &

&   write(51,1000)ipoin,s3poi(ipoin),s4poi(ipoin),  &

&                 s3pcoi(ipoin),s4pcoi(ipoin)

enddo

!

deallocate(npnum,STAT=status)

deallocate(s1poi,s2poi,s3poi,s4poi,s3pcoi,s4pcoi,stat=status)

!

1000 format(i8,2x,4f16.8)

1100 format(' NPNUM at point ',i8,' is zero')

!

close(ifile,status='keep')

!

return

end subroutine outimp

!

!     ***************************************************************

!     Subroutine to print wall pressure for Three-Dimensional Problems

!

Subroutine wallpres3d(jfile,nboun)

!

Use grid_info

Use bnd_cnd

Use flow_var

!

Implicit None

!

integer,intent(in) :: jfile,nboun

integer :: iboun,ipoin,kpoin

real,dimension(nboun) :: relx,rely,relp,cp

!

write(jfile,1000)

!

kpoin = 0

!

do iboun = 1,nboun

if(ibcon(2,iboun).eq.1.or.ibcon(2,iboun).eq.4) then

ipoin = ibcon(1,iboun)

kpoin = kpoin + 1

relx(kpoin) = x(1,ipoin)

rely(kpoin) = x(2,ipoin)

relp(kpoin) = press(ipoin)/press(1)

cp(kpoin) = 2.*(press(ipoin) - press(1))

endif

enddo

!

call quick_sort_2d(kpoin,relx,rely,relp,cp)

!

do ipoin = 1,kpoin

write(jfile,1005) relx(ipoin),rely(ipoin),relp(ipoin),cp(ipoin)

enddo

!

close(jfile,status='keep')

!

1000 format('     X        Y          P/Pinf      Pressure Coefficient'  &

&       )

1005 format(4(f12.8,1x))

!

return

end subroutine wallpres3d

!

!     *****************************************************************

!     Subroutine to sort the output for WALLPRES3D (sorted along Y at

!           constant x)

!

Recursive Subroutine quick_Sort_2d(npoin,x,y,a1,a2)

!

Implicit None

!

integer,intent(in) :: npoin

real,dimension(npoin),intent(inout) :: x,y,a1,a2

integer :: icoun,ipoin,ip1

!

!     sort in X-direction first

!

! Local variable

!

integer :: icoun,icoun1,j,left,right

real :: xref

!

CALL quick_sort_x(1, npoin)

!

!     Sort in Y-direction

!

icoun = 0

do

icoun = icoun + 1

if(icoun>=npoin) exit

xref = x(icoun)

left = icoun

icoun1 = icoun + 1

do j = icoun1,npoin

if(abs(x(j)-xref)>1.0e-08) exit

icoun = icoun + 1

enddo

right = icoun

call quick_sort_y(left,right)

!

end do

!

CONTAINS

!

RECURSIVE SUBROUTINE quick_sort_x(left_end, right_end)

!

INTEGER, INTENT(IN) :: left_end, right_end

!

!     Local variables

INTEGER             :: i, j

REAL                :: reference

real :: xtemp,ytemp,a1temp,a2temp

INTEGER, PARAMETER  :: max_simple_sort_size = 6

!

IF (right_end < left_end + max_simple_sort_size) THEN

! Use interchange sort for small lists

CALL interchange_sort_x(left_end, right_end)

!

ELSE

! Use partition ("quick") sort

reference = x((left_end + right_end)/2)

i = left_end - 1; j = right_end + 1

!

DO

! Scan list from left end until element >= reference is found

DO

i = i + 1

IF (x(i) >= reference) EXIT

END DO

! Scan list from right end until element <= reference is found

DO

j = j - 1

IF (x(j) <= reference) EXIT

END DO

!

IF (i < j) THEN

! Swap two out-of-order elements

xtemp = x(i); x(i) = x(j); x(j) = xtemp

ytemp = y(i); y(i) = y(j); y(j) = ytemp

a1temp = a1(i); a1(i) = a1(j); a1(j) = a1temp

a2temp = a2(i); a2(i) = a2(j); a2(j) = a2temp

ELSE IF (i == j) THEN

i = i + 1

EXIT

ELSE

EXIT

END IF

END DO

!

IF (left_end < j) CALL quick_sort_x(left_end, j)

IF (i < right_end) CALL quick_sort_x(i, right_end)

END IF

END SUBROUTINE quick_sort_x

SUBROUTINE interchange_sort_x(left_end, right_end)

!

INTEGER, INTENT(IN) :: left_end, right_end

!

!     Local variables

INTEGER             :: i, j

REAL                :: xtemp,ytemp,a1temp,a2temp

!

DO i = left_end, right_end - 1

DO j = i+1, right_end

IF (x(i) > x(j)) THEN

xtemp = x(i); x(i) = x(j); x(j) = xtemp

ytemp = y(i); y(i) = y(j); y(j) = ytemp

a1temp = a1(i); a1(i) = a1(j); a1(j) = a1temp

a2temp = a2(i); a2(i) = a2(j); a2(j) = a2temp

END IF

END DO

END DO

!

END SUBROUTINE interchange_sort_x

!

RECURSIVE SUBROUTINE quick_sort_y(left_end, right_end)

!

INTEGER, INTENT(IN) :: left_end, right_end

!

!     Local variables

INTEGER             :: i, j

REAL                :: reference

real :: xtemp,ytemp,a1temp,a2temp

INTEGER, PARAMETER  :: max_simple_sort_size = 6

!

IF (right_end < left_end + max_simple_sort_size) THEN

! Use interchange sort for small lists

CALL interchange_sort_x(left_end, right_end)

!

ELSE

! Use partition ("quick") sort

reference = y((left_end + right_end)/2)

i = left_end - 1; j = right_end + 1

!

DO

! Scan list from left end until element >= reference is found

DO

i = i + 1

IF (y(i) >= reference) EXIT

END DO

! Scan list from right end until element <= reference is found

DO

j = j - 1

IF (y(j) <= reference) EXIT

END DO

!

IF (i < j) THEN

! Swap two out-of-order elements

xtemp = x(i); x(i) = x(j); x(j) = xtemp

ytemp = y(i); y(i) = y(j); y(j) = ytemp

a1temp = a1(i); a1(i) = a1(j); a1(j) = a1temp

a2temp = a2(i); a2(i) = a2(j); a2(j) = a2temp

ELSE IF (i == j) THEN

i = i + 1

EXIT

ELSE

EXIT

END IF

END DO

!

IF (left_end < j) CALL quick_sort_y(left_end, j)

IF (i < right_end) CALL quick_sort_y(i, right_end)

END IF

END SUBROUTINE quick_sort_y

SUBROUTINE interchange_sort_y(left_end, right_end)

!

INTEGER, INTENT(IN) :: left_end, right_end

!

!     Local variables

INTEGER             :: i, j

REAL                :: xtemp,ytemp,a1temp,a2temp

!

DO i = left_end, right_end - 1

DO j = i+1, right_end

IF (y(i) > y(j)) THEN

xtemp = x(i); x(i) = x(j); x(j) = xtemp

ytemp = y(i); y(i) = y(j); y(j) = ytemp

a1temp = a1(i); a1(i) = a1(j); a1(j) = a1temp

a2temp = a2(i); a2(i) = a2(j); a2(j) = a2temp

END IF

END DO

END DO

!

END SUBROUTINE interchange_sort_y

!

END SUBROUTINE quick_sort_2d

!

!     ***************************************************************

!     The following subroutine calculates the skin friction at the wall

!

subroutine wallpropt(jfile,nelem,nface,nboun,npoin,lpoin)

!

Use param_var

Use grid_info

Use thermo_propt

Use bnd_cnd

Use flow_var

Use bface_info

!

Implicit None

!

integer,intent(in) :: jfile,nelem,nface,nboun,npoin

integer,intent(in) :: lpoin(mnode,melem)

integer :: kflag(npoin)

real :: cf(nboun),heatrt(nboun)

real :: relx(nboun),relp(nboun),cp(nboun)

!

integer :: ipoin,kpoin,iface,ielem,ibidx,inode,ibnod,lnode

integer :: ipp1,jpp1,kpp1

real :: dy,dudy,dtdy,cvisc,ccond,en1,en2,tan_a

real :: dx,y1,ydiff1,ydiff2,udif,tdif,ucalc,tcalc

real :: dudn,dtdn,dudx,dtdx

!

do ipoin = 1, npoin

kflag(ipoin) = 0

enddo

!

kpoin = 0

!

do iface = 1,nface

ielem=lface(7,iface)

ibidx=lface(8,iface)

!

if(ibidx.eq.1) then

do inode=1,2

ibnod = lface(inode+2,iface)

ipoin = ibcon(1,ibnod)

if(kflag(ipoin).eq.0) then

kpoin = kpoin + 1

relx(kpoin) = x(1,ipoin)

relp(kpoin) = press(ipoin)/press(1)

cp(kpoin) = 2.*(press(ipoin) - press(1))

lnode=lface(inode+4,iface)

if(lnode.eq.1) ipp1=lpoin(4,ielem)

if(lnode.eq.2) ipp1=lpoin(3,ielem)

dy=x(2,ipp1)-x(2,ipoin)

dudy=(vel(1,ipp1)-vel(1,ipoin))/dy

dtdy=(tempr(ipp1)-tempr(ipoin))/dy

if(tempr(ipoin)<1.0e-12) then

cvisc = 1.0

else

cvisc=(1.0+csuth)/(tempr(ipoin)+csuth)*   &

&               tempr(ipoin)**1.5

endif

ccond=cvisc/gm1m2pr

cf(kpoin)=2.*cvisc*dudy/creyn

heatrt(kpoin)=2.*ccond*dtdy/creyn

kflag(ipoin) = 1

!

endif

!

enddo

!

elseif(ibidx.eq.4) then

inode = 2

ibnod = lface(inode+2,iface)

ipoin = ibcon(1,ibnod)

!

kpoin = kpoin + 1

relx(kpoin) = x(1,ipoin)

relp(kpoin) = press(ipoin)/press(1)

cp(kpoin) = 2.*(press(ipoin) - press(1))

!

ipp1=lpoin(3,ielem)

en1 = -rbcon(2,ibnod)

en2 = rbcon(1,ibnod)

tan_a = rbcon(1,ibnod)/rbcon(2,ibnod)

ipp1 = lpoin(3,ielem)

jpp1 = lpoin(4,ielem)

kpp1 = lpoin(1,ielem)

dy = x(2,ipp1) - x(2,ipoin)

dx = x(1,ipoin) - x(1,kpp1)

dudy=(vel(1,ipp1)-vel(1,ipoin))/dy

dtdy=(tempr(ipp1)-tempr(ipoin))/dy

if(tempr(ipoin)<1.0e-12) then

cvisc = 1.0

else

cvisc=(1.0+csuth)/(tempr(ipoin)+csuth)*     &

&               tempr(ipoin)**1.5

endif

ccond=cvisc/gm1m2pr

y1 = x(2,kpp1) + dx*tan_a

ydiff1 = y1  - x(2,kpp1)

ydiff2 = x(2,jpp1) - x(2,kpp1)

udif = vel(1,jpp1) - vel(1,kpp1)

tdif = tempr(jpp1) - tempr(kpp1)

ucalc = vel(1,kpp1) + ydiff1/ydiff2*udif

tcalc = tempr(kpp1) +ydiff1/ydiff2*tdif

dudx = (vel(1,ipoin) - ucalc)/dx

dtdx = (tempr(ipoin) - tcalc)/dx

dudn = dudx*en1 + dudy*en2

dtdn = dtdx*en1 + dtdy*en2

cf(kpoin) = 2.*cvisc*dudn/creyn

heatrt(kpoin) = 2.*ccond*dtdn/creyn

kflag(ipoin) = 1

endif

enddo

!

call quick_sort(kpoin,relx,relp,cp,cf,heatrt)

!

write(jfile,2000)

write(jfile,1000) cmach,creyn,tinf,twall

write(jfile,2010)

write(jfile,1010) nelem,npoin,nboun,nface

!

write(jfile,2100)

!

do ipoin = 1,kpoin

write(jfile,1100) relx(ipoin),relp(ipoin),cp(ipoin),cf(ipoin),  &

&                     heatrt(ipoin)

enddo

!

close(jfile,status='keep')

!

1000 format(4e16.8)

1010 format(4i8)

1100 format(5e14.6)

!

2000 format('     cmach           creyn        tinf      twall')

2010 format('  nelem      npoin     nboun      nface')

2100 format('      X         Pressure      CP       Cf    Heat Rate')

!

return

end subroutine wallpropt

!

!     ****************************************************************

!     Subroutine to sort an array by ascending order

!

RECURSIVE SUBROUTINE quick_sort(npoin,x,a1,a2,a3,a4)

!

! Quick sort routine adapted from:

! Brainerd, W.S., Goldberg, C.H. & Adams, J.C. (1990) "Programmer's Guide to

! Fortran 90", McGraw-Hill  ISBN 0-07-000248-7, pages 149-150.

!

Implicit None

!

integer,intent(in) :: npoin

real,intent(inout) :: x(npoin),a1(npoin),a2(npoin)

real,intent(inout) :: a3(npoin),a4(npoin)

!

! Local variable

INTEGER :: i

CALL quick_sort_1(1, npoin)

CONTAINS

RECURSIVE SUBROUTINE quick_sort_1(left_end, right_end)

INTEGER, INTENT(IN) :: left_end, right_end

!     Local variables

INTEGER             :: i, j

REAL                :: reference

real :: xtemp,a1temp,a2temp,a3temp,a4temp

INTEGER, PARAMETER  :: max_simple_sort_size = 6

IF (right_end < left_end + max_simple_sort_size) THEN

! Use interchange sort for small lists

CALL interchange_sort(left_end, right_end)

ELSE

! Use partition ("quick") sort

reference = x((left_end + right_end)/2)

i = left_end - 1; j = right_end + 1

DO

! Scan list from left end until element >= reference is found

DO

i = i + 1

IF (x(i) >= reference) EXIT

END DO

! Scan list from right end until element <= reference is found

DO

j = j - 1

IF (x(j) <= reference) EXIT

END DO

IF (i < j) THEN

! Swap two out-of-order elements

xtemp = x(i); x(i) = x(j); x(j) = xtemp

a1temp = a1(i); a1(i) = a1(j); a1(j) = a1temp

a2temp = a2(i); a2(i) = a2(j); a2(j) = a2temp

a3temp = a3(i); a3(i) = a3(j); a3(j) = a3temp

a4temp = a4(i); a4(i) = a4(j); a4(j) = a4temp

ELSE IF (i == j) THEN

i = i + 1

EXIT

ELSE

EXIT

END IF

END DO

IF (left_end < j) CALL quick_sort_1(left_end, j)

IF (i < right_end) CALL quick_sort_1(i, right_end)

END IF

END SUBROUTINE quick_sort_1

SUBROUTINE interchange_sort(left_end, right_end)

INTEGER, INTENT(IN) :: left_end, right_end

!     Local variables

INTEGER             :: i, j

REAL                :: xtemp,a1temp,a2temp,a3temp,a4temp

DO i = left_end, right_end - 1

DO j = i+1, right_end

IF (x(i) > x(j)) THEN

xtemp = x(i); x(i) = x(j); x(j) = xtemp

a1temp = a1(i); a1(i) = a1(j); a1(j) = a1temp

a2temp = a2(i); a2(i) = a2(j); a2(j) = a2temp

a3temp = a3(i); a3(i) = a3(j); a3(j) = a3temp

a4temp = a4(i); a4(i) = a4(j); a4(j) = a4temp

END IF

END DO

END DO

END SUBROUTINE interchange_sort

END SUBROUTINE quick_sort

!

!   ****************************************************************

!   Subroutine to calculate boundary face connectivity for 3-D

!   isoparametric element

!

subroutine genbfac3(nelem,nboun,nnode,nface)

!

Use grid_info

Use bnd_cnd

Use bface_info

!

Implicit None

!

integer,intent(in) :: nelem,nboun,nnode

integer,intent(inout) :: nface

integer :: lnode(7)

integer :: ielem,lbnode,inode,ipoin,lflag,inod

integer :: msum,ibc1,ibc2,iface,iboun,jpoin,kboun

integer :: j

!

!     lface(1,iface) = point number of 1st boundary node

!           2        = point number of 2nd boundary node

!           3        = point number of 3rd boundary node

!           4        = point number of 4th boundary node

!           5        = orientation of boundary surface

!                  1 - parallel to x-y plane

!                  2 - parallel to y-z plane

!                  3 - parallel to x-z plane

!           6        = direction of vector normal to surface

!                  1 - positive direction

!                  2 - negative direction

!           7        = element number

!

! lfacnode(1-4,iface) = local node number of boundary nodes

! lfacnode(5-8,iface) = Boundary point number of boundary nodes

!

nface = 0

!

elemntlp: do ielem = 1,nelem

!

lbnode = 0

do inode = 1, nnode

ipoin = lnod(inode,ielem)

lflag = 0

!        write(35,651) ipoin,indbc(ipoin)

!  651 format(' ipoin = ',i8,' Boundary Condition:  ',i4)

if(indbc(ipoin).ne.0) then

lflag = 1

lbnode = lbnode + 1

goto 50

endif

50 continue

if(lflag.eq.1) lnode(lbnode) = inode

enddo

!

!      write(45,650) ielem,lbnode,(lnode(j),j=1,lbnode)

650 format(' Element No. ',i8,' lbnode:  ',i4,' Nodes:  ',7i4)

!

if(lbnode.eq.4) then     ! One Boundary Face

nface = nface + 1

lface(7,nface) = ielem

msum = 0

do inod = 1,4

msum = msum + lnode(inod)

enddo

!

!        write(45,655) msum

655 format(' MSUM = ',i8)

!

if(msum.eq.10) then           ! Bottom Face

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(3,ielem)

lface(4,nface) = lnod(4,ielem)

lface(5,nface) = 1

lface(6,nface) = 2

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 3

lfacnode(4,nface) = 4

!

ipoin = lnod(4,ielem)

ibc1 = indbc(ipoin)

ibc2 = indbc(lnod(1,ielem))

lface(8,nface) = ibc1

if(ibc2.eq.2) lface(8,nface) = ibc2

if(ibc1.eq.4) lface(8,nface) = ibc2

!

elseif(msum.eq.26) then        ! Top Face

lface(1,nface) = lnod(5,ielem)

lface(2,nface) = lnod(6,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 1

lface(6,nface) = 1

ipoin = lnod(7,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 5

lfacnode(2,nface) = 6

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

elseif(msum.eq.18) then         !  Left or Right Faces

if(lnode(1).eq.1) then

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(4,ielem)

lface(3,nface) = lnod(8,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 2

lface(6,nface) = 2

ipoin = lnod(1,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 4

lfacnode(3,nface) = 8

lfacnode(4,nface) = 5

else

lface(1,nface) = lnod(2,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(6,ielem)

lface(5,nface) = 2

lface(6,nface) = 1

ipoin = lnod(2,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 2

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 6

endif

elseif(msum.eq.22) then        ! Back Face

lface(1,nface) = lnod(4,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 3

lface(6,nface) = 2

ipoin = lnod(4,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 4

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

elseif(msum.eq.14) then        ! Front Face

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(6,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 3

lface(6,nface) = 1

ipoin = lnod(1,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 6

lfacnode(4,nface) = 5

endif

endif

!

if(lbnode.eq.6) then      ! Two Boundary Faces

!

msum = 0

do inod = 1,6

msum = msum + lnode(inod)

enddo

!

!      write(45,655) msum

!

if(msum.eq.21) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(6,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 3

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(6,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 6

lfacnode(4,nface) = 5

!

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(3,ielem)

lface(4,nface) = lnod(4,ielem)

lface(5,nface) = 1

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(3,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 3

lfacnode(4,nface) = 4

!

elseif(msum.eq.24) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(6,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 3

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(5,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 6

lfacnode(4,nface) = 5

!

nface = nface + 1

lface(1,nface) = lnod(2,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(6,ielem)

lface(5,nface) = 2

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(7,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 2

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 6

!

elseif(msum.eq.29) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(6,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 3

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(1,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 6

lfacnode(4,nface) = 5

!

nface = nface + 1

lface(1,nface) = lnod(5,ielem)

lface(2,nface) = lnod(6,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 1

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(7,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 5

lfacnode(2,nface) = 6

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

elseif(msum.eq.26) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(6,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 3

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(6,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 6

lfacnode(4,nface) = 5

!

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(4,ielem)

lface(3,nface) = lnod(8,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 2

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(8,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 4

lfacnode(3,nface) = 8

lfacnode(4,nface) = 5

!

elseif(msum.eq.31) then

nface = nface + 1

lface(1,nface) = lnod(5,ielem)

lface(2,nface) = lnod(6,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 1

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(7,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 5

lfacnode(2,nface) = 6

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

if(lnode(1).eq.1) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(4,ielem)

lface(3,nface) = lnod(8,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 2

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(1,ielem)

lface(8,nface) = indbc(ipoin)

jpoin = lnod(7,ielem)

lface(8,nface-1) = indbc(jpoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 4

lfacnode(3,nface) = 8

lfacnode(4,nface) = 5

else

!

nface = nface + 1

lface(1,nface) = lnod(2,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(6,ielem)

lface(5,nface) = 2

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(2,ielem)

lface(8,nface) = indbc(ipoin)

jpoin = lnod(8,ielem)

lface(8,nface-1) = indbc(jpoin)

lfacnode(1,nface) = 2

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 6

endif

!

elseif(msum.eq.23) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(3,ielem)

lface(4,nface) = lnod(4,ielem)

lface(5,nface) = 1

lface(6,nface) = 2

lface(7,nface) = ielem

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 3

lfacnode(4,nface) = 4

!

if(lnode(5).eq.5) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(4,ielem)

lface(3,nface) = lnod(8,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 2

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(5,ielem)

lface(8,nface) = indbc(ipoin)

jpoin = lnod(2,ielem)

lface(8,nface-1) = indbc(jpoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 4

lfacnode(3,nface) = 8

lfacnode(4,nface) = 5

else

!

nface = nface + 1

lface(1,nface) = lnod(2,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(6,ielem)

lface(5,nface) = 2

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(6,ielem)

lface(8,nface) = indbc(ipoin)

jpoin = lnod(1,ielem)

lface(8,nface-1) = indbc(jpoin)

lfacnode(1,nface) = 2

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 6

endif

elseif(msum.eq.33) then

nface = nface + 1

lface(1,nface) = lnod(5,ielem)

lface(2,nface) = lnod(6,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 1

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(6,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 5

lfacnode(2,nface) = 6

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

nface = nface + 1

lface(1,nface) = lnod(4,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 3

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(3,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 4

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

elseif(msum.eq.28) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(4,ielem)

lface(3,nface) = lnod(8,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 2

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(1,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 4

lfacnode(3,nface) = 8

lfacnode(4,nface) = 5

!

nface = nface + 1

lface(1,nface) = lnod(4,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 3

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(3,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 4

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

elseif(msum.eq.25) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(3,ielem)

lface(4,nface) = lnod(4,ielem)

lface(5,nface) = 1

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(1,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 3

lfacnode(4,nface) = 4

!

nface = nface + 1

lface(1,nface) = lnod(4,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 3

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(7,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 4

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

elseif(msum.eq.30) then

nface = nface + 1

lface(1,nface) = lnod(2,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(6,ielem)

lface(5,nface) = 2

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(2,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 2

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 6

!

nface = nface + 1

lface(1,nface) = lnod(4,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 3

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(8,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 4

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

endif

endif

!

if(lbnode.eq.7) then    ! Three Boundary Faces

!

msum = 0

do inod = 1,7

msum = msum +lnode(inod)

enddo

!

!      write(45,655)msum

!

if(msum.eq.33) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(6,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 3

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(2,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 6

lfacnode(4,nface) = 5

!

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(4,ielem)

lface(3,nface) = lnod(8,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 2

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(4,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 4

lfacnode(3,nface) = 8

lfacnode(4,nface) = 5

!

nface = nface + 1

lface(1,nface) = lnod(5,ielem)

lface(2,nface) = lnod(6,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 1

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(7,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 5

lfacnode(2,nface) = 6

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

elseif(msum.eq.32) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(6,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 3

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(1,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 6

lfacnode(4,nface) = 5

!

nface = nface + 1

lface(1,nface) = lnod(2,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(6,ielem)

lface(5,nface) = 2

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(3,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 2

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 6

!

nface = nface + 1

lface(1,nface) = lnod(5,ielem)

lface(2,nface) = lnod(6,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 1

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(8,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 5

lfacnode(2,nface) = 6

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

elseif(msum.eq.28) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(6,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 3

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(5,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 6

lfacnode(4,nface) = 5

!

nface = nface + 1

lface(1,nface) = lnod(2,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(6,ielem)

lface(5,nface) = 2

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(7,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 2

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 6

!

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(3,ielem)

lface(4,nface) = lnod(4,ielem)

lface(5,nface) = 1

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(4,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 3

lfacnode(4,nface) = 4

!

elseif(msum.eq.29) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(6,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 3

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(6,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 6

lfacnode(4,nface) = 5

!

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(3,ielem)

lface(4,nface) = lnod(4,ielem)

lface(5,nface) = 1

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(3,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 3

lfacnode(4,nface) = 4

!

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(4,ielem)

lface(3,nface) = lnod(8,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 2

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(8,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 4

lfacnode(3,nface) = 8

lfacnode(4,nface) = 5

!

elseif(msum.eq.34) then

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(4,ielem)

lface(3,nface) = lnod(8,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 2

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(1,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 4

lfacnode(3,nface) = 8

lfacnode(4,nface) = 5

!

nface = nface + 1

lface(1,nface) = lnod(5,ielem)

lface(2,nface) = lnod(6,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 1

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(6,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 5

lfacnode(2,nface) = 6

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

nface = nface + 1

lface(1,nface) = lnod(4,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 3

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(3,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 4

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

elseif(msum.eq.35) then

nface = nface + 1

lface(1,nface) = lnod(5,ielem)

lface(2,nface) = lnod(6,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 1

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(5,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 5

lfacnode(2,nface) = 6

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

nface = nface + 1

lface(1,nface) = lnod(4,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 3

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(4,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 4

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

nface = nface + 1

lface(1,nface) = lnod(2,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(6,ielem)

lface(5,nface) = 2

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(2,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 2

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 6

!

elseif(msum.eq.30) then

nface = nface + 1

lface(1,nface) = lnod(4,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 3

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(7,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 4

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(3,ielem)

lface(4,nface) = lnod(4,ielem)

lface(5,nface) = 1

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(2,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 3

lfacnode(4,nface) = 4

!

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(4,ielem)

lface(3,nface) = lnod(8,ielem)

lface(4,nface) = lnod(5,ielem)

lface(5,nface) = 2

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(5,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 4

lfacnode(3,nface) = 8

lfacnode(4,nface) = 5

!

elseif(msum.eq.31) then

nface = nface + 1

lface(1,nface) = lnod(4,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(8,ielem)

lface(5,nface) = 3

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(8,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 4

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 8

!

nface = nface + 1

lface(1,nface) = lnod(1,ielem)

lface(2,nface) = lnod(2,ielem)

lface(3,nface) = lnod(3,ielem)

lface(4,nface) = lnod(4,ielem)

lface(5,nface) = 1

lface(6,nface) = 2

lface(7,nface) = ielem

ipoin = lnod(1,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 1

lfacnode(2,nface) = 2

lfacnode(3,nface) = 3

lfacnode(4,nface) = 4

!

nface = nface + 1

lface(1,nface) = lnod(2,ielem)

lface(2,nface) = lnod(3,ielem)

lface(3,nface) = lnod(7,ielem)

lface(4,nface) = lnod(6,ielem)

lface(5,nface) = 2

lface(6,nface) = 1

lface(7,nface) = ielem

ipoin = lnod(6,ielem)

lface(8,nface) = indbc(ipoin)

lfacnode(1,nface) = 2

lfacnode(2,nface) = 3

lfacnode(3,nface) = 7

lfacnode(4,nface) = 6

endif

endif

!

enddo elemntlp

!

!      Store boundary point number in lfacnode(5-8,iface)

!

do iface = 1,nface

do inode = 1,4

ipoin = lface(inode,iface)

midloop: do iboun = 1,nboun

jpoin = ibcon(1,iboun)

if(ipoin.eq.jpoin) then

kboun = iboun

exit midloop

endif

enddo midloop

lfacnode(inode+4,iface) = kboun

enddo

enddo

!

open(unit=35,file='bface.out')

!

write(35,500)nface

500 format(' # of Boundary Faces = ',i8)

do iface = 1,nface

write(35,520) iface

write(35,510)(lface(j,iface),j=1,8)

write(35,510) (lfacnode(j,iface),j=1,8)

enddo

!

close(unit=35,status='keep')

!

510 format(8i8)

520 format(' Boundary Face Number ',i8)

!

return

end	subroutine genbfac3

!

!   ****************************************************************

!     SUBROUTINE TO CALCULATE DOMAIN SHAPE FUNCTIONS AND DERIVATIVES

!

SUBROUTINE SHAPDOM3

!

Use param_var

Use Gaus_quad

Use shap_fcns

Use shap3d_fcns

!

Implicit None

!

integer :: igaus,i,j,k

!

!     LINEAR INTERPOLATION FUNCTIONS

!

IGAUS=0

xiloop: DO I = 1, NGAUS

etaloop: DO J = 1, NGAUS

zetaloop: Do K = 1,ngaus

IGAUS = IGAUS + 1

PHI(1,IGAUS)=0.125*(1.-XI(I))*(1.-XI(J))*(1.-XI(K))

PHI(2,IGAUS)=0.125*(1.+XI(I))*(1.-XI(J))*(1.-XI(K))

PHI(3,IGAUS)=0.125*(1.+XI(I))*(1.+XI(J))*(1.-XI(K))

PHI(4,IGAUS)=0.125*(1.-XI(I))*(1.+XI(J))*(1.-XI(K))

PHI(5,IGAUS)=0.125*(1.-XI(I))*(1.-XI(J))*(1.+XI(K))

PHI(6,IGAUS)=0.125*(1.+XI(I))*(1.-XI(J))*(1.+XI(K))

PHI(7,IGAUS)=0.125*(1.+XI(I))*(1.+XI(J))*(1.+XI(K))

PHI(8,IGAUS)=0.125*(1.-XI(I))*(1.+XI(J))*(1.+XI(K))

!

!     derivatives of linear interpolation functions

!

dpxi(1,1,IGAUS)=-0.125*(1.-xi(j))*(1.-xi(k))

dpxi(2,1,IGAUS)=-0.125*(1.-xi(i))*(1.-xi(k))

dpxi(3,1,IGAUS)=-0.125*(1.-xi(i))*(1.-xi(j))

dpxi(1,2,IGAUS)=0.125*(1.-xi(j))*(1.-xi(k))

dpxi(2,2,IGAUS)=-0.125*(1.+xi(i))*(1.-xi(k))

dpxi(3,2,IGAUS)=-0.125*(1.+xi(i))*(1.-xi(j))

dpxi(1,3,IGAUS)=0.125*(1.+xi(j))*(1.-xi(k))

dpxi(2,3,IGAUS)=0.125*(1.+xi(i))*(1.-xi(k))

dpxi(3,3,IGAUS)=-0.125*(1.+xi(i))*(1.+xi(j))

dpxi(1,4,IGAUS)=-0.125*(1.+xi(j))*(1.-xi(k))

dpxi(2,4,IGAUS)=0.125*(1.-xi(i))*(1.-xi(k))

dpxi(3,4,IGAUS)=-0.125*(1.-xi(i))*(1.+xi(j))

dpxi(1,5,IGAUS)=-0.125*(1.-xi(j))*(1.+xi(k))

dpxi(2,5,IGAUS)=-0.125*(1.-xi(i))*(1.+xi(k))

dpxi(3,5,IGAUS)=0.125*(1.-xi(i))*(1.-xi(j))

dpxi(1,6,IGAUS)=0.125*(1.-xi(j))*(1.+xi(k))

dpxi(2,6,IGAUS)=-0.125*(1.+xi(i))*(1.+xi(k))

dpxi(3,6,IGAUS)=0.125*(1.+xi(i))*(1.-xi(j))

dpxi(1,7,IGAUS)=0.125*(1.+xi(j))*(1.+xi(k))

dpxi(2,7,IGAUS)=0.125*(1.+xi(i))*(1.+xi(k))

dpxi(3,7,IGAUS)=0.125*(1.+xi(i))*(1.+xi(j))

dpxi(1,8,IGAUS)=-0.125*(1.+xi(j))*(1.+xi(k))

dpxi(2,8,IGAUS)=0.125*(1.-xi(i))*(1.+xi(k))

dpxi(3,8,IGAUS)=0.125*(1.-xi(i))*(1.+xi(j))

WW(IGAUS)=W(I)*W(J)*W(K)

!

!     Second derivatives of interpolation functions

!

!     d2phidxde - d2phi/(dxi deta)

!     d2phidxdz - d2phi/(dxi dzeta)

!     d2phidedz - d2phi/(deta dzeta)

!

d2phidxde(1,igaus) = 0.125*(1. - xi(k))

d2phidxdz(1,igaus) = 0.125*(1. - xi(j))

d2phidedz(1,igaus) = 0.125*(1. - xi(i))

d2phidxde(2,igaus) = -0.125*(1. - xi(k))

d2phidxdz(2,igaus) = -0.125*(1. - xi(j))

d2phidedz(2,igaus) = 0.125*(1. + xi(i))

d2phidxde(3,igaus) = 0.125*(1. - xi(k))

d2phidxdz(3,igaus) = -0.125*(1. + xi(j))

d2phidedz(3,igaus) = -0.125*(1. + xi(i))

d2phidxde(4,igaus) = -0.125*(1. - xi(k))

d2phidxdz(4,igaus) = 0.125*(1. + xi(j))

d2phidedz(4,igaus) = -0.125*(1. - xi(i))

d2phidxde(5,igaus) = 0.125*(1. + xi(k))

d2phidxdz(5,igaus) = -0.125*(1. - xi(j))

d2phidedz(5,igaus) = -0.125*(1. - xi(i))

d2phidxde(6,igaus) = -0.125*(1. + xi(k))

d2phidxdz(6,igaus) = 0.125*(1. - xi(j))

d2phidedz(6,igaus) = -0.125*(1. + xi(i))

d2phidxde(7,igaus) = 0.125*(1. + xi(k))

d2phidxdz(7,igaus) = 0.125*(1. + xi(j))

d2phidedz(7,igaus) = 0.125*(1. + xi(i))

d2phidxde(8,igaus) = -0.125*(1. + xi(k))

d2phidxdz(8,igaus) = -0.125*(1. + xi(j))

d2phidedz(8,igaus) = 0.125*(1. - xi(i))

!

!          write(35,75) igaus

!          write(35,100)(phi(inode,igaus),inode=1,mnode)

!          write(35,100) (dpxi(1,inode,igaus),inode=1,mnode)

!          write(35,100)(dpxi(2,inode,igaus),inode=1,mnode)

!          write(35,100)(dpxi(3,inode,igaus),inode=1,mnode)

enddo zetaloop

enddo etaloop

enddo xiloop

!

75 format(' SHAPE FUNCTIONS AND DERIVATIVES AT GAUSSIAN POINT '   &

&       ,I4)

100 format(4(e16.8,1x))

!

RETURN

END

!    *************************************************************

SUBROUTINE DERSHA_3D(NNODE,ndimn,xloc,dpxi,d2phidxde,&

&                     d2phidxdz,d2phidedz,iflag,koun,dshpdx,    &

&                     detjac,d2phidx)

!

!     THIS SUBROUTINE CALCULATES THE DERIVATIVES OF THE SHAPE FUNCTIONS

!

Use param_var

!      Use shap_fcns

!      Use shap3d_fcns

!      Use grid_info

!      Use Gaus_quad

!

Implicit None

!

integer,intent(in) :: nnode,ndimn,iflag,koun

real,dimension(ndimn,nnode),intent(in) :: xloc

real,intent(in) :: dpxi(mdimn,mnode,mgaus)

real,intent(in),dimension(mnode,mgaus) :: d2phidxde,d2phidxdz

real,intent(in),dimension(mnode,mgaus) :: d2phidedz

real,intent(out) :: dshpdx(mdimn,mnode,mgaus),detjac(mgaus)

real,intent(out) :: d2phidx(mdimn,mdimn,mnode,mgaus)

integer :: i,igaus,l,j,k,inode

real :: dxdxi,dxdeta,dxdzeta,dydxi,dydeta,dydzeta,dzdxi,dzdeta

real :: dzdzeta,d2xdxde,d2xdxdz,d2xdedz,d2ydxde,d2ydxdz,d2ydedz

real :: d2zdxdz,d2zdxde,d2zdedz

real :: detjacinv,detjacinv2

real :: term1,term2,term3,term4,term5,term6

real :: term11,term12,term13,term21,term22,term23,term31,term32

real :: term33

real ::  djacdxi,djacdeta,djacdzeta

real :: jacinv(ndimn,ndimn),djacinvdxi(ndimn,ndimn)

real :: djacinvdet(ndimn,ndimn),djacinvdzt(ndimn,ndimn)

real :: Wn(nnode),Wnbar(nnode),Wntild(nnode)

real :: XELEM(NNODE),YELEM(NNODE),ZELEM(NNODE)

DO I=1,NNODE

XELEM(I)=Xloc(1,i)

YELEM(I)=Xloc(2,i)

ZELEM(I)=Xloc(3,I)

enddo

!

gausloop: DO IGAUS = 1, MGAUS

DXDXI = 0.0

DXDETA = 0.0

DXDZETA = 0.0

DYDXI = 0.0

DYDETA = 0.0

DYDZETA = 0.0

DZDXI = 0.0

DZDETA = 0.0

DZDZETA = 0.0

d2xdxde = 0.0

d2xdxdz = 0.0

d2xdedz = 0.0

d2ydxde = 0.0

d2ydxdz = 0.0

d2ydedz = 0.0

d2zdxdz = 0.0

d2zdxde = 0.0

d2zdedz = 0.0

DO L = 1, NNODE

DXDXI = DXDXI + dpxi(1,L,IGAUS)*xelem(L)

DXDETA = DXDETA + dpxi(2,L,IGAUS)*xelem(L)

DXDZETA = DXDZETA + dpxi(3,L,IGAUS)*Xelem(L)

DYDXI = DYDXI + dpxi(1,L,IGAUS)*yelem(L)

DYDETA = DYDETA + dpxi(2,L,IGAUS)*yelem(L)

DYDZETA = DYDZETA + dpxi(3,L,IGAUS)*yelem(L)

DZDXI = DZDXI + dpxi(1,L,IGAUS)*zelem(L)

DZDETA = DZDETA + dpxi(2,L,IGAUS)*zelem(L)

DZDZETA = DZDZETA + dpxi(3,L,IGAUS)*zelem(L)

d2xdxde = d2xdxde + d2phidxde(L,igaus)*xelem(L)

d2xdxdz = d2xdxdz + d2phidxdz(L,igaus)*xelem(L)

d2xdedz = d2xdedz + d2phidedz(L,igaus)*xelem(L)

d2ydxde = d2ydxde + d2phidxde(L,igaus)*yelem(L)

d2ydxdz = d2ydxdz + d2phidxdz(L,igaus)*yelem(L)

d2ydedz = d2ydedz + d2phidedz(L,igaus)*yelem(L)

d2zdxde = d2zdxde + d2phidxde(L,igaus)*zelem(L)

d2zdxdz = d2zdxdz + d2phidxdz(L,igaus)*zelem(L)

d2zdedz = d2zdedz + d2phidedz(L,igaus)*zelem(L)

enddo

!

!     determinant of Jacobian

!

detjac(IGAUS)=dxdxi*dydeta*dzdzeta +   &

&                 dxdeta*dydzeta*dzdxi +   &

&                 dxdzeta*dydxi*dzdeta -   &

&                 dxdzeta*dydeta*dzdxi -   &

&                 dxdxi*dydzeta*dzdeta -   &

&                 dxdeta*dydxi*dzdzeta

!

!      if(igaus.eq.1) then

!         write(25,201)

!         do inode = 1,nnode

!            write(25,*)xelem(inode),yelem(inode),zelem(inode)

!         enddo

!         write(25,202)igaus

!         do idimn=1,ndimn

!            write(25,*)(dxxi(jdimn,idimn,igaus),jdimn=1,ndimn)

!         enddo

!      endif

!

201 format(' X, Y, and Z coordinates')

202 format(' DXDXI AT GAUSSIAN POINT',i4)

!

if(detjac(IGAUS).lt.1.0e-12)then

write(*,*)'detjac = ',detjac(igaus)

write(*,*)'COORDINATES: '

write(*,100) (xelem(inode),yelem(inode),   &

&          zelem(inode),inode=1,nnode)

stop

endif

100 format(3e20.10)

!

!     calculate components of Jacobian inverse matrix

!

jacinv(1,1)=dydeta*dzdzeta - dydzeta*dzdeta

jacinv(1,2)=-(dydxi*dzdzeta - dydzeta*dzdxi)

jacinv(1,3)=dydxi*dzdeta - dydeta*dzdxi

jacinv(2,1)=-(dxdeta*dzdzeta - dxdzeta*dzdeta)

jacinv(2,2)=dxdxi*dzdzeta - dxdzeta*dzdxi

jacinv(2,3)=-(dxdxi*dzdeta - dxdeta*dzdxi)

jacinv(3,1)=dxdeta*dydzeta - dxdzeta*dydeta

jacinv(3,2)=-(dxdxi*dydzeta - dxdzeta*dydxi)

jacinv(3,3)=dxdxi*dydeta - dxdeta*dydxi

!

!     CALCULATE DERIVATIVES OF SHAPE FUNCTIONS

!

do k= 1,nnode

do j = 1,ndimn

dshpdx(j,k,igaus)=0.0

enddo

enddo

!

detjacinv = 1./detjac(igaus)

detjacinv2 = detjacinv/detjac(igaus)

!

do i = 1,ndimn

DO K = 1,NNODE

do j = 1,3

DSHPDX(i,K,IGAUS)=dshpdx(i,k,igaus)+(jacinv(i,j)*   &

&                          dpxi(j,k,igaus))*detjacinv

enddo

enddo

enddo

!

!          if(koun.le.100) then

!          write(25,*)igaus,detjac(igaus)

!          do i = 1,ndimn

!          write(25,101)(jacinv(i,j),k=1,ndimn)

!          enddo

!          endif

101 format(3(1x,e16.8))

!

if(iflag.eq.1) then

!

!     Derivatives of Jacobian inverse components wrt xi, eta, and zeta

!

djacinvdxi(1,1) = d2ydxde*dzdzeta + dydeta*     &

&                  d2zdxdz - d2ydxdz*dzdeta -    &

&                  dydzeta*d2zdxde

djacinvdet(1,1) = dydeta*d2zdedz -              &

&                  d2ydedz*dzdeta

djacinvdzt(1,1) = d2ydedz*dzdzeta -             &

&                  dydzeta*d2zdedz

djacinvdxi(1,2) = -(dydxi*d2zdxdz - d2ydxdz*    &

&                    dzdxi)

djacinvdet(1,2) = -(d2ydxde*dzdzeta + dydxi*    &

&                   d2zdedz - d2ydedz*dzdxi -    &

&                   dydzeta*d2zdxde)

djacinvdzt(1,2) = -(d2ydxdz*dzdzeta - dydzeta*  &

&                    d2zdxdz)

djacinvdxi(1,3) = dydxi*d2zdxde - d2ydxde*      &

&                  dzdxi

djacinvdet(1,3) = d2ydxde*dzdeta - dydeta*      &

&                  d2zdxde

djacinvdzt(1,3) = d2ydxdz*dzdeta + dydxi*       &

&                  d2zdedz - d2ydedz*dzdxi -     &

&                  dydeta*d2zdxdz

djacinvdxi(2,1) = -(d2xdxde*dzdzeta + dxdeta*   &

&                  d2zdxdz - d2xdxdz*dzdeta -    &

&                  dxdzeta*d2zdxde)

djacinvdet(2,1) = -(dxdeta*d2zdedz - d2xdedz*   &

&                   dzdeta)

djacinvdzt(2,1) = -(d2xdedz*dzdzeta - dxdzeta*  &

&                    d2zdedz)

djacinvdxi(2,2) = dxdxi*d2zdxdz - d2xdxdz*      &

&                  dzdxi

djacinvdet(2,2) = d2xdxde*dzdzeta + dxdxi*      &

&                  d2zdedz - d2xdedz*dzdxi -     &

&                  dxdzeta*d2zdxde

djacinvdzt(2,2) = d2xdxdz*dzdzeta - dxdzeta*    &

&                  d2zdxdz

djacinvdxi(2,3) = -(dxdxi*d2zdxde - d2xdxde*    &

&                  dzdxi)

djacinvdet(2,3) = -(d2xdxde*dzdeta - dxdeta*    &

&                   d2zdxde)

djacinvdzt(2,3) = -(d2xdxdz*dzdeta + dxdxi*     &

&                    d2zdedz - d2xdedz*dzdxi -   &

&                    dxdeta*d2zdxdz)

djacinvdxi(3,1) = d2xdxde*dydzeta + dxdeta*     &

&                  d2ydxdz - d2xdxdz*dydeta -    &

&                  dxdzeta*d2ydxde

djacinvdet(3,1) = dxdeta*d2ydedz - d2xdedz*     &

&                  dydeta

djacinvdzt(3,1) = d2xdedz*dydzeta - dxdeta*     &

&                  d2ydedz

djacinvdxi(3,2) = -(dxdxi*d2ydxdz - d2xdxdz*    &

&                    dydxi)

djacinvdet(3,2) = -(d2xdxde*dydzeta + dxdxi*    &

&                    d2ydedz - d2xdedz*dydxi -   &

&                    dxdzeta*d2ydxde)

djacinvdzt(3,2) = -(d2xdxdz*dydzeta - dxdzeta*  &

&                    d2ydxdz)

djacinvdxi(3,3) = dxdxi*d2ydxde - d2xdxde*      &

&                  dydxi

djacinvdet(3,3) = d2xdxde*dydeta - dxdeta*      &

&                  d2ydxde

djacinvdzt(3,3) = d2xdxdz*dydeta + dxdxi*       &

&                  d2ydedz -  d2xdedz*dydxi -    &

&                  dxdeta*d2ydxdz

!

!     Derivatives of determinant of Jacobian wrt to xi, eta, and zeta

!

term1 = dxdxi*(d2ydxde*dzdzeta +         &

&        dydeta*d2zdxdz)

term2 = dzdxi*(d2xdxde*dydzeta +         &

&        dxdeta*d2ydxdz)

term3 = dydxi*(d2xdxdz*dzdeta +          &

&        dxdzeta*d2zdxde)

term4 = dzdxi*(d2xdxdz*dydeta +          &

&        dxdzeta*d2ydxde)

term5 = dxdxi*(d2ydxdz*dzdeta +          &

&        dydzeta*d2zdxde)

term6 = dydxi*(d2xdxde*dzdzeta +         &

&        dxdeta*d2zdxdz)

!

djacdxi = term1 + term2 + term3 - term4 - term5 - term6

!

term1 = dydeta*(d2xdxde*dzdzeta +      &

&        dxdxi*d2zdedz)

term2 = dxdeta*(d2ydedz*dzdxi +        &

&        dydzeta*d2zdxde)

term3 = dzdeta*(d2xdedz*dydxi +        &

&        dxdzeta*d2ydxde)

term4 = dydeta*(d2xdedz*dzdxi +        &

&        dxdzeta*d2zdxde)

term5 = dzdeta*(d2xdxde*dydzeta +      &

&        dxdxi*d2ydedz)

term6 = dxdeta*(d2ydxde*dzdzeta +      &

&        dydxi*d2zdedz)

!

djacdeta = term1 + term2 + term3 - term4 - term5 - term6

!

term1 = dzdzeta*(d2xdxdz*dydeta +      &

&        dxdxi*d2ydedz)

term2 = dydzeta*(d2xdedz*dzdxi +       &

&        dxdeta*d2zdxdz)

term3 = dxdzeta*(d2ydxdz*dzdeta +      &

&        dydxi*d2zdedz)

term4 = dxdzeta*(d2ydedz*dzdxi +       &

&        dydeta*d2zdxdz)

term5 = dydzeta*(d2xdxdz*dzdeta +      &

&        dxdxi*d2zdedz)

term6 = dzdzeta*(d2xdedz*dydxi +       &

&        dxdeta*d2ydxdz)

!

djacdzeta = term1 + term2 + term3 - term4 - term5 -term6

!

!     Spatial second derivatives of shape functions

!

do inode = 1,nnode

wn(inode) = dshpdx(1,inode,igaus)*detjac(igaus)

wnbar(inode) = dshpdx(2,inode,igaus)*detjac(igaus)

wntild(inode) = dshpdx(3,inode,igaus)*detjac(igaus)

enddo

!

do inode = 1,nnode

!

term1 = -detjacinv2*djacdxi*wn(inode)

term2 = (djacinvdxi(1,1)*dpxi(1,inode,igaus) +              &

&               djacinvdxi(1,2)*dpxi(2,inode,igaus) + jacinv(1,2)* &

&              d2phidxde(inode,igaus) + djacinvdxi(1,3)*           &

&              dpxi(3,inode,igaus) + jacinv(1,3)*                  &

&              d2phidxdz(inode,igaus))*detjacinv

term11 = jacinv(1,1)*(term1 + term2)

!

term1 = -detjacinv2*djacdeta*wn(inode)

term2 = (djacinvdet(1,1)*dpxi(1,inode,igaus) +              &

&               jacinv(1,1)*d2phidxde(inode,igaus) +               &

&               djacinvdet(1,2)*dpxi(2,inode,igaus) +              &

&               djacinvdet(1,3)*dpxi(3,inode,igaus) + jacinv(1,3)* &

&              d2phidedz(inode,igaus))*detjacinv

term12 = jacinv(1,2)*(term1 + term2)

!

term1 = -detjacinv2*djacdzeta*wn(inode)

term2 = (djacinvdzt(1,1)*dpxi(1,inode,igaus) +       &

&               jacinv(1,1)*d2phidxdz(inode,igaus) +        &

&               djacinvdzt(1,2)*dpxi(2,inode,igaus) +       &

&               jacinv(1,2)*d2phidedz(inode,igaus) +        &

&               djacinvdzt(1,3)*dpxi(3,inode,igaus))*detjacinv

term13 = jacinv(1,3)*(term1 + term2)

!

d2phidx(1,1,inode,igaus) = detjacinv*(term11 + term12 +   &

&                                 term13)

!

term1 = -detjacinv2*djacdxi*wnbar(inode)

term2 = (djacinvdxi(2,1)*dpxi(1,inode,igaus) +             &

&               djacinvdxi(2,2)*dpxi(2,inode,igaus) + jacinv(2,2)*&

&              d2phidxde(inode,igaus) + djacinvdxi(2,3)*          &

&              dpxi(3,inode,igaus) + jacinv(2,3)*                 &

&              d2phidxdz(inode,igaus))*detjacinv

term21 = jacinv(2,1)*(term1 + term2)

!

term1 = -detjacinv2*djacdeta*wnbar(inode)

term2 = (djacinvdet(2,1)*dpxi(1,inode,igaus) +             &

&               jacinv(2,1)*d2phidxde(inode,igaus) +              &

&               djacinvdet(2,2)*dpxi(2,inode,igaus) +             &

&               djacinvdet(2,3)*dpxi(3,inode,igaus) + jacinv(2,3)*&

&              d2phidedz(inode,igaus))*detjacinv

term22 = jacinv(2,2)*(term1 + term2)

!

term1 = -detjacinv2*djacdzeta*wnbar(inode)

term2 = (djacinvdzt(2,1)*dpxi(1,inode,igaus) +         &

&               jacinv(2,1)*d2phidxdz(inode,igaus) +          &

&               djacinvdzt(2,2)*dpxi(2,inode,igaus) +         &

&               jacinv(2,2)*d2phidedz(inode,igaus) +          &

&               djacinvdzt(2,3)*dpxi(3,inode,igaus))*detjacinv

term23 = jacinv(2,3)*(term1 + term2)

!

d2phidx(2,2,inode,igaus) = detjacinv*(term21 + term22 +   &

&                                 term23)

!

term1 = -detjacinv2*djacdxi*wntild(inode)

term2 = (djacinvdxi(3,1)*dpxi(1,inode,igaus) +             &

&               djacinvdxi(3,2)*dpxi(2,inode,igaus) + jacinv(3,2)*&

&              d2phidxde(inode,igaus) + djacinvdxi(3,3)*          &

&              dpxi(3,inode,igaus) + jacinv(3,3)*                 &

&              d2phidxdz(inode,igaus))*detjacinv

term31 = jacinv(3,1)*(term1 + term2)

!

term1 = -detjacinv2*djacdeta*wntild(inode)

term2 = (djacinvdet(3,1)*dpxi(1,inode,igaus) +          &

&               jacinv(3,1)*d2phidxde(inode,igaus) +           &

&               djacinvdet(3,2)*dpxi(2,inode,igaus) +          &

&               djacinvdet(3,3)*dpxi(3,inode,igaus) + jacinv(3,3)*&

&              d2phidedz(inode,igaus))*detjacinv

term32 = jacinv(3,2)*(term1 + term2)

!

term1 = -detjacinv2*djacdzeta*wntild(inode)

term2 = (djacinvdzt(3,1)*dpxi(1,inode,igaus) +         &

&               jacinv(3,1)*d2phidxdz(inode,igaus) +          &

&               djacinvdzt(3,2)*dpxi(2,inode,igaus) +         &

&               jacinv(3,2)*d2phidedz(inode,igaus) +          &

&               djacinvdzt(3,3)*dpxi(3,inode,igaus))*detjacinv

term33 = jacinv(3,3)*(term1 + term2)

!

d2phidx(3,3,inode,igaus) = detjacinv*(term31 + term32 +   &

&                                 term33)

!

term1 = -detjacinv2*djacdxi*wn(inode)

term2 = (djacinvdxi(1,1)*dpxi(1,inode,igaus) +            &

&               djacinvdxi(1,2)*dpxi(2,inode,igaus) + jacinv(1,2)*&

&              d2phidxde(inode,igaus) + djacinvdxi(1,3)*         &

&              dpxi(3,inode,igaus) + jacinv(1,3)*                &

&              d2phidxdz(inode,igaus))*detjacinv

term21 = jacinv(2,1)*(term1 + term2)

!

term1 = -detjacinv2*djacdeta*wn(inode)

term2 = (djacinvdet(1,1)*dpxi(1,inode,igaus) +           &

&               jacinv(1,1)*d2phidxde(inode,igaus) +            &

&               djacinvdet(1,2)*dpxi(2,inode,igaus) +           &

&               djacinvdet(1,3)*dpxi(3,inode,igaus) + jacinv(1,3)*&

&              d2phidedz(inode,igaus))*detjacinv

term22 = jacinv(2,2)*(term1 + term2)

!

term1 = -detjacinv2*djacdzeta*wn(inode)

term2 = (djacinvdzt(1,1)*dpxi(1,inode,igaus) +          &

&               jacinv(1,1)*d2phidxdz(inode,igaus) +           &

&               djacinvdzt(1,2)*dpxi(2,inode,igaus) +          &

&               jacinv(1,2)*d2phidedz(inode,igaus) +           &

&               djacinvdzt(1,3)*dpxi(3,inode,igaus))*detjacinv

term23 = jacinv(2,3)*(term1 + term2)

!

d2phidx(1,2,inode,igaus) = detjacinv*(term21 + term22 +   &

&                                 term23)

d2phidx(2,1,inode,igaus) = d2phidx(1,2,inode,igaus)

!

term1 = -detjacinv2*djacdxi*wntild(inode)

term2 = (djacinvdxi(3,1)*dpxi(1,inode,igaus) +             &

&               djacinvdxi(3,2)*dpxi(2,inode,igaus) + jacinv(3,2)*&

&              d2phidxde(inode,igaus) + djacinvdxi(3,3)*          &

&              dpxi(3,inode,igaus) + jacinv(3,3)*                 &

&              d2phidxdz(inode,igaus))*detjacinv

term21 = jacinv(1,1)*(term1 + term2)

!

term1 = -detjacinv2*djacdeta*wntild(inode)

term2 = (djacinvdet(3,1)*dpxi(1,inode,igaus) +             &

&               jacinv(3,1)*d2phidxde(inode,igaus) +              &

&               djacinvdet(3,2)*dpxi(2,inode,igaus) +             &

&               djacinvdet(3,3)*dpxi(3,inode,igaus) + jacinv(3,3)*&

&              d2phidedz(inode,igaus))*detjacinv

term22 = jacinv(1,2)*(term1 + term2)

!

term1 = -detjacinv2*djacdzeta*wntild(inode)

term2 = (djacinvdzt(3,1)*dpxi(1,inode,igaus) +         &

&               jacinv(3,1)*d2phidxdz(inode,igaus) + 		   &

&               djacinvdzt(3,2)*dpxi(2,inode,igaus) +		   &

&               jacinv(3,2)*d2phidedz(inode,igaus) +		   &

&               djacinvdzt(3,3)*dpxi(3,inode,igaus))*detjacinv

term23 = jacinv(1,3)*(term1 + term2)

!

d2phidx(1,3,inode,igaus) = detjacinv*(term21 + term22 +  &

&                                 term23)

d2phidx(3,1,inode,igaus) = d2phidx(1,3,inode,igaus)

!

term1 = -detjacinv2*djacdxi*wntild(inode)

term2 = (djacinvdxi(3,1)*dpxi(1,inode,igaus) +             &

&               djacinvdxi(3,2)*dpxi(2,inode,igaus) + jacinv(3,2)*&

&              d2phidxde(inode,igaus) + djacinvdxi(3,3)*		   &

&              dpxi(3,inode,igaus) + jacinv(3,3)*				   &

&              d2phidxdz(inode,igaus))*detjacinv

term21 = jacinv(2,1)*(term1 + term2)

!

term1 = -detjacinv2*djacdeta*wntild(inode)

term2 = (djacinvdet(3,1)*dpxi(1,inode,igaus) +             &

&               jacinv(3,1)*d2phidxde(inode,igaus) + 			   &

&               djacinvdet(3,2)*dpxi(2,inode,igaus) +			   &

&               djacinvdet(3,3)*dpxi(3,inode,igaus) + jacinv(3,3)*&

&              d2phidedz(inode,igaus))*detjacinv

term22 = jacinv(2,2)*(term1 + term2)

!

term1 = -detjacinv2*djacdzeta*wntild(inode)

term2 = (djacinvdzt(3,1)*dpxi(1,inode,igaus) +        &

&               jacinv(3,1)*d2phidxdz(inode,igaus) + 		  &

&               djacinvdzt(3,2)*dpxi(2,inode,igaus) +		  &

&               jacinv(3,2)*d2phidedz(inode,igaus) +		  &

&               djacinvdzt(3,3)*dpxi(3,inode,igaus))*detjacinv

term23 = jacinv(2,3)*(term1 + term2)

!

d2phidx(2,3,inode,igaus) = detjacinv*(term21 + term22 +  &

&                                 term23)

d2phidx(3,2,inode,igaus) = d2phidx(2,3,inode,igaus)

!

enddo

!

endif

!

!        DETJAC(IGAUS) = DETJAC(IGAUS)*WW(IGAUS)

enddo gausloop

!      write(25,125)(detjac(ig),ig = 1,mgaus)

125 format(1x,4(e16.8,1x))

!

return

end subroutine dersha_3d

!

!     ***************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATE THE SHAPE FUNCTIONS AND THEIR

!     DERIVATIVES FOR A TWO-D BOUNDARY SURFACE

!

SUBROUTINE SHAPSURF_3d

!

Use Shapbou_3d

Use Gaus_quad

!

Implicit None

!

integer :: igaus,i,j

!

!     TWO-DIMENSIONAL LINEAR SHAPE FUNCTIONS

!

igaus = 0

!

DO I = 1, NGAUS

do j = 1,ngaus

igaus = igaus + 1

!

PHI(1,igaus)=0.25*(1.-XI(I))*(1.-xi(j))

PHI(2,igaus)=0.25*(1.+XI(I))*(1.-xi(j))

phi(3,igaus) = 0.25*(1.+xi(i))*(1.+xi(j))

phi(4,igaus) = 0.25*(1.-xi(i))*(1.+xi(j))

!

!     DERIVATIVE OF SHAPE FUNCTIONS

!

dpxi(1,1,igaus) = -0.25*(1.-xi(j))

dpxi(2,1,igaus) = -0.25*(1.-xi(i))

dpxi(1,2,igaus) = 0.25*(1.-xi(j))

dpxi(2,2,igaus) = -0.25*(1.+xi(i))

dpxi(1,3,igaus) = 0.25*(1.+xi(j))

dpxi(2,3,igaus) = 0.25*(1.+xi(i))

dpxi(1,4,igaus) = -0.25*(1.+xi(j))

dpxi(2,4,igaus) = 0.25*(1.-xi(i))

!

w_w(igaus) = w(i)*w(j)

!

!     Second derivatives

!

d2phi(1,igaus) = 0.25

d2phi(2,igaus) = -0.25

d2phi(3,igaus) = 0.25

d2phi(4,igaus) = -0.25

!

enddo

enddo

!

RETURN

end subroutine shapsurf_3d

!

!  **************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE CONVECTIVE FLUX

!     AT A NODAL POINT

!

SUBROUTINE CONVFLX3(NPOIN)

!

Use flow_var

Use Conv_flux

!

Implicit None

!

integer,intent(in) :: npoin

integer :: k

real :: r,uvel,vvel,wvel,P,E

!

DO K=1,NPOIN

!

R = DENSE(K)

UVEL = VEL(1,K)

VVEL = VEL(2,K)

WVEL = VEL(3,K)

P = PRESS(K)

E = ENERG(K)

!

F(K,1,1)=R*UVEL

F(K,1,2)=R*UVEL*UVEL + P

F(K,1,3)=R*UVEL*VVEL

F(K,1,4)=R*UVEL*WVEL

F(K,1,5)=(R*E + P)*UVEL

!

F(K,2,1)=R*VVEL

F(K,2,2)=R*UVEL*VVEL

F(K,2,3)=R*VVEL*VVEL + P

F(K,2,4)=R*VVEL*WVEL

F(K,2,5)=(R*E + P)*VVEL

!

F(K,3,1)=R*WVEL

F(K,3,2)=R*UVEL*WVEL

F(K,3,3)=R*VVEL*WVEL

F(K,3,4)=R*WVEL*WVEL + P

F(K,3,5)=(R*E + P)*WVEL

!

enddo

!

RETURN

end subroutine convflx3

!

!     ********************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE CHARACTERISTIC

!     LENGTH OF EACH ELEMENT

!

SUBROUTINE EVALELE3d(adap,ndimn,nnode,NELEM)

!

Use param_var

Use grid_info

Use shap_fcns

Use Shap3d_fcns

Use Gaus_quad

!

Implicit None

!

integer,intent(in) :: ndimn,nnode,nelem

integer :: iel,inode,koun,i,idimn

integer,dimension(nnode) :: iel_nod

real,dimension(ndimn,nnode) :: xloc

real :: dshpdx(mdimn,mnode,mgaus),detjac(mgaus)

real :: d2phidx(mdimn,mdimn,mnode,mgaus)

real :: exp

!

real :: ELEAREA(NELEM)

!

logical,intent(in) :: adap

!

open(unit=26,file='eleleng.out')

elemntlp: DO IEL=1,NELEM

!         write(*,1000)iel,nnode

do inode = 1, nnode

iel_nod(inode) = lnod(inode,iel)

do idimn = 1,ndimn

xloc(idimn,inode) = x(idimn,iel_nod(inode))

enddo

enddo

koun = 1000

CALL DERSHA_3D(NNODE,ndimn,xloc,dpxi,d2phidxde,d2phidxdz, &

&                  d2phidedz,2,koun,dshpdx,detjac,d2phidx)

ELEAREA(IEL)=0.0

DO I=1,MGAUS

ELEAREA(IEL)=ELEAREA(IEL)+DETJAC(I)*ww(i)

enddo

!

!         write(26,1150)(detjac(ig),ig=1,mgaus)

exp = 1./float(ndimn)

CLENG(IEL)=ELEAREA(IEL)**exp

write(26,1200) iel,elearea(iel),cleng(iel)

enddo elemntlp

close(unit=26,status='keep')

!

1000 format(1x,'ELEMENT ',I8,' NNODE ',i20)

!      write(*,1100)

1100 format(' EXITING SUBROUTINE EVALELE')

1150 format(' DETERMINANT OF JACOBIAN ',/,4(e12.6,1x))

1200 format(1x,'ELEMENT ',I8,' AREA ',e12.6,' CLENG ',e12.6)

RETURN

end subroutine evalele3d

!

!   *******************************************************************

!   Subroutine to calculate Petrov-Galerkin factor, TAU

!

Subroutine GPG(ndimn,nnode,xloc,detjac,ig,velo,rho,vis,hk,  &

&GPG_tau)

!

Use param_var

Use shap_fcns

Use grid_info

!

Implicit None

!

integer,intent(in) :: ndimn,nnode,ig

real,dimension(ndimn,nnode),intent(in) :: xloc

real,dimension(mgaus),intent(in) :: detjac

real,intent(in) :: velo(ndimn),rho,vis,hk

real,intent(out) :: GPG_tau

!

integer :: idimn,inode,L

real :: veloc,diff

real,dimension(ndimn) :: uxi,hxi

real,dimension(nnode) :: xelem,yelem,zelem

real :: dxdxi,dydxi,dzdxi,dxdeta,dydeta,dzdeta

real :: dxdzeta,dydzeta,dzdzeta

real,dimension(ndimn) :: exi,eeta,ezeta

real :: jacinv

real,dimension(ndimn) :: pt1,pt2

real :: onesix

real,dimension(ndimn) :: Re,alphabar

!

diff = vis/rho

onesix = 1./6.

!

veloc = 0.0

!

do idimn = 1,ndimn

veloc = veloc + velo(idimn)*velo(idimn)

enddo

!

do inode = 1,nnode

xelem(inode) = xloc(1,inode)

yelem(inode) = xloc(2,inode)   ! Coordinates of local

zelem(inode) = xloc(3,inode)   !      Nodes

enddo

!

!     unit vector in xi, eta, and zeta direction

!

DXDXI = 0.0

DXDETA = 0.0

DXDZETA = 0.0

DYDXI = 0.0

DYDETA = 0.0

DYDZETA = 0.0

DZDXI = 0.0

DZDETA = 0.0

DZDZETA = 0.0

DO L = 1, NNODE

DXDXI = DXDXI + dpxi(1,L,ig)*xelem(L)

DXDETA = DXDETA + dpxi(2,L,ig)*xelem(L)

DXDZETA = DXDZETA + dpxi(3,L,ig)*Xelem(L)

DYDXI = DYDXI + dpxi(1,L,ig)*yelem(L)

DYDETA = DYDETA + dpxi(2,L,ig)*yelem(L)

DYDZETA = DYDZETA + dpxi(3,L,ig)*yelem(L)

DZDXI = DZDXI + dpxi(1,L,ig)*zelem(L)

DZDETA = DZDETA + dpxi(2,L,ig)*zelem(L)

DZDZETA = DZDZETA + dpxi(3,L,ig)*zelem(L)

enddo

!

jacinv = 1./detjac(ig)

!

exi(1) = jacinv*dxdxi

exi(2) = jacinv*dydxi

exi(3) = jacinv*dzdxi

eeta(1) = jacinv*dxdeta

eeta(2) = jacinv*dydeta

eeta(3) = jacinv*dzdeta

ezeta(1) = jacinv*dxdzeta

ezeta(2) = jacinv*dydzeta

ezeta(3) = jacinv*dzdzeta

!

uxi = 0.0

!

do idimn = 1,ndimn

uxi(1) = uxi(1) + velo(idimn)*exi(idimn)

uxi(2) = uxi(2) + velo(idimn)*eeta(idimn)

uxi(3) = uxi(3) + velo(idimn)*ezeta(idimn)

enddo

!     hxi

pt1(1) = 0.25*(xelem(1) + xelem(4) + xelem(5) + xelem(8))

pt1(2) = 0.25*(yelem(1) + yelem(4) + yelem(5) + yelem(8))

pt1(3) = 0.25*(zelem(1) + zelem(4) + zelem(5) + zelem(8))

!

pt2(1) = 0.25*(xelem(2) + xelem(3) + xelem(6) + xelem(7))

pt2(2) = 0.25*(yelem(2) + yelem(3) + yelem(6) + yelem(7))

pt2(3) = 0.25*(zelem(2) + zelem(3) + zelem(6) + zelem(7))

!

hxi(1) = sqrt((pt2(1)-pt1(1))**2 + (pt2(2)-pt1(2))**2 +      &

(pt2(3)-pt1(3))**2)

!     heta

pt1(1) = 0.25*(xelem(1) + xelem(2) + xelem(5) + xelem(6))

pt1(2) = 0.25*(yelem(1) + yelem(2) + yelem(5) + yelem(6))

pt1(3) = 0.25*(zelem(1) + zelem(2) + zelem(5) + zelem(6))

!

pt2(1) = 0.25*(xelem(3) + xelem(4) + xelem(7) + xelem(8))

pt2(2) = 0.25*(yelem(3) + yelem(4) + yelem(7) + yelem(8))

pt2(3) = 0.25*(zelem(3) + zelem(4) + zelem(7) + zelem(8))

!

hxi(2) = sqrt((pt2(1)-pt1(1))**2 + (pt2(2)-pt1(2))**2 +      &

(pt2(3)-pt1(3))**2)

!     hzeta

pt1(1) = 0.25*(xelem(1) + xelem(2) + xelem(3) + xelem(4))

pt1(2) = 0.25*(yelem(1) + yelem(2) + yelem(3) + yelem(4))

pt1(3) = 0.25*(zelem(1) + zelem(2) + zelem(3) + zelem(4))

!

pt2(1) = 0.25*(xelem(5) + xelem(6) + xelem(7) + xelem(8))

pt2(2) = 0.25*(yelem(5) + yelem(6) + yelem(7) + yelem(8))

pt2(3) = 0.25*(zelem(5) + zelem(7) + zelem(7) + zelem(8))

!

hxi(3) = sqrt((pt2(1)-pt1(1))**2 + (pt2(2)-pt1(2))**2 +      &

(pt2(3)-pt1(3))**2)

!

! Calculate alphabar and isoparametric Reynolds number

!

do idimn = 1,ndimn

Re(idimn) = uxi(idimn)*hxi(idimn)/diff

alphabar(idimn) = 1./tanh(Re(idimn)/2.) - 2./Re(idimn)

enddo

!

GPG_tau = 0.0

!

do idimn = 1,ndimn

GPG_tau = GPG_tau + alphabar(idimn)*hxi(idimn)*uxi(idimn)

enddo

!

GPG_tau = onesix*GPG_tau/veloc

!

return

end subroutine GPG

!

!  ****************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE THREE-D JACOBIAN MATRICES

!     "A SUB I" FOR A SAMPLE POINT

!

!     THESE MATRICES ARE FOR THE NAVIER-STOKES SYSTEM OF EQUATION

!     WITHOUT PRESSURE CORRECTIONS

!

SUBROUTINE AJACOB_3D(velo,E,gamma,gamm1,gm3d2,gm1d2,gamm3,aj)

!

!DIR$ INLINEALWAYS AJACOB_3D

!

Use Param_var

!      Use thermo_propt

!

Implicit None

!

real,intent(in) :: velo(mdimn),E

real,intent(in) :: gamma,gamm1,gm3d2,gm1d2,gamm3

real,dimension(mdimn,mequa,mequa),intent(out) :: aj

real :: u,v,w,u2,v2,w2,uv,uw,vw,u2pv2pw2,u2pv2,u2pw2,v2pw2

!

!     Zero Jacobian

aj = 0.0

!

u = velo(1)

v = velo(2)

w = velo(3)

u2 = u*u

v2 = v*v

w2 = w*w

uv = u*v

uw = u*w

vw = v*w

u2pv2pw2 = u2 + v2 + w2

u2pv2 = u2 + v2

u2pw2 = u2 + w2

v2pw2 = v2 + w2

!

aj(1,1,2)=1.0

aj(1,2,1)=GM3D2*u2+GM1D2*v2pw2

aj(1,2,2)=-GAMM3*u

aj(1,2,3)=-GAMM1*v

aj(1,2,4)=-GAMM1*w

aj(1,2,5)=GAMM1

aj(1,3,1)=-uv

aj(1,3,2)=v

aj(1,3,3)=u

aj(1,4,1)=-uw

aj(1,4,2)=w

aj(1,4,4)=u

aj(1,5,1)=-GAMMA*E*u+GAMM1*u*u2pv2pw2

aj(1,5,2)=GAMMA*E-GM1D2*(3.*u2+v2pw2)

aj(1,5,3)=-GAMM1*uv

aj(1,5,4)=-GAMM1*uw

aj(1,5,5)=GAMMA*u

!

aj(2,1,3)=1.0

aj(2,2,1)=-uv

aj(2,2,2)=v

aj(2,2,3)=u

aj(2,3,1)=GM1D2*u2pw2+GM3D2*v2

aj(2,3,2)=-GAMM1*u

aj(2,3,3)=-GAMM3*v

aj(2,3,4)=-GAMM1*w

aj(2,3,5)=GAMM1

aj(2,4,1)=-vw

aj(2,4,3)=w

aj(2,4,4)=v

aj(2,5,1)=-GAMMA*E*v+GAMM1*v*u2pv2pw2

aj(2,5,2)=-GAMM1*uv

aj(2,5,3)=GAMMA*E-GM1D2*(u2pw2+3.*v2)

aj(2,5,4)=-GAMM1*vw

aj(2,5,5)=GAMMA*v

!

aj(3,1,4)=1.0

aj(3,2,1)=-uw

aj(3,2,2)=w

aj(3,2,4)=u

aj(3,3,1)=-vw

aj(3,3,3)=w

aj(3,3,4)=v

aj(3,4,1)=GM1D2*u2pv2 + GM3D2*w2

aj(3,4,2)=-GAMM1*u

aj(3,4,3)=-GAMM1*v

aj(3,4,4)=-GAMM3*w

aj(3,4,5)=GAMM1

aj(3,5,1)=-GAMMA*E*w + GAMM1*w*u2pv2pw2

aj(3,5,2)=-GAMM1*uw

aj(3,5,3)=-GAMM1*vw

aj(3,5,4)=GAMMA*E - GM1D2*(u2pv2 + 3.*w2)

aj(3,5,5)=GAMMA*w

!

RETURN

end subroutine ajacob_3d

!

!  ******************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE JACOBIAN "B SUB I"

!     FOR A SAMPLE POINT

!

SUBROUTINE BJACOB_3D(reinv,cv,VIS,HK,TAU,DUNDX,Unkn,velo,E,bj)

!

!DIR$ INLINEALWAYS BJACOB_3D

!

Use Param_var

!      Use thermo_propt

!

Implicit None

!

real,intent(in) :: reinv,cv,vis,hk,tau(mdimn,mdimn)

real,intent(in) :: DUNDX(mdimn,MEQUA),VELO(MDIMN),UNKN(MEQUA)

real,intent(in) :: E

real,dimension(mdimn,mequa,mequa),intent(out) :: bj

real :: vislam,visr,denom,term1

real :: r,ru,rv,rw,re,u,v,w,u2,v2,w2,u2pv2pw2

real :: drdx,dldx,dmdx,dndx,dedx,drdy,dldy,dmdy,dndy,dedy

real :: drdz,dldz,dmdz,dndz,dedz

!

!     Zero Jacobian

bj = 0.0

!

VISLAM=-2.*VIS/3.

visr=2.*vis+vislam

!

DENOM=1./UNKN(1)**2

TERM1=HK*DENOM/CV

!

r = unkn(1)

ru = unkn(2)

rv = unkn(3)

rw = unkn(4)

re = unkn(5)

u = velo(1)

v = velo(2)

w = velo(3)

u2 = u*u

v2 = v*v

w2 = w*w

u2pv2pw2 = u2 + v2 + w2

!

drdx = dundx(1,1)

dldx = dundx(1,2)

dmdx = dundx(1,3)

dndx = dundx(1,4)

dedx = dundx(1,5)

drdy = dundx(2,1)

dldy = dundx(2,2)

dmdy = dundx(2,3)

dndy = dundx(2,4)

dedy = dundx(2,5)

drdz = dundx(3,1)

dldz = dundx(3,2)

dmdz = dundx(3,3)

dndz = dundx(3,4)

dedz = dundx(3,5)

!

bj(1,2,1)=(DENOM*(VISR*dldx+VISLAM*(dmdy+dndz)+     &

&         visr*(-2.*u*drdx+v*drdy+ w*drdz)))*reinv

bj(1,2,2)=VISR*drdx*DENOM*reinv

bj(1,2,3)=VISLAM*drdy*DENOM*reinv

bj(1,2,4)=VISLAM*drdz*DENOM*reinv

bj(1,3,1)=(VIS*DENOM*(dldy+dmdx-2.*(u*drdy+v*drdx)))*reinv

bj(1,3,2)=VIS*drdy*DENOM*reinv

bj(1,3,3)=VIS*drdx*DENOM*reinv

bj(1,4,1)=(VIS*DENOM*(dldz+dndx-2.*(u*drdz+w*drdx)))*reinv

bj(1,4,2)=VIS*drdz*DENOM*reinv

bj(1,4,4)=VIS*drdx*DENOM*reinv

bj(1,5,1)=u*bj(1,2,1)+v*bj(1,3,1)+w*bj(1,4,1)+(DENOM*(ru*TAU(1,1) +     &

&         rv*TAU(2,1)+rw*TAU(3,1))      &

&         -TERM1*(-dedx+(2.*E-3.*u2pv2pw2)*drdx    &

&         +2.*(u*dldx+v*dmdx+w*dndx)))*reinv

bj(1,5,2)=u*bj(1,2,2)+v*bj(1,3,2)+w*bj(1,4,2) + (       &

&         -TAU(1,1)/r - TERM1*(2.*u*drdx-dldx))*reinv

bj(1,5,3)=u*bj(1,2,3)+v*bj(1,3,3)+w*bj(1,4,3) + (-TAU(1,2)/r    &

&         -term1*(-dmdx+2.*v*drdx))*reinv

bj(1,5,4)=u*bj(1,2,4)+v*bj(1,3,4)+w*bj(1,4,4) + (-TAU(1,3)/r - &

&         TERM1*(2*w*drdx-dndx))*reinv

bj(1,5,5)=TERM1*drdx*reinv

!

bj(2,2,1)=bj(1,3,1)

bj(2,2,2)=bj(1,3,2)

bj(2,2,3)=bj(1,3,3)

bj(2,3,1)=(DENOM*(VISLAM*(dldx+dndz) + VISR*(dmdy+u*drdx-2.*v*  &

&         drdy+w*drdz)))*reinv

bj(2,3,2)=VISLAM*drdx*DENOM*reinv

bj(2,3,3)=VISR*drdy*DENOM*reinv

bj(2,3,4)=VISLAM*drdz*DENOM*reinv

bj(2,4,1)=(VIS*DENOM*(dmdz+dndy-2.*(v*drdz +w*drdy)))*reinv

bj(2,4,3)=VIS*DENOM*drdz*reinv

bj(2,4,4)=VIS*DENOM*drdy*reinv

bj(2,5,1)=u*bj(2,2,1)+v*bj(2,3,1)+w*bj(2,4,1) + (DENOM*(ru*      &

&         TAU(1,2)+rv*TAU(2,2)+rw*TAU(2,3))		&

&         -TERM1*(-dedy+(2.*E-3.*u2pv2pw2)	&

&         *drdy + 2.*(u*dldy+v*dmdy+w*dndy)))*reinv

bj(2,5,2)=u*bj(2,2,2) + v*bj(2,3,2) + w*bj(2,4,2)+(-TAU(2,1)/r - &

&         TERM1*(-dldy+2.*u*drdy))*reinv

bj(2,5,3)=u*bj(2,2,3)+v*bj(2,3,3)+w*bj(2,4,3)+            &

&         (-TAU(2,2)/r - TERM1*(2.*v*drdy - dmdy))*reinv

bj(2,5,4)=u*bj(2,2,4)+v*bj(2,3,4)+w*bj(2,4,4) + (-TAU(2,3)/r    &

&         -TERM1*(2.*w*drdy-dndy))*reinv

bj(2,5,5)=TERM1*drdy*reinv

!

bj(3,2,1)=bj(1,4,1)

bj(3,2,2)=bj(1,4,2)

bj(3,2,4)=bj(1,4,4)

bj(3,3,1)=bj(2,4,1)

bj(3,3,3)=bj(2,4,3)

bj(3,3,4)=bj(2,4,4)

bj(3,4,1)=(DENOM*(VISR*(dndz+u*drdx+v*         &

&         drdy-2.*w*drdz)+VISLAM*(dldx+ dmdy)))*reinv

bj(3,4,2)=VISLAM*drdx*DENOM*reinv

bj(3,4,3)=VISLAM*drdy*DENOM*reinv

bj(3,4,4)=VISR*drdz*DENOM*reinv

bj(3,5,1)=u*bj(3,2,1)+v*bj(3,3,1)+w*bj(3,4,1) + (  &

&         DENOM*(ru*TAU(3,1)+rv*TAU(3,2)+rw*	&

&         TAU(3,3))-term1*(-dedz+(2.*E-3.*u2pv2pw2)*drdz  &

&         +2.*(u*dldz+v*dmdz+w*dndz)))*reinv

bj(3,5,2)=u*bj(3,2,2)+v*bj(3,3,2)+w*bj(3,4,2) + (-TAU(3,1)/r  &

&         -TERM1*(2.*u*drdz-dldz))*reinv

bj(3,5,3)=u*bj(3,2,3)+v*bj(3,3,3)+w*bj(3,4,3) + (-TAU(3,2)/r  &

&         -TERM1*(2.*v*drdz-dmdz))*reinv

bj(3,5,4)=u*bj(3,2,4)+v*bj(3,3,4)+w*bj(3,4,4) +               &

&         (-TAU(3,3)/r - TERM1*(2.*w*drdz - dndz))*reinv

bj(3,5,5)=TERM1*drdz*reinv

!

RETURN

end subroutine BJACOB_3D

!

!    **************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE JACOBIAN "C SUB IJ"

!     ON A SAMPLE POINT

!

SUBROUTINE CJACOB_3D(reinv,cv,VIS,HK,UNKN,cj)

!

!DIR$ INLINEALWAYS CJACOB_3D

!

Use Param_var

!      Use thermo_propt

!

Implicit None

!

real,intent(in) :: reinv,cv,vis,hk

real,intent(in) :: unkn(mequa)

real,dimension(mdimn,mdimn,mequa,mequa),intent(out) :: cj

real :: velo(mdimn)

real :: vislam,visr,r,ru,rv,rw,re,ru2,rv2,rw2,ru2prv2prw2

real :: denom,term1,hkocv

!

!     Zero Jacobian

cj= 0.0

!

VISLAM=-2.*VIS/3.

visr=2.*vis+vislam

!

r = unkn(1)

ru = unkn(2)

rv = unkn(3)

rw = unkn(4)

re = unkn(5)

ru2 = ru*ru

rv2 = rv*rv

rw2 = rw*rw

ru2prv2prw2 = ru2 + rv2 + rw2

!

DENOM=1./r**2

TERM1=HK/CV*DENOM

hkocv = hk/cv

!

cj(1,1,2,1)=VISR*ru*DENOM*reinv

cj(1,1,2,2)=-VISR/r*reinv

cj(1,1,3,1)=VIS*rv*DENOM*reinv

cj(1,1,3,3)=-VIS/r*reinv

cj(1,1,4,1)=VIS*rw*DENOM*reinv

cj(1,1,4,4)=-VIS/r*reinv

cj(1,1,5,1)=(ru*cj(1,1,2,1)+ rv*cj(1,1,3,1) + rw*cj(1,1,4,1))/r   &

&           -TERM1*(-re + ru2prv2prw2/r)*reinv

cj(1,1,5,2)=ru/r*cj(1,1,2,2) + TERM1*ru*reinv

cj(1,1,5,3)=rv/r*cj(1,1,3,3) + TERM1*rv*reinv

cj(1,1,5,4)=rw/r*cj(1,1,4,4) + TERM1*rw*reinv

cj(1,1,5,5)=-hkocv/r*reinv

!

cj(1,2,2,1)=VISLAM*rv*DENOM*reinv

cj(1,2,2,3)=-VISLAM/r*reinv

cj(1,2,3,1)=VIS*ru*DENOM*reinv

cj(1,2,3,2)=-VIS/r*reinv

cj(1,2,5,1)=ru/r*cj(1,2,2,1)+rv/r*cj(1,2,3,1)

cj(1,2,5,2)=rv/r*cj(1,2,3,2)

cj(1,2,5,3)=ru/r*cj(1,2,2,3)

!

cj(1,3,2,1)=VISLAM*rw*DENOM*reinv

cj(1,3,2,4)=-VISLAM/r*reinv

cj(1,3,4,1)=VIS*ru*DENOM*reinv

cj(1,3,4,2)=-VIS/r*reinv

cj(1,3,5,1)=ru/r*cj(1,3,2,1)+rw/r*cj(1,3,4,1)

cj(1,3,5,2)=rw/r*cj(1,3,4,2)

cj(1,3,5,4)=ru/r*cj(1,3,2,4)

!

cj(2,1,2,1)=cj(1,1,3,1)

cj(2,1,2,3)=cj(1,1,3,3)

cj(2,1,3,1)=VISLAM*ru*DENOM*reinv

cj(2,1,3,2)=-VISLAM/r*reinv

cj(2,1,5,1)=ru/r*cj(2,1,2,1)+rv/r*cj(2,1,3,1)

cj(2,1,5,2)=rv/r*cj(2,1,3,2)

cj(2,1,5,3)=ru/r*cj(2,1,2,3)

!

cj(2,2,2,1)=cj(1,2,3,1)

cj(2,2,2,2)=cj(1,2,3,2)

cj(2,2,3,1)=VISR*rv*DENOM*reinv

cj(2,2,3,3)=-VISR/r*reinv

cj(2,2,4,1)=VIS*rw*DENOM*reinv

cj(2,2,4,4)=-VIS/r*reinv

cj(2,2,5,1)=(ru*cj(2,2,2,1)+rv*cj(2,2,3,1)+rw*cj(2,2,4,1))/r    &

&           - TERM1*(-re + ru2prv2prw2/r)*reinv

cj(2,2,5,2)=ru/r*cj(2,2,2,2) + TERM1*ru*reinv

cj(2,2,5,3)=rv/r*cj(2,2,3,3) + TERM1*rv*reinv

cj(2,2,5,4)=rw/r*cj(2,2,4,4) + TERM1*rw*reinv

cj(2,2,5,5)=-hkocv/r*reinv

!

cj(2,3,3,1)=VISLAM*rw*DENOM*reinv

cj(2,3,3,4)=-VISLAM/r*reinv

cj(2,3,4,1)=VIS*rv*DENOM*reinv

cj(2,3,4,3)=-VIS/r*reinv

cj(2,3,5,1)=rv/r*cj(2,3,3,1)+rw/r*cj(2,3,4,1)

cj(2,3,5,3)=rw/r*cj(2,3,4,3)

cj(2,3,5,4)=rv/r*cj(2,3,3,4)

!

cj(3,1,2,1)=cj(1,1,4,1)

cj(3,1,2,4)=cj(1,1,4,4)

cj(3,1,4,1)=VISLAM*ru*DENOM*reinv

cj(3,1,4,2)=-VISLAM/r*reinv

cj(3,1,5,1)=ru/r*cj(3,1,2,1)+rw/r*cj(3,1,4,1)

cj(3,1,5,2)=rw/r*cj(3,1,4,2)

cj(3,1,5,4)=ru/r*cj(3,1,2,4)

cj(3,2,3,1)=cj(2,2,4,1)

cj(3,2,3,4)=cj(2,2,4,4)

cj(3,2,4,1)=VISLAM*rv*DENOM*reinv

cj(3,2,4,3)=-VISLAM/r*reinv

cj(3,2,5,1)=rv/r*cj(3,2,3,1)+rw/r*cj(3,2,4,1)

cj(3,2,5,3)=rw/r*cj(3,2,4,3)

cj(3,2,5,4)=rv/r*cj(3,2,3,4)

!

cj(3,3,2,1)=cj(1,3,4,1)

cj(3,3,2,2)=cj(1,3,4,2)

cj(3,3,3,1)=cj(2,3,4,1)

cj(3,3,3,3)=cj(2,3,4,3)

cj(3,3,4,1)=VISR*rw*DENOM*reinv

cj(3,3,4,4)=-VISR/r*reinv

cj(3,3,5,1)=(ru*cj(3,3,2,1)+rv*cj(3,3,3,1)+rw*cj(3,3,4,1))/r  &

&           - TERM1*(-re + ru2prv2prw2/r)*reinv

cj(3,3,5,2)=ru/r*cj(3,3,2,2) + TERM1*ru*reinv

cj(3,3,5,3)=rv/r*cj(3,3,3,3) + TERM1*rv*reinv

cj(3,3,5,4)=rw/r*cj(3,3,4,4) + TERM1*rw*reinv

cj(3,3,5,5)=-hkocv/r*reinv

!

RETURN

end subroutine cjacob_3d

!

!     ************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE VISCOUS FLUX AT A

!     NODAL POINT

!

SUBROUTINE VISCFLX_3D(reinv,TAU,HK,vis,dudx,dtdx,velo,d2tdx2,  &

&                      d2udx2,G,dgjdxj)

!

!DIR$ INLINEALWAYS VISCFLX_3D

!

Use param_var

!

Implicit None

!

real,intent(in) :: reinv,hk,vis

real,intent(in) :: TAU(mdimn,mdimn),DTDX(mdimn)

REAL,intent(out) :: G(MDIMN,MEQUA),dgjdxj(mequa)

real,intent(in) :: dudx(mdimn,mdimn),velo(mdimn)

real,intent(in) :: d2tdx2(mdimn),d2udx2(mdimn,mdimn,mdimn)

real :: dg1dx1(mequa),dg2dx2(mequa),dg3dx3(mequa)

real :: vis43,vis23

integer :: iequa

!

vis43 = 4./3.*vis

vis23 = 2./3.*vis

!

G(1,1)=0.0

G(1,2)=-TAU(1,1)*reinv

G(1,3)=-TAU(1,2)*reinv

G(1,4)=-TAU(1,3)*reinv

G(1,5)=(-TAU(1,1)*VELO(1) - TAU(1,2)*VELO(2) -TAU(1,3)*VELO(3) - &

&       HK*DTDX(1))*reinv

!

G(2,1)=0.0

G(2,2)=-TAU(2,1)*reinv

G(2,3)=-TAU(2,2)*reinv

G(2,4)=-TAU(2,3)*reinv

G(2,5)=(-TAU(2,1)*VELO(1) - TAU(2,2)*VELO(2) -TAU(2,3)*VELO(3) - &

&       HK*DTDX(2))*reinv

!

G(3,1) = 0.0

G(3,2)=-TAU(3,1)*reinv

G(3,3)=-TAU(3,2)*reinv

G(3,4)=-TAU(3,3)*reinv

G(3,5)=(-TAU(3,1)*VELO(1) - TAU(3,2)*VELO(2) -TAU(3,3)*VELO(3) - &

&       HK*DTDX(3))*reinv

!

!     DGJDXJ

!

dg1dx1(1) = 0.0

dg1dx1(2) = (-vis43*d2udx2(1,1,1)+vis23*(d2udx2(2,1,2) +         &

&            d2udx2(3,1,3)))*reinv

dg1dx1(3) = -vis*(d2udx2(1,1,2) + d2udx2(2,1,1))*reinv

dg1dx1(4) = -vis*(d2udx2(1,1,3) + d2udx2(3,1,1))*reinv

dg1dx1(5) = G(1,2)*dudx(1,1) + velo(1)*dg1dx1(2) + G(1,3)*      &

&            dudx(2,1) + velo(2)*dg1dx1(3) + G(1,4)*dudx(3,1) +  &

&            velo(3)*dg1dx1(4) - hk*d2tdx2(1)*reinv

!

dg2dx2(1) = 0.0

dg2dx2(2) = -vis*(d2udx2(1,2,2) + d2udx2(2,2,1))*reinv

dg2dx2(3) = (-vis43*d2udx2(2,2,2)+vis23*(d2udx2(1,2,1) +   &

&            d2udx2(3,2,3)))*reinv

dg2dx2(4) = -vis*(d2udx2(2,2,3) + d2udx2(3,2,2))*reinv

dg2dx2(5) = G(2,2)*dudx(1,2) + velo(1)*dg2dx2(2) + G(2,3)*   &

&            dudx(2,2) + velo(2)*dg2dx2(3) + G(2,4)*dudx(3,2) &

+ velo(3)*dg2dx2(4) - hk*d2tdx2(2)*reinv

!

dg3dx3(1) = 0.0

dg3dx3(2) = -vis*(d2udx2(1,3,3) + d2udx2(3,1,3))*reinv

dg3dx3(3) = -vis*(d2udx2(2,3,3) + d2udx2(3,2,3))*reinv

dg3dx3(4) = (-vis43*d2udx2(3,3,3) + vis23*(d2udx2(1,1,3) +   &

&            d2udx2(2,2,3)))*reinv

dg3dx3(5) = G(3,2)*dudx(1,3) + velo(1)*dg3dx3(2) + G(3,3)*   &

&            dudx(2,3) + velo(2)*dg3dx3(3) + G(3,4)*dudx(3,3) &

&            + velo(3)*dg3dx3(4) - hk*d2tdx2(3)*reinv

!

do iequa = 1,mequa

dgjdxj(iequa) = dg1dx1(iequa) + dg2dx2(iequa) + dg3dx3(iequa)

!         dgjdxj(iequa) = 0.0

enddo

!

RETURN

end subroutine viscflx_3d

!

!    **************************************************************

!     THE FOLLOWING SUBROUTINE CALCULATES THE "U" VECTOR AND

!     JACOBIANS AT THE GAUSSIAN POINT OF INTEREST FOR THE

!     BOUNDARY TERMS

!

SUBROUTINE EVALBOU3D(NDIMN,NEQUA,ilbou,DPIDX,IG,VELO,   &

&                   E,T,unkn,DUDX,dtdx,dundx,cflux,dfxdx,&

&                   d2tdx2,d2udx2)

!

Use flow_var

Use thermo_propt

Use Conv_flux

Use shapbou_3d

!

Implicit None

!

integer,intent(in) :: ndimn,nequa,ig

integer,intent(in) :: ilbou(4)

real,intent(in) :: dpidx(mdimn,4,ngsurf)

real,intent(out) :: E,T

REAL,intent(out) :: CFLUX(NDIMN,NEQUA),DFXDX(NDIMN,NEQUA)

real,intent(out) :: dtdx(ndimn)

REAL,intent(out) :: DUNDX(NDIMN,NEQUA),unkn(nequa),velo(ndimn)

real,intent(out) :: dudx(mdimn,mdimn)

real,intent(out) :: d2tdx2(mdimn),d2udx2(mdimn,mdimn,mdimn)

!

integer :: idimn,jdimn,iequa,i,L,iordr

integer :: ipoin

!

T = 0.0

E=0.0

!

do idimn = 1, ndimn

do jdimn = 1,ndimn

dudx(idimn,jdimn) = 0.0

d2udx2(1,idimn,jdimn) = 0.0

d2udx2(2,idimn,jdimn) = 0.0

d2udx2(3,idimn,jdimn) = 0.0

enddo

velo(idimn) = 0.0

dtdx(idimn) = 0.0

d2tdx2(idimn) = 0.0

enddo

DO IEQUA = 1, NEQUA

unkn(iequa) = 0.00

do idimn = 1,ndimn

DUNDX(idimn,IEQUA) = 0.0

dfxdx(idimn,iequa) = 0.0

cflux(idimn,iequa) = 0.0

enddo

ENDDO

!

DO I=1,4

L=ILBOU(I)

iordr = nequa*(L-1)

E=E+ENERG(L)*PHI(I,IG)

T = T + TEMPR(L)*PHI(I,IG)

!

do idimn = 1, ndimn

VELO(idimn)=VELO(idimn)+VEL(idimn,L)*PHI(I,IG)

dtdx(idimn) = dtdx(idimn) + dpidx(idimn,i,ig)*tempr(L)

d2tdx2(idimn) = d2tdx2(idimn) + d2phidx(idimn,idimn,i,ig)* &

&                   tempr(L)

do jdimn = 1,ndimn

dudx(idimn,jdimn) = dudx(idimn,jdimn) + dpidx(jdimn,i,ig) &

&                          *vel(idimn,L)

d2udx2(1,idimn,jdimn) = d2udx2(1,idimn,jdimn) +           &

&                              d2phidx(idimn,jdimn,i,ig)*vel(1,L)

d2udx2(2,idimn,jdimn) = d2udx2(2,idimn,jdimn) +           &

&                              d2phidx(idimn,jdimn,i,ig)*vel(2,L)

d2udx2(3,idimn,jdimn) = d2udx2(3,idimn,jdimn) +           &

&                              d2phidx(idimn,jdimn,i,ig)*vel(3,L)

enddo

enddo

!

DO IEQUA = 1, NEQUA

Unkn(iequa)=Unkn(iequa)+U(iordr+iequa)*PHI(I,IG)

do jdimn = 1,ndimn

DUNDX(jdimn,IEQUA) = DUNDX(jdimn,IEQUA) + DPIDX(jdimn,I,IG)*  &

&                  U(iordr+IEQUA)

enddo

ENDDO

enddo

!

DO IEQUA = 1, NEQUA

DO I = 1, 4

IPOIN = ILBOU(I)

do jdimn = 1,ndimn

DFXDX(jdimn,IEQUA) = DFXDX(jdimn,IEQUA) + DPIDX(jdimn,I,IG)*  &

&                     F(IPOIN,jdimn,IEQUA)

CFLUX(jdimn,IEQUA) = CFLUX(jdimn,IEQUA)+PHI(I,IG)*            &

&                         F(IPOIN,jdimn,IEQUA)

enddo

enddo

enddo

!

RETURN

end subroutine evalbou3d

!

!     ***************************************************************

!     Subroutine to calculate the normal vector and the spatial

!     derivatives of the shape functions for a two-d boundary surface

!

Subroutine calnorm3(ndimn,ndbou,ifacdir,locnode,enorm,en)

!

Use param_var

Use grid_info

Use shapbou_3d

!

Implicit None

!

integer,intent(in) :: ndimn,ifacdir,locnode(4)

integer,intent(in) :: ndbou

real,intent(out) :: en(ndimn,ngsurf),enorm(ndimn)

!

real :: xelem(ndimn,4),xdiff(ndimn)

real :: Lambda(ndimn),gama(ndimn),Mu(ndimn)

real :: acoortr(ndimn,ndimn)

real :: dpdxp(4),dpdyp(4)        !  Deriviative in prime directions

real :: d2phidx2(4),d2phidy2(4)

!

integer :: idimn,inode,jnode,j,ig

real :: sum,el,dx_dxi,dy_dxi,dz_dxi,dx_deta,dy_deta,dz_deta

real :: dx_dxieta,dy_dxieta,dz_dxieta,dxdxi,dxdeta,dydxi,dydeta

real :: dxxieta,dyxieta,djacinv

real :: endir

!

do idimn = 1,ndimn

do inode = 1,4

xelem(idimn,inode) = x(idimn,locnode(inode))

enddo

enddo

!

!     calculate unit vectors (e12)

!

sum = 0.0

!

do idimn = 1,ndimn

xdiff(idimn) = xelem(idimn,2) - xelem(idimn,1)

sum = sum + xdiff(idimn)*xdiff(idimn)

enddo

!

el = sqrt(sum)

!

do j = 1,ndimn

Lambda(j) = xdiff(j)/el

enddo

!

!     vector e14

!

sum = 0.0

!

do idimn = 1,ndimn

xdiff(idimn) = xelem(idimn,4) - xelem(idimn,1)

sum = sum + xdiff(idimn)*xdiff(idimn)

enddo

!

el = sqrt(sum)

!

do j = 1,ndimn

Mu(j) = xdiff(j)/el

enddo

!

!     direction of the normal vector

!

if(ifacdir.eq.1) endir = 1.

if(ifacdir.eq.2) endir = -1.

!

!     vector normal to surface

!

enorm(1) = endir*(Lambda(2)*Mu(3) - Lambda(3)*Mu(2))

enorm(2) = endir*(Lambda(3)*Mu(1) - Lambda(1)*Mu(3))

enorm(3) = endir*(Lambda(1)*Mu(2) - Lambda(2)*Mu(1))

!

!      write(45,1000)(enorm(j), j=1,ndimn)

1000 format(' Direction Cosines:  ',3e16.8)

!

!     y-prime axis

!

gama(1) = endir*(enorm(2)*Lambda(3) - enorm(3)*Lambda(2))

gama(2) = endir*(enorm(3)*Lambda(1) - enorm(1)*Lambda(3))

gama(3) = endir*(enorm(1)*Lambda(2) - enorm(2)*Lambda(1))

!

!     coordinate transformation matrix

!

do idimn = 1,ndimn

acoortr(1,idimn) = Lambda(idimn)

acoortr(2,idimn) = gama(idimn)

acoortr(3,idimn) = enorm(idimn)

enddo

!

!     components of surface Jacobian

!

do ig = 1,ngsurf

!

dx_dxi = 0.0

dy_dxi = 0.0

dz_dxi = 0.0

dx_deta = 0.0

dy_deta = 0.0

dz_deta = 0.0

dx_dxieta = 0.0

dy_dxieta = 0.0

dz_dxieta = 0.0

!

do inode = 1,4

dx_dxi = dx_dxi + dpxi(1,inode,ig)*xelem(1,inode)

dy_dxi = dy_dxi + dpxi(1,inode,ig)*xelem(2,inode)

dz_dxi = dz_dxi + dpxi(1,inode,ig)*xelem(3,inode)

dx_deta = dx_deta + dpxi(2,inode,ig)*xelem(1,inode)

dy_deta = dy_deta + dpxi(2,inode,ig)*xelem(2,inode)

dz_deta = dz_deta + dpxi(2,inode,ig)*xelem(3,inode)

dx_dxieta = dx_dxieta + d2phi(inode,ig)*xelem(1,inode)

dy_dxieta = dy_dxieta + d2phi(inode,ig)*xelem(2,inode)

dz_dxieta = dz_dxieta + d2phi(inode,ig)*xelem(3,inode)

enddo

!

dxdxi = acoortr(1,1)*dx_dxi +     &

&           acoortr(1,2)*dy_dxi +	   &

&           acoortr(1,3)*dz_dxi

!

dxdeta = acoortr(1,1)*dx_deta +   &

&            acoortr(1,2)*dy_deta +   &

&            acoortr(1,3)*dz_deta

!

dydxi = acoortr(2,1)*dx_dxi +     &

&           acoortr(2,2)*dy_dxi +	   &

&           acoortr(2,3)*dz_dxi

!

dydeta = acoortr(2,1)*dx_deta +   &

&            acoortr(2,2)*dy_deta +   &

&            acoortr(2,3)*dz_deta

!

dxxieta = acoortr(1,1)*dx_dxieta + acoortr(1,2)*dy_dxieta +  &

&             acoortr(1,3)*dz_dxieta

dyxieta = acoortr(2,1)*dx_dxieta + acoortr(2,2)*dy_dxieta +  &

&             acoortr(2,3)*dz_dxieta

!

detjac(ig) = dxdxi*dydeta - dydxi*dxdeta

!

if(detjac(ig).lt.1.0e-12) then

write(*,2000)detjac(ig)

do inode = 1,4

write(*,2100)locnode(inode),(xelem(idimn,inode),idimn=1,ndimn)

enddo

write(*,*)' Normal Direction:  ',endir

write(*,*)' Transformation Matrix, ifacdir = ',ifacdir

write(*,*)(Lambda(jnode),jnode=1,3)

write(*,*)(Mu(jnode),jnode=1,3)

write(*,*)(enorm(jnode),jnode=1,3)

endif

!

2000 format(' SURFACE DETERMINANT OF JACOBIAN IS NEGATIVE ',/,   &

&       ' DETERMINANT OF JACOBIAN IS:  ',e16.8,/,            &

&       ' COORDINATES ARE:')

2100 format(i8,3e16.8)

!

djacinv = 1./detjac(ig)

!

do inode = 1,4

dpdxp(inode) = (dydeta*dpxi(1,inode,ig) -        &

&                      dydxi*dpxi(2,inode,ig))*djacinv

!

dpdyp(inode) = (-dxdeta*dpxi(1,inode,ig) +       &

&                       dxdxi*dpxi(2,inode,ig))*djacinv

!

enddo

!

!     Spatial derivatives of shape functions and normal vector

!

do idimn = 1,ndimn

do inode = 1,4

!

dshpdx(idimn,inode,ig) = acoortr(1,idimn)*dpdxp(inode)  &

&                                  + acoortr(2,idimn)*dpdyp(inode)

!

enddo

!

en(idimn,ig) = enorm(idimn)*w_w(ig)*detjac(ig)

!

enddo

!

!     Second derivatives of shape functions

!

call d2shapsurf(4,ig,ndbou,dxxieta,dyxieta,dxdxi,dxdeta,  &

&                   dydxi,dydeta,d2phidx2,d2phidy2)

!

do inode = 1,4

!

d2phidx(1,1,inode,ig) = acoortr(1,1)*acoortr(1,1)*       &

&                              d2phidx2(inode) + acoortr(2,1)*	 &

&                              acoortr(2,1)*d2phidy2(inode)

!

d2phidx(2,2,inode,ig) = acoortr(1,2)*acoortr(1,2)*       &

&                              d2phidx2(inode) + acoortr(2,2)*	 &

&                              acoortr(2,2)*d2phidy2(inode)

!

d2phidx(3,3,inode,ig) = acoortr(1,3)*acoortr(1,3)*       &

&                              d2phidx2(inode) + acoortr(2,3)*	 &

&                              acoortr(2,3)*d2phidy2(inode)

!

d2phidx(1,2,inode,ig) = acoortr(1,1)*acoortr(1,2)*       &

&                              d2phidx2(inode) + acoortr(2,1)*	 &

&                              acoortr(2,2)*d2phidy2(inode)

!

d2phidx(2,1,inode,ig) = d2phidx(1,2,inode,ig)

!

d2phidx(1,3,inode,ig) = acoortr(1,1)*acoortr(1,3)*       &

&                              d2phidx2(inode) + acoortr(2,1)*	 &

&                              acoortr(2,3)*d2phidy2(inode)

!

d2phidx(3,1,inode,ig) = d2phidx(1,3,inode,ig)

!

d2phidx(2,3,inode,ig) = acoortr(1,2)*acoortr(1,3)*      &

&                              d2phidx2(inode) + acoortr(2,2)*	&

&                              acoortr(2,3)*d2phidy2(inode)

!

d2phidx(3,2,inode,ig) = d2phidx(2,3,inode,ig)

!

enddo

!

enddo

!

return

end subroutine calnorm3

!

!     ***************************************************************

!     Subroutine to calculate spatial second derivative of shape

!     functions for numerical damping

!

subroutine d2shapsurf(nnode,igaus,ndbou,dxxieta,dyxieta,dxdxi,  &

&                      dxdeta,dydxi,dydeta,d2phidx2,d2phidy2)

!

Use param_var

Use Gaus_quad

Use grid_info

Use shapbou_3d

!

Implicit None

!

integer,intent(in) :: ndbou

!

integer,intent(in) :: nnode,igaus

real,intent(in) :: dxxieta,dyxieta,dxdxi,dxdeta,dydxi,dydeta

!

real,intent(out) :: d2phidx2(ndbou),d2phidy2(ndbou)

integer :: inode

real :: djacdxi,djacdeta,detjac2,detjac3

real :: term1,term2,term3,term4

!

djacdxi = dxdxi*dyxieta - dxxieta*dydeta

djacdeta = dxxieta*dydeta - dxdeta*dyxieta

!

detjac2 = 1./detjac(igaus)**2

detjac3 = 1./detjac(igaus)**3

!

do inode = 1, nnode

term1 = -detjac3*djacdxi*(dydeta**2*      &

&              dpxi(1,inode,igaus) - dydeta* &

&              dydxi*dpxi(2,inode,igaus))

!

term2 = detjac2*(dydeta*dyxieta*dpxi(1,inode,igaus)   &

&             - dydeta*dydxi*d2phi(inode,igaus))

!

term3 = detjac3*djacdeta*(dydxi*dydeta*       &

&             dpxi(1,inode,igaus) - dydxi**2*	 &

&             dpxi(2,inode,igaus))

!

term4 = -detjac2*(dydxi*dydeta*                  &

&             d2phi(inode,igaus) - dydxi*dyxieta*	&

&             dpxi(2,inode,igaus))

!

!     d2phi/dx2

!

d2phidx2(inode) = term1 + term2 + term3 + term4

!

term1 = detjac3*djacdxi*(-dxdeta**2*     &

&             dpxi(1,inode,igaus) + dxdeta*  &

&             dxdxi*dpxi(2,inode,igaus))

!

term2 = -detjac2*(-dxdeta*dxxieta*        &

&           dpxi(1,inode,igaus)  + dxdeta*	 &

&           dxdxi*d2phi(inode,igaus))

!

term3 = -detjac3*djacdeta*(-dxdxi*dxdeta*    &

&             dpxi(1,inode,igaus) + dxdxi**2*	&

&             dpxi(2,inode,igaus))

!

term4 = detjac2*(-dxdxi*dxdeta*                 &

&             d2phi(inode,igaus) + dxdxi*dxxieta* &

&             dpxi(2,inode,igaus))

!

!     d2phi/dy2

!

d2phidy2(inode) = term1 + term2 + term3 + term4

!

term1 = -detjac3*djacdxi*(-dydeta*dxdeta*    &

&             dpxi(1,inode,igaus) + dydeta*	&

&           dxdxi*  dpxi(2,inode,igaus))

!

term2 = detjac2*(-dydeta*dxxieta*dpxi(1,inode,igaus)   &

&             + dydeta*dxdxi*							  &

&           d2phi(inode,igaus))

!

term3 = detjac3*djacdeta*(-dydxi*dxdeta*     &

&           dpxi(1,inode,igaus) + dydxi*		&

&           dxdxi*dpxi(2,inode,igaus))

!

term4 = -detjac2*(-dydxi*dxdeta*                 &

&             d2phi(inode,igaus) + dydxi*dxxieta*	&

&             dpxi(2,inode,igaus))

!

!     d2phi/dxdy

!

!	   d2phidx(1,2,inode,igaus) = term1 + term2 + term3 + term4

!

!        term1 = detjac3*djacdxi*(dxdeta*dydeta*

!     &           dpxi(1,inode,igaus) - dxdeta*

!     &         dydxi*dpxi(2,inode,igaus))

!

!        term2 = -detjac2*(dxdeta*dyxieta*dpxi(1,inode,igaus)

!     &           - dxdeta*dydxi*d2phi(inode,igaus))

!

!        term3 = -detjac3*djacdeta*(dxdxi*dydeta*

!     &           dpxi(1,inode,igaus) - dxdxi*

!     &          dydxi*dpxi(2,inode,igaus))

!

!        term4 = detjac2*(dxdxi*dydeta*

!     &         d2phi(inode,igaus) - dxdxi*dyxieta*

!     &         dpxi(2,inode,igaus))

!

!     d2phidydx

!

!      d2phidx(2,1,inode,igaus) = term1 + term2 + term3 + term4

!

enddo

!

return

end subroutine d2shapsurf

!