Problems Set

## Chapter 4. Semi-analytical parameter extraction

### Problem 1.

In compact device modeling,  layout pads for measurement are not the part of  the device which needs to be removed ( Deembedding procedure) from the measurement data. In industry, open/short  structures are used.

- Design your own open/short structure.  How do you think, the pad size, ground layout, probe type or frequency affect your design ?

- what the equivalent circuit for open structure looks like ?

- how about short structure ?

- Attached is  S parameter files for open/short, what information you can extract from it ?

### Problem 2

Some engineer is developing the diode model at the particular bias. Outline the procedure he/she is going to work on.  Attached is S parameter file.    If there is inductance in the structure he/she forget to remove, how that will affect the final result ?  if ignoring the resistor and inductor, do you get the right result ? Why ?

### Problem 3

There are a set of S parameter files were taken for the diode.  Use the files to extract the diode capacitance model parameters.  Assuming the parasitic elements have been deembedded out.

### Problem 4

Attached is  HBT S parameter measured at cold condition.  Extract Cbe, Cbc from the data.

### Problem 5

Attached is  HBT S parameters measured at Constant Vce condition.  Use what you learned  to extract the model parameters.

### Problem 6

Attached is  HBT S parameters measured at Constant Ib condition.  Use what you learned  to extract the model parameters.

Problem 7

Given  the  intrinsic  part of small signal equivalent circuit of MESFET  which is similar to that of hybrid – pi model of HBT.  Outline the  extraction procedure to extract the model parameters assuming negligible gds.

The full equivalent circuit of  MESFET is as follows.  Assume Lg = Ld =25pH, Ls =10pH and Rs =Rd  =10 Ω, Rg =1 Ω,  Cpg =Cpd =25fF, , gds =0 Ω.   Extract the other model parameters based on the given set of S parameters.

## Chapter 5. Basic expressions

## Problem 1

Assume that  base collector extrinsic capacitance Cf  could be lumped into intrinsic Cbc, develop the strategy to small single model parameter extraction procedure.

## Chapter 6. The inverted HBT

### Problem 1

What is the advantages of inverted HBT?  If it is a promising in some power application, why is it not popular yet?  Compare emitter-up HBT and emitter-down HBT in term of  device parameter?

Problem 2

Base collector capacitance Cf is  much smaller in inverted HBT, does that make the model parameter extraction easily ?