13. MATLAB® Program listings

The following Matlab programs are listed herein:

SH2D_FULL	SH Line load in full space

SVP2D_FULL	SVP Line load in full space

BLAST2D	Blast line source in full space

CAVITY2D	2D Cavity in full space

POINT_FULL	Point source in full 3D space

TORSION_FULL	Torsional point source in full space

CAVITY3D	3D Cavity in full space

SH2D_HALF	SH Line source in half-space

GARVIN	Blast line source in half-space

LAMB2D	SVP line source in half-space

LAMB3D	Point source in half-space (horizontal and vertical)

TORSION_HALF	Torsional point source in half-space

SH_PLATE	SH line load in homogeneous plate

SH_STRATUM	SH line source in homogeneous stratum

SVP_PLATE	SVP line source in plate with mixed boundary conditions

SPHEROIDAL	Spheroidal and torsional modes of homogeneous sphere

In addition, the following functions are called by some of the programs

EASYPLOT	Writes text files for plotting (most programs)

CISIB	Sine and cosine integrals (Cavity 2D and 3D)

ELLIPINT3	Elliptic integrals (Lamb3D)

Note: Some programs have same-named functions embedded within, but they are not identical to each other, even if functionally related. Thus, they are not interchangeable.

function [] = SH2D_Full()

% SH line source in a full, homogeneous 2D space

% Default data

cs = 1;		% Shear wave velocity

rho = 1;		% Mass density

np = 200;		% Number of time and/or frequency intervals

tmax = 10;		% Maximum dimensionless time for plotting

wmax = 10;		% maximum dimensionless frequency for plotting (rad/s)

mu = rho*cs^2;	% Shear modulus

dw = wmax/np;

dt = tmax/np;

W = [dw:dw:wmax];

G = -i*0.25*besselh(0,2,W)/mu;

tit = 'Full-space displacement due to SH line load';

titx = 'Dimensionless frequency w*r/Cs = kr';

plot (W,real(G));

hold on;

plot (W,imag(G),'r');

grid on;

title(tit);

xlabel(titx);

pause;

hold off;

% Frequency domain

W2 = W.^0.5;

W = [0, W];

G = [0, G.*W2];

plot (W,real(G));

hold on;

plot (W,imag(G),'r');

tit = 'Full-space displacement due to SH line load * sqrt(kr)';

title(tit);

xlabel(titx);

grid on;

pause;

hold off;

EasyPlot('SH1-FD.ezp', tit, titx, W, G, 'c');

% Time domain

T1 = [0 1 1];

T2 = [1+dt:dt:tmax];

Uyy = 2*pi*mu*sqrt(T2.^2-1);

Uyy = [0 0 inf 1./Uyy];

Uyy(3) = Uyy(4);	% avoid singularity and discontinuity in plot

T = [T1, T2];

plot (T, Uyy);

tit = 'Full-space displacement due to SH line load';

title(tit);

titx = 'Dimensionless time, t*Cs/r';

xlabel(titx);

grid on;

pause;

EasyPlot('SH1-TD.ezp', tit, titx, T, Uyy, 'r');

close;

return

function [] = SVP2D_Full(x, z, poi);

% SVP line source in a full 2D space

% Arguments:

%	x, z	= coordinates of receiver

%	pois	= Poisson's ratio

% Note: Uxz = Uzx, Gxz = Gzx

% To obtain the components in cylindrical coordinates,

% simply run with x=1,z=0, and then x=0,z=1

% Default data

rho = 1;		% mass density

cs = 1;		% shear wave velocity

nt = 500;		% number of time points

tmax = 5;   		% maximum time

nf = 500;		% number of frequency points

wmax = 20;		% max. frequency

mu = rho*cs^2;	% Shear modulus

alfa = sqrt((1-2*poi)/(2-2*poi));		% cs/cp

r = sqrt(x^2+z^2);

theta = atan2(z, x);

% Time domain solution

dt = tmax/nt;

T = [0:nt]*dt;

T2 = T.^2;

U = impulse_response(r, theta, cs, mu, alfa, T2);

T = T*cs/r;		% dimensionless time

plot (T, U(1,:));

tit = sprintf(...

'Uxx at x=%5.3f, z=%5.3f due to SVP line load in full space', x, z);

titx = 'Dimensionless time, t*Cs/r';

grid on;

title(tit);

xlabel(titx);

EasyPlot('SVP_TDxx.ezp', tit, titx, T, U(1,:), ‘r’);

pause;

plot (T, U(2,:));

tit = sprintf(...

'Uxz at x=%5.3f, z=%5.3f due to SVP line load in full space', x, z);

grid on;

title(tit);

xlabel(titx);

EasyPlot('SVP_TDxz.ezp', tit, titx, T, U(2,:), ‘r’);

pause;

plot (T, U(3,:));

tit = sprintf(...

'Uzz at x=%5.3f, z=%5.3f due to SVP line load in full space', x, z);

grid on;

title(tit);

xlabel(titx);

EasyPlot('SVP_TDzz.ezp', tit, titx, T, U(3,:), 'r');

pause;

% Frequency domain solution

dw = wmax/nf;

W = [dw:dw:wmax];

H = Green(r, theta, cs, mu, alfa, W);

W = W*r/cs;	% dimensionless frequency

plot (W, real(H(1,:)));

hold on;

plot (W, imag(H(1,:)),'r');

tit = sprintf(...

'Gxx at x=%5.3f, z=%5.3f due to SVP line load in full space', x, z);

titx = 'Dimensionless frequency w*r/Cs = k*r';

grid on;

title(tit);

xlabel(titx);

EasyPlot('SVP_FDxx.ezp', tit, titx, W, H(1,:), 'c');

pause;

hold off;

plot (W, real(H(2,:)));

hold on;

plot (W, imag(H(2,:)),'r');

tit = sprintf(...

'Gxz at x=%5.3f, z=%5.3f due to SVP line load in full space', x, z);

grid on;

title(tit);

xlabel(titx);

EasyPlot('SVP_FDxz.ezp', tit, titx, W, H(2,:), 'c');

pause;

hold off;

plot (W, real(H(3,:)));

hold on;

plot (W, imag(H(3,:)),'r');

tit = sprintf(...

'Gzz at x=%5.3f, z=%5.3f due to SVP line load in full space', x, z);

grid on;

title(tit);

xlabel(titx);

EasyPlot('SVP_FDzz.ezp', tit, titx, W, H(3,:), 'c');

pause;

hold off;

close all;

return

function [G] = Green(r, theta, cs, mu, alfa, W)

% Green's functions for SVP line load in full space

Ws = W*r/cs;

Wp = alfa*Ws;

psi = (besselh(1,2,Ws)-alfa*besselh(1,2,Wp))./Ws - besselh(0,2,Ws) ;

chi = alfa^2*besselh(2,2,Wp)- besselh(2,2,Ws);

c = cos(theta);

s = sin(theta);

f = 0.25*i/mu;

G = f*[psi+chi*c^2; chi*c*s; psi+chi*s^2];	% [gxx; gxz=gzx; gzz]

% G = f*[psi+chi; psi]; 	% radial & tangential components

% Radial changes as cos(theta), tangential as -sin(theta)

return

function [U] = impulse_response(r, theta, cs, mu, alfa, T2)

% Impulse response functions for SVP line load in full space

ts = r/cs;

tp = alfa*ts;

ts2 = ts^2;

tp2 = tp^2;

k = find(tp2<T2);

if isempty(k)

PSI = zeros(size(T2));

CHI = PSI;

else

k = k(1);

k1 = k-1;

n = length(T2);

S = sqrt((T2(k:n)-tp2));

PSI = [zeros(1,k1),-S/ts2];	% yes, we divide by ts2, not tp2

CHI = [zeros(1,k1),alfa^2./S]-2*PSI;

if tp2==T2(k1), CHI(k1)=inf; end

k = find(ts2<T2);

if ~isempty(k)

k = k(1);

k1 = k-1;

n = length(T2);

S = sqrt((T2(k:n)-ts2));

PSI = PSI + [zeros(1,k1),S/ts2+1./S];

CHI = CHI - [zeros(1,k1),2*S/ts2+1./S];

if tp2==T2(k1), PSI(k1)=inf; CHI(k1)=inf; end

end

end

c = cos(theta);

s = sin(theta);

f = 0.5/pi/mu;

U = f*[PSI+CHI*c^2; CHI*c*s; PSI+CHI*s^2];	% [uxx; uxz=uzx; uzz]

% U = f*[PSI+CHI; PSI];	% radial & tangential components

% Radial changes as cos(theta), tangential as -sin(theta)

return

function [] = Blast2D(pois)

% Computes the plane-strain response elicited by an

% SV-P line blast source in a full, homogeneous 2D space

% Arguments:

% 	pois = Poisson's ratio

% Default data

nt = 300;			% Number of time intervals

nf = 200;			% Number of frequency intervals

cs = 1;			% Shear wave velocity

rho = 1;			% Mass density

r = 1;				% epicentral distance

tmax = 5;			% Maximum time for plotting

wmax = 20;			% maximum frequency for plotting (rad/s)

mu = rho*cs^2;		% Shear modulus

cp = cs*sqrt( (2-2*pois)/(1-2*pois));	% P-wave velocity

a = cs/cp;

a2 = a^2;

% Frequency domain

f = 0.25/mu/cp;

dw = wmax/nf;

Ws = [dw:dw:wmax]*r/cs;	% dimensionless frequency for S waves

Wp = a*Ws;			% dimensionless frequency for P waves

H1p = besselh(1,2,Wp);

Gr = f*Wp.*H1p;		% Green's function for radial displacement

tit = 'Radial displacement in full space due to line blast load';

plot (Ws,real(Gr));

hold on;

plot (Ws,imag(Gr),'r');

grid on;

title(tit);

titx = 'Dimensionless frequency w*r/Cs = k*r';

xlabel(titx);

pause;

hold off;

EasyPlot('B2D_FDr.ezp', tit, titx, Ws, Gr, 'c');

clear G* H*

% Time domain

f = 1/(2*pi*mu*r);

tp = r/cp;		% arrival time of P waves

ts = r/cs;		% arrival time of SV waves

ts2 = ts^2;

tp2 = tp^2;

dt = tmax/nt;

T = [0:dt:tmax]+eps;

P = sqrt(T.^2-tp2);

Ur = f*T.*real(1./P);

T = T*cs/r;		% dimensionless time

plot (T, Ur);

tit = 'Radial displacement in full space due to line blast load';

title(tit);

titx = 'Dimensionless time, t*Cs/r';

xlabel(titx);

grid on;

pause;

EasyPlot('B2D_TDr.ezp', tit, titx, T, Ur, 'r');

close;

return

function [] = Cavity2D(r, r0, pois);

% Cylindrical (2-D) cavity in a full, homogeneous space

% subjected to a harmonic/impulsive pressure

% The pressure is scaled so that p*pi*r0^2=1

%

% Arguments (in consistent units)

%	r     = distance to receiver

%	r0    = radius of cavity

%	pois  = Poisson's ratio

%

% Returned transfer functions are   g*mu*r     vs.  w*r/Cp

% Returned time histories are       u*rho*r^2  vs.  t*Cp/r

% in which mu is the shear modulus, rho is the mass density,

% and Cp is the P-wave velocity

% Basic data

Cs = 1;     % S-wave velocity (the results do not depend on this value)

a2 = (2-2*pois)/(1-2*pois);

a = sqrt(a2);   	% Cp/Cs

Cp = a*Cs;  		% P-wave velocity

rr = r0/r;

nf = 1024;

wmax = 2*pi*Cp/r0;

dw = wmax/nf;

w = [dw:dw:wmax]; % true frequency

w0 = w*r0/Cp;

w1 = w*r/Cp;      % dimensionless frequency

wmax = wmax*r/Cp; % max. dimensionless freq.

dw = dw*r/Cp;     % dimensionless freq. step

% Frequency domain solution

f1 = r/r0/pi;     		% factor for cavity

f2 = 0.25/i;	       	% factor forline of pressure

h0 = besselh(0,2,w0);

h1 = besselh(1,2,w0);

y = 2*h1-a2*w0.*h0;

h1 = besselh(1,2,w1);

H = [0.5/pi, f1*h1./y];	% transfer function for cylindrical cavity

G = [0.5/pi, f2*w1.*h1];	% transfer function for line of pressure

w1 = [0, w1];			% frequency vector

plot(w1,real(H));

hold on;

plot(w1,imag(H),’r’);

%plot(w1,real(G),':');	% Compare with TF forline of pressure

%plot(w1,imag(G),'r:');

tit = sprintf('T.F. for pressure in cylindrical cavity, r0=%5.4f, \\nu=%5.2f', rr, pois);

titx = 'Frequency (rad/s)';

title (tit);

xlabel(titx);

grid on;

%axis([0 wmax -1 1]);

pause;

hold off;

close;

EasyPlot('Cav2D-FD.ezp', tit, titx, w1, [H;G], 'c');

% Response at receiver in the time domain

t0 = 1-rr;           	% dimensionless arrival time

H = H.*exp(i*w1*t0); 	% delay TF by arrival time

tmax = 2;			% maximum dimensionless (delayed) time

nt = 100;		% No. of time steps

dt = tmax/nt;		% time step

T = [0:nt]*dt;	% dimensionless, delayed time axis

U = zeros(size(T));

for j=1:length(T)	% Fourier transform by direct integration (no FFT here)

t = T(j);		% Advantage: can use arbitrary size & No. of time steps

U(j) = sum(real(H.*exp(i*w1*t)));

end

U = U*dw*a/pi;

f = 2/pi;

fac = 0.5/a/pi/rr^1.5;

u = fac*(1-f*cisib(wmax*T));	% tail

U = U+u;

T = [0 t0 T+t0];    	% shift delayed time axis by wave arrival time

U = [0 0 U];

plot(T,U);			% plot response

grid on;

tit = sprintf( ...

'Cylindrical cavity under pressure, response at r/r0=%5.2f,\\nu=%5.2f', ...

1/rr, pois);

title(tit);

titx = 'Time (s)';

xlabel(titx);

pause;

close;

EasyPlot('Cav2D-TD.ezp', tit, titx, T, U, ‘r’);

return

function [] = Point_Full(pois)

% Computes the response elicited by a point load

% in a full, 3D homogeneous space

% Input arguments:

%	pois = Poisson's ratio

% Basic data

nt = 400;		% Number of time intervals

nf = 200;		% Number of frequency intervals

cs = 1;		% Shear wave velocity

rho = 1;		% Mass density

mu = rho*cs^2;	% Shear modulus

r = 1;			% epicentral distance

tmax = 2;		% Maximum dimensionless time for plotting

wmax = 20;		% maximum dimensionless frequency for plotting (rad/s)

% Frequency domain

dw = wmax/nf;

dt = tmax/nt;

cp = cs*sqrt( (2-2*pois)/(1-2*pois));	% P-wave velocity

a = cs/cp;

a2 = a^2;

Ws = [dw:dw:wmax];	% Dimensionless frequency for S waves

Wp = a*Ws;		% Dimensionless frequency for P waves

Es = exp(-i*Ws);

Ep = a2*exp(-i*Wp);

Fp = Ep.*(i+1./Wp)./Wp;

Fs = Es.*(i+1./Ws)./Ws;

% Displacements in spherical coordinates due to point load in x direction

PSI = Es-Fs+Fp;		% Rigid-body component

CHI = Ep-Es-3*(Fp-Fs);	% Distortional component

clear E* F*

tit = sprintf('\\psi in full 3D space, \\nu=%5.2f',pois);

plot (Ws,real(PSI));

hold on;

plot (Ws,imag(PSI),’r’);

grid on;

title(tit);

titx = 'Dimensionless frequency w*r/Cs';

xlabel(titx);

ylabel(‘\psi’);

pause;

EasyPlot('PSI-3D-FD.ezp', tit, titx, Ws, PSI, ‘c’);

hold off;

tit = sprintf('\\chi in full 3D space, \\nu=%5.2f',pois);

plot (Ws,real(CHI));

hold on;

plot (Ws,imag(CHI),’r’);

grid on;

title(tit);

titx = 'Dimensionless frequency w*r/Cs';

xlabel(titx);

ylabel('\chi');

hold off;

EasyPlot('CHI-3D-FD.ezp', tit, titx, Ws, CHI, ‘c’);

pause;

% Time domain

clear G*

tp = r/cp;			% arrival time of P waves

ts = r/cs;			% arrival time of SV waves

ts2 = ts^2;

T = [0:dt:tmax];

Tp = T.*(T>=tp)/ts2;

Ts = T.*(T>=ts)/ts2;

PSI = Ts-Tp;

CHI = -3*PSI;

jp = ceil(tp/dt)+1;

js = floor(ts/dt);

amax = 10*max(abs(PSI)); 	% to simulate a dirac delta

mm = length(PSI);

PSI = [0,0,0,PSI(jp),PSI(jp:js),PSI(js),amax,0,0];

CHI = [0,0,amax,CHI(jp),CHI(jp:js),CHI(js),-amax,0,0];

tp1 = 0.5*(T(jp-1)+T(jp));

ts1 = 0.5*(T(js)+T(js+1));

T = [0,tp1,tp1,tp1,T(jp:js),ts1,ts1,ts1,tmax];

plot (T, PSI);

tit = sprintf('\\psi in full 3D space, \\nu=%5.2f',pois);

title(tit);

titx = 'Dimensionless time, t*Cs/r';

xlabel(titx);

grid on;

axis([0 tmax -1.5 0.5]);

EasyPlot('PSI-3D-TD.ezp', tit, titx, T, PSI, ‘r’);

pause;

plot (T, CHI);

tit = sprintf('\\chi in full 3D space, \\nu=%5.2f',pois);

title(tit);

titx = 'Dimensionless time, t*Cs/r';

xlabel(titx);

axis([0 tmax -0.5 5]);

grid on;

EasyPlot('CHI-3D-TD.ezp', tit, titx, T, CHI, ‘r’);

pause;

close;

return

function Torsion_Full(x,y,z,td,rho,cs)

% Computes the response of a full 3D space to a torsional

% point source with time variation (pi/2/td)*sin(pi*t/td), 0<=t<=td

% (i.e. a sine pulse with unit area, or unit impulse)

% The torsional moment is applied at the origin and has vertical axis

% Arguments

%   x, y, z     = coordinates of receiver

%   td          = duration of pulse

%   rho         = mass density

%   cs          = shear wave velocity

mu = rho*cs^2;      % shear modulus

nt = 200;           % Number of points

dt = td/nt;         % time step

r = sqrt(x^2 + y^2);

R = sqrt (x^2 + y^2 + z^2);

phi = r/R;          % cos of vertical angle

ap = pi/2/td;       % amplitude of sine pulse

ap = 1;

A = ap*phi/8/pi/mu; % Scaling factor for response

ta = R/cs;          % arrival time

ang = pi/td;

a = A/R^2;

b = ang*A/cs/R;

T = [0:dt:td];

tau = ang*T;

v = a*sin(tau)+b*cos(tau);

T = [0,ta,T+ta,ta+td,ta+2*td];

v = [0,0,v,0,0];

% plot actual response vs. actual time

plot(T,v)

tit = sprintf(...

'Response at (x,y,z)=(%5.2f,%5.2f,%5.2f) due to torsional sine pulse',x,y,z);

xlabel(' Time');

title(tit)

grid on;

pause;

% plot dimensionless response & time

tau = T*cs/R;

v = (mu*R^2)*v;

plot(tau,v);

grid on;

tit = sprintf(...

'Response at (x/R,y/R,z/R)=(%5.2f,%5.2f,%5.2f) due to torsional sine pulse'...

, x/R, y/R, z/R);

ylabel('v*mu*R^2');

xlabel('Dimensionless time t*Cs/R');

title(tit);

pause;

close

% Make file for plotting with EasyPlot

fout = fopen ('tors.ezp','w');

fprintf (fout, '/et g "%s"\n', tit);

fprintf (fout, '/et x "t*Cs/R"\n');

fprintf (fout, '/og on\n');

fprintf (fout, '/sd off\n');

fprintf (fout, '/sm off\n');

fprintf (fout, '%15.5e  %15.5e\n', [tau;v]);

fclose (fout);

return

function [] = Cavity3D(r, pois);

% Spherical (3-D) cavity in a full, homogeneous space

% subjected to a harmonic/impulsive pressure p

% Arguments:

%		r = R/R0 	= (distance to receiver)/(radius of cavity)

%		pois		= Poisson's ratio

%

% This program assumes p0=1 (unit pressure), and r0=1 (radius of cavity)

% Basic parameters

a2 = (2-2*pois)/(1-2*pois);

a = sqrt(a2);   % Cp/Cs

R0 = 1;         % Radius of cavity

R = r*R0;       % Receiver

rho = 1;        % mass density

Cs = 1;         % S-wave velocity

Cp = a*Cs;      % P-wave velocity

mu = rho*Cs^2;  % shear modulus

% Frequency and time vectors

nf = 1024;

wmax = 16*pi;

dw = wmax/nf;

w = [dw:dw:wmax];

tmax = 6;		% maximum dimensionless time

nt = 200;		% No. of time steps

dt = tmax/nt;		% time step

T = [0:nt]*dt;	% dimensionless, delayed time (= t*Cp/R)

% Response in frequency domain

fac = 0.25*R0/mu;   % assuming p=1 here

w0 = w*R0/Cp;

w1 = w0*r;

G0 = 1+i*w0;

G1 = 1+i*w1;

G = fac*(G0./(G0-0.25*a2*w0.^2));  % Cavity wall at R0

plot(w,real(G));

hold on;

plot(w,imag(G),’r’);

grid on;

tit = sprintf(...

'T.F. for pressure in spherical cavity, R/R0=%5.4f, \\nu=%5.2f'...

, r, pois);

titx = 'Frequency (rad/s)';

title (tit);

xlabel(titx);

H = G.*exp(-i*(r-1)*w0).*G1./G0/r^2;  % Receiver at R

plot(w,real(H),':');

plot(w,imag(H),’r:’);

grid on;

hold off;

pause;

fout = fopen('Cav3D-FD.ezp', 'w');

fprintf (fout, '/et g "%s"\n', tit);

fprintf (fout, '/et x "%s"\n', titx);

fprintf (fout, '/og on\n');

f0 = 0.75*mu/R0/pi;    % factor for unit strength pressure

f1 = f0*r^2;

fprintf (fout, '%15.5f  %15.5f  %15.5f\n', [w1;f1*real(H);f1*imag(H)]);

fprintf (fout, '%s \n', '//nc');

fprintf (fout, '%15.5f  %15.5f  %15.5f\n', [w0;f0*real(G);f0*imag(G)]);

fclose (fout);

% a) Time domain, response at wall by numerical Fourier transform

U = zeros(size(T));

u0 = 0.5*fac;

for j=1:length(T)	% Fourier transform by direct integration (no FFT here)

t = T(j);		% Advantage: can use arbitrary size & No. of time steps

U(j) = u0+sum(real(G.*exp(i*w*t)));

end

U = U*dw/pi;

tail = (1-2/pi*cisib(wmax*T))*2*fac*Cs^2/Cp/R0;  %  tail of Fourier integral

U = U+tail;

plot(T,U);		    % plot response by FT

grid on;

% b) Time domain, response at wall by exact formula

fac = sqrt((1-pois)/2)/rho/Cs;

wn = 2*Cs/R0;           % natural frequency

xi = Cs/Cp;             % damping

wd = wn*sqrt(1-xi^2);   % damped frequency

b = xi*wn;

phi = asin(pois/(1-pois));

U = fac*exp(-b*T).*cos(wd*T-phi);   % Exact reponse

hold on;

plot(T,U,'r:');

tit = sprintf(...

'Cylindrical cavity under pressure, response at wall, \\nu=%5.2f', pois);

title(tit);

titx = 'Time (s)';

xlabel(titx);

pause;

% Create EasyPlot file

fout = fopen('Cav3D-TD0.ezp', 'w');

fprintf (fout, '/et g "%s"\n', tit);

fprintf (fout, '/et x "%s"\n', titx);

fprintf (fout, '/og on\n');

f0 = 0.75*rho*Cs*R0^3/pi;    % factor for unit strength pressure

f1 = f0*r^3;

fprintf (fout, '%15.5f  %15.5f\n', [T;U*f0]);

fclose (fout);

% c) Response at receiver by numerical Fourier transform

hold off;

t0 = r-1;			% dimensionless arrival time

H = H.*exp(i*w0*t0);	% delay TF by arrival time

V = zeros(size(T));

u0 = 0.5*fac/r^2;

for j=1:length(T)		% Fourier transform by direct integration (no FFT here)

t = T(j);			% Advantage: can use arbitrary size & No. of time steps

V(j) = u0+sum(real(H.*exp(i*w*t)));

end

V = V*dw/pi;

V = V+tail/r;

T1 = [0 t0 T+t0];		% shift delayed time by wave arrival time

V = [0 0 V];

plot(T1,V);			% plot response by FT

grid on;

% d) Response at receiver by exact formula

fac = sqrt((1-pois)/2)/rho/Cs;

V = fac*exp(-b*T).*(cos(wd*T-phi)-(1-1/r)*sin(wd*T))/r;   % Exact reponse

V = [0 0 V];

hold on;

plot(T1,V,'r:');

tit = sprintf(...

'Cylindrical cavity under pressure, response at r/r0=%5.2f, \\nu=%5.2f'...

, r, pois);

title(tit);

titx = 'Time (s)';

xlabel(titx);

pause;

close;

% Create EasyPlot file

fout = fopen('Cav3D-TD1.ezp', 'w');

fprintf (fout, '/et g "%s"\n', tit);

fprintf (fout, '/et x "%s"\n', titx);

fprintf (fout, '/og on\n');

fprintf (fout, '%15.5f  %15.5f\n', [T1;V*f1]);

fclose (fout);

return

function [] = SH2D_Half(xs, zs, xr, zr)

% Computes displacements due to SH line source in a homogeneous halfspace

% The source and the receiver can be placed anywhere

%

% Input arguments:

%	xs, zs	= Coordinates of source

%	xr, zr	= Coordinates of receiver

%

%	The origin of coordinates is taken at the free surface.

%	The vertical axis may be taken either up or down. Hence, zs and zr

%      must have the same sign (either both positive or both negative)

if (zs>0 & zr<0) | (zs<0 & zr>0)

'Error, zs and zr in call to SH2 must have the same sign!'

return

end

% Basic data

cs = 1;		% Shear wave velocity

rho = 1;		% Mass density

mu = rho*cs^2;	% Shear modulus

tmax = 10;		% Maximum dimensionless time for plotting

wmax = 10;		% maximum dimensionless frequency for plotting (rad/s)

np = 500;		% Number of time and/or frequency intervals

dw = wmax/np;

dt = tmax/np;

dx = xr-xs;

dz1 = zr-zs;

dz2 = zr+zs;

r1 = sqrt(dx^2+dz1^2);	% source-receiver distance

r2 = sqrt(dx^2+dz2^2);	% image source - receiver distance

r = r2/r1;

if r<1, r=1/r; end

W = [dw:dw:wmax];

G = -i*0.25*(besselh(0,2,W)+besselh(0,2,W*r))/mu;

tit = sprintf( ...

'Halfspace SH displacement, Source at (%5.2f, %5.2f), Receiver at (%5.2f, %5.2f)' , ...

xs, zs, xr, zr);

plot (W,real(G));

hold on;

plot (W,imag(G),’r’);

grid on;

title(tit);

titx = 'Dimensionless frequency w*r/Cs';

xlabel(titx);

pause;

hold off;

% Create EasyPlot file

fout = fopen('SH2-FD.ezp', 'w');

fprintf (fout, '/et g "%s"\n', tit);

fprintf (fout, '/et x "%s"\n', titx);

fprintf (fout, '/sm off\n');

fprintf (fout, '/sd off\n');

fprintf (fout, '/og on\n');

fprintf (fout, '%15.5f  %15.5f  %15.5f\n', [W;real(G);imag(G)]);

fclose (fout);

f = 1/(2*pi*mu);

T1 = [0 1 1];

T2 = [1+dt:dt:tmax];

Uyy = f./sqrt(T2.^2-1)+real(f./sqrt((T2.^2-r^2)));

Uyy = [0 0 inf Uyy];

Uyy(3) = Uyy(4);	% avoid singularity & discontinuity in plot

T = [T1, T2];

plot (T, Uyy);

titx = 'Dimensionless time, t*Cs/r';

xlabel(titx);

title(tit);

grid on;

pause;

% Create EasyPlot file

fout = fopen('SH2-TD.ezp', 'w');

fprintf (fout, '/et g "%s"\n', tit);

fprintf (fout, '/et x "%s"\n', titx);

fprintf (fout, '/sm off\n');

fprintf (fout, '/sd off\n');

fprintf (fout, '/og on\n');

fprintf (fout, '%15.5f  %15.5f\n', [T;Uyy]);

fclose (fout);

close;

function [T, Ux, Uz] = Garvin(x,z,pois);

% Garvin's line blast problem, plane strain (2-D)

% Step blast line load applied at depth z below surface of elastic halfspace

%

% Written by Eduardo Kausel, MIT, Room 1-271, Cambridge, MA

%

%  [T, Ux, Uz] = garvin(x,z,pois)

%  Input arguments:

%        x   = range of receiver on surface (z = 0)

%        z   = depth of source at x = 0

%        pois = Poisson's ratio

%  Output arguments

%        T   = Dimensionless time vector, tau = t*Cs/r

%        Ux  = Horizontal displacement at surface

%        Uz  = Vertical displacement at surface

% Unit soil properties are assumed (Cs=1, rho=1)

%

% Sign convention:

%   x from left to right, z=0 at the surface, z points up.

%   Displacements are positive up and to the right.

%   If z > 0 ==> an upper halfspace is assumed, z=0 is the lower boundary

%   If z < 0 ==>  a lower halfspace is assumed, z=0 is the upper boundary

% Reference:

%   W.W. Garvin, Exact transient solution of the buried line source problem,

%                Proceedings of the Royal Society of London, Series A

%                Vol. 234, No. 1199, March 1956, pp. 528-541

% Basic data

cs = 1;		% Shear wave velocity

rho = 1;		% Mass density

np = 200;		% Number of time intervals

tmax = 2.;		% Max. time for plotting

mu = rho*cs^2;	% Shear modulus

a2 = (1-2*pois)/(2-2*pois);

a = sqrt(a2);		% Cs/Cp

dt = (tmax-a)/np;	% time step

r = sqrt(x^2+z^2);	% Source-receiver distance

c = abs(z)/r;		% direction cosine w.r.t. z axis

s = abs(x)/r;		%     "        "     "    x axis

theta = asin(s);	% Source-receiver angle w.r.t. vertical

fac = 1/(pi*mu*r);	% scaling factor for displacements

T0 = [0 a];		% time interval before arrival of P waves

Ux0 = [0, 0];

Uz0 = [0, 0];

T = [a+dt:dt:tmax];		% Time vector

T2 = T.^2;

T1 = conj(sqrt(T2-a2));	% make the imaginary part negative

q1 = c*T1+i*s*T;		% Complex horizontal slowness, P waves

p1 = c*T+i*s*T1;		%   "     vertical      "      "   "

Q1 = q1.^2;

s1 = sqrt(Q1+1);

S1 = 2*Q1+1;

R1 = S1.^2 - 4*Q1.*p1.*s1;	% Rayleigh function, P waves

D1 = p1./T1./R1;		% Derivative of q1 divided by R1

Ux1 = 2*fac*sign(x)*imag(q1.*s1.*D1);

Uz1 = -fac*sign(z)*real(S1.*D1);

T = [T0,T];

Ux = [Ux0,Ux1];

Uz = [Uz0,Uz1];

% Plot results

plot (T,r*Ux);

tmax = max(T);

tit = sprintf(...

'Horizontal displacement due to line blast load, \\nu=%5.2f',pois);

title (tit);

grid on;

%axis([0 tmax -2 2]);

pause;

% Create EasyPlot file

fout = fopen('garvin-x.ezp', 'w');

fprintf (fout, '/et g "%s"\n', tit);

fprintf (fout, '/og on\n');

fprintf (fout, '/sd off\n');

fprintf (fout, '/sm off\n');

fprintf (fout, '%15.5f  %15.5f\n', [T;r*Ux]);

fclose (fout);

plot (T,r*Uz);

tit = sprintf(...

'Vertical displacement due to line blast load, \\nu=%5.2f',pois);

title (tit);

grid on;

%axis([0 tmax -2 2]);

pause;

% Create EasyPlot file

fout = fopen('garvin-z.ezp', 'w');

fprintf (fout, '/et g "%s"\n', tit);

fprintf (fout, '/og on\n');

fprintf (fout, '/sd off\n');

fprintf (fout, '/sm off\n');

fprintf (fout, '%15.5f  %15.5f\n', [T;r*Uz]);

fclose (fout);

close;

function [T, Uxx, Uxz, Uzz] = lamb2D(x,z,pois);

% Lamb's problem in plane strain (2-D)

% Implusive line load applied onto the surface of an elastic

% half-space and receiver at depth z

%

% To obtain the solution for an interior source and receiver at the

% surface, exchange the coupling terms, and reverse their signs.

%

% Written by Eduardo Kausel, MIT, Room 1-271, Cambridge, MA

%

% [T, Uxx, Uxz, Uzz] = lamb2(x,z,pois)

%  Input arguments:

%        x, z = coordinates of receiver relative to source at (0,0)

%        pois = Poisson's ratio

%  Output arguments

%        T   = Dimensionless time vector, tau = t*Cs/r

%        Uxx = Horizontal displacement caused by a horizontal load

%        Uxz = Horizontal displacement caused by a vertical load

%        Uzx = Vertical displacement caused by a horizontal load

%        Uzz = Vertical displacement caused by a vertical load

%

% Unit soil properties are assumed (Cs=1, rho=1)

%

% Sign convention:

%   x from left to right, z=0 at the surface, z points up.

%   Displacements are positive up and to the right.

%   If z > 0 ==> an upper halfspace is assumed, z=0 is the lower boundary

%                Vertical impulse at z=0 is compressive, i.e. up)

%   If z < 0 ==>  a lower halfspace is assumed, z=0 is the upper boundary

%                Vertical impulse at z=0 is tensile, i.e. up)

%   If z = 0 ==>  a lower halfspace is assumed, z=0 is the upper boundary

%                 both the load and the displacements are at the surface

%

% References:

%   Walter L. Pilant, Elastic Waves in the Earth, Elsevier, 1979

%   Eringen & Suhubi, Elastodynamics, Vol. 2, page 615, eq. 7.16.5,

%                     Pages 606/607, eqs. 7.14.22 and 7.14.27

%                     Page 612, eq. 7.15.9, page 617, eqs. 7.16.8-10

% Basic data

cs = 1;		% Shear wave velocity

rho = 1;		% Mass density

np = 200;		% Number of time intervals

tmax = 2.;		% Max. time for plotting

mang = 0.4999*pi;	% Max. angle for interior solution

mu = rho*cs^2;	% Shear modulus

a2 = (1-2*pois)/(2-2*pois);

a = sqrt(a2);		% Cs/Cp

dt = (tmax-a)/np;	% time step

r = sqrt(x^2+z^2);	% Source-receiver distance

c = abs(z)/r;		% direction cosine w.r.t. z axis

s = abs(x)/r;		%     "        "     "    x axis

theta = asin(s);	% Source-receiver angle w.r.t. vertical

crang = asin(a);	% Critical angle w.r.t. vertical

fac = cs/(pi*mu*r);	% scaling factor for displacements

% t < tp=a (two points suffice)

T0 = [0 a];

Uxx0 = [0, 0];

Uxz0 = [0, 0];

Uzx0 = [0, 0];

Uzz0 = [0, 0];

T = [a+dt:dt:tmax];		% Time vector

T2 = T.^2;

jl = length(T);

if theta > mang

% Displacements on surface

T1 = [a:dt:1];		% interval from tp to ts

T2 = [1+dt:dt:tmax];	% t > ts=1

% a=tp <= t <= ts=1

t2 = T1.^2;

p = t2-a2;

s = 1-t2;

q = 2*t2-1;

d = q.^4 + 16*p.*s.*t2.^2;

p = sqrt(p);

Uxx1 = 4*fac*t2.*s.*p./d;

Uzz1 = -fac*q.^2.*p./d;

s = sqrt(s);

Uxz1 = 2*fac*T1.*q.*s.*p./d;

% t > ts=1

t2 = T2.^2;

p = sqrt(t2-a2);

s = sqrt(t2-1);

q = 2*t2-1;

d = q.^2-4*t2.*p.*s;

Uxx2 = -fac*s./d;

Uzz2 = -fac*p./d;

Uxz2 = zeros(size(Uxx2));

cr = cs*(0.874+(0.197-(0.056+0.0276*pois)*pois)*pois);

tr = cs/cr;		% dimensionless arrival time of R waves

xr = (cr/cs)^2;

wr = 0.25*fac*pi*(1-0.5*xr)^3/(1-0.5*xr^2+0.125*xr^3-a2);

jr =  floor((tr-a)/dt+1);

% Combine solutions

Uxx = [Uxx1, Uxx2];

Uzz = [Uzz1, Uzz2];

Uxz = [Uxz1, Uxz2];

Uzx = -Uxz;

T = [T1 T2];

elseif theta < 1e-3

% Displacements on epicentral line

D1 = (T2-a2+0.5).^2 - T.*(T2-a2).*sqrt(T2-a2+1);

Uxx = -T.*sqrt(T2-a2).*sqrt(T2-a2+1)./D1;

Uzz = T2.*(T2-a2+0.5)./sqrt(T2-a2)./D1;

js = 1+floor((1-a)/dt+1); % first element after arrival of S waves

T2 = T2(js:jl);

T1 = T(js:jl);

D2 = (T2-0.5).^2 - T1.*(T2-1).*sqrt(T2+a2-1);

Uxx(js:jl) = Uxx(js:jl) + T2.*(T2-0.5)./D2./sqrt(T2-1);

Uzz(js:jl) = Uzz(js:jl) - T1.*sqrt(T2-1).*sqrt(T2-1+a2)./D2;

clear T1;

f = 0.5*fac;

Uxx = f*Uxx;

Uzz = f*Uzz;

Uxz = zeros(size(T));

Uzx = Uxz;

else

% Displacements in the interior

T1 = conj(sqrt(T2-a2));% make the imaginary part negative

T2 = conj(sqrt(T2-1));

q1 = c*T1+i*s*T;	% Complex slowness, P waves (Cagniard - De Hoop path)

q2 = c*T2+i*s*T;	%    "        "     S   "

p1 = c*T+i*s*T1;

p2 = c*T+i*s*T2;

Q1 = q1.^2;

Q2 = q2.^2;

s1 = sqrt(Q1+1);

s2 = sqrt(Q2+a2);

S1 = 2*Q1+1;

S2 = 2*Q2+1;

R1 = S1.^2 - 4*Q1.*p1.*s1;	% Rayleigh function, P waves

R2 = S2.^2 - 4*Q2.*p2.*s2;	% Rayleigh function, S waves

D1 = p1./T1./R1;		% Derivative of q1 divided by R1

D2 = p2./T2./R2;		%     "         q2    "     " R2

% Check critical angle

if (theta > crang)

tcrit = cos(theta-crang);

else

tcrit = 1;

end

% Apply Heaviside step function

k = floor((tcrit-a)/dt+1);

if k >=1; D2(1:k)=0; end

S1 = S1.*D1;

S2 = S2.*D2;

D1 = 2*D1.*s1;

D2 = 2*D2.*s2;

% Displacements due to impulsive line load

sgn = fac*sign(x)*sign(z);

Uxx = fac*real(p2.*S2-Q1.*D1);

Uzz = fac*real(p1.*S1-Q2.*D2);

Uzx = sgn*imag(q1.*p1.*D1-q2.*S2);

Uxz = sgn*imag(q1.*S1-q2.*p2.*D2);

end

T = [T0,T];

Uxx = [Uxx0,Uxx];

Uzz = [Uzz0,Uzz];

Uxz = [Uxz0,Uxz];

Uzx = [Uzx0,Uzx];

% Plot results

plot (T,Uxx);

tmax = max(T);

tit = sprintf(...

'Horizontal displacement due to horizontal line load, \\nu=%5.2f',pois);

titx = 'Time';

title (tit);

grid on;

%axis([0 tmax -2 2]);

pause;

plot (T,Uzz);

tit = sprintf((...

'Vertical displacement due to vertical line load, \\nu=%5.2f',pois);

title (tit);

grid on;

%axis([0 tmax -2 2]);

pause;

plot (T,Uzx);

tit = sprintf((...

'Vertical displacement due to horizontal line load, \\nu=%5.2f',pois);

title (tit);

grid on;

%axis([0 tmax -0.5 0.5]);

if theta > mang

hold on

plot ([tr tr], [0 -wr]);

end

hold off;

pause;

plot (T,Uxz);

tit = sprintf(...

'Horizontal displacement due to vertical line load, \\nu=%5.2f',pois);

title (tit);

grid on;

%axis([0 tmax -0.5 0.5]);

if theta > mang

hold on

plot ([tr tr], [0 wr]);

end

pause;

hold off;

pfac = rho*cs*r;

EasyPlot('Lamb2Dxx.ezp', tit, titx, T, pfac*Uxx, ‘r’);

EasyPlot('Lamb2Dzz.ezp', tit, titx, T, pfac*Uzz, ‘r’);

if z~=0

% reverse sign & coupling term for source at depth, receiver at surface

EasyPlot('Lamb2Dxz.ezp', tit, titx, T, -pfac*Uzx, ‘r’);

EasyPlot('Lamb2Dzx.ezp', tit, titx, T, -pfac*Uxz, ‘r’);

else

% source+ receiver are at the surface of a lower half-space

EasyPlot('Lamb2Dxz.ezp', tit, titx, T, pfac*Uxz, ‘r’);

EasyPlot('Lamb2Dzx.ezp', tit, titx, T, pfac*Uzx, ‘r’);

end

close;

function [T, Uxx, Utx, Uzz, Urz] = Lamb3D(r, pois);

% Point loads suddenly applied onto the surface of a lower elastic halfspace.

% Time variation of load is a unit step function (Heaviside), which is

% applied at the origin on the surface (x=0, z=0).

% Both horizontal and vertical loads are considered.

% Unit soil properties are assumed (Cs=1, rho=1)

%

% Written by Eduardo Kausel, MIT, Room 1-271, Cambridge, MA

%

% [T, Uxx, Utx, Uzz, Urz] = point3D(r, pois)

%  Input arguments:

%        r = range of receiver relative to source at (0,0)

%        pois = Poisson's ratio

%  Output arguments (cylindrical coordinates)

%        T   = Dimensionless time vector, tau = t*Cs/r

%        Urx = Radial displacement caused by a horizontal load

%              Varies as  cos(theta) with azimuth

%        Utx = Tangential displacement caused by a horizontal load

%              Varies as  -sin(theta) with azimuth

%        Uzz = Vertical displacement caused by a vertical load

%              (no variation with azimuth)

%        Urz = Radial displacement caused by a vertical load

%              (no variation with azimuth)

%        Uzx = Vertical displacement caused by a horizontal load

%              Not returned, because  Uzx = -Urz, except that

%              Uzx varies as  cos(theta) (reciprocity)

%

% Sign convention:

% Vertical load and vertical displacements at the surfce point up

% Response given in terms of dimensionless time tau=t*Cs/r

%

% Note:

% Solution for vertical loads for any Poisson's ratio, but coupling term

% Urz available only up to Poisson's ratio 0.2631. This is the value

% for which the two false roots of the Rayleigh equation turn complex.

% References:

%    a) Vertical loads:

%	Eringen and Suhubi, Elastodynamics, Vol. II, 748-750, pois<0.26

% 	Mooney, 1974, BSSA, V. 64, No.2, pp. 473-491, pois > 0.2631, but

%      vertical component only

%    b) Horizontal loads: (pois = 0.25 only)

%      Chao, C.C. Dynamical response of an elastic halfspace to

%      tangential surface loading, Journal of Applied Mechanics,

%      Vol 27, September 1960, pp 559-567

mu = 1;		% Shear modulus

Nt = 400;

tmax = 2;		% Maximum time for plotting (t=1 => arrival os S waves)

fac = 1/16/pi/r;

% Vertical displacements due to vertical step load

f = (1-pois)*fac;

a2 = (1-2*pois)./(2 - 2*pois);

b2 = 1-a2;

p = [-16*(1-a2), 8*(3-2*a2), -8, 1];

x = sort(roots(p));

x1 = x(1);

x2 = x(2);

x3 = x(3);

n1 = b2/(a2-x1);

if pois==0

x2 = a2;

n2 = inf;

else

n2 = b2/(a2-x2);

end

n3 = b2/(a2-x3);

tp = sqrt(a2);		% Arrival time of P waves

ts = 1;			%    "      "   " S   "

tr = sqrt(x3);		%    "      "   " R   "

dt = (ts-tp)/Nt;

T0 = [0 tp];

U0 = [0 0];

T1 = [tp+dt:dt:ts];

t2 = T1.^2;

if (imag(x1)==0)

A1 = (1-2*x1)^2*sqrt(a2-x1)/((x1-x2)*(x1-x3));

A2 = (1-2*x2)^2*sqrt(a2-x2)/((x2-x1)*(x2-x3));

A3 = (1-2*x3)^2*sqrt(x3-a2)/((x3-x1)*(x3-x2));

U1 = f*(4-A1./sqrt(t2-x1)-A2./sqrt(t2-x2)-A3./sqrt(x3-t2));

else

D = t2-a2;

Z = (a2-x1)./D;

Q = 1+2*Z+2*sqrt((Z+1).*Z);

for k=1:length(Q)

if abs(Q(k))>1, Q(k)=1/Q(k); end

end

Q = (1./Q - Q).*D;

A1 = (1-2*x1)^2*(a2-x1)/((x1-x2)*(x1-x3));

Q = real(A1./Q);

A3 = (1-2*x3)^2*sqrt(x3-a2)/real((x3-x1)*(x3-x2));

U1 = f*(4 - 8*Q - A3./sqrt(x3-t2));

end

T2 = [ts+dt:dt:tr-dt];

t2 = T2.^2;

U2 = 2*f*(4 - A3./sqrt(x3-t2));

y = U2(length(U2)); 	% save last point

Uzz = 8*f;

T3 = [tr];

U3 = [-sign(A3)*inf];

T4 = [tr+dt:dt:tmax];

U4 = Uzz*ones(size(T4));

U4(1) = y;

T = [T0, T1, T2, T3, T4];

Uzz = [U0, U1, U2, U3, U4];

plot(T, Uzz);

grid on;

%axis ([0 tmax -0.1 0.2]);

tit = sprintf( ...

'Vertical displacement due to vertical point (step) load, \\nu=%5.3f',pois);

title (tit);

titx = 'Dimensionless time';

xlabel(titx);

EasyPlot('Lamb3Dzz.ezp', tit, titx, T, Uzz, ‘r’);

pause;

hold off;

% Radial displacements due to vertical load

fac = 1/(8*pi*pi*r);

if (imag(x1)==0)

t2 = T1.^2;

k2 = (t2-a2)/b2;		% = k^2

B1 = ellipint3(90,n1*k2,k2)*(1-2*x1)*(1-x1)/(x1-x2)/(x1-x3);

B2 = ellipint3(90,n2*k2,k2)*(1-2*x2)*(1-x2)/(x2-x1)/(x2-x3);

B3 = ellipint3(90,n3*k2,k2)*(1-2*x3)*(1-x3)/(x3-x1)/(x3-x2);

U1 = 2*ellipint3(90,0,k2) - B1 - B2 - B3;

f = fac/sqrt(b2^3);

U1 = f*U1.*T1;

t2 = T2.^2;

k2 = b2./(t2-a2);	% inverse of k^2

B1 = ellipint3(90,n1,k2)*(1-2*x1)*(1-x1)/(x1-x2)/(x1-x3);

B2 = ellipint3(90,n2,k2)*(1-2*x2)*(1-x2)/(x2-x1)/(x2-x3);

B3 = ellipint3(90,n3,k2)*(1-2*x3)*(1-x3)/(x3-x1)/(x3-x2);

U2 = 2*ellipint3(90,0,k2) - B1 - B2 - B3;

U2 = f*U2.*T2.*sqrt(k2);

U3 = U2(length(U2));	% save last point

t2 = T4.^2;

C = (2*x3-1)^3/(1-4*x3+8*b2*x3^3);

U4 = 2*pi*fac*C*T4./sqrt(t2-x3);

Urz = [U0, U1, U2, U3, U4];

plot (T,Urz);

grid on;

axis ([0 tmax -0.2 0.6]);

tit = sprintf( ...

'Radial displacement due to vertical point (step) load, \\nu=%5.3f',pois);

title (tit);

xlabel(titx);

EasyPlot('Lamb3Drz.ezp', tit, titx, T, Urz, ‘r’);

pause;

else

% THIS PART NOT YET FUNCTIONAL FOR Urz

Warning = ...

'Sorry: solution for Urz not available for Poisson'' ratio > 0.263'

break

end

% Horizontal step load (Chao's solution)

% Note: Chao's figures are upside down, and multiplied by pi (but not here)

% Radial displacements

fac = 0.5/pi/mu/r;

c1 = 0.75*sqrt(3);

c2 = 0.125*sqrt(6*sqrt(3)+10);

c3 = 0.125*sqrt(6*sqrt(3)-10);

x1 = 0.25;

x2 = 0.25*(3-sqrt(3));

x3 = 0.25*(3+sqrt(3));

t2 = T1.^2;

U1 = fac*t2.*(c1./sqrt(t2-x1)-c2./sqrt(t2-x2)-c3./sqrt(x3-t2));

t2 = T2.^2;

U2 = fac*(1 -2*c3*t2./sqrt(x3-t2));

U4 = fac*ones(size(T4));

T = [T0, T1, T2, T4];

Urx = [U0, U1, U2, U4];

plot (T,Urx);

grid on;

tit = 'Radial displacement due to horizontal point (step) load, \\nu=0.25';

title (tit);

xlabel(titx);

EasyPlot('Chao3Drx.ezp', tit, titx, T, Urx, ‘r’);

pause;

% Tangential displacements

fac = -0.375/pi/mu/r;

t2 = T1.^2;

f = 8/3;

U1 = 0.5*fac*(1- f*(c1*sqrt(t2-x1)-c2*sqrt(t2-x2)+c3*sqrt(x3-t2)));

t2 = T2.^2;

U2 = fac*(1 - f*c3*sqrt(x3-t2));

U4 = fac*ones(size(T4));

Utx = [U0, U1, U2, U4];

plot (T,Utx);

grid on;

tit = 'Tangential displacement due to horizontal point (step) load, \\nu=0.25';

title (tit);

xlabel(titx);

EasyPlot('Chao3Dtx.ezp', tit, titx, T, Utx, ‘r’);

pause;

% Chao's functions on the axis  (point load, displ. under load.)

fac = 0.5/pi/mu/r;

t = T1;

t2 = t.^2;

f = (2*t2+1/3).^2-4*t.*(t2-1/3).*sqrt(t2+2/3);

f = t.*(t2-1/3).*sqrt(t2+2/3)./f;

U1 = -fac*f;

t = [T2,T4];

t2 = t.^2;

f = (2*t2+1/3).^2-4*t.*(t2-1/3).*sqrt(t2+2/3);

f = t.*(t2-1/3).*sqrt(t2+2/3)./f;

g = (2*t2-1).^2-4*t.*(t2-1).*sqrt(t2-2/3);

g = t2.*(t2-1).*(2*t.*sqrt(t2-2/3)-2*t2+1)./g;

U2 = fac*(0.5*(t2+1)-f+g);

T = [T0,T1,t];

U = [U0,U1,U2];

plot (T,U);

grid on;

tit = 'Displacement at axis under horizontal point (step) load, \\nu=0.25';

title(tit);

xlabel(titx);

EasyPlot('Chao3Daxis.ezp', tit, titx, T, U, ‘r’);

pause;

close all;

function [] = Torsion_Half(r, z, z0);

% Torsional point source with vertical axis acting within an

% elastic half-space at depth zo. The source has a time

% variation of the form f(t) = sin(pi*t/td), 0<=t<=td

% Origin is at surface

% Arguments:

%	r, z	= coordinates of receiver (horizontal range and depth)

%	z0 		= depth of source

if (z>0&z0<0) | (z<0&z0>0)

disp('Error: z and z0 must have the same sign');

return

end

mu = 1;			% Shear modulus

rho = 1;		% mass density

cs = 1;			% shear wave velocity

td = 1.0;		% duration of pulse

fac = 1/8/pi/mu;

R1 = sqrt(r^2+(z-z0)^2);

R2 = sqrt(r^2+(z+z0)^2);

S1 = fac*r/R1^3;	% fac*sin(phi-1)/R1^2

S2 = fac*r/R2^3;

% Time domain solution

m = 200;		% Number of intervals in pulse

dt = td/m;

t1 = R1/cs;

t2 = R2/cs;

nt = m;

tarr = 2*max(t1,t2);	% latest arrival

while nt*dt<tarr, nt=2*nt; end

T = [0:nt]*dt;

U = zeros(size(T));

Tp = pi/m*[0:m];

M  = sin(Tp);

dM = pi/td/cs*cos(Tp);

dM(1) = 0.5*dM(1);		% smooth out discontinuity for FFT

dM(m+1) = 0.5*dM(m+1);

k1 = floor(t1/dt)+1;

k2 = floor(t2/dt)+1;

U(k1:k1+m) = S1*(M+R1*dM);

U(k2:k2+m) = U(k2:k2+m)+S2*(M+R2*dM);

plot (T, U);

tit = sprintf(...

'Response at r=%5.3f, z=%5.3f due to torsional point load at z=%5.3f', r, z, z0);

titx = 'Time';

grid on;

title(tit);

xlabel(titx);

pause;

EasyPlot('TorHalf_TD.ezp', tit, titx, T, U, ‘r’);

% Transfer function by brute force Fourier Transform

% Reason: Only a dense set of low frequencies is of interest

% (difficult and computationally expensive to accomplish with FFT)

nf = 200;

fmax = 2;

df = fmax/nf;

f = [0:df:fmax];

Tp = dt*[0:m];

U = U(k1:k2+m);

T = T(k1:k2+m);

H = zeros(1,nf+1);

for j=1:nf+1

w = i*2*pi*f(j);

Uf = sum(U.*exp(-w*T));

Um = sum(M.*exp(-w*Tp));

H(j) = Uf/Um;

end

plot (f, real(H));

hold on;

plot (f, imag(H),’r’);

tit = 'Transfer function by FT ';

title (tit);

titx = 'Frequency';

xlabel(titx);

grid on;

pause;

hold off;

A1 = abs(H);

plot(f, A1);

grid on;

title ('Abs. Val. transfer function by FT ');

xlabel(titx);

pause;

hold off;

ang1 = unwrap(angle(H));

plot(f, ang1);

grid on;

title ('Angle transfer function by FT ');

xlabel(titx);

pause;

% Transfer function by frequency domain solution

W1 = i*2*pi*R1/cs*f;

W2 = i*2*pi*R2/cs*f;

%H = S1*(1+W1).*exp(-W1)+S2*(1+W2).*exp(-W2);

H = S1*(1+W1).*exp(-W1)+S2*(1+W2).*exp(-W2);

plot (f, real(H));

hold on;

plot(f,imag(H),’r’);

tit = sprintf(...

'Transfer function at r=%5.3f, z=%5.3f due to torsional load at z=%5.3f'...

, r, z, z0);

title (tit);

xlabel(titx);

grid on;

pause;

hold off;

EasyPlot('TorHalf_FD.ezp', tit, titx, f, H, ‘c’);

A2 = abs(H);

plot(f, A2);

grid on;

tit = 'Abs. Val. transfer function, direct';

title (tit);

xlabel(titx);

pause;

ang2 = unwrap(angle(H));

plot(f, ang2);

grid on;

title ('Angle transfer function, direct');

xlabel(titx);

pause;

plot(f, A1./A2);

grid on;

title ('A1/A2');

xlabel(titx);

pause;

ang = (ang2-ang1)/pi;

plot(f, ang);

xlabel(titx);

grid on;

title ('Diff. Angle / pi');

xlabel(titx);

pause;

close all;

return

function [] = SH_Plate(x, z, z0);

% SH line source at elevation zo above the origin in an elastic plate.

% Plate has unit thickness and material properties

% Origin is at the bottom surface, z is positive up

% Arguments:

%	x, z	= coordinates of receiver

%	z0 	= elevation of source

% Default data

mu = 1;		% Shear modulus

rho = 1;		% mass density

cs = 1;		% shear wave velocity

h = 1;			% plate thickness

xi = 0.0001;		% material damping

N = 10;		% parameter for max. time and max. frequency

if z>h | z0>h

disp('Error, source/receiver z must be <= than thickness of plate');

return

end

% Time domain solution via method of images

% Choose number of images ni=N such that other images arrive after tmax

tfund = 2*h/cs;	% fundamental period of plate

ffund = 1./tfund;	% fundamental frequency (Hz)

fac = 1/2/pi/mu;

tmax = N*tfund;

nt = 400;

dt = tmax/nt;

cs2 = cs^2;

x2 = x^2;

dz = z-z0;

sz = z+z0;

h2 = 2*h;

T = [0:nt]*dt;

T2 = T.^2;

U = zeros(size(T));

for j=-N:N

t1 = (x2+(dz-j*h2)^2)/cs2;

t2 = (x2+(sz-j*h2)^2)/cs2;

U = U + response(T2, t1) + response(T2, t2);

end

U = fac*U;

plot (T, U);

tit = sprintf(...

'Impulse response at x=%5.3f, z=%5.3f due to SH line load in plate at z=%5.3f', x, z, z0);

grid on;

title(tit);

titx = 'Time';

xlabel(titx);

pause;

EasyPlot('SH_plate_TD.ezp', tit, titx, T, U, ‘r’);

% Frequency domain solution via method of images

ni = 20;		% Number of images

fac = 0.25/mu/i;

fmax = N*ffund;

nf = 600;

df = fmax/nf;

F = [df:df:fmax];

W = 2*pi*F;

H = zeros(size(F));

for j=-ni:ni

t1 = sqrt(x2+(dz-j*h2)^2)/cs;

t2 = sqrt(x2+(sz-j*h2)^2)/cs;

H = H + besselh(0,2,t1*W) + besselh(0,2,t2*W);

end

H = fac*H;

plot (F, real(H));

hold on;

plot (F, imag(H),’r’);

tit = sprintf(...

'Green function at x=%5.3f, z=%5.3f due to SH line load in plate at z=%5.3f'...

, x, z, z0);

titx = 'Frequency (rad/s)';

grid on;

title(tit);

xlabel(titx);

pause;

hold off;

EasyPlot('SH_plate_FD1.ezp', tit, titx, F, H, ‘c’);

% Frequency domain solution via normal modes

nm = 20;		% number of normal modes

fac = 1/mu/i;

tit = sprintf(...

'Green function at x=%5.3f, z=%5.3f due to SH line load in plate at z=%5.3f'...

, x, z, z0);

x = abs(x/h);

z = z/h;

z0 = z0/h;

W = W*h*(1-i*xi)/cs;

H = 0.5*exp(-i*x*W)./W;

W2 = W.^2;

for j=1:nm

j1 = pi*j;

K = sqrt(W2-j1^2);

H = H + cos(j1*z)*cos(j1*z0)*exp(-i*x*K)./K;

end

H = fac*H;

plot (F, real(H));

hold on;

plot (F, imag(H),’r’);

grid on;

title(tit);

xlabel(titx);

pause;

hold off;

EasyPlot('SH_plate_FD2.ezp', tit, titx, F, H, ‘c’);

close all;

return

function [U] = response(T2, t2)

k = find(t2<T2);

if isempty(k)

U = zeros(size(T2));

else

k = k(1);

k1 = k-1;

n = length(T2);

U = [zeros(1,k1), 1./real(sqrt((T2(k:n)-t2)))];

if t2==T2(k1), U(k1)=inf; end

end

return

function [] = SH_Stratum(x, z, z0);

% SH line source at elevation zo above the origin in an elastic stratum.

% Stratum has unit thickness and material properties

% Origin is at the bottom surface, z is positive up

% Arguments:

%	x, z	= coordinates of receiver

%	z0 	= elevation of source

% Method of images used for both the time domain and the frequency domain

% In addition, the frequency domain solution is determined again with the

% normal modes method (gives more accurate answers).

mu = 1;		% Shear modulus

rho = 1;		% mass density

cs = 1;		% shear wave velocity

h = 1;			% plate thickness

xi = 0.0001;		% material damping

N = 10;		% parameter for max. time and max. frequency

if z>h | z0>h

disp('Error, source/receiver z must be <= than thickness of plate');

return

end

% Time domain solution via method of images

% Choose number of images ni=N such that other images arrive after tmax

tfund = 2*h/cs;	% fundamental period of plate

ffund = 1./tfund;	% fundamental frequency (Hz)

fac = 1/2/pi/mu;

tmax = N*tfund;

nt = 400;

dt = tmax/nt;

cs2 = cs^2;

x2 = x^2;

dz = z-z0;

sz = z+z0;

h2 = 2*h;

T = [0:nt]*dt;

T2 = T.^2;

U = zeros(size(T));

for j=-N:N

t1 = (x2+(dz-j*h2)^2)/cs2;

t2 = (x2+(sz-j*h2)^2)/cs2;

U = U + (response(T2, t1) - response(T2, t2))*(-1)^j;

end

U = fac*U;

plot (T, U);

tit = sprintf(...

'Impulse response at x=%5.3f, z=%5.3f due to SH line load in stratum at z=%5.3f', x, z, z0);

titx = 'Time';

xlabel(titx);

grid on;

title(tit);

pause;

EasyPlot('SH_stratum_TD.ezp', tit, titx, T, U, ‘r’);

% Frequency domain solution via method of images

ni = 100;		% Number of images

fac = 0.25/mu/i;

fmax = N*ffund;

nf = 600;

df = fmax/nf;

F = [df:df:fmax];

W = 2*pi*F;

H = zeros(size(F));

for j=-ni:ni

t1 = sqrt(x2+(dz-j*h2)^2)/cs;

t2 = sqrt(x2+(sz-j*h2)^2)/cs;

H = H + (besselh(0,2,t1*W) - besselh(0,2,t2*W))*(-1)^j;

end

H = fac*H;

plot (F, real(H));

hold on;

plot (F, imag(H),’r’);

tit = sprintf(...

'Green function at x=%5.3f, z=%5.3f due to SH load in stratum at z=%5.3f'...

, x, z, z0);

grid on;

title(tit);

titx = 'Frequency (rad/s)';

xlabel(titx);

pause;

hold off;

EasyPlot('SH_stratum_FD1.ezp', tit, titx, F, H, ‘c’);

% Frequency domain solution via normal modes

nm = 20;		% number of normal modes

fac = 1/mu/i;

tit = sprintf(...

'Green function at x=%5.3f, z=%5.3f due to SH load in stratum at z=%5.3f'...

, x, z, z0);

x = abs(x/h);

z = z/h;

z0 = z0/h;

W = W*h*(1-i*xi)/cs;

H = zeros(size(W));

W2 = W.^2;

for j=1:nm

j1 = pi*(j-0.5);

K = sqrt(W2-j1^2);

H = H + sin(j1*z)*sin(j1*z0)*exp(-i*x*K)./K;

end

H = fac*H;

plot (F, real(H));

hold on;

plot (F, imag(H),’r’);

grid on;

title(tit);

xlabel(titx);

pause;

hold off;

EasyPlot('SH_stratum_FD2.ezp', tit, titx, F, H, ‘c’);

close all;

return

function [U] = response(T2, t2)

k = find(t2<T2);

if isempty(k)

U = zeros(size(T2));

else

k = k(1);

k1 = k-1;

n = length(T2);

U = [zeros(1,k1), 1./real(sqrt((T2(k:n)-t2)))];

if t2==T2(k1), U(k1)=inf; end

end

return

function [] = SVP_Plate(x, z, z0, poi, BC1, BC2);

% SVP line source at elevation zo in an elastic plate

% with mixed boundary conditions

% Plate has unit thickness and material properties

% Origin is at the bottom surface, z is positive up

% Arguments:

%	x, z	= coordinates of receiver

%	z0 	= elevation of source

%	pois	= Poisson's ratio

%	BC1	= Boundary condition at bottom ('FC' or 'CF')

%	BC2	= Boundary condition at top    ('FC' or 'CF')

% Method of images used for both the time domain and the frequency domain

% In addition, the frequency domain solution is determined again with the

% normal modes method (Gives more accurate answers, and it is MUCH faster).

% Default data

mu = 1;			% Shear modulus

rho = 1;			% mass density

cs = 1;			% shear wave velocity

h = 1;				% plate thickness

xi = 0.0001;			% material damping

N = 10;			% parameter for max. time and max. frequency

if z>h | z0>h

disp('Error, source/receiver z must be <= than thickness of plate');

return

end

alfa = sqrt((1-2*poi)/(2-2*poi));	% cs/cp

tfund = 2*h*alfa/cs;	% fundamental period of plate (dilatation)

ffund = 1./tfund;		% fundamental frequency (Hz)

x2 = x^2;

dz = z-z0;

sz = z+z0;

h2 = 2*h;

BC1 = upper(BC1);

BC2 = upper(BC2);

BC = strcat(BC1,BC2);	% boundary conditions

% Method of images

% Parameters for time domain

%tmax = N*tfund;	% choose ni=N such that other images arrive after tmax

tmax = 10;	%%%%%%%%%%%%

nt = 400;

dt = tmax/nt;

cs2 = cs^2;

T = [0:nt]*dt;

T2 = T.^2;

n = length(T);

U = zeros(4,n);

for j=-N:N

z1 = dz-j*h2;

z2 = sz-j*h2;

r1 = sqrt(x2+z1^2);

r2 = sqrt(x2+z2^2);

U1 = impulse_response(r1, x, z1, mu, cs, alfa, T2);

U2 = impulse_response(r2, x, z2, mu, cs, alfa, T2);

if BC=='FCFC'

U = U+[U1(1:2,:)+U2(1:2,:);U1(2:3,:)-U2(2:3,:)];

elseif BC=='CFCF'

U = U+[U1(1:2,:)-U2(1:2,:);U1(2:3,:)+U2(2:3,:)];

elseif BC=='FCCF'

U = U+(-1)^j*[U1(1:2,:)+U2(1:2,:);U1(2:3,:)-U2(2:3,:)];

elseif BC=='CFFC'

U = U+(-1)^j*[U1(1:2,:)-U2(1:2,:);U1(2:3,:)+U2(2:3,:)];

else

disp('Not a valid boundary condition');

return

end

end

tit = sprintf(...

'Uxx at x=%5.3f, z=%5.3f due to SVP line load in plate at z=%5.3f', x, z, z0);

titx = 'Time';

direc = ['xx', 'zx', 'xz', 'zz'];

fname = 'Uxx_SVP_plate_TD.ezp';

for j=1:4

j2 = 2*j;

j1 = j2-1;

tit(2:3) = direc(j1:j2);

fname(2:3) = direc(j1:j2);

plot (T, U(j,:));

grid on;

title(tit);

xlabel(titx);

pause;

EasyPlot(fname, tit, titx, T, U(j,:), ‘r’);

end

% Frequency domain solution via method of images

% Parameters frequency domain

ni = 50;		% Number of images frequency domain

fmax = 5;

nf = 500;

df = fmax/nf;

F = [df:df:fmax];

W = 2*pi*F;

n = length(W);

H = zeros(4,n);

for j=-ni:ni

z1 = dz-j*h2;

z2 = sz-j*h2;

r1 = sqrt(x2+z1^2);

r2 = sqrt(x2+z2^2);

H1 = Green(r1, x, z1, mu, cs, alfa, W);

H2 = Green(r2, x, z2, mu, cs, alfa, W);

if BC=='FCFC'

H = H+[H1(1:2,:)+H2(1:2,:);H1(2:3,:)-H2(2:3,:)];

elseif BC=='CFCF'

H = H+[H1(1:2,:)-H2(1:2,:);H1(2:3,:)+H2(2:3,:)];

elseif BC=='FCCF'

H = H+(-1)^j*[H1(1:2,:)+H2(1:2,:);H1(2:3,:)-H2(2:3,:)];

elseif BC=='CFFC'

H = H+(-1)^j*[H1(1:2,:)-H2(1:2,:);H1(2:3,:)+H2(2:3,:)];

else

disp('Not a valid boundary condition');

return

end

end

tit = sprintf(...

'Hxx at x=%5.3f, z=%5.3f due to SVP line load in plate at z=%5.3f', x, z, z0);

direc = ['xx', 'zx', 'xz', 'zz'];

fname = 'Hxx_SVP_plate_FD1.ezp';

titx = 'Frequency (rad/s)';

for j=1:4

j2 = 2*j;

j1 = j2-1;

tit(2:3) = direc(j1:j2);

fname(2:3) = direc(j1:j2);

plot (F, real(H(j,:)));

hold on;

plot (F, imag(H(j,:)), ‘r’);

hold off;

grid on;

title(tit);

xlabel(titx);

pause;

EasyPlot(fname, tit, titx, F, H(j,:), ‘c’);

end

clear U H T

% Frequency domain solution via normal modes

nm = 20;	% number of normal modes

tit = sprintf(...

'Green function at x=%5.3f, z=%5.3f due to SVP load in plate at z=%5.3f'...

, x, z, z0);

sgn = sign(x);

x = abs(x/h);

z = z/h;

z0 = z0/h;

Ws = W*h*(1-i*xi)/cs;

Wp = alfa*Ws;

Ws2 = Ws.^2;

Wp2 = Wp.^2;

H = zeros(4,length(Ws));

if BC=='FCFC'

H(1,:) = 0.5*Wp.*exp(-i*x*Wp);

elseif BC=='CFCF'

H(4,:) = 0.5*Ws.*exp(-i*x*Ws);

end

for j=1:nm

if BC1==BC2

j1 = pi*j;

else

j1 = pi*(j-0.5);

end

j2 = j1^2;

Ks = sqrt(Ws2-j2);

Kp = sqrt(Wp2-j2);

Es = exp(-i*x*Ks);

Ep = exp(-i*x*Kp);

sr = sin(j1*z);

ss = sin(j1*z0);

cr = cos(j1*z);

cs = cos(j1*z0);

H1  = Kp.*Ep+j2*Es./Ks;

H2 = j1*(Ep-Es);

H4 = Ks.*Es+j2*Ep./Kp;

if BC1=='FC'

H(1,:) = H(1,:) + cs*cr*H1;

H(2,:) = H(2,:) - cs*sr*H2;

H(3,:) = H(3,:) + ss*cr*H2;

H(4,:) = H(4,:) + ss*sr*H4;

elseif BC1=='CF'

H(1,:) = H(1,:) + ss*sr*H1;

H(2,:) = H(2,:) + ss*cr*H2;

H(3,:) = H(3,:) - cs*sr*H2;

H(4,:) = H(4,:) + cs*cr*H4;

end

end

clear H1 H2 H3

tit = sprintf(...

'Hxx at x=%5.3f, z=%5.3f due to SVP line load in plate at z=%5.3f'...

, x, z, z0);

direc = ['xx', 'zx', 'xz', 'zz'];

fname = 'Hxx_SVP_plate_FD2.ezp';

f = [-i, sgn, sgn, -i]/mu;

for j=1:4

j2 = 2*j;

j1 = j2-1;

tit(2:3) = direc(j1:j2);

fname(2:3) = direc(j1:j2);

H(j,:) = f(j)*H(j,:)./Ws2;

plot (F, real(H(j,:)));

hold on;

plot (F, imag(H(j,:)), ‘r’);

hold off;

grid on;

title(tit);

xlabel(titx);

pause;

EasyPlot(fname, tit, titx, F, H(j,:), ‘c’);

end

close all;

return

function [G] = Green(r, x, z, mu, cs, alfa, W)

% Returns the Green's functions for an SVP line load in a full space

% r=source receiver distance, c=cos(theta), s=sintheta), alfa=cs/cp

ts = r/cs;

Ws = ts*W;

Wp = alfa*Ws;

psi = (besselh(1,2,Ws)-alfa*besselh(1,2,Wp))./Ws - besselh(0,2,Ws);

chi = alfa^2*besselh(2,2,Wp)- besselh(2,2,Ws);

f = 0.25*i/mu;

c = x/r;		% cos(theta)

s = z/r;		% sin(theta)

G = f*[psi+chi*c^2; chi*c*s; psi+chi*s^2];	% [gxx; gzx; gzz]

return

function [U] = impulse_response(r, x, z, mu, cs, alfa, T2)

% Returns the impulse response functions for an SVP line load in a full space

ts = r/cs;

tp = alfa*ts;

tp2 = tp^2;

k = find(tp2<T2);

if isempty(k)

U = zeros(3,length(T2));

return

else

ts2 = ts^2;

k = k(1);

k1 = k-1;

n = length(T2);

S = sqrt((T2(k:n)-tp2));

PSI = [zeros(1,k1),-S/ts2];	% yes, we must divide by ts2, not tp2

CHI = [zeros(1,k1),alfa^2./S]-2*PSI;

if tp2==T2(k1), CHI(k1)=inf; end

k = find(ts2<T2);

if ~isempty(k)

k = k(1);

k1 = k-1;

n = length(T2);

S = sqrt((T2(k:n)-ts2));

PSI = PSI + [zeros(1,k1),S/ts2+1./S];

CHI = CHI - [zeros(1,k1),2*S/ts2+1./S];

if tp2==T2(k1), PSI(k1)=inf; CHI(k1)=inf; end

end

end

f = 1/2/pi/mu;

c = x/r;	% cos(theta)

s = z/r;	% sin(theta)

U = f*[PSI+CHI*c^2; CHI*c*s; PSI+CHI*s^2];	% [uxx; uzx; uzz]

return

function [ztors,zspher] = spheroidal (pois, m, wmin, wmax)

% Computes the torsional and spheroidal modes of a solid sphere, and

% plots the two determinant functions whose zero crossings define the roots

% Assumes a sphere of radius R=1 and shear wave velocity Cs=1

% Note: the modes are independent of the azimuthal index n

%

% Input arguments:

% 		pois	= Poisson's ratio

%		m		= radial index

%		wmin	= minimum frequency for search

%		wmax	= maximum frequency for search

% Output arguments:

%		ztors	= dimensionless frequencies, torsional modes

%		zspher	= dimensionless frequencies, spheroidal modes

% Actual frequencies (rad/s) are obtained multiplying by Cs/R

% where Cs is the shear wave velocity, and R is the radius

%

% For enhanced accuracy, it uses the Bessel functions of half order

% instead of the actual spherical Bessel functions. Reason:

% The factors sqrt(pi/2z) cancel out in the determinant equations

%

% Makes first a rough search in the search interval to determine approximate

% values of the roots, then refines the results using the bisection method

%

% Results agree with Eringen & Suhubi, Elastodynamics, Vol. II, pp. 814,818

dz = 0.1;		% initial search step

if wmin<dz

zs=[dz:dz:wmax];

else

zs=[wmin:dz:wmax];

end

tit = sprintf('Torsional modes for m = %d', m);

titx = 'Mode number';

fname = sprintf('Tors%d.ezp',m);

if m>0

f = dettors(m,zs);

ztors = zerox(f,dz,zs(1));

nz = length(ztors);

for k=1:nz

z1 = ztors(k)-dz;

z2 = ztors(k)+dz;

ztors(k) = bisect2('dettors', m, z1, z2, 1.e-5,pois);

end

plot(zs,f);

axis([0 wmax -1 1]);

grid on;

pause;

close;

EasyPlot(fname, tit, titx, [1:nz], ztors, ‘r’)

else

disp('No torsional modes exist for m=0');

ztors = [];

end

tit = sprintf('Spheroidal modes for m = %d', m);

fname = sprintf('Spher%d.ezp',m);

f = detspher(m,zs,pois);

plot(zs,f);

grid on;

axis([0 wmax -1 1]);

pause;

close;

zspher= zerox(f,dz,zs(1));

nz = length(zspher);

for k=1:nz

z1 = zspher(k)-dz;

z2 = zspher(k)+dz;

z = bisect2('detspher', m, z1, z2, 1.e-5,pois);

if z~=[], zspher(k) = z; end

end

EasyPlot(fname, tit, titx, [1:nz], zspher, ‘r’)

return

function [f] = dettors(m,zs,pois)

jsm = besselj(m+0.5,zs);

jsm1 = besselj(m-0.5,zs);

f = jsm1-(m+2)*jsm./zs;

return

function [f] = detspher(m,zs,pois)

cm = 2*(1-pois)/(1-2*pois);	% Constrained modulus

a=sqrt(1/cm);	% Cs/Cp

zp = a*zs;

jpm  = besselj(m+0.5,zp);

jpm1 = besselj(m-0.5,zp);

if m>0

jsm  = besselj(m+0.5,zs);

jsm1 = besselj(m-0.5,zs);

f11 = -cm*jpm- 2*(2*jpm1-(m+1)*(m+2)*jpm./zp)./zp;

f12 = 2*m*(m+1)*(jsm1-(m+2)*jsm./zs)./zs;

f21 = 2*(jpm1-(m+2)*jpm./zp)./zp;

f22 = -jsm-2*(jsm1-m*(m+2)*jsm./zs)./zs;

f = f11.*f22-f12.*f21;

else

f = -cm*jpm- 4*jpm1./zp;

end

return

function center = bisect2(f, m, left, right, tolerance, pois)

% Finds the root of a function f in interval [left, right]

% where a root is known to exist. Uses the bisection method

fleft = feval(f,m,left,pois);

fright = feval(f,m,right,pois);

if (fleft == 0)

center = left;

return

elseif fright == 0

center = right;

return

elseif fleft* fright > 0

center = [];

return

end

while (right-left) > tolerance

center = (left + right)/ 2;

if feval(f,m,left,pois)* feval(f,m,center,pois) <= 0

right = center;

else

left = center;

end

end

return

function z = zerox(a, dz, z1)

% Finds the zero crossings of a vector

n = length(a);

nz = 0;

z = [];

if (n < 2) return; end

j = 1;

while j<n

sg1 = sign(a(j));

while (sg1 == 0)

nz = nz+1;

z = [z,j];

j = j + 1;

sg1 = sign(a(j));

end

j = j + 1;

while j<=n & sign(a(j))==sg1

j = j+1;

end

if j<=n

% found a root

nz = nz+1;

dj = abs(a(j))/(abs(a(j-1))+abs(a(j)));

z = [z,j-dj];

end

end

z = (z-1)*dz+z1;

return

function EasyPlot(fname, tit, titx, T, U, flag)

% Creates files for display with the EasyPlot program

fout = fopen (fname, 'w');

fprintf (fout, '/et g "%s"\n', tit);

fprintf (fout, '/et x "%s"\n', titx);

fprintf (fout, '/og on\n');

fprintf (fout, '/sd off\n');

fprintf (fout, '/sm off\n');

[nr,nc] = size(U);

for n=1:nr

fprintf (fout, '//nc\n');

if flag==‘r’

fprintf (fout, '%15.5f  %15.5f\n', [T;U(n,:)]);

elseif flag==‘c’

fprintf (fout, '%15.5f  %15.5f  %15.5f\n', [T;real(U(n,:));imag(U(n,:))]);

else

'Not a valid option in EasyPlot';

end

end

fclose (fout);

return

function [si]=cisib(x);

%       =============================================

%       Purpose: Compute cosine and sine integrals

%                Si(x) and Ci(x)

%       Input :  x  --- Argument of Ci(x) and Si(x)

%       Output:  CI --- Ci(x)

%                SI --- Si(x)

%       =============================================

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%% added stuff

n = length(x);

n1 = 1;

while (n1<n & x(n1)<=0)

x(n1) = 0;

n1 = n1+1;

end

n3 = n1;

while (n3<n & x(n3)<=1)

n3 = n3+1;

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if (n1>1)

n0 = n1-1;

ci(1:n0) = -1.0d+300;

si(1:n0) =  0.0d0;

end

if (n3>n1)

n2 = n3-1;

xx = x(n1:n2);

x2 = xx.^2;

ci(n1:n2) = ((((-3.0d-8*x2+3.10d-6).*x2-2.3148d-4).*x2+ ...

1.041667d-2).*x2-0.25).*x2+0.577215665d0+log(xx);

si(n1:n2) = ((((3.1d-7*x2-2.834d-5).*x2+ ...

1.66667d-003) .*x2-5.555556d-002).*x2+1.0).*xx;

end

if (n3<=n)

xx = x(n3:n);

x2 = xx.^2;

fx = ((((x2+38.027264d0).*x2+265.187033d0).*x2+ ...

335.67732d0).*x2+38.102495d0)./((((x2 +40.021433d0).*x2+ ...

322.624911d0).*x2+570.23628d0).*x2+157.105423d0);

gx = ((((x2+42.242855d0).*x2+302.757865d0).*x2+352.018498d0).*x2+ ...

21.821899d0)./((((x2 +48.196927d0).*x2+ ...

482.485984d0).*x2+1114.978885d0).*x2+449.690326d0)./xx;

ci(n3:n) = fx.*sin(xx)./xx-gx.*cos(xx)./xx;

si(n3:n) = 1.570796327d0-fx.*cos(xx)./xx-gx.*sin(xx)./xx;

end

function [el3]=ellipint3(phi,N,M);

%    [EL3] = ELLIPINT3 (phi,N,M) returns the elliptic integral of

%            the third kind, evaluated for each value of N, M

%            Can also be used to obtain the elliptic integral

%            of the first kind by setting N=0.

%    Arguments :  phi -- Upper limit of integration (in degrees)

%                 M   -- Modulus   (some authors use k=sqrt(m))

%                        M can be a scalar or vector

%                 N  --- parameter (some authors use c=-n)

%                        N can be also be scalar or vector, but if

%                        the latter, it must agree in size with M

%    Definition: If n, m are elements of N, M, then

%

%                phi

%    el3 = integ  |   [dt/((1+n*sin^2(t))*sqrt(1-m*sin^2(t)))

%                 0

%

%    Observe that m = k^2 is the square of the argument

%    used by some authors for the elliptic integrals

%    Method:  10-point Gauss-Legendre quadrature

if (phi < 0)

'Error, first argument in ellipint3 cannot be negative'

el3 = 0;

return

end

if length(N) ==1

N = N*ones(size(M));

elseif length(N) ~= length(M)

'Error, wrong size of second argument in ellipint3.'

'Should be size(N)=1 or size(N)=size(M)'

el3 = 0;

return

end

tol = 1-1d-8;

ang = phi*pi/180;

psi = ang/2;

t = [.9931285991850949,.9639719272779138,.9122344282513259, ...

.8391169718222188,.7463319064601508,.6360536807265150, ...

.5108670019508271,.3737060887154195,.2277858511416451, ...

.7652652113349734d-1];

w = [.1761400713915212d-1,.4060142980038694d-1,.6267204833410907d-1, ...

.8327674157670475d-1,.1019301198172404,.1181945319615184, ...

.1316886384491766,.1420961093183820,.1491729864726037,.1527533871307258];

t1 = psi*(1+t);

t2 = psi*(1-t);

s1 = sin(t1).^2;

s2 = sin(t2).^2;

el3 = zeros(size(M));

s = sin(ang)^2;

for j=1:length(M)

k2 = M(j);

n = N(j);

flag = 1;

if phi<=90	% assuming phi is in degrees here

if k2*s>=tol | -n*s >= tol

flag = 0;

end

elseif k2 >= tol | -n >=tol

flag = 0;

end

if flag

f1 = 1./( (1+n*s1).*sqrt(1-k2*s1) );

f2 = 1./( (1+n*s2).*sqrt(1-k2*s2) );

el3(j) = (f1+f2)*w';

else

el3(j) = inf;

end

end

el3 = psi*el3;

return