Instruction on how to use the standing wave AC larger signal solver

- Prepare input files “lcavity.dci”, “locmodu.lgi”, and “effc001.dci” for laser cavity structure description, modulation set up, and cross-sectional effective parameter provision, respectively. “lcavity.dci” and “effc001.dci” are the same files used for the DC solver.

Use a plain text edit tool, e.g., notepad, to form “locmodu.lgi” as follow (where the underlined text in red is for explanation only, please remove them in the actual input file).

#

# Simulation Control Parameters

#

Please don’t remove these three lines, otherwise file reading errors will appear.

0                          # wavelength flag (if=1, use reference wavelength below)

Integer, the wavelength flag, 0 for taking the self-consistent wavelength in input file “effc001.dci”, 1 for taking the reference wavelength shown below

1.300E+00          # reference wavelength [um]

Real number, the reference wavelength in [m], will be used only if the above integer is set to 1

5.000E+00          # bandwidth [nm]

Real number, the lasing wavelength bandwidth in [nm]

1                          # chirp flag (if=1, calculate chirp)

Integer, the chirp flag, 1 for chirp output on, 0 for chirp output off; for FP laser, there is no chirp output even if this flag is set to 1.

#

# Local Bias Parameters

#

Please don’t remove these three lines, otherwise file reading errors will appear.

1                          # total section number

Integer, the total number of sections in the laser structure; if this number is N, following lines (starting from the section index to the end) must repeat N times for the description of the modulation current applied to each section.

1                          # section index

Integer, the section index number

5.000E-05           # noise coupling coefficient

Real number, the product of the spatial and spectral coupling ratios between the spontaneous emission noise and the lasing mode

#

# Define Binary Bit Sequence

#

Please don’t remove these three lines, otherwise file reading errors will appear.

2.500E+00           # bit rate B [Gbits/s]

Real number, the binary bit rate in [Gbits/s]

1.000E+00           # duty cycle D

Real number, the duty cycle

2.000E+01           # DC bias (minimum) current [mA]

Real number, the DC bias current, i.e., the minimum current (in symbol 0), in [mA]

4.000E+01           # peak (maximum) current [mA]

Real number, the peak current, i.e., the maximum current (in symbol 1), in [mA]

2.000E+00          # bit sequence starting time [ns]

Real number, the bit sequence starting time [ns]; the DC bias current always starts from 0.

3                          # bit sequence length

Integer, the bit sequence length; in the eye-diagram pattern, it indicates the word length, i.e., the bit sequence will go through all possible binary bit combinations within the word length. For example, in the eye-diagram pattern, 3 means the actual bit sequence is 000 001 010 011 … 111 with a total bit length of 323=24.

3                          # bit sequence pattern flag: 1-user-defined/2-random/3-eye-pattern

Integer, the bit sequence pattern flag, 1 for the user defined bit sequence, 2 for the random bit sequence, 3 for the eye-diagram pattern

#

# Symbol 0 Shape

#

Please don’t remove these three lines, otherwise file reading errors will appear.

0                                          # shape flag: 0-non/1-Gaussian/2-square/3-trapezoid

Integer, the symbol 0 shape flag, 0 for all zero, 1 for the Gaussian pulse, 2 for the square pulse, 3 for the trapezoid pulse

0.000E+00 0.000E+00        # 1st/2nd turning point x1, x2 (0<x1<x2<D/B) [ns]

1st real number – the 1st turning point time in [ns]; 2nd real number – the 2nd turning point time in [ns]; they will be used only if the above shape flag is set to 3; the time must be counted from the beginning of the local bit, i.e., 0 < 1st turning point time < 2nd turning pint time < (1/bit-rate)(duty-cycle).

#

# Symbol 1 Shape

#

Please don’t remove these three lines, otherwise file reading errors will appear.

2                                         # shape flag: 0-non/1-Gaussian/2-square/3-trapezoid

Integer, the symbol 1 shape flag, 0 for all zero, 1 for the Gaussian pulse, 2 for the square pulse, 3 for the trapezoid pulse

1.500E-01 2.500E-01        # 1st/2nd turning point x1, x2 (0<x1<x2<D/B) [ns]

1st real number – the 1st turning point time in [ns]; 2nd real number – the 2nd turning point time in [ns]; they will be used only if the above shape flag is set to 3; the time must be counted from the beginning of the local bit, i.e., 0 < 1st turning point time < 2nd turning pint time < (1/bit-rate)(duty-cycle).

The following part will be used if the bit sequence pattern flag is set to 1, the user defined bit sequence. Please note that, for the laser with multiple sections, the following part must be deleted if the bit sequence pattern flag is not set to 1.

#

# User Defined Bit Sequence

#

Please don’t remove these three lines, otherwise file reading errors will appear.

1

0

1

0

Only 1 or 0 are allowed in each of the row, which defines the binary bit sequence from top down.

- Execute “lg.exe”. The execution can be forced to stop by: 1 – open “sigstop.tmp”; 2 – change 0 to 1; 3 – save the change made. For re-execution, 1 in “sigstop.tmp” must be changed back to 0. In the execution process, a command prompt widow will pop out and display the time domain marching results (the counting step, the actual time instant in [ns], and the optical output power from the left facet in [mW]) every 100 time marching steps.

- Output files:

1. “op_tdsw.lgo” contains the time dependent modulation current, averaged carrier density, and optical output power:

1st column – time in [ns]

2nd column – output power from the left facet in [mW]

3rd column – output power from the right facet in [mW]

4th column – averaged carrier density in [1018/cm3]

5th column – modulation current in [mA]

2. “op_wave.lgo” contains the time dependent lasing wavelength:

1st column – time in [ns]

2nd column – lasing wavelength from the left facet in [m]

3rd column – lasing wavelength from the right facet in [m]

Using a third party plotting routine, the calculated results can be shown as:

2.5Gbits/s NRZ square pulse modulation current applied to the asymmetrically coated FP laser (Binary bit sequence 000 001 010 011 100 101 110 111 starts at 2ns; each bit has a duration of 0.4ns.)

Optical output power v.s. time of the asymmetrically coated FP laser under 2.5Gbits/s NRZ square pulse modulation

Eye-diagram of the asymmetrically coated FP laser under 2.5Gbits/s NRZ square pulse modulation

Averaged carrier density v.s. time of the asymmetrically coated FP laser under 2.5Gbits/s NRZ square pulse modulation

Optical output power v.s. time of the asymmetrically coated uniform grating DFB laser under 5Gbits/s NRZ trapezoid pulse modulation

Lasing wavelength v.s. time of the asymmetrically coated uniform grating DFB laser under 5Gbits/s NRZ trapezoid pulse modulation

- File handling table:

| Input files | Execution files | Output files |
| --- | --- | --- |
| lcavity.dci locmodu.lgi effc001.dci~effc0nn.dci sigstop.tmp | lg.exe | op_tdsw.lgo op_wave.lgo |