A List of Solver Routines

1. Input – convert the user supplied physical structure to standard solver input files

2. Mode – cross-sectional optical mode solver

3. Gain – strained-layer multiple quantum well gain solver

4. DC – standing wave based laser static solver I: current dependent external performance

5. DS – standing wave based laser static solver II: internal distributions and spectrum

6. LG – standing wave based laser large signal dynamic solver

7. TW – traveling wave based time domain solver for laser diodes, semiconductor optical amplifiers, and superluminescent light emitting diodes

Chapter 2 Problems

Note: In the following problems, we will assume that all geometrical dimensions involved are given.

1. Extract a one-dimensional (1D) longitudinal optical model for an edge-resonant surface-emitting index-coupled distributed feedback (DFB) laser diode (LD) with three sections; the intermediate section has a 2nd order grating whereas the two identical sections at the rear and front ends have 1st order gratings. We will assume that all the cross-sectional optical modal parameters, such as the effective index and confinement factor, and the grating related parameters, such as the 1st and 2nd order grating periods and the 1st and 2nd order grating index coupling coefficients, are all known.

2. Extract a 1D longitudinal optical model for a uniform grating index-coupled DFB-LD monolithically integrated with an electro-absorption modulator (EAM), by assuming that all the cross-sectional optical modal parameters, such as the effective index and confinement factor, and the grating related parameters, such as the grating period and the index coupling coefficient, are known.

3. Extract a 1D longitudinal optical model for a monolithically integrated semiconductor optical amplifier (SOA) and a waveguide type (i.e., edge incidence) resonant cavity PIN photodetector (PD), by assuming that all the cross-sectional optical modal parameters, such as the effective index and the confinement factor, are known.

4. Extract a full three-dimensional (3D) optical model for a gain-clamped broadband SOA. With a ridge waveguide (RW) structure, the amplifier is designed to have optical gains covering the entire C-band (from 1530nm to 1565nm). It also uses a detuned uniform index-coupled 1st order grating to generate a lasing wavelength at 1520nm for gain clamping.

5. Extract a full 3D optical model for an RW two-sectional super-luminescent light emitting diode (SLED). The front part is a straight gain section whereas the rear part is a horn-shaped absorbing or gain section.

6. Extract a full 3D optical model for an RW quarter-wavelength phase-shifted (QWS) DFB-LD monolithically integrated with an (RW) horn-shaped amplification region.

Chapter 3 Problems

1. Write the 66 Hamiltonian for the InGaN-AlGaN system (group-III nitrides with the wurtzite structure). Details must be given to make the Hamiltonian ready for numerical implementation (coding).

2. Write the 1010 Hamiltonian for the GaInNAs-AlGaAs-GaAs system (diluted nitrides). Details must be given to make the Hamiltonian ready for numerical implementation (coding).

3. Following the effective mass theory, derive the governing equation for solving the quantum wire band structure. We will assume that the quantum wire structure is InGaAsP-InP based along the <100> direction and each wire has the square cross-section with negligible anisotropy in the (100) plane.

4. Following the effective mass theory, derive the governing equation for solving the quantum dot band structure. We will assume that the quantum dot structure is InGaAs-AlGaAs-GaAs based and each dot takes the cubic shape.

The material parameters can be found from various sources, e.g., Landolt-Bornstein, Vol. 17 Semiconductors, ed. O. Madelung, M. Schulz, and H. Weiss, in Numerical Data and Fundamental Relationships in Science and Technology, ed. K. H. Hellwege. (Berlin: Springer-Verlag, 1982).

Chapter 4 Problems

1. Derive the expression for the spontaneous emission gain under the free-carrier model framework.

2. Derive dipole matrix elements for TE and TM polarizations in the InGaAsP-InP quantum well structure by assuming that the well is in the x-y (100) plane and the TE polarization is along x and TM polarization along z, respectively.

3. From (4-110a) and (4-110b), derive expressions for the material absorption and refractive index change for quantum well structures, with contributions from both bound (discrete) and continuum states included.

4. Discuss the free-carrier, screened Coulomb interaction, and many-body correlation model adequacy in material gain/absorption calculations for laser diode (LD), electro-absorption modulator (EAM), and epitaxial wafer photoluminescence (PL) spectroscopy under normal operating conditions.

Chapter 5 Problems

1. Extract a two-dimensional (2D) carrier transport and thermal diffusion model in the cross-section of the field-effect quantum-well laser with appropriate approximations. (An example of the field-effect quantum-well laser structure can be found in D. Ahn and S. L. Chuang, Model of the field-effect quantum-well laser with free-carrier screening and valence band mixing. Journal of Appl. Phys., 64:11 1988, 6143-6149.)

2. Extract a 2D carrier transport and thermal diffusion model in the cross-section of the transverse junction stripe (TJS) laser with appropriate approximations.

3. Extract a carrier rate equation model for a cleaved-coupled cavity (C3) laser with the rear section applied by a fixed DC current and the front section directly modulated.

Chapter 6 Problems

1. Develop a finite difference method based one-dimensional (1D) optical traveling wave equation time domain (TD) solver. Observe the computation accuracy and efficiency by changing η=vgΔtΔz.

2. Develop a split-step method based 1D optical traveling wave equation TD solver. Contrast and discuss schemes (6-49) and (6-51).

3. Develop a transfer matrix method based 1D static optical standing wave equation (6-65) solver for the longitudinal mode. (Muller’s method is suggested for complex root-searching.)

4. Develop a Runge-Kutta method based 1D dynamic optical standing wave equation (2-127) solver for the time-dependent coefficient.

Chapter 7 Problems

1. Implement the free-carrier model for the bulk active region material gain and refractive index change calculation.

2. Implement the free-carrier model for the strained-layer multiple quantum well active region material gain and refractive index change calculation.

3. Implement the semi-analytical many-body correlation model for the strained-layer multiple quantum well active region material gain and refractive index change calculation.

Chapter 8 Problems

1. Implement the one-dimensional (1D) carrier transport and thermal diffusion equations along the current injection path (i.e., perpendicular to the PN junction) in a broad area contact InGaAsP-InP double-heterojunction (DH) laser cross-section.

2. Develop a Runge-Kutta method based 1D carrier rate equation solver.

Chapter 9 Problems

1. Following the optical traveling wave approach, in conjunction with the carrier rate equation and the free-carrier gain model, develop a full one-dimensional (1D) solver for laser diode (LD), semiconductor optical amplifier (SOA), and superluminescent light emitting diode (SLED) simulations along the longitudinal (cavity) direction.

2. Following the optical standing wave approach, in conjunction with the carrier rate equation and the free-carrier gain model, develop a full 1D solver for the LD simulation along the longitudinal (cavity) direction.

3. Following the 1D optical standing wave approach, derive analytical expressions for AC small-signal intensity and frequency modulation responses, 2nd and 3rd order inter-modulation and harmonic distortion responses. Implement these expressions for numerical simulation of LD AC small signal characteristics.

4. Following the 1D longitudinal optical traveling wave approach, in conjunction with the 1D vertical carrier transport (Poisson) equation and the screened Coulomb interaction material absorption model, develop a quasi two-dimensional (2D) electro-absorption modulator (EAM) solver, where the longitudinal direction indicates the device cavity (wave propagation) direction and the vertical direction is perpendicular to the device PN junction (along the current flow path).