Instruction on how to use the material band and gain solvers

- Prepare input files “cstruct.opt” and “cmaterl.opt” which can be obtained by the execution of “input.exe”.

- Execute “band.exe”. The execution can be forced to stop by: 1 – open “sigstop.tmp”; 2 – change 0 to 1; 3 – save the change made. For re-execution, 1 in “sigstop.tmp” must be changed back to 0.

- Output files:

1. “bandstr.qgo” contains the band structure information and is used as the input file for “gain.exe”, where:

7th row – 1st integer: total number of heavy hole bands

2nd integer: total number of light hole bands

3rd integer: total number of electron bands

2. “wavefun.qgo” contains the wave function related information and is used as the input file for “gain.exe”.

3. “bds1wel.qgo” contains band structure E~kt dispersion curves:

1st column – normalized in-plane carrier wave number kt

2nd column – unstrained well valence band edge energy in [eV]

3rd column – barrier valence band edge energy in [eV]

4th column – unstrained well conduction band edge energy in [eV]

5th column – barrier conduction band edge energy in [eV]

6th column – 1st valence subband energy in [eV]

7th column – 1st conduction subband energy in [eV]

8th column – 2nd valence subband energy in [eV]

9th column – 2nd conduction subband energy in [eV]

10th column – 3rd valence subband energy in [eV]

11th column – 3rd conduction subband energy in [eV]

Number 1 in the file name indicates that this file contains the information related to the 1st quantum well numbered from the top. If the composition or thickness of the quantum wells are different, similar information for other different quantum wells will be stored as “bds2wel.qgo”, “bds3wel.qgo”, …, etc., with number 2, 3, …, etc. in the file name indicate the 2nd, 3rd, …, etc. quantum well numbered from the top to the bottom, respectively. For example, if an active region contains 4 quantum wells with identical 1st, 2nd, and 4th, but different 3rd well designs, numbered from the top to the bottom, output files “bds1wel.qgo” and “bds3wel.qgo” will be generated, where the former contains the information for the identical 1st, 2nd, and 3rd wells and the latter contains the information for the different 3rd well.

Using a third party plotting routine, the calculated band structure can be shown as:

Full band structure

Valence band structure

- Execute “gain.exe”. The execution can be forced to stop by: 1 – open “sigstop.tmp”; 2 – change 0 to 1; 3 – save the change made. For re-execution, 1 in “sigstop.tmp” must be changed back to 0.

- Output files:

1. “gte1wel.qgo” contains the material TE gain and differential gain profiles:

1st column – wavelength in [m]

2nd column – gain in [1/cm]

3rd column – differential gain in [10-16cm2]

2. “gtm1wel.qgo” contains the material TM gain and differential gain profiles:

1st column – wavelength in [m]

2nd column – gain in [1/cm]

3rd column – differential gain in [10-16cm2]

3. “ite1wel.qgo” contains the material TE refractive index change and differential refractive index change profiles:

1st column – wavelength in [m]

2nd column – refractive index change

3rd column – differential refractive index change in [10-18cm3]

4. “itm1wel.qgo” contains the material TM refractive index change and differential refractive index change profiles:

1st column – wavelength in [m]

2nd column – refractive index change

3rd column – differential refractive index change in [10-18cm3]

5. “ate1wel.qgo” contains the TE linewidth enhancement factor:

1st column – wavelength in [m]

2nd column – linewidth enhancement factor

6. “atm1wel.qgo” contains the TM linewidth enhancement factor:

1st column – wavelength in [m]

2nd column – linewidth enhancement factor

Number 1 in these file names indicate that these files contain the information related to the 1st quantum well numbered from the top. If the composition or thickness of the quantum wells are different, similar information for other different quantum wells will be stored as “gte2wel.qgo”, …, “atm2wel.qgo”, “gte3wel.qgo”, …, “atm3wel.qgo”, …, etc., with number 2, 3, …, etc. in the file names indicate the 2nd, 3rd, …, etc. quantum well numbered from the top to the bottom, respectively.

7. “matpara.qgo” contains the extracted effective gain parameters.

8. “plotfig.tmp” contains figure plotting information:

1st row – 1st integer: valence subband starting column

2nd integer: valence subband ending column

3rd integer: valence subband column increment number

2nd row – 1st integer: conduction subband starting column

2nd integer: conduction subband ending column

3rd integer: conduction subband column increment number

This information is for plotting “bds1wel.qgo”.

3rd row – total number of gain profiles under different carrier densities

4th row – starting carrier density in [cm2]

5th row – ending carrier density in [cm2]

6th row – carrier density increment in [cm2]

This information is for plotting “gte1wel.qgo”, “gtm1wel.qgo”, “ite1wel.qgo”, “itm1wel.qgo”, “ate1wel.qgo”, and “atm1wel.qgo”.

Using a third party plotting routine, the calculated gain, refractive index change, and linewidth enhancement factor profiles can be shown as:

TE (red) and TM (green) gain profiles under sheet carrier densities: 1.61012/cm2, 1.81012/cm2, 2.01012/cm2, 2.01012/cm2, and 2.41012/cm2 from bottom to top

TE (red) and TM (green) refractive index change profiles under sheet carrier densities: 1.61012/cm2, 1.81012/cm2, 2.01012/cm2, 2.01012/cm2, and 2.41012/cm2 from bottom to top

TE (red) and TM (green) linewidth enhancement factor profiles under sheet carrier densities: 1.61012/cm2, 1.81012/cm2, 2.01012/cm2, 2.01012/cm2, and 2.41012/cm2 from bottom to top

- File handling table:

| Input files | Execution files | Output files |
| --- | --- | --- |
| cstruct.opt cmaterl.opt sigstop.tmp | band.exe | bandstr.qgo wavefun.qgo bds1wel.qgo |
| cstruct.opt cmaterl.opt bandstr.qgo wavefun.qgo sigstop.tmp | gain.exe | gte1wel.qgo gtm1wel.qgo ite1wel.qgo itm1wel.qgo ate1wel.qgo atm1wel.qgo matpara.qgo plotfig.tmp |