Instruction on how to use the time domain traveling wave solver

- Prepare input files “lcavity.dci”, “twinput.twi”, and “effc001.dci” for the device (e.g., LD, SOA, or SLED) cavity structure description, bias set up, and cross-sectional effective parameter provision, respectively. “lcavity.dci” and “effc001.dci” are the same files used for the DC solver.

Use a plain text edit tool, e.g., notepad, to form “twinput.twi” as follow (where the underlined text in red is for explanation only, please remove them in the actual input file).

#

# Simulation Control and Local Bias/Input Parameters

#

Please don’t remove these three lines, otherwise file reading errors will appear.

0.5E-01  0.5E-01   1  # starting bias [A]/bias increment [A]/# of biases

1st real number – the starting bias current in [A]; 2nd real number – the bias increment in [A]; 3rd integer – the total number of bias currents applied

1.0E-12  1.0E-09      # starting time [s]/ending time [s]

1st real number – the biasing starting time in [s]; 2nd real number – the biasing ending time in [s]

1        100                  # # of wavelength slices/# of cavity sections

1st integer – the total number of wavelength channels/slices in the spectrum; 2nd integer – the total number of cavity sections artificially sliced for simulation

1                                # # of input signal channels

Integer, the total number of input optical signal wavelength channels; if this number is N, the line underneath must repeat N times for the description of the input optical signal applied to each wavelength channel.

0.0E+00  1                # input signal power [W]/at channel #

1st real number – the input optical signal power in [W]; 2nd integer – the input optical signal wavelength channel number

1.0E-12                     # input signal starting time [s]

Real number, the input optical signal starting time in [s]

1                                # output sampling wavelength indicator

Integer, the wavelength channel at which the output is sampled

1.8E+00                    # population inversion factor

Real number, the population inversion factor (i.e., the ratio between the spontaneous emission gain and the stimulated emission gain)

3.0E-08  3.0E-08      # left/right side maximum wavelength range [m]

1st real number – the left (blue) side maximum wavelength rage in [m]; 2nd integer – the right (red) side maximum wavelength rage in [m]

1.0                            # current injection efficiency

Real number, the current injection efficiency

For device with multiple sections along the cavity, the input and operating conditions described in the above file are assumed to be the same for each section. Therefore, there is only one “twinput.twi” file even if there are multiple sections in the device cavity, i.e., there are multiple files “effc001.dci”, “effc002.dci”, …, “effc0nn.dci”.

- Execute “tw.exe”. In the execution process, a command prompt widow will pop out and display the time domain marching results (the actual time instant in [ns], the averaged material gain in [1/cm], and the total optical output power from both facets in [mW]) every 100 time marching steps.

- Output files:

1. “op_tdtw.lgo” contains the time dependent averaged carrier density, averaged material gain, and optical output power:

1st column – time in [ns]

2nd column – output power from the left facet in [mW]

3rd column – output power from the right facet in [mW]

4th column – averaged carrier density in [1018/cm3]

5th column – averaged material gain in [1/cm]

Using a third party plotting routine, the calculated results can be shown as:

Optical output power v.s. time of the asymmetrically coated FP laser under 50mA bias current

Averaged carrier density v.s. time of the asymmetrically coated FP laser under 50mA bias current

Averaged material gain v.s. time of the asymmetrically coated FP laser under 50mA bias current

Optical output power v.s. time of the SOA under 100mA bias current; red line – the input signal power is 1W, green line – the input signal power is 10W; input signals are switched on at 1ns.

Optical output power v.s. time of the SOA under 100mA bias current; red line – the input signal power is 100W, green line – the input signal power is 1mW, blue line – the input signal power is 10mW; input signals are switched on at 1ns.

Averaged carrier density v.s. time of the SOA under 100mA bias current; from the top down, the input signal powers are 1W, 10W, 100W, 1mW, and 10mW, respectively; input signals are switched on at 1ns.

Averaged material gain v.s. time of the SOA under 100mA bias current; from the top down, the input signal powers are 1W, 10W, 100W, 1mW, and 10mW, respectively; input signals are switched on at 1ns.

Optical output power v.s. time of the symmetric SLED with both facets anti-reflection coated; from the bottom up, the bias currents are 25mA, 50mA, 75mA, 100mA, 125mA, 150mA, 175mA, and 200mA.

Optical output power v.s. time of the asymmetric SLED with rear (left) facet cleaved and front (right) facet anti-reflection coated; from the bottom up, the bias currents are 25mA, 50mA, 75mA, 100mA, 125mA, 150mA, 175mA, and 200mA.

Averaged carrier density v.s. time of the symmetric (red) and asymmetric (green) SLEDs; from the bottom up, the bias currents are 25mA, 50mA, 75mA, 100mA, 125mA, 150mA, 175mA, and 200mA.

Averaged material gain v.s. time of the symmetric (red) and asymmetric (green) SLEDs; from the bottom up, the bias currents are 25mA, 50mA, 75mA, 100mA, 125mA, 150mA, 175mA, and 200mA.

- File handling table:

| Input files | Execution files | Output files |
| --- | --- | --- |
| lcavity.dci twinput.twi effc001.dci~effc0nn.dci | tw.exe | op_tdtw.lgo |