Instruction on how to use the standing wave DC solver

- Prepare input files “lcavity.dci”, “locbias.dci”, and “effc001.dci” for laser cavity structure description, bias set up, and cross-sectional effective parameter provision, respectively.

Use a plain text edit tool, e.g., notepad, to form input files as follow (where the underlined text in red is for explanation only, please remove them in the actual input file).

“lcavity.dci”:

#

# Longitudinal Parameters (section flag 0=AR/2=WG)

#

Please don’t remove these three lines, otherwise file reading errors will appear.

1  3.000E+02                            # total section number/length [um]

1st integer – the total number of sections in the laser structure; if this number is N, there are must be N lines underneath for the description of the N sections in the laser cavity; moreover, there must be N “effc0nn.dci” files with nn = 01, 02, 03, …, N; 0<N<100; 2nd real number – the total cavity length in [m]

0  0.000E+00  3.000E+02  1  1  # section flag/grating phase [degree]/length [um]/material flag/A. R. flag

1st integer – the section type flag, 0 for the active region, 2 for the passive waveguide region; 2nd real number – the section grating phase in [degree]; 3rd real number – the section length in [m]; 4th integer – the material flag, use 1 for InGaAsP/InP, 2 for InGaAsP/GaAs, 3 for InAlGaAs/InP, 4 for InAlGaAs/GaAs, and 5 for InGaAs-AlGaAs/GaAs; 5th integer – the active region flag, 0 for bulk, 1 for quantum well

5.650E-01  0.000E+00            # left facet amplitude/phase [degree]

1st real number – the left facet amplitude reflectivity; 2nd real number – the left facet phase in [degree]

3.300E-01  0.000E+00            # right facet amplitude/phase [degree]

1st real number – the right facet amplitude reflectivity; 2nd real number – the right facet phase in [degree]

“locbias.dci”:

#

# Local Bias Parameters

#

Please don’t remove these three lines, otherwise file reading errors will appear.

1                      # total section number

Integer, the total number of sections in the laser structure; if this number is N, there are must be N lines underneath for the description of the bias current applied to each section.

1.000E+02       # section bias [mA]

Real number, the maximum bias current applied to the local section in [mA]

“effc001.dci”:

#

# Modal Parameters

#

Please don’t remove these three lines, otherwise file reading errors will appear.

1.60000E-02       # A.R. total thickness [um]

Real number, the active region total thickness in [m], i.e., the total thickness of the quantum wells, can be found in “modpara.mdo” if the mode solver has been called

2.00000E+00      # A.R. width [um]

Real number, the active region width in [m], can be found in “modpara.mdo” if the mode solver has been called

3.22358E+00      # effective index

Real number, the effective index, can be found in “modpara.mdo” if the mode solver has been called

3.77572E+00      # group index

Real number, the average group index, can be found in “modpara.mdo” if the mode solver has been called

2.52915E-02       # confinement factor

Real number, the confinement factor, can be found in “modpara.mdo” if the mode solver has been called

0.00000E+00      # grating period [um]

Real number, the grating period in [m]

0.00000E+00      # index coup. coeff. [1/um]

Real number, the grating index coupling coefficient in [1/m]

0.00000E+00  0  # loss coup. ratio/flag

1st real number – the grating loss/index coupling coefficient ratio; 2nd integer – the loss coupling flag, 0 for no loss coupling, 1 for in-phase loss coupling, -1 for anti-phase loss coupling

0.00000E+00  0  # gain coup. ratio/flag

1st real number – the grating gain/index coupling coefficient ratio; 2nd integer – the gain coupling flag, 0 for no gain coupling, 1 for in-phase gain coupling, -1 for anti-phase gain coupling

9.25448E-01       # A.R. band gap energy [eV]

Real number, the active region band gap energy in [eV], can be found in “modpara.mdo” if the mode solver has been called

#

# Gain Parameters

#

Please don’t remove these three lines, otherwise file reading errors will appear.

1.00000E+01      # modal loss [1/cm]

Real number, the modal loss in [1/cm]

0.00000E+00      # bias induced modal loss change rate [1/cm/mA] for passive region

Real number, the bias induced modal loss change rate in [1/cm/mA] for the passive waveguide region with section flag = 2 in “lcavity.dci”, must be zero for the active region with section flag = 0 in “lcavity.dci”

1.33237E+03      # gain coefficient [1/cm] for QW [cm^2] for Bulk

Real number, the material gain coefficient in [1/cm] for the quantum well active region and in [cm2] for the bulk active region, can be found in “matpara.qgo” if the gain solver has been called for quantum well active region design

6.13275E-01       # transparency carrier [10^18/cm^3]

Real number, the transparency carrier density in [1018/cm3], can be found in “matpara.qgo” if the gain solver has been called for quantum well active region design

1.00000E-01       # nonlinear gain saturation factor [10^-17 cm^3]

Real number, the nonlinear gain saturation factor in [10-17cm3]

-1.10000E+00      # LEF

Real number, the linewidth enhancement factor

0.00000E+00      # bias induced index change rate [1/mA] for passive region

Real number, the bias induced refractive index change rate in [1/mA] for the passive waveguide region with section flag = 2 in “lcavity.dci”, must be zero for the active region with section flag = 0 in “lcavity.dci”

1.00000E+00      # SRH and surface recom. constant [1/ns]

Real number, the SRH and surface recombination constant in [1/ns]

2.00000E+00      # spont. and bimolecular recomb. constant [10^-10 cm^3/s]

Real number, the spontaneous and bimolecular recombination constant in [10-10cm3/s]

3.00000E+00      # Auger recomb.constant [10^-29 cm^6/s]

Real number, the Auger recombination constant in [10-29cm6/s]

9.51575E-01       # gain peak position [eV]

Real number, the gain peak position in [eV], can be found in “matpara.qgo” if the gain solver has been called for quantum well active region design

The number of “effc0nn.dci” files must be the same as the total section number in the laser cavity, with number nn indicating the section index number ordered from left to right. “effc0nn.dci” contains the cross-sectional effective parameters for section indexed nn. For example, for conventional FP or uniform grating DFB lasers, nn = 01; for the quarter-wavelength phase-shifted DFB laser, nn = 01 and 02; for the DBR laser with gratings on both sides, nn = 01, 02, and 03.

- Execute “dc.exe”. The execution can be forced to stop by: 1 – open “sigstop.tmp”; 2 – change 0 to 1; 3 – save the change made. For re-execution, 1 in “sigstop.tmp” must be changed back to 0.

- Output files:

1. “op_wave.dco” contains the current dependent lasing wavelength:

1st column – bias current in [mA]

2nd column – 1st mode lasing wavelength in [m]

3rd column – 2nd mode lasing wavelength in [m]

4th column – 1st and 2nd mode spacing in [nm]

2. “op_powr.dco” contains the current dependent optical output power:

1st column – bias current in [mA]

2nd column – 1st mode output power from the left facet in [mW]

3rd column – 1st mode output power from the right facet in [mW]

4th column – 1st mode external efficiency from the left facet in [mW/mA]

5th column – 1st mode external efficiency from the right facet in [mW/mA]

6th column – total internal efficiency

3. “op_smsr.dco” contains the current dependent side-mode-suppression-ratio (SMSR):

1st column – bias current in [mA]

2nd column – SMSR from the left facet in [dB]

3rd column – SMSR from the right facet in [dB]

4th column – ratio of the normalized optical field at the left and right facets in [dB]

5th column – ratio of the normalized optical field at the right and left facets in [dB]

4. “op_para.dco” contains the current dependent other parameters:

1st column – bias current in [mA]

2nd column – averaged carrier density in [1018/cm3]

3rd column – 1st mode averaged material gain in [1/cm]

4th column – 1st mode averaged material differential gain in [10-16cm2]

5th column – 1st mode photon lifetime in [ps]

6th column – 2nd mode photon lifetime in [ps]

Using a third party plotting routine, the calculated results can be shown as:

Optical output power v.s. bias current of the asymmetrically coated FP laser in the example batch

Optical output power v.s. bias current of the quarter-wavelength phase-shifted DFB laser in the example batch

Lasing wavelength v.s. bias current of the asymmetrically coated uniform grating DFB laser in the example batch

SMSR v.s. bias current of the in-phase gain-coupled DFB laser in the example batch

Averaged carrier density v.s. bias current of the quarter-wavelength phase-shifted DFB laser in the example batch

1st mode averaged material gain v.s. current of the asymmetrically coated uniform grating DFB laser in the example batch

Photon lifetime v.s. bias current of the in-phase loss-coupled DFB laser in the example batch

- File handling table:

| Input files | Execution files | Output files |
| --- | --- | --- |
| lcavity.dci locbias.dci effc001.dci~effc0nn.dci sigstop.tmp | dc.exe | op_wave.dco op_powr.dco op_smsr.dco op_para.dco |