Instruction on how to use the standing wave DS solver

- Prepare input files “lcavity.dci”, “locbias.dci”, and “effc001.dci” for laser cavity structure description, bias set up, and cross-sectional effective parameter provision, respectively. They are the same files used for the DC solver.

- Execute “ds.exe”. The execution can be forced to stop by: 1 – open “sigstop.tmp”; 2 – change 0 to 1; 3 – save the change made. For re-execution, 1 in “sigstop.tmp” must be changed back to 0.

- Output files contain various optical field and carrier density distributions and the optical spectrum sampled at the maximum bias current set in input file “locbias.dci”:

1. “op_dist.dso” contains the normalized optical field intensity, conjugate-cross field, and normalized carrier density distributions along the cavity:

1st column – cavity length in [m]

2nd column – 1st mode field intensity (|F1|2+|R1|2)

3rd column – 2nd mode field intensity (|F2|2+|R2|2)

4th column – 1st mode conjugate-cross field (2F1R1), real part

5th column – 1st mode conjugate-cross field (2F1R1), imaginary part

6th column – 2nd mode conjugate-cross field (2F2R2), real part

7th column – 2nd mode conjugate-cross field (2F2R2), imaginary part

8th column – carrier density

2. “op_cros.dso” contains various normalized optical cross field distributions along the cavity:

1st column – cavity length in [m]

2nd column – 1st mode self-cross field (2F1R1), real part

3rd column – 1st mode self-cross field (2F1R1), imaginary part

4th column – 1st-2nd mode mutual-cross field (F1R2+R1F2)/(2F2R2), real part

5th column – 1st-2nd mode mutual-cross field (F1R2+R1F2)/(2F2R2), imaginary part

6th column – 2nd-1st mode mutual-cross field (F2R1+R2F1)/(2F1R1), real part

7th column – 2nd-1st mode mutual-cross field (F2R1+R2F1)/(2F1R1), imaginary part

8th column – 2nd mode self-cross field (2F2R2), real part

9th column – 2nd mode self-cross field (2F2R2), imaginary part

3. “op_spec.dso” contains the optical spectra:

1st column – wavelength in [m]

2nd column – optical spectrum intensity from the left facet

3rd column – optical spectrum intensity from the right facet

4th column – reference spectrum

Using a third party plotting routine, the calculated results can be shown as:

Normalized carrier density distribution along the cavity in the FP laser in the example batch

Normalized optical field intensity distributions along the cavity in the quarter-wavelength phase-shifted DFB laser in the example batch

Optical spectra of the asymmetrically coated uniform grating DFB (red) and quarter-wavelength phase-shifted DFB (green) lasers in the example batch

- File handling table:

| Input files | Execution files | Output files |
| --- | --- | --- |
| lcavity.dci locbias.dci effc001.dci~effc0nn.dci sigstop.tmp | ds.exe | op_dist.dco op_cros.dco op_spec.dco |