Instruction on how to use the input file convertor

- Prepare input file “crosect.ipt” for cross-sectional structure description.

Use a plain text edit tool, e.g., notepad, to form a file as follow (where the underlined text in red is for explanation only, please remove them in the actual input file):

#

# Cross Section Structure

#

Please don’t remove these three lines, otherwise file reading errors will appear.

- # total number of layers

The total number of layers in the cross-sectional structure must be consistent with the number of columns in the chart shown below.

#

# lyr      x(lb)                  y(lb)                  x(rt)                    y(rt)            doping     x(comp)      y(comp)      flag

#

Please don’t remove these three lines, otherwise file reading errors will appear.

1  0.000000E+00  0.000000E+00  1.000000E+02  1.250000E+02 -5.00E+18  0.00E+00  0.00E+00       1

2  1.000000E+02  0.000000E+00  1.013440E+02  1.250000E+02 -1.00E+18  0.00E+00  0.00E+00       1

3  1.013440E+02  0.000000E+00  1.020000E+02  1.000000E+00 -1.00E+18  0.00E+00  0.00E+00       1

4  1.020000E+02  0.000000E+00  1.020500E+02  1.000000E+00 -5.00E+17  1.08E-01  2.36E-01         1

5  1.020500E+02  0.000000E+00  1.021000E+02  1.000000E+00 -5.00E+17  1.81E-01  3.95E-01         1

6  1.021000E+02  0.000000E+00  1.021040E+02  1.000000E+00  1.00E+17  1.10E-01  5.48E-01        -1

7  1.021040E+02  0.000000E+00  1.021140E+02  1.000000E+00  1.00E+17  1.81E-01  3.95E-01         1

8  1.021140E+02  0.000000E+00  1.021180E+02  1.000000E+00  1.00E+17  1.10E-01  5.48E-01        -1

9  1.021180E+02  0.000000E+00  1.021280E+02  1.000000E+00  1.00E+17  1.81E-01  3.95E-01         1

10  1.021280E+02  0.000000E+00  1.021320E+02  1.000000E+00  1.00E+17  1.10E-01  5.48E-01        -1

11  1.021320E+02  0.000000E+00  1.021420E+02  1.000000E+00  1.00E+17  1.81E-01  3.95E-01         1

12  1.021420E+02  0.000000E+00  1.021460E+02  1.000000E+00  1.00E+17  1.10E-01  5.48E-01        -1

13  1.021460E+02  0.000000E+00  1.021900E+02  1.000000E+00  5.00E+17  1.81E-01  3.95E-01         1

14  1.021900E+02  0.000000E+00  1.022440E+02  1.000000E+00  5.00E+17  1.08E-01  2.36E-01         1

15  1.022440E+02  0.000000E+00  1.047440E+02  1.000000E+00  5.00E+17  0.00E+00  0.00E+00       1

16  1.047440E+02  0.000000E+00  1.048440E+02  1.000000E+00  1.00E+19  4.68E-01  1.00E+00        1

17  1.013440E+02  1.000000E+00  1.020440E+02  1.250000E+02  5.00E+17  0.00E+00  0.00E+00       1

18  1.020440E+02  1.000000E+00  1.048440E+02  1.250000E+02 -5.00E+17  0.00E+00  0.00E+00       1

Every row represents a different layer in the cross-sectional structure. 1st column – integer, the layer index; 2nd and 3rd columns – real numbers, the layer box left bottom vertical and horizontal coordinates in [m]; 4th and 5th columns – real numbers, the layer box right top vertical and horizontal coordinates in [m]; 6th column – real number, the layer doping concentration in [1/cm3]; 7th and 8th columns – real numbers, the layer material composition (x, y) for In(1-x)Ga(x)As(y)P(1-y), In(1-x-y)Al(x)Ga(y)As, or In(1-x)Ga(x)As-Al(1-x)Ga(x)As, for InGaAs-AlGaAs system, use -|x| as the In composition in InGaAs and |x| as the As composition in AlGaAs; 9th column – integer, the layer type flag, use -1 for active region, 1 for other semiconductor layer, 0 for dielectric (including air) layer.

#

# Other Information

#

Please don’t remove these three lines, otherwise file reading errors will appear.

2.000E+00    # stripe width [um]

Real number, the electrode strip width in [m]

1.340E+00    # reference wavelength [um]

Real number, the reference wavelength in [m]

1                   # material flag

Integer, the material flag, use 1 for InGaAsP/InP, 2 for InGaAsP/GaAs, 3 for InAlGaAs/InP, 4 for InAlGaAs/GaAs, and 5 for InGaAs-AlGaAs/GaAs

6.400E-01    # band offset ratio

Real number, the valence band offset ratio, e.g., use 0.64 for InGaAsP/InP system, used for the band structure calculation only

1.000E-13    # intraband relaxation time [s]

Real number, the intraband relaxation time in [s], used for the gain calculation only

3.440E+00    # A. R. index

Real number, the active region material index, used for refractive index change and linewidth enhancement factor calculations only

This input file describes a symmetric half of a buried heterojunction (BH) structure with a compressively strained InGaAsP multiple quantum well active region on InP substrate shown below.

N-InP                                  51018/cm3 100m (substrate)

N-InP                                  11018/cm3      2m (N-cladding)

N-In0.892Ga0.108As0.236P0.764 51017/cm3   50nm (waveguide)

N-In0.819Ga0.181As0.395P0.605 51017/cm3   50nm (barrier)

P-In0.890Ga0.110As0.548P0.452 11017/cm3     4nm (well 4)

P-In0.819Ga0.181As0.395P0.605 11017/cm3   10nm (barrier)

P-In0.890Ga0.110As0.548P0.452 11017/cm3     4nm (well 3)

P-In0.819Ga0.181As0.395P0.605 11017/cm3   10nm (barrier)

P-In0.890Ga0.110As0.548P0.452 11017/cm3     4nm (well 2)

P-In0.819Ga0.181As0.395P0.605 11017/cm3   10nm (barrier)

P-In0.890Ga0.110As0.548P0.452 11017/cm3     4nm (well 1)

P-In0.819Ga0.181As0.395P0.605 51017/cm3   44nm (barrier)

P-In0.892Ga0.108As0.236P0.764 51017/cm3   54nm (waveguide)

P-InP                                  51017/cm3  2.5m (P-cladding)

P+-In0.532Ga0.468As              11019/cm3   100nm (cap)

N-InP 51017/cm3  2.8m (N-blocking)

P-InP 51017/cm3  0.7m

(P-blocking)

1m

125m

- Execute “input.exe”.

- Output files:

1. “cstruct.opt” contains the standard cross-sectional structure that will be used for the optical mode and material gain calculation.

2. “cmaterl.opt” contains the material parameters that will be used for the optical mode and material gain calculation.

- File handling table:

| Input files | Execution files | Output files |
| --- | --- | --- |
| crosect.ipt | input.exe | cstruct.opt cmaterl.opt |