Corrections and additions for Chapter 3: Precoding design (A. Paulraj and M. Vu)

pg 121. section 3.5.6. paragraph 1

Old text:

This allocation follows the water-filling principle for all criteria, where more power is

allocated to stronger modes, and weak modes are dropped, depending on the SNR.

New text:

This allocation follows the water-filling principle for most criteria (except notably the MMSE), where more power is allocated to stronger modes, and weak modes are dropped, depending on the SNR.

---------------------

pg 104, section 3.4, new paragraph 2: (just before subsection 3.4.1)

In designing a precoder in Figure 3.8, we assume that the encoder, and hence its codeword covariance Q, is pre-determined and is not a design target. A total power constraint would correspond to tr(FQF*)=1, but as Q is fixed, a simpler constraint is tr(FF*) = 1. These two constraints lead to a precoder with the same optimal beam directions; only the power loadings are different. When Q = I (such as with spatial multiplexing), the two constraints coincide and the precoder then represents a linear operator (similar to the structure studied in [C-E] with perfect CSIT). Subsequently, the power constraint in all the optimization problems will be tr(FF*)= 1 as in (3.24).

-------------------------

pg 104, section 3.4.1, paragraph 1: (after equation 3.27)

old text:

When C is a STBC, it provides the system capacity.

new text:

When C is a STBC, it provides the system capacity, which depends on Q.

-----------------------------

Bibliography updated:

[55] add month: July.

New references:

[A] M. Vu and A. Paulraj, "On the Capacity of MIMO Wireless Channels with Dynamic CSIT", IEEE Journal on Selected Areas in Comm., Special Issue on Optimization of MIMO Transceivers for Realistic Communication Networks, Sept. 2007.

(add the citation to this reference in pg. 88, second paragraph as:

“In this chapter, we focus on the CSIT given by a channel estimate with a known error covariance [A].” )

[B] M. Vu and A. Paulraj, “MIMO wireless linear precoding,” IEEE Signal Processing Mag., Sept. 2007.

(add the citation to this reference in pg. 88, third paragraph as:

“Precoding design for MIMO wireless has been an active research area in recent years

[... , B] ...” )

[C] A. Scaglione, P. Stoica, S. Barbarossa, G. Giannakis, and H. Sampath, “Optimal designs for space-time linear precoders and decoders,” IEEE Trans. on Signal Processing, vol. 50, no. 5, pp. 1051–1064, May 2002.

[D] D. Palomar, J. Cioffi, and M. Lagunas, “Joint Tx-Rx beamforming design for multicarrier MIMO channels: A unified framework for convex optimization,” IEEE Trans. on Signal Processing, vol. 51, no. 9, pp. 2381–2401, Sept. 2003.

[E] Y.W. Ding, T.N. Davidson, Z.Q. Luo, and K. M. Wong, “Minimum BER block precoders for zero-forcing equalization,” IEEE Trans. on Signal Processing, Vol. 51, No. 9, pp. 2410-2423, Sept. 2003.

(citation to these references was added in the new paragraph to pg. 104 above)